| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17 Oct – 24 Oct 2014 |  |  | Document: JCTVC-S0187 |
| --- | --- | --- | --- |
| Title: | Non-SCCE: Video coding with extended largest coding unit |  |  |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dong Jiang, Xuerong Zhang, Yaqiang Wu, Zhepeng Wang Lenovo Research Institute Building B5,No.6,ShangDi West Rd, Beijing, P.R. China, 100085 | Tel: Email: | (86)18610861019 Jiangdong1@lenovo.com |
| Source: | Lenovo |  |  |

_____________________________

# Abstract

This contribution proposes an extension of current coding framework by extending the largest coding unit. Since there are more repeated and flat blocks in SCC testing sequences, conspicuous BD-rate gain could be achieved by using an enlarged largest coding unit.

Experimental results show the proposed approach provides bitrate reductions of -2.8 %, -3.6 % for AI 1080p text and graphics RGB and YUV categories respectively over SCM2.0 anchor. At the same time, the decoder complexity is visibly reduced for AI, LB and RA. The encoding time is increased with 142%, 147% and 139% for AI, LB, and RA lossy configurations respectively.

# Introduction

As we know, there are many repeated textures in SCC related sequences. Extending the current largest coding unit is an effective tool to remove such correlation. Consequently, PU size needs to be enlarged while TU keeps the current size.

# Proposal

Considering the trade-off between computational complexity and BD-rate gain, 128*128 is proposed to be used as the SCC LCU size.

# Test results

The coding gain of the proposed method is implemented on SCM-2.0. The experimental conditions are subject to the test conditions issued by JCT-VC in the 17th meeting [2].

Results below are BD-rate and encoding/decoding time comparison between proposed scheme and anchor. The proposed method can get an obvious BD-rate reduction as shown in the following tables. .

Table 1: Results for lossy All Intra configuration

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -2.8% | -2.4% | -2.2% |
| RGB, text & graphics with motion,720p | -2.0% | -1.3% | -1.4% |
| RGB, mixed content, 1440p | -1.3% | -1.1% | -0.9% |
| RGB, mixed content, 1080p | -1.8% | -1.6% | -1.3% |
| RGB, Animation, 720p | -0.1% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | -0.4% | -0.3% | -0.3% |
| YUV, text & graphics with motion, 1080p | -3.6% | -3.7% | -3.6% |
| YUV, text & graphics with motion,720p | -2.3% | -2.2% | -2.2% |
| YUV, mixed content, 1440p | -1.5% | -1.3% | -1.3% |
| YUV, mixed content, 1080p | -2.3% | -1.9% | -2.2% |
| YUV, Animation, 720p | -0.2% | 0.3% | 0.0% |
| YUV, camera captured, 1080p | -0.4% | -0.7% | -0.7% |
| Enc Time[%] | 142% |  |  |
| Dec Time[%] | 99% |  |  |

Table 2: Results for lossy Random Access configuration

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.3% | -1.0% | -1.0% |
| RGB, text & graphics with motion,720p | -1.5% | -0.9% | -0.9% |
| RGB, mixed content, 1440p | -1.5% | -1.2% | -1.0% |
| RGB, mixed content, 1080p | -1.9% | -1.4% | -0.9% |
| RGB, Animation, 720p | 0.0% | 0.2% | 0.0% |
| RGB, camera captured, 1080p | -0.5% | -0.9% | -0.8% |
| YUV, text & graphics with motion, 1080p | -1.6% | -1.6% | -1.6% |
| YUV, text & graphics with motion,720p | -1.6% | -1.7% | -1.5% |
| YUV, mixed content, 1440p | -1.6% | -1.4% | -1.3% |
| YUV, mixed content, 1080p | -2.0% | -2.0% | -2.0% |
| YUV, Animation, 720p | -0.2% | 1.0% | 0.4% |
| YUV, camera captured, 1080p | -0.5% | -1.2% | -0.9% |
| Enc Time[%] | 147% |  |  |
| Dec Time[%] | 96% |  |  |

Table 3: Results for lossy Low delay B configuration

|  | Low delay B |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.3% | -1.1% | -1.1% |
| RGB, text & graphics with motion,720p | -1.3% | -0.8% | -0.9% |
| RGB, mixed content, 1440p | -1.6% | -1.2% | -1.0% |
| RGB, mixed content, 1080p | -2.1% | -1.1% | -0.6% |
| RGB, Animation, 720p | 0.3% | 0.8% | 0.5% |
| RGB, camera captured, 1080p | -0.6% | -0.8% | -0.7% |
| YUV, text & graphics with motion, 1080p | -1.2% | -1.0% | -1.1% |
| YUV, text & graphics with motion,720p | -1.2% | -0.8% | -0.6% |
| YUV, mixed content, 1440p | -2.0% | -1.7% | -1.7% |
| YUV, mixed content, 1080p | -1.9% | -1.2% | -1.4% |
| YUV, Animation, 720p | 0.3% | 2.3% | 1.7% |
| YUV, camera captured, 1080p | -0.6% | -2.0% | -1.1% |
| Enc Time[%] | 139% |  |  |
| Dec Time[%] | 107% |  |  |

Table 4: Results for lossless configuration

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | -1.3% | -1.6% | -4.4% | 0.2% |
| RGB, text & graphics with motion,720p | 0.1% | 0.1% | 0.0% | 0.2% |
| RGB, mixed content, 1440p | -0.1% | -0.2% | -0.2% | -0.1% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 2.5% | 2.5% | 1.2% | 3.4% |
| YUV, text & graphics with motion,720p | 0.7% | 0.7% | 0.2% | 1.1% |
| YUV, mixed content, 1440p | 0.4% | 0.4% | 0.3% | 0.5% |
| YUV, mixed content, 1080p | 0.8% | 0.8% | 0.8% | 0.8% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 146% |  |  |  |
| Dec Time[%] | 97% |  |  |  |

|  | Random Access |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | -0.2% | -1.2% | -3.5% | 0.2% |
| RGB, text & graphics with motion,720p | -0.7% | -0.3% | -0.9% | 0.0% |
| RGB, mixed content, 1440p | -0.3% | -0.3% | -0.4% | -0.2% |
| RGB, mixed content, 1080p | -0.3% | -0.3% | -0.3% | -0.3% |
| RGB, Animation, 720p | -1.0% | -1.0% | -1.0% | -1.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 2.0% | 1.7% | 0.6% | 2.3% |
| YUV, text & graphics with motion,720p | 0.1% | 0.3% | 0.0% | 0.9% |
| YUV, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.2% |
| YUV, mixed content, 1080p | 0.2% | 0.2% | 0.2% | 0.2% |
| YUV, Animation, 720p | -0.2% | -0.2% | -0.2% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 136% |  |  |  |
| Dec Time[%] | 98% |  |  |  |

|  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.0% | -0.8% | -3.2% | 0.4% |
| RGB, text & graphics with motion,720p | -0.8% | -0.4% | -1.0% | 0.2% |
| RGB, mixed content, 1440p | -0.2% | -0.3% | -0.4% | -0.1% |
| RGB, mixed content, 1080p | -0.4% | -0.4% | -0.4% | -0.4% |
| RGB, Animation, 720p | -1.0% | -1.0% | -1.0% | -1.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 2.1% | 1.5% | 0.6% | 2.3% |
| YUV, text & graphics with motion,720p | 0.1% | 0.2% | 0.0% | 0.6% |
| YUV, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.2% |
| YUV, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | -0.3% | -0.3% | -0.3% | -0.3% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 135% |  |  |  |
| Dec Time[%] | 105% |  |  |  |

# Conclusion

In this contribution, an enlarged LCU size was proposed to exploiting the spatial and temporal correlation more effectively. According to the obvious BD-rate gain and the reduction of decoding complexity, it is recommended to consider the method.

# References

- B. Li, J. Xu, F. Wu, X. Guo, G. J. Sullivan, “Description of screen content coding technology proposal by Microsoft,” Document of Joint Collaborative Team on Video Coding, JCTVC-Q0035, Mar. 2014.

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common conditions for screen content coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-R1015, July. 2014.

# Patent rights declaration(s)

Lenovo may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).