| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0191 |
| --- | --- |

| Title: | CE9: Cross-check of JCTVC-S0102, CE9 Test A.1: Optionally disabling the usage of the intra boundary filters |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Robert Cohen Mitsubishi Electric Research Laboratories 201 Broadway, Cambridge, MA 02139 U.S.A | Tel: Email: | +1-617-621-7500 cohen@merl.com |
| Source: | Mitsubishi Electric Research Laboratories (MERL) |  |  |

_____________________________

# Abstract

This document provides the results of a crosscheck of CE9 Test A.1, Optionally disabling the usage of the intra boundary filters, from JCTVC-S0102. Crosschecks of two tests are performed: Test 1 always disables the intra boundary filter, and Test 2 selectively disables the intra boundary filters. The results of this crosscheck match the corresponding results in JCTVC-S0102.

# Cross-check overview

Two methods are cross-checked: Method 1 uses a candidate list of size 2, using spatial neighboring samples, and Method 2 uses a candidate list of size 4, using both spatial neighboring samples and colour candidates from the palette predictor table. The software provided by the authors of JCTVC-S0102 was used for this cross-check, and results are compared to the SCM-2.0 reference using the test conditions specified in the CE9 description JCTVC-R1109. Note that decode time ratios may vary due to differences in computing platforms.

# Experimental Results

## Test 1: Intra boundary filter is always disabled

### Lossy coding results

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.5% | -0.3% | -0.4% |
| RGB, text & graphics with motion,720p | -0.6% | -0.3% | -0.4% |
| RGB, mixed content, 1440p | -1.2% | -0.7% | -0.8% |
| RGB, mixed content, 1080p | -0.9% | -0.6% | -0.6% |
| RGB, Animation, 720p | -0.2% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | -0.1% | 0.0% | -0.1% |
| YUV, text & graphics with motion, 1080p | -0.2% | -0.1% | -0.1% |
| YUV, text & graphics with motion,720p | 0.1% | 0.2% | 0.1% |
| YUV, mixed content, 1440p | 0.2% | 0.1% | 0.2% |
| YUV, mixed content, 1080p | 0.2% | 0.1% | 0.2% |
| YUV, Animation, 720p | 0.2% | 0.0% | 0.2% |
| YUV, camera captured, 1080p | 0.3% | 0.3% | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.2% | -0.1% | -0.1% |
| RGB, text & graphics with motion,720p | -0.4% | -0.2% | -0.3% |
| RGB, mixed content, 1440p | -1.1% | -0.7% | -0.6% |
| RGB, mixed content, 1080p | -0.6% | -0.4% | -0.4% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.2% |
| RGB, camera captured, 1080p | -0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.1% | 0.0% | 0.2% |
| YUV, mixed content, 1440p | 0.1% | -0.2% | 0.1% |
| YUV, mixed content, 1080p | 0.2% | 0.1% | 0.0% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.1% |
| YUV, camera captured, 1080p | 0.1% | 0.2% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 103% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | -0.1% | -0.1% |
| RGB, text & graphics with motion,720p | -0.5% | -0.3% | -0.3% |
| RGB, mixed content, 1440p | -0.9% | -0.6% | -0.6% |
| RGB, mixed content, 1080p | -0.5% | -0.4% | 0.5% |
| RGB, Animation, 720p | -0.1% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion,720p | -0.1% | -0.1% | -0.2% |
| YUV, mixed content, 1440p | -0.3% | 0.0% | 0.2% |
| YUV, mixed content, 1080p | -0.1% | -0.1% | -0.3% |
| YUV, Animation, 720p | 0.1% | -0.3% | 0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.2% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 99% |  |  |

### Lossless coding results

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 103% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 99% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 100% |  |  |  |

## Test 2: Intra boundary filter is selectively disabled

### Lossy coding results

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.5% | -0.3% | -0.4% |
| RGB, text & graphics with motion,720p | -0.6% | -0.3% | -0.4% |
| RGB, mixed content, 1440p | -1.2% | -0.7% | -0.8% |
| RGB, mixed content, 1080p | -0.9% | -0.6% | -0.6% |
| RGB, Animation, 720p | -0.2% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | -0.1% | -0.1% | -0.1% |
| YUV, text & graphics with motion, 1080p | -0.2% | -0.1% | -0.1% |
| YUV, text & graphics with motion,720p | -0.1% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.2% | -0.1% | -0.1% |
| RGB, text & graphics with motion,720p | -0.4% | -0.2% | -0.3% |
| RGB, mixed content, 1440p | -1.1% | -0.7% | -0.6% |
| RGB, mixed content, 1080p | -0.6% | -0.4% | -0.4% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | -0.1% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 104% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | -0.1% | -0.1% |
| RGB, text & graphics with motion,720p | -0.5% | -0.3% | -0.3% |
| RGB, mixed content, 1440p | -0.9% | -0.6% | -0.6% |
| RGB, mixed content, 1080p | -0.5% | -0.4% | 0.5% |
| RGB, Animation, 720p | -0.1% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion,720p | -0.1% | 0.1% | -0.2% |
| YUV, mixed content, 1440p | -0.3% | -0.1% | 0.2% |
| YUV, mixed content, 1080p | 0.0% | 0.2% | -0.4% |
| YUV, Animation, 720p | 0.0% | -0.3% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.2% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 106% |  |  |

### Lossless coding results

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 103% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  |  |
| Dec Time[%] | 102% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  |  |
| Dec Time[%] | 102% |  |  |  |

# Conclusions

The results for both methods in this cross-check match the corresponding results from JCTVC-S0102.