| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0192 |
| --- | --- |

| Title: | CE10: Results of test 2, 4, 7 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xianyi Chen, Shuhui Wang, Tao Lin Jing Ye, Shan Liu, Shawmin Lei 1239 Siping Ave, Shanghai 200092, China 2860 Junction Ave, San Jose, CA 95134, USA | Email: | lintao@tongji.edu.cn jing.ye@mediatek.com shan.liu@mediatek.com |
| Source: | Tongji University, MediaTek Inc |  |  |

_____________________________

# Abstract

This document reports the experimental results of test 2 and 4 of HEVC CE 10 Intra String Copy, as described in document JCTVC-R1110. The ISC implementation is based on JCTVC-R0060 with 2 modifications: 1) changed from sample-based to pixel-based, 2) added 2D matching (copy). Memory access calculation code based on IBC and ILC memory access calculation code is added to measure memory access bandwidth of ISC. The experiments using SCM-2.0 as anchor report the following main results:

| SCM20IBC_FF vs | All Intra |  |  |  | Random Access |  |  |  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ISC 2D 1MB | Total | Avg | Min | Max | Total | Avg | Min | Max | Total | Avg | Min | Max |
| RGB, TGM, 1080p | 14.9% | 15.5% | 11.2% | 18.0% | 8.8% | 10.3% | 8.4% | 11.7% | 8.6% | 8.7% | 7.5% | 10.1% |
| RGB,TGM, 720p | 5.0% | 3.8% | 0.9% | 8.9% | 1.1% | 1.3% | 0.4% | 2.1% | 0.8% | 1.0% | 0.3% | 1.5% |
| RGB, MC, 1440p | 1.6% | 1.9% | 0.8% | 3.0% | 0.3% | 0.3% | 0.3% | 0.3% | 0.2% | 0.2% | 0.1% | 0.2% |
| RGB, MC, 1080p | 2.2% | 2.2% | 2.2% | 2.2% | 0.3% | 0.3% | 0.3% | 0.3% | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, TGM, 1080p | 15.2% | 15.7% | 9.7% | 19.9% | 8.3% | 10.9% | 7.3% | 13.9% | 8.0% | 9.4% | 7.2% | 13.2% |
| YUV, TGM, 720p | 4.6% | 3.8% | 0.7% | 7.7% | 0.8% | 1.3% | 0.3% | 2.7% | 0.6% | 1.0% | 0.2% | 1.9% |
| YUV, MC, 1440p | 1.6% | 1.8% | 0.6% | 2.9% | 0.3% | 0.2% | 0.2% | 0.3% | 0.1% | 0.1% | 0.1% | 0.2% |
| YUV, MC, 1080p | 2.1% | 2.1% | 2.1% | 2.1% | 0.3% | 0.3% | 0.3% | 0.3% | 0.1% | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 136% |  |  |  | 136% |  |  |  | 136% |  |  |  |
| Dec Time[%] | 187% |  |  |  | 187% |  |  |  | 187% |  |  |  |

| SCM20IBC_FF vs | All Intra |  |  |  | Random Access |  |  |  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ISC 1D 1MB | Total | Avg | Min | Max | Total | Avg | Min | Max | Total | Avg | Min | Max |
| RGB, TGM, 1080p | 12.2% | 12.6% | 9.3% | 14.6% | 6.9% | 8.0% | 6.6% | 8.7% | 6.8% | 6.5% | 4.8% | 8.1% |
| RGB,TGM, 720p | 4.2% | 3.0% | 0.7% | 8.2% | 0.9% | 0.8% | 0.3% | 1.2% | 0.6% | 0.6% | 0.2% | 0.8% |
| RGB, MC, 1440p | 1.3% | 1.5% | 0.5% | 2.4% | 0.2% | 0.2% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, MC, 1080p | 1.8% | 1.8% | 1.8% | 1.8% | 0.3% | 0.3% | 0.3% | 0.3% | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, TGM, 1080p | 12.3% | 12.8% | 7.9% | 17.2% | 6.4% | 8.4% | 5.4% | 11.6% | 6.1% | 7.1% | 5.1% | 10.9% |
| YUV, TGM, 720p | 3.8% | 2.9% | 0.5% | 7.1% | 0.6% | 0.8% | 0.2% | 1.5% | 0.4% | 0.6% | 0.1% | 1.1% |
| YUV, MC, 1440p | 1.2% | 1.4% | 0.4% | 2.4% | 0.2% | 0.2% | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, MC, 1080p | 1.7% | 1.7% | 1.7% | 1.7% | 0.3% | 0.3% | 0.3% | 0.3% | 0.1% | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 118% |  |  |  | 105% |  |  |  | 103% |  |  |  |
| Dec Time[%] | 185% |  |  |  | 246% |  |  |  | 222% |  |  |  |

| SCM20IBC_FF vs | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ISC 2D 1MB | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, TGM, 1080p | -2.9% | -3.1% | -3.0% | -1.8% | -2.1% | -2.0% | -1.0% | -1.4% | -1.3% |
| RGB,TGM, 720p | -1.6% | -1.7% | -1.8% | -1.9% | -1.9% | -2.1% | -1.0% | -1.0% | -1.2% |
| RGB, MC, 1440p | -0.1% | -0.1% | -0.1% | -0.7% | -0.6% | -0.6% | -0.4% | -0.5% | -0.4% |
| RGB, MC, 1080p | -1.1% | -1.2% | -1.1% | -1.0% | -1.2% | -1.2% | -1.3% | -1.4% | -0.8% |
| YUV, TGM, 1080p | -2.0% | -2.3% | -2.1% | -1.5% | -2.1% | -2.0% | -0.9% | -1.2% | -1.2% |
| YUV, TGM, 720p | -0.8% | -1.4% | -1.8% | -1.3% | -2.5% | -2.9% | -1.0% | -1.3% | -2.2% |
| YUV, MC, 1440p | 0.0% | -0.3% | -0.2% | -0.3% | -0.8% | -0.5% | -0.2% | -0.9% | -0.2% |
| YUV, MC, 1080p | -1.0% | -2.7% | -2.2% | -0.8% | -3.2% | -2.5% | -1.0% | -2.4% | -2.6% |
| Enc Time[%] | 146% |  |  | 114% |  |  | 109% |  |  |
| Dec Time[%] | 233% |  |  | 302% |  |  | 274% |  |  |

| SCM20IBC_FF vs | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ISC 1D 1MB | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, TGM, 1080p | -1.3% | -1.5% | -1.4% | -0.9% | -1.0% | -0.9% | -0.4% | -0.6% | -0.5% |
| RGB,TGM, 720p | -1.1% | -1.0% | -1.2% | -1.4% | -1.4% | -1.5% | -0.8% | -0.7% | -1.1% |
| RGB, MC, 1440p | -0.1% | -0.1% | -0.1% | -0.6% | -0.6% | -0.6% | -0.3% | -0.4% | -0.5% |
| RGB, MC, 1080p | -1.1% | -1.1% | -1.0% | -1.1% | -1.2% | -1.0% | -0.8% | -0.8% | -0.5% |
| YUV, TGM, 1080p | -0.9% | -1.2% | -1.1% | -0.7% | -1.3% | -1.2% | -0.4% | -0.7% | -0.7% |
| YUV, TGM, 720p | -0.4% | -0.9% | -1.2% | -1.0% | -2.2% | -2.5% | -0.5% | -0.5% | -1.4% |
| YUV, MC, 1440p | -0.1% | -0.4% | -0.2% | -0.3% | -0.9% | -0.7% | -0.4% | -0.7% | -0.1% |
| YUV, MC, 1080p | -1.0% | -2.7% | -2.1% | -0.8% | -3.1% | -2.2% | -0.8% | -3.2% | -2.8% |
| Enc Time[%] | 123% |  |  | 107% |  |  | 105% |  |  |
| Dec Time[%] | 225% |  |  | 306% |  |  | 273% |  |  |

Memory access data:

Lossless coding

| SCM20IBC_FF vs | All Intra |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ISC 1D 1MB | 8-8 | 32-64 | 32-128 | 64-128 | 64-256 | 64-512 | 64-256 Cache | 64-512 Cache |
| RGB, TGM, 1080p | 99.37% | 99.37% | 99.10% | 99.10% | 95.89% | 89.10% | 101.07% | 100.79% |
| RGB,TGM, 720p | 101.46% | 101.46% | 101.40% | 101.40% | 99.01% | 93.24% | 102.73% | 102.67% |
| RGB, MC, 1440p | 100.57% | 100.57% | 100.48% | 100.48% | 98.79% | 95.12% | 102.37% | 102.34% |
| RGB, MC, 1080p | 99.74% | 99.74% | 99.48% | 99.48% | 97.12% | 93.33% | 100.15% | 99.90% |
| YUV, TGM, 1080p | 97.56% | 97.56% | 97.30% | 97.30% | 94.51% | 88.59% | 99.92% | 99.69% |
| YUV, TGM, 720p | 99.58% | 99.58% | 99.45% | 99.45% | 97.34% | 92.66% | 101.70% | 101.64% |
| YUV, MC, 1440p | 99.15% | 99.15% | 99.03% | 99.03% | 97.57% | 94.54% | 100.19% | 100.02% |
| YUV, MC, 1080p | 98.57% | 98.57% | 98.36% | 98.36% | 96.69% | 94.07% | 98.90% | 98.71% |

| SCM20IBC_FF vs | Random Access |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ISC 1D 1MB | 8-8 | 32-64 | 32-128 | 64-128 | 64-256 | 64-512 | 64-256 Cache | 64-512 Cache |
| RGB, TGM, 1080p | 98.17% | 98.17% | 98.05% | 98.05% | 97.24% | 95.62% | 98.04% | 97.84% |
| RGB,TGM, 720p | 99.89% | 99.89% | 99.89% | 99.89% | 99.78% | 99.50% | 99.92% | 99.90% |
| RGB, MC, 1440p | 99.93% | 99.93% | 99.93% | 99.93% | 99.90% | 99.80% | 99.99% | 99.99% |
| RGB, MC, 1080p | 99.98% | 99.98% | 99.98% | 99.98% | 99.93% | 99.79% | 99.97% | 99.96% |
| YUV, TGM, 1080p | 98.18% | 98.18% | 98.07% | 98.07% | 97.28% | 95.74% | 97.99% | 97.80% |
| YUV, TGM, 720p | 99.87% | 99.87% | 99.87% | 99.87% | 99.79% | 99.57% | 99.90% | 99.89% |
| YUV, MC, 1440p | 100.02% | 100.02% | 100.02% | 100.02% | 99.98% | 99.86% | 100.00% | 99.99% |
| YUV, MC, 1080p | 99.93% | 99.93% | 99.93% | 99.93% | 99.88% | 99.75% | 99.94% | 99.93% |

| SCM20IBC_FF vs | Low Delay B |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ISC 1D 1MB | 8-8 | 32-64 | 32-128 | 64-128 | 64-256 | 64-512 | 64-256 Cache | 64-512 Cache |
| RGB, TGM, 1080p | 98.54% | 98.54% | 98.41% | 98.41% | 97.45% | 95.65% | 98.36% | 98.16% |
| RGB,TGM, 720p | 99.92% | 99.92% | 99.91% | 99.91% | 99.82% | 99.58% | 99.89% | 99.88% |
| RGB, MC, 1440p | 99.97% | 99.97% | 99.97% | 99.97% | 99.94% | 99.85% | 99.98% | 99.97% |
| RGB, MC, 1080p | 99.93% | 99.93% | 99.93% | 99.93% | 99.91% | 99.85% | 99.96% | 99.95% |
| YUV, TGM, 1080p | 98.42% | 98.42% | 98.30% | 98.30% | 97.49% | 96.01% | 98.23% | 98.07% |
| YUV, TGM, 720p | 99.92% | 99.92% | 99.92% | 99.92% | 99.85% | 99.68% | 99.91% | 99.90% |
| YUV, MC, 1440p | 99.91% | 99.91% | 99.91% | 99.91% | 99.88% | 99.80% | 99.92% | 99.91% |
| YUV, MC, 1080p | 99.93% | 99.93% | 99.93% | 99.93% | 99.90% | 99.82% | 99.95% | 99.95% |

Other result summaries are

1) ISC coding parameter mapping does not affect coding performance due to changing to pixel-based ISC (Test 4).

2) SNL (string number limited to 1/4 of pixel number per CU) almost affects neither coding performance nor average memory access bandwidth (Test 7).

# Introduction

ISC is first officially proposed by Tongji University [1] in the 11th JCT-VC meeting. In the 18th JCT-VC meeting, in July 2014 Sapporo, a Core Experiment 10 (CE 10) is defined to further study:

- Benefit and complexity of 1D copy vs 2D copy (Test 2)

- Benefit of ISC coding parameter (pos and len) mapping (Test 4)

- Impact of constraining the number of string per CU (Test 7)

# Proposed changes

## Adding 2D ISC and changing from sample-based ISC to pixel-based ISC

The current ISC implementation includes both 1D matching (ISC1D) and 2D matching (ISC2D). Both ISC1D and ISC2D are changed from sample-based to pixel-based.

## Constraining the number of string per CU to a fraction of the number of pixel per CU (for Test 7)

The current ISC code implemented a String Number Limited (SNL) feature. The purpose of SNL is to reduce decoder memory access bandwidth.

Since at least one dedicated general purpose local working buffer of CTU size (4K pixels) is used in decoder implementation, access to strings inside the working buffer does not involve outside memory access, SNL is only applied to strings outside of current CTU in ISC2D case or 4K pixels distance in ISC1D case.

## Adding memory access calculation code

Based on memory access calculation code for IBC (IntraBCMemAccess02062014.patch) and ILC (SCM-2.0+MemCalc_r1.patch), memory access calculation code is added to measure memory access bandwidth of ISC.

Since at least one dedicated general purpose local working buffer of CTU size (4K pixels) is used in decoder implementation, access to strings inside the working buffer does not involve outside memory access, memory access calculation is only applied to strings outside of current CTU in ISC2D case or 4K pixels distance in ISC1D case.

# Simulation results

As an implementation example, the proposed methods were tested using SCM-2.0 as anchor. All the results were generated under the common test conditions and software reference configurations defined in Q1015 for SCC. The encoding and decoding runtime is not very accurate. Especially, decoding runtime may include memory access calculation time.

CE10_Test2_SCM20_FF_vs_ISC1D_1MB_SNLoff_vs_ISC2D_1MB_SNLoff_lossless.xls is Test 2 lossless coding performance data.

CE10_Test2_SCM20_FF_vs_ISC1D_1MB_SNLoff_vs_ISC2D_1MB_SNLoff_lossy.xls is Test 2 lossy coding performance data.

More data to be provided in a later version.

# Conclusions

ISC method shows significant performance gain in lossless coding, while the average memory access bandwidth increment is almost zero in ISC1D and less than 10% in ISC2D. ISC still has significant room for improvement. We suggest further study in CE and TE.

# References

[1]. T. Lin, S. Wang, P. Zhang, K. Zhou, “AHG7: Full-chroma (YUV444) dictionary+hybrid dual-coder extension of HEVC”, JCT-VC Document, JCTVC-K0133, Shanghai, China, October 2012.

[2]. Tao Lin, Shuhui Wang, Peijun Zhang, Kailun Zhou, “AHG8: P2M based dual-coder extension of HEVC”, JCT-VC Document, JCTVC-L0303, Geneva, Switzerland,14–23 Jan. 2013.

[3]. Jing Ye, Shan Liu, Shawmin Lei, Xianyi Chen, Liping Zhao, Tao Lin, “Improvements on 1D dictionary coding”, JCT-VC Document, JCTVC-Q0124, Valencia, ES, 27 March – 4 April 2014

# Patent rights declaration(s)

MediaTek Inc. Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Tongji University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).