| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0197 |
| --- | --- |

| Title: | VUI code point for SMPTE ST 2085 (YDzDx) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chad Fogg, Jim Helman | Email: | chadfogg@gmail.com jhelman@movielabs.com |
| Source: | Motion Picture Laboratories, Inc. |  |  |

_____________________________

# Abstract

Changes to accommodate a code point to indicate YDzDx (SMPTE CD 2085) are proposed for the HEVC and AVC specification’s respective VUI (Annex E) matrix_coeffs (Table E-5) elements. The Society of Motion Pictures and Television Engineers ST 2085 completes by February 2015. A liaison statement (m34700) from SMPTE includes the latest draft of CD 2085.

# Introduction

The YDzDx signal follows a practice in digital cinema to provide a 4:4:4 difference signal that is less correlated than a planar array of 4:4:4 YZX samples (matrix_coeffs=0 “Identity”). YDzDx can also be coded in the 4:2:0 chroma subsampled format that can, for example, be passed through equipment chains carrying YCbCr 4:2:0 video.

Modified text from the latest HEVC specification [1] that adds a VUI entry for YDzDx SMPTE FCD 2085 [2] is copied below in section 1.1 of this document. Differences from [1] are highlighted in red by the Microsoft Word “Track Changes” feature.

The CD draft document for ST 2085 is included in the SMPTE liaison statement m34700 to MPEG [3].

## Modified HEVC section E.3.1: VUI parameters semantics

- matrix_coeffs describes the matrix coefficients used in deriving luma and chroma signals from the green, blue, and red, or Y, Z, and X primaries, as specified in Table E.1.

- matrix_coeffs shall not be equal to 0 unless one or more of the following conditions are true:

- –	BitDepthC is equal to BitDepthY.

- –	chroma_format_idc is equal to 3 (4:4:4).

- The specification of the use of matrix_coeffs equal to 0 under all other conditions is reserved for future use by ITU-T | ISO/IEC.

- matrix_coeffs shall not be equal to 8 unless one of the following conditions is true:

- –	BitDepthC is equal to BitDepthY,

- –	BitDepthC is equal to BitDepthY + 1 and chroma_format_idc is equal to 3 (4:4:4).

- The specification of the use of matrix_coeffs equal to 8 under all other conditions is reserved for future use by ITU-T | ISO/IEC.

- When the matrix_coeffs syntax element is not present, the value of matrix_coeffs is inferred to be equal to 2 (unspecified).

- The interpretation of matrix_coeffs, together with colour_primaries and transfer_characteristics, is specified by the equations below.

NOTE 1 – For purposes of YZX representation when matrix_coeffs is equal to 0, the symbols R, G, and B are substituted for X, Y, and Z, respectively, in the following descriptions of Equations E-1 to E-3, E-7 to E-9, E-13 to E-15, E-19 to E-21, and E-47 to E-51.

- ER, EG, and EB are defined as "linear-domain" real-valued signals based on the indicated colour primaries before application of the transfer characteristics function. The application of the transfer characteristics function is denoted by ( x )′ for an argument x. The signals E′R, E′G, and E′B are determined by application of the transfer characteristics function as follows:

E′R = ( ER )′		(E-1)

E′G = ( EG )′		(E-2)

E′B = ( EB )′		(E-3)

- The range of E′R, E′G, and E′B is specified as follows:

–	If transfer_characteristics is not equal to 11 or 12, E′R, E′G, and E′B are real numbers with values in the range of 0 to 1 inclusive.

–	Otherwise, (transfer_characteristics is equal to 11 (IEC 61966-2-4) or 12 (Rec. ITU-R BT.1361 extended colour gamut system) ), E′R, E′G and E′B are real numbers with a larger range not specified in this Specification.

Nominal white is specified as having E′R equal to 1, E′G equal to 1, and E′B equal to 1.

Nominal black is specified as having E′R equal to 0, E′G equal to 0, and E′B equal to 0.

- The interpretation of matrix_coeffs is specified as follows:

–	If video_full_range_flag is equal to 0, the following applies:

–	If matrix_coeffs is equal to 1, 4, 5, 6, 7, 9, 10, or 11, the following equations apply:

Y = Clip1Y( Round( ( 1  <<  ( BitDepthY − 8 ) ) * ( 219 * E′Y + 16 ) ) )	(E-4)

Cb = Clip1C( Round( ( 1  <<  ( BitDepthC − 8 ) ) * ( 224 * E′PB + 128 ) ) )	(E-5)

Cr = Clip1C( Round( ( 1  <<  ( BitDepthC − 8 ) ) * ( 224 * E′PR + 128 ) ) )	(E-6)

–	Otherwise, if matrix_coeffs is equal to 0 or 8, the following equations apply:

R = Clip1Y( ( 1  <<  ( BitDepthY − 8 ) ) * ( 219 * E′R + 16 ) )	(E-7)

G = Clip1Y( ( 1  <<  ( BitDepthY − 8 ) ) * ( 219 * E′G + 16 ) )	(E-8)

B = Clip1Y( ( 1  <<  ( BitDepthY − 8 ) ) * ( 219 * E′B + 16 ) )	(E-9)

–	Otherwise, if matrix_coeffs is equal to 2, the interpretation of the matrix_coeffs syntax element is unknown or is determined by the application.

–	Otherwise (matrix_coeffs is not equal to 0, 1, 2, 4, 5, 6, 7, 8, 9, 10, or 11), the interpretation of the matrix_coeffs syntax element is reserved for future definition by ITU-T | ISO/IEC.

–	Otherwise (video_full_range_flag is equal to 1), the following applies:

–	If matrix_coeffs is equal to 1, 4, 5, 6, 7, 9, 10 or 11 the following equations apply:

Y = Clip1Y( Round( ( ( 1  <<  BitDepthY ) − 1 ) * E′Y ) )	(E-10)

Cb = Clip1C( Round( ( ( 1  <<  BitDepthC ) − 1 )  * E′PB + ( 1  <<  ( BitDepthC − 1 ) ) ) )	(E-11)

Cr = Clip1C( Round( ( ( 1  <<  BitDepthC ) − 1 )  * E′PR + ( 1  <<  ( BitDepthC − 1 ) ) ) )	(E-12)

–	Otherwise, if matrix_coeffs is equal to 0 or 8, the following equations apply:

R = Clip1Y( ( ( 1  <<  BitDepthY ) − 1 ) * E′R )		(E-13)

G = Clip1Y( ( ( 1  <<  BitDepthY ) − 1 ) * E′G )		(E-14)

B = Clip1Y( ( ( 1  <<  BitDepthY ) − 1 ) * E′B )		(E-15)

–	Otherwise, if matrix_coeffs is equal to 2, the interpretation of the matrix_coeffs syntax element is unknown or is determined by the application.

–	Otherwise (matrix_coeffs is not equal to 0, 1, 2, 4, 5, 6, 7, 8, 9, 10 or 11), the interpretation of the matrix_coeffs syntax element is reserved for future definition by ITU-T | ISO/IEC. Reserved values for matrix_coeffs shall not be present in bitstreams conforming to this version of this Specification. Decoders shall interpret reserved values of matrix_coeffs as equivalent to the value 2.

- The variables E′Y, E′PB, and E′PR (for matrix_coeffs not equal to 0 or 8) or Y, Cb, and Cr (for matrix_coeffs equal to 0 or 8) are specified as follows:

–	If matrix_coeffs is not equal to 0, 8, 10 or 11, the following equations apply:

E′Y = KR * E′R + ( 1 − KR − KB ) * E′G + KB * E′B		(E-16)

E′PB = 0.5 * ( E′B − E′Y ) ÷ ( 1 − KB )		(E-17)

E′PR = 0.5 * ( E′R − E′Y ) ÷ ( 1 − KR )		(E-18)

NOTE 2 – E′Y is a real number with the value 0 associated with nominal black and the value 1 associated with nominal white. E′PB and E′PR are real numbers with the value 0 associated with both nominal black and nominal white. When transfer_characteristics is not equal to 11 or 12, E′Y is a real number with values in the range of 0 to 1 inclusive. When transfer_characteristics is not equal to 11 or 12, E′PB and E′PR are real numbers with values in the range of −0.5 to 0.5 inclusive. When transfer_characteristics is equal to 11 (IEC 61966-2-4), or 12 (ITU-R BT.1361 extended colour gamut system), E′Y, E′PB and E′PR are real numbers with a larger range not specified in this Specification.

–	Otherwise, if matrix_coeffs is equal to 0, the following equations apply:

Y   = Round( G )		(E-19)

Cb = Round( B )		(E-20)

Cr = Round( R )		(E-21)

–	Otherwise, if matrix_coeffs is equal to 8, the following applies:

–	If BitDepthC is equal to BitDepthY, the following equations apply:

Y   = Round( 0.5 * G + 0.25 * ( R + B ) )		(E-22)

Cb = Round( 0.5 * G − 0.25 * ( R + B ) ) + ( 1  <<  ( BitDepthC − 1 ) )	(E-23)

Cr = Round( 0.5 * (R − B ) ) + ( 1  <<  ( BitDepthC − 1 ) )	(E-24)

NOTE 3 – For purposes of the YCgCo nomenclature used in Table E.1, Cb and Cr of Equations E-23 and E-24 may be referred to as Cg and Co, respectively. The inverse conversion for the above three equations should be computed as:

t   = Y − ( Cb − ( 1  <<  ( BitDepthC − 1 ) ) )		(E-25)

G = Clip1Y( Y + ( Cb − ( 1  <<  ( BitDepthC − 1 ) ) ) )	(E-26)

B = Clip1Y( t − ( Cr − ( 1  <<  ( BitDepthC − 1 ) ) ) )	(E-27)

R = Clip1Y( t + ( Cr − ( 1  <<  ( BitDepthC − 1 ) ) ) )	(E-28)

–	Otherwise (BitDepthC is not equal to BitDepthY), the following equations apply:

Cr = Round( R ) − Round( B ) + ( 1  <<  ( BitDepthC − 1 ) )	(E-29)

t = Round( B ) + ( ( Cr − ( 1  <<  ( BitDepthC − 1 ) ) )  >>  1 )	(E-30)

Cb = Round( G ) − t + ( 1  <<  ( BitDepthC − 1 ) )	(E-31)

Y = t + ( ( Cb − ( 1  <<  ( BitDepthC − 1 ) ) )  >>  1 )	(E-32)

NOTE 4 – For purposes of the YCgCo nomenclature used in Table E.1, Cb and Cr of Equations E-31 and E-29 may be referred to as Cg and Co, respectively. The inverse conversion for the above four equations should be computed as.

t   = Y − ( ( Cb − ( 1  <<  ( BitDepthC − 1 ) ) )  >>  1 )	(E-33)

G = Clip1Y( t + ( Cb − ( 1  <<  ( BitDepthC − 1 ) ) ) )	(E-34)

B = Clip1Y( t − ( ( Cr − ( 1  <<  ( BitDepthC − 1 ) ) )  >>  1 ) )	(E-35)

R = Clip1Y( B + ( Cr − ( 1  <<  ( BitDepthC − 1 ) ) ) )	(E-36)

–	Otherwise, if matrix_coeffs is equal to 10, the signal E′Y is determined by application of the transfer characteristics function as follows, and Equations E-39 to E-46 apply for specification of the signals E′PB and E′PR:

EY = KR * ER + ( 1 − KR − KB ) * EG + KB * EB		(E-37)

E′Y = ( EY )′			(E-38)

NOTE 5 – In this case, EY is defined from the "linear-domain" signals for ER, EG, and EB, prior to application of the transfer characteristics function, which is then applied to produce the signal E′Y. EY and E′Y are analogue with the value 0 associated with nominal black and the value 1 associated with nominal white.

E′PB = ( E′B − E′Y ) ÷ ( 2 * NB )	for − NB  <=  E′B − E′Y  <=  0	(E-39)

E′PB = ( E′B − E′Y ) ÷ ( 2 * PB )	for 0 < E′B − E′Y  <=  PB	(E-40)

E′PR = ( E′R − E′Y ) ÷ ( 2 * NR )	for − NR  <=  E′R − E′Y   <=  0	(E-41)

E′PR = ( E′R − E′Y ) ÷ ( 2 * PR )	for 0 < E′R − E′Y   <=  PR	(E-42)

where the constants NB, PB, NR, and PR are determined by application of the transfer characteristics function to expressions involving the constants KB and KR as follows:

NB = ( 1 − KB )′		(E-43)

PB = 1 – ( KB )′ 		(E-44)

NR = ( 1 − KR )′		(E-45)

PR = 1 – (  KR )′			(E-46)

–	Otherwise (if matrix_coeffs is equal to 11), the signal E′Y is determined by application of the transfer characteristics function as follows, and Equations E-39 and E-46 apply for specification of the signals E′PB and E′PR:

E′Y = ( EY )′			(E-47)

E′PB = (c1 *E′B − E′Y ) ÷ 2	 	(E-48)

E′PR = (E′R – c2 *E′Y ) ÷ 2		(E-49)

where the constants c1 and c2 are:

c1 = 0.986566		(E-50)

c2 = 0.991902		(E-51)

[ Note 9:  E′PB =  D′z  and E′PR = D′x ]

Table E.1 – Matrix coefficients

| - Value | - Matrix | - Informative remark |
| --- | --- | --- |
| - 0 | - Identity | - The identity matrix. - Typically used for GBR (often referred to as RGB); however, may also be used for YZX (often referred to as XYZ); see Equations E-19 to E-21 IEC 61966-2-1 (sRGB) Society of Motion Picture and Television Engineers ST 428-1 |
| - 1 | - KR = 0.2126; KB = 0.0722 | ITU-R Rec. BT.709-5 ITU-R Rec. BT.1361 conventional colour gamut system and extended colour gamut system IEC 61966-2-1 (sYCC) IEC 61966-2-4 xvYCC709 Society of Motion Picture and Television Engineers RP 177 (1993) Annex B |
| - 2 | - Unspecified | - Image characteristics are unknown or are determined by the application. |
| - 3 | - Reserved | - For future use by ITU-T ¦ ISO/IEC |
| - 4 | - KR = 0.30; KB = 0.11 | - United States Federal Communications Commission Title 47 Code of Federal Regulations (2003) 73.682 (a) (20) |
| - 5 | - KR = 0.299; KB = 0.114 | - ITU-R Rec. BT.470-6 System B, G (historical) - ITU-R Rec. BT.601-6 625 - ITU-R Rec. BT.1358 625 - ITU-R Rec. BT.1700 625 PAL and 625 SECAM - IEC 61966-2-4 xvYCC601 - (functionally the same as the value 6) |
| - 6 | - KR = 0.299; KB = 0.114 | - ITU-R Rec. BT.601-6 525 - ITU-R Rec. BT.1358 525 - ITU-R Rec. BT.1700 NTSC - Society of Motion Picture and Television Engineers 170M (2004) - (functionally the same as the value 5) |
| - 7 | - KR = 0.212; KB = 0.087 | - Society of Motion Picture and Television Engineers 240M (1999) |
| - 8 | - YCgCo | - See Equations E-22 to E-36 |
| - 9 | - KR = 0.2627; KB = 0.0593 | - Rec. ITU-R BT.2020 non-constant luminance system - See Equations E-16 to E-18 |
| - 10 | - KR = 0.2627; KB = 0.0593 | - Rec. ITU-R BT.2020 constant luminance system - See Equations E-37 to E-46 |
| - 11 | - Y′D′z D′x | - Society of Motion Picture and Television Engineers ST 2085 (2015) |
| - 12..255 | - Reserved | - For future use by ITU-T ¦ ISO/IEC |

[ Note to editor: add ST 2085 reference [1] to the Reference Annexes of AVC and HEVC]

# References

[1] SMPTE CD 2085, “Y’D’zD’x Color-Difference Encoding for X’Y’Z’ Signals”,(2014)

[2] JCTVC-R1013,  “Edition 2 Draft Text of High Efficiency Video Coding (HEVC), Including Format Range (RExt), Scalability (SHVC), and Multi-View (MV-HEVC) Extensions”,  Sapporo, JP, 30 June – 9 July 2014.

[3] m34700, “Liaison to MPEG” [from SMPTE], Strasbourg, FR, 17–24 Oct. 2014

# Patent rights declaration(s)

MovieLabs does not have any current or pending patent rights relating to the technology described in this contribution.