| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0198 |
| --- | --- |

| Title: | Conversion tools update |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | B. Mandel, C. Fogg | Email: | bill.mandel@nbcuni.com chadfogg@gmail.com |
| Source: | Universal Pictures; Motion Picture Laboratories, Inc. |  |  |

_____________________________

# Abstract

This information document provides notice of recent updates to conversion tools that feature scripts written in ctlrender (Color Transform Language) by the Academy of Motion Pictures Arts and Sciences (AMPAS) that performs video conversion tasks, in particular High Dynamic Range (HDR) & Wide Color Gamut (WCG) signals.  New scripts added since the Sapporo meeting in July 2014 utilize ctlrender’s internal tone mapping algorithm to render Standard Dynamic Range /Standard Color Gamut (SDR/SCG) signals, conforming to Bt.709, from larger color volumes in higher bitdepth HDR/WCG signal containers such as PQ (SMPTE ST 2084) / Bt.2020.

# Introduction

Since the Sapporo update [1], new scripts added to the source code repository [2].

Instructions to build ctrlrender and its dependent libraries (ILMbase, OpenEXR, LibTIFF, etc.) are given in [1]. A key distinction of ctlrender is its domain for most operations:  32-bit float linear light ACES primaries. Particular attention is paid to empirically driven detail such as: LAD, black point, reference white, etc.

The internal tone mapping tool is utilized in both the Reference Rendering Transform (RRT) and Output Device Transform (ODT) scripts. The tone mapper applies sigmoid rolloff on the upper intensity ranges, and a spline fit is applied to fit the mid-range above 5 nits. In the latter stages, a hue preservation step (a pixel-wise operation computed in log10 space with 6-axis gain) attempts to correct for color shifts created by the tone mapping process.

The ctrlender scripts and associated C programs at the repository have been found to match within -65dB of the recent HDRTools created for the HDR/WCG adhoc studies [3]

# References

[1] JCTVC-R0214, “Video Conversion Tools Update”

[2] https://github.com/quantizationbit

[3] m35156, “HDRTools: A Software Package for Video Processing and Analysis.