| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0201 |
| --- | --- |

| Title: | Non-CE5: CU dependent color palette maximum size |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Wang, Zhan Ma, Meng Xu, H. Yu 2330 Central Expy, Santa Clara, USA | Tel: Email: | +1-408-330-5142 wei.wang.sc@huawei.com |
| Source: | Huawei USA R&D |  |  |

_____________________________

# Abstract

This contribution recommends a CU dependent color palette maximum size to balance the trade-off between coding efficiency and complexity throughput. .  Simulation results demonstrate averaged 0.5%/0.3%/0.1% lossy BD-Rate loss, and averaged -0.8%/-0.2%/-0.1% lossless bit rate increase for AI/RA/LB encoder configurations, respectively.  It provides gain compared with the scenarios that max. palette size and palette predictor size fixed as respective 15 and 32, but keeps the same throughput at small CU.

# Proposed Method

In SCM 2.0, the maximum palette size and maximum reference table size are configured as 31 and 64 respectively. As a result, the table derivation requires tremendous amount of resource to decide the re-use flag for each entry.  At the worst case, for any CU, it requires 31x64 = 1984 operations.  This problem becomes rather critical for smaller CUs, especially for 8x8.  With such a large table size, it is impractical to realize the palette mode for smaller CU using the reasonable resource.  Even it is doable, with such a computation intensive operation, 8x8 CU processing will be the bottleneck for the overall system.   On the other hand, since palette mode at smaller CU size also contributes noticeable coding efficiency, it would compromise the overall coding performance if the palette mode is completely disabled for 8x8/16x16 CUs.

CE5 [1] is investigating the impact to the coding efficiency from using different table sizes. In addition, JCTVC S0097 [2] provides additional information on using different maximum table sizes that are not specified in CE5.

In this contribution, instead of assigning a fixed table size for CUs, it is suggested to have the CU dependent maximum palette size for both current palette and palette predictor, shown in Table 1.

Table 1 CU dependent maximum size

| CU Size | Max. Palette Size | Max. Palette Predictor Size |
| --- | --- | --- |
| 64x64 | 31 | 32 |
| 32x32 | 31 | 32 |
| 16x16 | 15 | 32 |
| 8x8 | 15 | 32 |

As seen, for 8x8/16x16 CUs, it requires 15*32 = 480 operations, which is about a quarter of the operations resulted from default settings in SCM 2.0.

# Simulations

Experiments are performed on top of the SCM 2.0 with default settings using common test condition [3] (w/ full frame intra block copy). Results for lossy and lossless are summarized in Table 1 to 2.

Table 1 Lossy Coding Efficiency of Proposed Method vs. SCM 2.0

# Table 2 Lossless Coding Efficiency of Proposed Method vs. SCM 2.0

Please note that the running time is not reliable because of the heterogeneous computing nodes utilized for this simulation.

Alternatively, it also provides the data using another anchor where palette size is 15 and palette predictor size is 32.  From the simulation results, this proposed CU dependent maximum palette size improves the performance but keeps the same throughput as the fixed palette size 15 and palette predictor size 32.

Table 3 Lossy Coding Efficiency of Proposed Method vs. SCM 2.0 with palette size = 15 and palette predictor size = 32

Table 4 Lossless Coding Efficiency of Proposed Method vs. SCM 2.0 with palette size = 15 and palette predictor size = 32

# Recommendation

It is recommended to add this feature into HEVC SCC WD and SCM 3.0 s to improve the system throughput.

# Reference

[1] R. Joshi, and X. Xiu, “Description of Core Experiment 5 (CE5): Maximum Palette Size and Maximum Palette Predictor Size”, JCTVC Doc. R1105, Sapporo, JP, July 2014.

[2] P. Lai, S. Liu and S. Lei, “CE5: Informational tests on reducing both maximum palette size and maximum palette predictor size”, JCTVC Doc. S0097, Strasbourg, FR, Oct. 2014.

[3] H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common Test Conditions for Screen Content Coding”, Doc. JCT-VC R1015, Sapporo, Japan, July 2014.

# Patent rights declaration(s)

Huawei USA R&D may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).