| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0202 |
| --- | --- |

| Title: | Cross check of On deblocking for screen content coding (JCTVC-S0045) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ohji Nakagami Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2746 ohji.nakagami@jp.sony.com |
| Source: | Sony corporation |  |  |

_____________________________

# Abstract

This contribution is a cross check report of JCTVC-S0045.

JCTVC-S0045 proposes to modify the bS derivation on IntraBC boundary.

The cross checker confirmed that the provided software is consistent with the proposal.

Also, it is reported that the cross check result is identical to the proponent result.

# Cross check

JCTVC-S0045 [3] proposes to modify the bS derivation on the IntraBC boundary in the deblocking filter process. The proposal is summarized in Table 1.

The proponent provided the source code on top of HM-15.0+RExt-8.0+SCM-2.0. It is confirmed that the code is consistent with the proposal.

The simulation is conducted under SCC CTC [1]. Table 2 reportedly shows the cross check result is identical to the result in [3]. The complete result is attached in the package. It is noted that the run-time in the cross checker result is unstable due to the simulation cluster environment.

Table 1 derived bS value

| PU combination across the boundary | JCTVC-R1005/SCM-2.0 | JCTVC-S0045 |
| --- | --- | --- |
| BC/BC | 2 | 1 |
| BC/Intra | 2 | 1 |
| BC/Inter | 2 | 1 |
| Intra/Intra | 2 | 2 |
| Intra/Inter | 2 | 2 |
| Inter/Inter | 1 or 0 (depend on MV/ref/TU) | 1 or 0 (depend on MV/ref/TU) |

Table 2 cross check result

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.0% | -1.9% | -1.7% |
| RGB, text & graphics with motion,720p | 0.0% | -1.2% | -1.3% |
| RGB, mixed content, 1440p | 0.0% | -0.6% | -0.7% |
| RGB, mixed content, 1080p | 0.0% | -0.7% | -0.8% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | -0.1% | -2.5% | -2.3% |
| YUV, text & graphics with motion,720p | 0.0% | -1.0% | -1.4% |
| YUV, mixed content, 1440p | 0.0% | -0.8% | -0.8% |
| YUV, mixed content, 1080p | 0.0% | -1.6% | -1.4% |
| YUV, Animation, 720p | 0.0% | 0.2% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.2% |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.2% | -2.4% | -2.1% |
| RGB, text & graphics with motion,720p | 0.0% | -1.8% | -1.9% |
| RGB, mixed content, 1440p | -0.1% | -1.4% | -1.6% |
| RGB, mixed content, 1080p | -0.1% | -1.6% | -1.7% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | -0.2% | -2.7% | -2.6% |
| YUV, text & graphics with motion,720p | -0.1% | -1.7% | -2.3% |
| YUV, mixed content, 1440p | 0.0% | -1.8% | -1.9% |
| YUV, mixed content, 1080p | -0.1% | -3.1% | -2.9% |
| YUV, Animation, 720p | 0.0% | -0.3% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | -0.1% |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.3% | -1.9% | -1.7% |
| RGB, text & graphics with motion,720p | -0.2% | -1.4% | -1.6% |
| RGB, mixed content, 1440p | 0.0% | -1.1% | -1.2% |
| RGB, mixed content, 1080p | -0.2% | -2.2% | -1.9% |
| RGB, Animation, 720p | -0.1% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | -0.1% | -0.1% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.2% | -2.2% | -2.0% |
| YUV, text & graphics with motion,720p | -0.1% | -1.3% | -1.9% |
| YUV, mixed content, 1440p | -0.1% | -1.6% | -1.6% |
| YUV, mixed content, 1080p | -0.4% | -4.8% | -4.3% |
| YUV, Animation, 720p | 0.0% | -0.1% | 0.2% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.0% |

# Comment from the cross checker

The gain provided by [3] shows that the bS derivation process in the current draft [2] is not optimal on the IntraBC boundary. One of the reason would be IntraBC has a similar characteristics to Inter PU in terms of prediction image quality, rather than Intra PU.

The cross checker thinks the proposed change doesn’t increase the complexity from the current draft since the current process already checks the IntraBC boundary.

It is also commented that, when bS is equal to 1, the chroma deblocking filter is not applied on the edge.

# Reference

- JCTVC-R1015, “Common Test Conditions for Screen Content Coding”

- JCTVC-R1005, “HEVC Screen Content Coding Draft Text 1”

- JCTVC-S0045, “AHG13: On deblocking for screen content coding”

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).