| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0208 |
| --- | --- |

| Title: | CE8-related: Crosscheck for combination of CE8 Test A and CE8 Test B for screen content coding (JCTVC-S0050) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Yi-Wen Chen +886-3-5670766 ext. 24652 yiwen.chen@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The purpose of this document is to crosscheck JCTVC-S0050 on combination of CE8 Test A and CE8 Test B proposed by ITRI. The verification task was done, and the BD-rates results matched those provided by ITRI.

# Verification

ITRI provided the source code of their proposed method described in JCTVC-S0050 [1]. The verification is carried out by setting the following parameters in the provided code:

#define ITRI_TWO_SAMPLE_MODE 1

#if ITRI_TWO_SAMPLE_MODE

#define ITRI_TWO_SAMPLE_MODE_CANDIDATE_LIST_SIZE         1   // the number of candidates of bi-color mode

#define ITRI_TWO_SAMPLE_MODE_ENCODING_CHOICE_THRESHOLD   0.1

#define ITRI_HIST_NEIGHBORS                              1   // 1:use Hist to get neighboring colors

#if ITRI_HIST_NEIGHBORS

#define MAX_NEIGHBOR_SIZE                                2   // the number of neighboring colors required

// it is related with ITRI_TWO_SAMPLE_MODE_CANDIDATE_LIST_SIZE

#endif

#define ITRI_TWO_SAMPLE_MODE_DISABLE_PSLICE              1   // 1:disable slice-level flag (not completed)

#define ITRI_FEW_SAMPLE_MODE                             1   // 1:inclusion of single color mode

#define ITRI_CUPARTITION_LIMITATION                      0   // 1:disable 64; 2:disable 64-32; 3:disable 64-16

#define ITRI_FAST_SKIP_REPEAT_MODE                       1   // 1:skip repeated candidate

#define ITRI_LOSSLESS_BUGFIX                             1

#endif

The tests are conducted under the common test conditions.

# Results

The BD-rates from our experimental results match exactly those provided by ITRI. The simulations for the anchor and the tested method are not on the clusters with the same capability. Therefore, the run time is not reliable. The detailed results can be seen in the attached excel files.

# Conclusion

The BD-rates results presented by ITRI in JCTVC-S0050 are confirmed.

# References

- Y.-J. Chang, C.-C. Lin, C.-L. Lin, J.-S. Tu, C.-H. Hung, “Non-CE8: Combination of CE8 Test A and CE8 Test B for screen content coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-S0050, Oct. 2014.