| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0209 |
| --- | --- |

| Title: | CE8-related: Crosscheck for Multi-colour intra mode for screen content coding (JCTVC-S0051) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Yi-Wen Chen +886-3-5670766 ext. 24652 yiwen.chen@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The purpose of this document is to crosscheck JCTVC-S0051 on combination of multi-colour intra mode proposed by ITRI. The verification task was done, and the BD-rates results of Test 1 matched those provided by ITRI.

# Verification

ITRI provided the source code of their proposed method described in JCTVC-S0051 [1]. The verification is carried out by setting the following parameters in the provided code:

Test 1:

#define ITRI_FEW_SAMPLE_MODE 1

#if ITRI_FEW_SAMPLE_MODE

#define ITRI_FEW_SAMPLE_MODE_CANDIDATE_LIST_SIZE         10   // Can be any size

#define ITRI_FEW_SAMPLE_MODE_ENCODING_CHOICE_THRESHOLD   0.1

#define ITRI_HIST_NEIGHBORS                              1    // 1:Use Hist. to get neighboring colors

#define ITRI_FEW_SAMPLE_MODE_DISABLE_PSLICE              1    // 1:Disable slice-level flag

#define ITRI_CUPARTITION_LIMITATION                      0    // 1:Disable 64; 2:Disable 64-32; 3:Disable 64-16

#define ITRI_FAST_SKIP_REPEAT_MODE                       1    // 1:Skip repeated candidate

#define ITRI_FAST_SKIP_UNUSED_DEFAULT                    1    // 1:Skip unused default value

#define ITRI_LOSSLESS_BUGFIX                             1

#endif

Additional tests are also verified by another software provided by ITRI and the parameter in the provided code is listed as below:

Test 2:

#define ITRI_FEW_SAMPLE_MODE 1

#if ITRI_FEW_SAMPLE_MODE

#define ITRI_FEW_SAMPLE_MODE_CANDIDATE_LIST_SIZE         10   // Can be any size

#define ITRI_FEW_SAMPLE_MODE_ENCODING_CHOICE_THRESHOLD   0.1

#define ITRI_HIST_NEIGHBORS                              1    // 1:Use Hist. to get neighboring colors

#define ITRI_FEW_SAMPLE_MODE_DISABLE_PSLICE              1    // 1:Disable slice-level flag

#define ITRI_CUPARTITION_LIMITATION                      0    // 1:Disable 64; 2:Disable 64-32; 3:Disable 64-16

#define ITRI_FAST_SKIP_REPEAT_MODE                       1    // 1:Skip repeated candidate

#define ITRI_FAST_SKIP_UNUSED_DEFAULT                    1    // 1:Skip unused default value

#define ITRI_LOSSLESS_BUGFIX                             1

#endif

Test 3:

#define ITRI_FEW_SAMPLE_MODE 1

#if ITRI_FEW_SAMPLE_MODE

#define ITRI_FEW_SAMPLE_MODE_CANDIDATE_LIST_SIZE         10   // Can be any size

#define ITRI_FEW_SAMPLE_MODE_ENCODING_CHOICE_THRESHOLD   0.1

#define ITRI_HIST_NEIGHBORS                              4    // 1:Use Hist. to get neighboring colors

#define ITRI_FEW_SAMPLE_MODE_DISABLE_PSLICE              1    // 1:Disable slice-level flag

#define ITRI_CUPARTITION_LIMITATION                      0    // 1:Disable 64; 2:Disable 64-32; 3:Disable 64-16

#define ITRI_FAST_SKIP_REPEAT_MODE                       1    // 1:Skip repeated candidate

#define ITRI_FAST_SKIP_UNUSED_DEFAULT                    1    // 1:Skip unused default value

#define ITRI_LOSSLESS_BUGFIX                             1

#endif

The tests are conducted under the common test conditions.

# Results

For Test 1, the BD-rates from our experimental results match exactly those provided by ITRI. The simulations for the anchor and the tested method are not on the clusters with the same capability. Therefore, the run time is not reliable. The detailed results can be seen in the attached excel files.

The simulations for Test 2 and Test 3 are still on-going and the results will be updated in next revision of this report.

# Conclusion

The BD-rates results of Test1 presented by ITRI in JCTVC-S0051 are confirmed.

# References

- Y.-J. Chang, C.-C. Lin, C.-L. Lin, J.-S. Tu, C.-H. Hung, “Non-CE8: Multi-colour intra mode for screen content coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-S0051, Oct. 2014.