| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0212 |
| --- | --- |

| Title: | CE8: Cross-Check Results of Test C |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | 1Jih-Sheng Tu 2Chao-Hsiung Hung Ching-Chieh Lin Yao-Jen Chang 3Chun-Lung Lin | Tel: Email: | 1+886-3-5915639 1sunriseJSTu@itri.org.tw 2chhung@itri.org.tw 3Chunlung@itri.org.tw |
| Source: | ITRI International |  |  |

_____________________________

# Abstract

This document provides the cross-check results of CE8, Test C, independent uniform prediction intra mode [1][2][3]. Bitrates and PSNR all match those provided by proponent exactly.

The source code of CE8, Test C, was provided by MERL. ITRI compiled the source code by Microsoft Visual Studio Express 2013 for Windows Desktop, 64 bits. The results were simulated under the common test conditions [4]. Table 1 gives the environment of PCs.

Table 1. Environment of PCs

| Platform | CPU | Memory |
| --- | --- | --- |
| Windows Server 2008 R2 Enterprise 64 bit | Intel Xeon E5620@2.4GHz | 32 GB |

# Introduction

CE8 Test C provides an independent uniform prediction method to a block by a single sample. The sample candidate number and values are signalled at slice level. One additional CU-level flag indicates corresponding CU is coded by this mode or not. The colour index will be signalled if the candidate number is more than one.

# Simulation Results

Table 2 and Table 3 show the simulation results. The detailed results of each test sequences can be found in attached files.

Table 2. Lossless results.

Table 3. Lossy results.

# Reference

- X. Zhang, R. Cohen, “SCCE3 Test D.2: Independent Uniform Prediction (IUP) Mode,” 18th JCT-VC meeting, JCTVC-R0200, Sapporo, JP, June, 2014.

- R. Cohen, X. Zhang, “CE8 Test C: Independent Uniform Prediction Intra Mode (IUP),” 19th JCT-VC meeting, JCTVC-S0176, Strasbourg, FR, Oct. 2014.

- R. Cohen, T.-D. Chuang, Y.-J. Chang, “Description of Core Experiment 8 (CE8): Single-Colour and Two-Colour Modes,” 18th JCT-VC meeting, JCTVC-R1108, Sapporo, JP, June, 2014.

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common test conditions for screen content coding,” 18th JCT-VC meeting,  JCTVC-R1015, Sapporo, JP, June, 2014..