| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0216 |
| --- | --- |

| Title: | CE7: crosscheck report of CE7 test 5.1 1-D string-based index coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhanglei Xu 29F, Kerry Parkside NO.1155 Fangdian Road Shanghai, 201204, China | Tel: Email: | +86-21-63350606-5817 xuzhanglei@cn.fujitsu.com |
| Source: | Fujitsu R&D Center Co., Ltd. |  |  |

_____________________________

# Abstract

This document reports the cross-check results presented for CE7 test 5.1 on 1-D string-based index coding (JCTVC-R0268 [1]). Simulation result matches the data provided by the proponent.

# Introduction

JCTVC- R0268 [1] describes a color index map coding method using 1D string match based index compression. 1-D string match is performed within current CU with the representation of matched pairs. More detail of this technique can be found in JCTVC- R0268.

# Experimental results

CE7 Test 5.1 is conducted under the CE7 test conditions [2] [3] and summarized as below. The anchor is base software of SCM2 [4]. The detailed results can be found in the attached excel datasheets.

Table 1. Simulation results for lossless coding with full frame search

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.5% | 0.5% | 0.3% | 0.9% |
| RGB, text & graphics with motion,720p | 0.2% | 0.2% | 0.0% | 0.4% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.5% | 0.5% | 0.1% | 0.9% |
| YUV, text & graphics with motion,720p | 0.2% | 0.2% | 0.0% | 0.4% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 117% |  |  |  |
| Dec Time[%] | 116% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.2% | 0.2% | 0.1% | 0.4% |
| RGB, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.3% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.1% | 0.2% | 0.1% | 0.5% |
| YUV, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.3% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 104% |  |  |  |
| Dec Time[%] | 122% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.1% | 0.2% | 0.1% | 0.3% |
| RGB, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.2% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.1% | 0.1% | 0.0% | 0.3% |
| YUV, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.2% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |  |
| Dec Time[%] | 127% |  |  |  |

Table 2. Simulation results for lossy coding with full frame search

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.6% | -0.7% | -0.7% |
| RGB, text & graphics with motion,720p | -0.7% | -0.7% | -0.7% |
| RGB, mixed content, 1440p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | -1.0% | -1.3% | -1.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.6% | -0.8% | -0.8% |
| YUV, text & graphics with motion,720p | -0.2% | -0.4% | -0.4% |
| YUV, mixed content, 1440p | 0.0% | -0.4% | -0.3% |
| YUV, mixed content, 1080p | -0.8% | -2.4% | -2.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 110% |  |  |
| Dec Time[%] | 121% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.4% | -0.4% | -0.4% |
| RGB, text & graphics with motion,720p | -0.7% | -0.7% | -0.7% |
| RGB, mixed content, 1440p | -0.2% | -0.2% | -0.2% |
| RGB, mixed content, 1080p | -0.8% | -0.9% | -1.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.3% | -0.6% | -0.6% |
| YUV, text & graphics with motion,720p | -0.4% | -0.7% | -0.7% |
| YUV, mixed content, 1440p | -0.1% | -0.7% | -0.3% |
| YUV, mixed content, 1080p | -0.6% | -2.6% | -2.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | -0.1% |
| YUV, camera captured, 1080p | 0.1% | 0.2% | 0.0% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 126% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.3% | -0.4% | -0.4% |
| RGB, text & graphics with motion,720p | -0.6% | -0.6% | -0.6% |
| RGB, mixed content, 1440p | -0.2% | -0.4% | -0.2% |
| RGB, mixed content, 1080p | -0.8% | -0.6% | -0.4% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | -0.3% | -0.6% | -0.5% |
| YUV, text & graphics with motion,720p | -0.3% | -0.5% | -0.9% |
| YUV, mixed content, 1440p | -0.3% | -0.7% | -0.4% |
| YUV, mixed content, 1080p | -1.0% | -3.5% | -3.0% |
| YUV, Animation, 720p | 0.1% | -0.1% | 0.2% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 131% |  |  |

Table 3. Simulation results for lossless coding with 4-CTU search

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.8% | 0.9% | 0.6% | 1.1% |
| RGB, text & graphics with motion,720p | 0.4% | 0.5% | 0.0% | 1.0% |
| RGB, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.2% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.7% | 0.8% | 0.2% | 1.1% |
| YUV, text & graphics with motion,720p | 0.3% | 0.4% | 0.0% | 1.1% |
| YUV, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.1% |
| YUV, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 115% |  |  |  |
| Dec Time[%] | 116% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.2% | 0.4% | 0.2% | 0.6% |
| RGB, text & graphics with motion,720p | 0.1% | 0.2% | 0.0% | 0.7% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.2% | 0.4% | 0.1% | 0.7% |
| YUV, text & graphics with motion,720p | 0.0% | 0.2% | 0.0% | 0.8% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 104% |  |  |  |
| Dec Time[%] | 119% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.2% | 0.2% | 0.1% | 0.3% |
| RGB, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.3% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.1% | 0.3% | 0.1% | 0.4% |
| YUV, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.4% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  |  |
| Dec Time[%] | 123% |  |  |  |

Table 4. Simulation results for lossy coding with 4-CTU search

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.0% | -1.2% | -1.2% |
| RGB, text & graphics with motion,720p | -0.9% | -1.0% | -1.0% |
| RGB, mixed content, 1440p | -0.1% | -0.2% | -0.2% |
| RGB, mixed content, 1080p | -0.6% | -0.7% | -0.5% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.8% | -1.1% | -1.1% |
| YUV, text & graphics with motion,720p | -0.5% | -0.8% | -0.9% |
| YUV, mixed content, 1440p | -0.1% | -0.5% | -0.4% |
| YUV, mixed content, 1080p | -0.3% | -1.7% | -1.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 109% |  |  |
| Dec Time[%] | 120% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.6% | -0.7% | -0.6% |
| RGB, text & graphics with motion,720p | -0.9% | -1.0% | -1.0% |
| RGB, mixed content, 1440p | -0.1% | -0.3% | -0.2% |
| RGB, mixed content, 1080p | -0.5% | -0.5% | -0.6% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.1% | 0.1% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.5% | -0.8% | -0.8% |
| YUV, text & graphics with motion,720p | -0.6% | -0.8% | -1.0% |
| YUV, mixed content, 1440p | -0.2% | -0.7% | -0.7% |
| YUV, mixed content, 1080p | -0.3% | -1.7% | -1.5% |
| YUV, Animation, 720p | 0.0% | 0.0% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 124% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.4% | -0.5% | -0.5% |
| RGB, text & graphics with motion,720p | -0.6% | -0.8% | -0.8% |
| RGB, mixed content, 1440p | 0.0% | -0.1% | -0.3% |
| RGB, mixed content, 1080p | -0.9% | -1.1% | -0.7% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.1% | 0.1% |
| YUV, text & graphics with motion, 1080p | -0.4% | -0.9% | -0.9% |
| YUV, text & graphics with motion,720p | -0.4% | -0.9% | -1.1% |
| YUV, mixed content, 1440p | -0.3% | -0.6% | -0.6% |
| YUV, mixed content, 1080p | -0.4% | -1.9% | -1.2% |
| YUV, Animation, 720p | 0.1% | -0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.1% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 129% |  |  |

# References

- Zhan Ma, Wei Wang, Meng Xu, F, “Non-SCCE3: Results of SCCE3 with support of 2x4 CTUs as reference buffer”, JCTVC-R0268, Jul. 2014.

- Zhan Ma, “Description of Core Experiment 7 (CE7): String matching for palette index coding”, JCTVC-R1107, Jul. 2014.

- Haoping Yu, Robert Cohen, Krishna Rapaka, Jizheng Xu, “Common conditions for screen content coding tests”, JCTVC-R1015, Jul. 2014.

- R. Joshi, J. Xu, R. Cohen, S. Liu, Z. Ma, Y. Ye, “Screen Content Coding Test Model 2 Encoder Description (SCM 2),” JCTVC-R1014, Jul. 2014.