| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0219 |
| --- | --- |

| Title: | Cross-verification of JCTVC-S0155: Non-CE6: Palette copy above mode for the first row |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yuwen He, Yan Ye 9710 Scranton Rd, #250 San Diego, CA 92121, USA | Tel: Email: | + 1-858-210-4830 Xiaoyu.Xiu@InterDigital.com Yan.Ye@InterDigital.com Yuwen.He@InterDigital.com |
| Source: | InterDigital Communications, LLC |  |  |

_____________________________

# Abstract

This document presents the cross-verification results of JCTVC-S0155 on palette copy above mode for the first row for palette coding by Qualcomm. It is verified that the implementation is consistent with the proposal description and all the simulation results are perfectly matched with that provided by the proponent.

# Introduction

In the existing design of palette-based coding in HEVC SCC working draft, the copy mode is not used for the first row of each palette-coded CU, such that the palette mode is not signaled and always inferred to be run mode. In JCTVC-S0155, it is proposed to enable copy above mode for the samples in the first row of one CU by assigning a default palette index 0 for the palette indices of the samples outside the current CU. Specifically, the following changes are proposed to the current palette design:

- Enable both run mode and copy above mode for the first row of one CU with one additional context included in palette mode coding.

- Enable the palette index adjustment for the samples in the first row.

- For the case when the left neighbor is not escape color, and coded in run mode with the same palette index as that of the above palette index, enable copy above mode by using the palette index 0 (when the palette index of left neighbor is greater than 0) or 1 (when the palette index of left neighbor is 0) for the current sample to form the run-length in copy above mode.

The source codes of these methods are provided by proponent. This verification is carried out by using the provided source code. It is confirmed that the software implementation is consistent with the proposal description and all the simulation results are perfected matching the results provided from the proponent.

# Simulation results

So far, only the lossy results are verified while the lossless tests are still in running which will be updated as soon as they become available. The detailed results are presented and bundled in the same archive.

## Enabling copy above mode for the first row

Table 1 BD-rate performance of enabling copy above mode for the first row, compared to SCM-2.0 lossy coding

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.4% | -0.4% | -0.4% |
| RGB, text & graphics with motion,720p | -0.2% | -0.2% | -0.2% |
| RGB, mixed content, 1440p | 0.0% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.5% | -0.4% | -0.5% |
| YUV, text & graphics with motion,720p | -0.1% | -0.1% | -0.2% |
| YUV, mixed content, 1440p | -0.1% | -0.1% | -0.2% |
| YUV, mixed content, 1080p | -0.1% | -0.1% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 89% |  |  |
| Dec Time[%] | 90% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.2% | -0.2% | -0.2% |
| RGB, text & graphics with motion,720p | -0.1% | -0.2% | -0.1% |
| RGB, mixed content, 1440p | 0.0% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.2% | -0.3% | -0.3% |
| YUV, text & graphics with motion,720p | -0.1% | -0.2% | -0.1% |
| YUV, mixed content, 1440p | 0.0% | -0.2% | -0.1% |
| YUV, mixed content, 1080p | 0.0% | -0.3% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 88% |  |  |
| Dec Time[%] | 87% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.2% | -0.2% | -0.2% |
| RGB, text & graphics with motion,720p | 0.0% | -0.2% | -0.1% |
| RGB, mixed content, 1440p | 0.0% | -0.2% | -0.3% |
| RGB, mixed content, 1080p | -0.1% | 0.2% | 0.0% |
| RGB, Animation, 720p | -0.1% | 0.0% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.2% | -0.3% | -0.3% |
| YUV, text & graphics with motion,720p | -0.2% | 0.1% | -0.5% |
| YUV, mixed content, 1440p | -0.1% | 0.0% | 0.2% |
| YUV, mixed content, 1080p | -0.1% | -0.6% | -0.1% |
| YUV, Animation, 720p | 0.0% | -0.4% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 97% |  |  |

Table 2 Bit-rate performance of enabling copy above mode for the first row, compared to SCM-2.0 lossless coding

## Enabling copy above mode for the first row & copy above adjustment

Table 3 BD-rate performance of enabling copy above mode for the first row & copy above adjustment, compared to SCM-2.0 lossy coding

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.5% | -0.4% | -0.4% |
| RGB, text & graphics with motion,720p | -0.3% | -0.3% | -0.3% |
| RGB, mixed content, 1440p | 0.0% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.6% | -0.5% | -0.6% |
| YUV, text & graphics with motion,720p | -0.2% | -0.2% | -0.3% |
| YUV, mixed content, 1440p | -0.1% | -0.2% | -0.3% |
| YUV, mixed content, 1080p | -0.1% | -0.1% | -0.1% |
| YUV, Animation, 720p | 0.0% | -0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 93% |  |  |
| Dec Time[%] | 94% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.3% | -0.3% | -0.3% |
| RGB, text & graphics with motion,720p | -0.2% | -0.2% | -0.1% |
| RGB, mixed content, 1440p | 0.0% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.3% | -0.3% | -0.4% |
| YUV, text & graphics with motion,720p | -0.1% | -0.3% | -0.2% |
| YUV, mixed content, 1440p | -0.1% | -0.3% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | -0.1% | -0.2% |
| YUV, Animation, 720p | 0.0% | 0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 87% |  |  |
| Dec Time[%] | 87% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.3% | -0.3% | -0.3% |
| RGB, text & graphics with motion,720p | -0.2% | -0.2% | -0.1% |
| RGB, mixed content, 1440p | -0.1% | -0.2% | -0.1% |
| RGB, mixed content, 1080p | -0.3% | 0.2% | 0.4% |
| RGB, Animation, 720p | -0.1% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.3% | -0.4% | -0.3% |
| YUV, text & graphics with motion,720p | -0.2% | 0.0% | -0.3% |
| YUV, mixed content, 1440p | 0.0% | -0.1% | 0.0% |
| YUV, mixed content, 1080p | -0.1% | -0.2% | -0.2% |
| YUV, Animation, 720p | 0.1% | -0.4% | 0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |

Table 4 Bit-rate performance of enabling copy above mode for the first row & copy above adjustment, compared to SCM-2.0 lossless coding