| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0220r1 |
| --- | --- |

| Title: | On parallel processing capability of intra block copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Krishna Rapaka Vadim Seregin Chao Pang Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-658-2042 krapaka@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes methods to enhance parallel processing capability when intra block copy (IBC) mode is enabled. IBC mode uses previously decoded unfiltered samples within the same picture for its prediction. This introduces dependency that prediction samples of the current block have to be reconstructed before processing the current block.  In this contribution some restrictions and indications are proposed on IBC block vectors (BV) such that to allow decoder to be able to parallel process multiple coded tree blocks CTB’s in non-raster scan.

In the revision 1 of the this document, simulation results are provided for different delay restrictions proposed for intra BC

# Introduction and Problem statement

IBC mode uses previously decoded unfiltered samples within the same picture for its prediction. In the current test model, for the IBC mode the search range is unrestricted and can use any unfiltered decoded samples of the current picture (full search IBC). In such a scenario, some parallel processing configurations (e.g processing multiple CTB’s in wave front) might have synchronization issues when processed in parallel.

This contribution identifies following problems when non-raster multi-core parallel processing is enabled

- The decoder specification imposes restrictions only on entropy parsing when WPP is enabled and the availability of prediction samples for IBC is unrestricted. It is asserted that information of IBC prediction region distribution is useful for decoders in enhancing parallel processing capability.

- There have been some methods proposals to restrict the IBC prediction region when WPP is enabled. However, it is asserted that a decoder needs to be able to support every configuration (that is when WPP is disabled) and hence restricting prediction region only for specific setting (e.g. restriction only when WPP is enabled) does not benefit the system overall.

- It is asserted that even when WPP is disabled, the decoders may decode multiple CTB’s in parallel if there is some kind of indication on IBC prediction region availability. Such an indication is argued to be flexible, efficient in terms of coding efficiency and also in line with motion constraint prediction restriction in other HEVC extension like MV-HEVC, 3D-HEVC, SHVC.

- Unlike Intra mode which used only one line of spatial neighbouring pixels for prediction, IBC could use any pixels from neighboring CTB’s. In such scenarios, it is typical for spatial left CTB’s to be maintained in cache/local memory to avoid delay is storing and fetching from SRAM. When multiples cores are used to decode CTB’s in the wavefront order, it introduces the need to share cache across cores. Hence it is asserted pipelining based on just intra prediction sample availability may not be optimal for the system.

# Proposal

In this contribution several methods are proposed  in order to address problems detailed above. The methods can be classified as

- Fixed bitstream restriction on IBC prediction region (2.1, 2.2 and 2.3)

- Flexible IBC prediction region indication to decoders to enhance parallel processing capabilities.

## IBC processing order with 1 CTB delay w.r.t above CTB row.

It is proposed to always apply following restriction to IBC prediction region availability as shown in the figure 4.1 below such that no IBC block predicts from red CTB’s. In the figure below, the region marked with blue is valid prediction CTB’s that need to be reconstructed.

Figure 4.1

The

### 8.4.4 Derivation process for block vector components in intra block copying prediction mode

–	It is a requirement of bitstream conformance that

(xPb + bvIntra[ xPb ][ yPb ][ 0 ] + nPbSw − 1)/CtbSizeY – xCurr/CtbSizeY <=

yCurr/CtbSizeY – (yPb + bvIntra[ xPb ][ yPb ][ 1 ] + nPbSh – 1 )/CtbSizeY

## IBC processing order with 2 CTB delay w.r.t above CTB row.

It is proposed to always apply following restriction to IBC prediction region availability as shown in the figure 4.2 below such that no IBC block predicts from red CTB’s. In the figure below, the region marked with blue is valid prediction CTB’s that need to be reconstructed.

Figure 4.2

### 8.4.4 Derivation process for block vector components in intra block copying prediction mode

–	It is a requirement of bitstream conformance that

(xPb + bvIntra[ xPb ][ yPb ][ 0 ] + nPbSw − 1)/CtbSizeY – xCurr/CtbSizeY <=

2* (yCurr/CtbSizeY – (yPb + bvIntra[ xPb ][ yPb ][ 1 ] + nPbSh – 1 )/CtbSizeY)

## IBC processing order with tile shaped regions.

It is proposed to always apply following restriction to IBC prediction region availability as shown in the figure 4.3 below such that no IBC block predicts from red CTB’s. In the figure below, the region marked with blue is valid prediction CTB’s that need to be reconstructed. Such a restriction may remove the need to share cache across cores.

Figure 4.3

### 8.4.4 Derivation process for block vector components in intra block copying prediction mode

–	It is a requirement of bitstream conformance that

xCurr/CtbSizeY – (xPb + bvIntra[ xPb ][ yPb ][ 0 ] + nPbSw – 1 )/CtbSizeY >=0

yCurr/CtbSizeY – (yPb + bvIntra[ xPb ][ yPb ][ 1 ] + nPbSh – 1 )/CtbSizeY >=0

## IBC processing order with flexible CTB delay w.r.t above CTB row:

Wavefront parallel processing enables to parallel process each CTB row in the picture. For example, in a 1080p picture, upto 17 CTB rows can be processed in parallel if the system has 17 parallel processing cores. However, in most multi-core systems, it is typical that only limited number of parallel processing cores are used (e.g 4). In this scenario, only 4 CTB rows are processed in parallel and 5th CTB row is processed after completion of one of above 4 CTB rows. In such a scenario, it is possible for the 5th CTB row to predict from the regions that are already decoded from the previous 4 CTB rows. In this contribution it is proposed to signal for each CTB row, the valid decoded region (CTB’s) for all its previous decoded CTB rows.

It is recommended the following are considered

- Signalling at VPS or PPS level the IBC prediction region availability for the entire sequence

- Allow flexibility to modify the IBC prediction region availability at slice level.

- Provide indication of IBC prediction region availability at VUI or SEI.

Below is an example implementation of the proposed method at slice header

| pps_scc_extension ( ) { | Descriptor |
| --- | --- |
| … |  |
| if(intra_block_copy_enabled_flag){ |  |
| pps_ibc_ref_avail_signal_present_flag |  |
| } |  |
| … |  |
| } |  |

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| if( tiles_enabled_flag ¦ ¦ entropy_coding_sync_enabled_flag ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| } |  |
| if(pps_ibc_ref_avail_restriction_present_flag) { |  |
| num_ctbY_in_slice_minus1 | ue(v) |
| for(i=0;i< num_ctbY_in_slice;i++) |  |
| for(j=0;j< i;i++) |  |
| max_delay_IBCPred_in_CTBs[ i ][ j ] | ue(v) |
| } |  |
| byte_alignment( ) |  |
| } |  |

pps_ibc_ref_avail_ restriction _present_flag equal to 1 specifies that intra block copy reference usage restrictions are present and are signaled in the slice segment header for all the coded tree blocks of the slice. pps_ibc_ref_avail_ restriction _present_flag equal to 0 specifies that intra block copy reference usage info is not present in the slice segment header  When pps_ibc_ref_avail_ restriction _present_flag is not present, it is inferred to be 0.

num_ctbY_in_slice_minus1 plus 1 specifies the number of CTB rows in the slice.

max_delay_IBCPred_in_CTBs[ i ][ j ] specifies for the current CTB row  i  that maximum delay in terms of CTB’s for each the previous CTB row  j  that is available for IBC prediction. When not present it is inferred to be equal to number of CTB in the row.

# Results

This section summarizes the results of different configuration

## IBC processing order with 1 CTB delay w.r.t above CTB row .

Table 1: Test 3.1 vs SCM 2.0 (lossy)

## IBC processing order with 2 CTB delay w.r.t above CTB row.

Table 2: Test 3.2 vs SCM 2.0 (lossy)

## IBC processing order with 0 CTB delay w.r.t above CTB row.

Table 2: Test 3.2 vs SCM 2.0 (lossy)

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).