| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0226 |
| --- | --- |

| Title: | Cross-check of ‘CE1: Test 2.2 – MVD coding using a universal entropy coding scheme’ by Mediatek |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Rosewarne | Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au |
|  | V. Kolesnikov Building A, The Park Estate, 5 Talavera Road, Macquarie Park NSW 2113, Australia. | Tel: Email: | +61 2 9805 2973 vladimir.kolesnikov@cisra.canon.com.au |
|  | M. Maeda 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan. | Tel: Email: | +81-3-3758-2111 maeda.mitsuru@canon.co.jp |
| Source: | Canon Information Systems Research Australia, Canon Inc. |  |  |

_____________________________

# Abstract

This contribution provides a cross-check result of “CE1: Test 2.2 – MVD coding using a universal entropy coding scheme” by Mediatek. The result reported by the proponent is confirmed in this cross-check.

# Cross check procedure

The source code for CE1: Test 2.2 was provided by Mediatek. Canon compiled the source code using GCC 4.7.1 and the simulation was performed on a Linux cluster under (non-intra) lossy and lossless conditions, as defined in [2]. Note that timing results are not comparable due to differences in the simulation environment.

# Analysis

The source code, the description provided in the proposal document [1] and proposed WD text modifications provided by Mediatek were studied. The normative changes described in the proposal [1] and in accompanying WD modification document were found to match the source code.

# Coding performance measurement

Table 1 and Table 2 show the lossy and lossless simulation results respectively for CE1: Test 2.2 implemented on top of HM-15.0-RExt8.0-SCM2.0 base software.  The anchor of the provided result is the HM-15.0-RExt8.0-SCM2.0 base software.  The simulation result of the provided source code matches the result reported by the proponent.

Table 1. Lossy simulation result with HM-15.0-RExt8.0-SCM2.0 base software as anchor.

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -2.8% | -2.7% | -2.7% |
| RGB, text & graphics with motion,720p | -0.6% | -0.6% | -0.6% |
| RGB, mixed content, 1440p | -0.2% | -0.2% | -0.2% |
| RGB, mixed content, 1080p | -0.2% | -0.2% | -0.2% |
| RGB, Animation, 720p | -0.2% | -0.2% | -0.2% |
| RGB, camera captured, 1080p | -0.3% | -0.2% | -0.2% |
| YUV, text & graphics with motion, 1080p | -3.1% | -3.0% | -3.0% |
| YUV, text & graphics with motion,720p | -0.7% | -0.7% | -0.7% |
| YUV, mixed content, 1440p | -0.1% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | -0.2% | -0.2% | -0.2% |
| YUV, Animation, 720p | -0.2% | -0.1% | 0.1% |
| YUV, camera captured, 1080p | -0.2% | -0.1% | -0.2% |
| Enc Time[%] | 95% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -3.6% | -3.5% | -3.5% |
| RGB, text & graphics with motion,720p | -1.3% | -1.4% | -1.2% |
| RGB, mixed content, 1440p | -0.3% | -0.2% | -0.3% |
| RGB, mixed content, 1080p | -0.5% | 0.0% | -0.2% |
| RGB, Animation, 720p | -0.1% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | -0.1% | -0.1% | -0.1% |
| YUV, text & graphics with motion, 1080p | -4.0% | -3.8% | -3.8% |
| YUV, text & graphics with motion,720p | -1.7% | -1.6% | -1.7% |
| YUV, mixed content, 1440p | -0.3% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | -0.4% | -0.6% | -0.2% |
| YUV, Animation, 720p | 0.0% | -0.1% | 0.1% |
| YUV, camera captured, 1080p | -0.2% | -0.3% | -0.3% |
| Enc Time[%] | 94% |  |  |
| Dec Time[%] | 101% |  |  |

Table 2. Lossless simulation result with HM-15.0-RExt8.0-SCM2.0 base software as anchor.

|  | Random Access |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.4% | 1.5% | 0.1% | 2.7% |
| RGB, text & graphics with motion,720p | 0.1% | 0.2% | 0.0% | 0.3% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.4% | 1.5% | 0.2% | 2.7% |
| YUV, text & graphics with motion,720p | 0.1% | 0.2% | 0.1% | 0.4% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 94% |  |  |  |
| Dec Time[%] | 102% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.4% | 2.2% | 0.1% | 4.6% |
| RGB, text & graphics with motion,720p | 0.1% | 0.3% | 0.0% | 0.7% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.4% | 2.1% | 0.2% | 4.6% |
| YUV, text & graphics with motion,720p | 0.1% | 0.3% | 0.0% | 0.9% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 93% |  |  |  |
| Dec Time[%] | 102% |  |  |  |

# Conclusion

The results provided by the proponent are verified in this cross-check report.

# References

- S.-T. Hsiang, T.-D. Chuang, S. Lei “CE1: Results of Tests 1.4, 2.2, & 3.3 on unified BVD & MVD coding”, JCTVC-S0162, Strasbourg, FR, October 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” Document of Joint Collaborative Team on Video Coding, JCTVC-R1015, 18th Meeting: Sapporo, Japan, 30 June – 9 July 2014.