| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0238 |
| --- | --- |

| Title: | Cross-check of JCTVC-S0129: On indication of IBC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | For information |  |  |
| Author(s) or Contact(s): | Jani Lainema | Tel: Email: | +358 7180 08000 firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution provides a cross-verification of JCTVC-S0129. JCTVC-S0129 proposes to signal the maximum CU size for which intra block copy mode is signalled. It is confirmed that by enabling the proposed signalling and setting the maximum intra block copy CU size to 16x16 the objective compression performance is increased on average by 0.1% under the common test conditions.

# Objective coding efficiency impact

In order to measure the effect of limiting the intra block copy CU size, the software provided by the proponent of JCTVC-S0129 was run under the common test conditions. Results when setting the maximum size to 16x16 are reported in the attached spreadsheet and summarized in the tables below. The simulation was done in a different computing cluster than the anchor. Thus, the encoding and decoding times are not comparable.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | -0.1% | -0.1% |
| RGB, text & graphics with motion,720p | -0.1% | 0.0% | -0.1% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.1% | -0.1% | -0.1% |
| YUV, text & graphics with motion,720p | -0.1% | -0.1% | -0.1% |
| YUV, mixed content, 1440p | -0.1% | -0.1% | -0.2% |
| YUV, mixed content, 1080p | -0.1% | -0.1% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | N/A |  |  |
| Dec Time[%] | N/A |  |  |

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | -0.1% | -0.1% |
| RGB, mixed content, 1440p | -0.1% | 0.0% | -0.1% |
| RGB, mixed content, 1080p | 0.1% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | -0.1% | 0.0% |
| YUV, text & graphics with motion,720p | -0.2% | -0.2% | -0.1% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | -0.1% | -0.3% | -0.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | N/A |  |  |
| Dec Time[%] | N/A |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | -0.1% | -0.1% |
| RGB, mixed content, 1440p | -0.1% | -0.1% | -0.2% |
| RGB, mixed content, 1080p | -0.3% | 0.4% | 0.3% |
| RGB, Animation, 720p | -0.1% | -0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | -0.2% | 0.0% | -0.2% |
| YUV, mixed content, 1440p | -0.2% | -0.4% | 0.1% |
| YUV, mixed content, 1080p | 0.1% | 0.5% | -0.4% |
| YUV, Animation, 720p | -0.1% | -0.2% | 0.3% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | N/A |  |  |
| Dec Time[%] | N/A |  |  |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).