| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0239 |
| --- | --- |

| Title: | CE1: Cross-check report of JCTVC-S0085 on Adaptive motion vector resolution for screen content |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Krishna Rapaka 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-658-2042 krapaka@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document reports cross checking results of JCTVC-S0085 on adaptive motion vector resolution for screen content. It was confirmed that the cross checking results exactly match the results provided by the proponents and the software implementation matches the proposed method description in JCTVC-S0068.

# Introduction

The document JCTVC-S0085 asserts that most of the motion vectors in the encoded bitstream point to a integer values for video that is captured from a computer desktops. It argues that if decoder could interpret the motion vectors adaptively as integer and fractional bits, encoders could save bits based on this interpretation. A normative change to the decoding process is proposed to scale the motion vectors based on the indication at slice level.

The document JCTVC-S0085, presents results following three configurations and under common test conditions specified for lossy and lossless coding.

- Single pass encoding decision process: - The process requires hash based search to be enabled. Three metrics are computed before coding the current picture (possibly in pre-processing stage) a) SAD for every pixel in the picture is calculated w.r.t to collocated picture to find matching blocks b) A hash based metric is computed to identify smooth regions c) hash based metric to compute number of matching block. The decision of MV accuracy is done based on above three metrics.

- Multi-pass encoding decision process

- Non-normative method of decision process

The cross checking simulations were performed using x64 Linux cluster. The software was provided by the proponent and compiled with GCC 4.1.2. The run time information may not be accurate.

# Simulation Results

The results were tested under the SCC common test conditions specified in [2].  Complete results can be found in the attached spreadsheets.

Table I. JCTVC-S0085_single_pass v.s. SCM2.0 Anchor (Lossy)

Table II. JCTVC-S0085_single_pass v.s. SCM2.0 Anchor (Lossless)

Table III. JCTVC-S0085_multi_pass v.s. SCM2.0 Anchor (Lossy)

Table IV. JCTVC-S0085_multi_pass v.s. SCM2.0 Anchor (Lossless)

Table V. JCTVC-S0085_multi_pass v.s. SCM2.0 Anchor (Lossy)

Table VI. JCTVC-S0085_multi_pass v.s. SCM2.0 Anchor (Lossless)

# Reference

- B. Li, J. Xu, G. Sullivan, Y. Zhou, B. Lin (Microsoft),“Adaptive motion vector resolution for screen content,” JCTVC-S0085.

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common test conditions for screen content coding,” JCTVC-R1015.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).