| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0250 |
| --- | --- |

| Title: | Non-CE10: Improvement on coding of ISC parameters and comparison to Palette Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kailun Zhou, Liping Zhao, Xianyi Chen, Tao Lin 1239 Siping Ave, Shanghai 200092, China | Email: | lintao@tongji.edu.cn |
| Source: | Tongji University |  |  |

_____________________________

# Abstract

This contribution proposes an ISC scheme which applies and extends a few schemes used in Palette coding to code three ISC parameters, i.e. pos, len, no_reference_pixel (NRP, previously called unmatched pixel or unmatchable pixel). Using SCM20FF as anchor, SCM20 integrated with the proposed ISC (1D version) is tested and the lossless and lossy results are summarized in the following table:

| SCM20IBC_FF vs | All Intra |  |  |  | Random Access |  |  |  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ISC 1D 1MB | Total | Avg | Min | Max | Total | Avg | Min | Max | Total | Avg | Min | Max |
| RGB, TGM, 1080p | 12.3% | 12.8% | 9.4% | 14.8% | 7.0% | 8.0% | 6.7% | 8.9% | 6.8% | 6.5% | 4.8% | 8.2% |
| RGB,TGM, 720p | 4.3% | 3.1% | 0.7% | 8.5% | 1.1% | 0.9% | 0.3% | 1.3% | 0.8% | 0.7% | 0.2% | 1.0% |
| RGB, MC, 1440p | 1.3% | 1.5% | 0.6% | 2.4% | 0.2% | 0.2% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, MC, 1080p | 1.8% | 1.8% | 1.8% | 1.8% | 0.3% | 0.3% | 0.3% | 0.3% | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, TGM, 1080p | 12.8% | 13.3% | 8.1% | 17.8% | 6.5% | 8.7% | 5.5% | 11.9% | 6.2% | 7.2% | 5.2% | 11.2% |
| YUV, TGM, 720p | 4.0% | 3.0% | 0.6% | 7.5% | 0.6% | 0.8% | 0.2% | 1.6% | 0.4% | 0.6% | 0.2% | 1.1% |
| YUV, MC, 1440p | 1.3% | 1.4% | 0.4% | 2.5% | 0.2% | 0.2% | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, MC, 1080p | 1.7% | 1.7% | 1.7% | 1.7% | 0.3% | 0.3% | 0.3% | 0.3% | 0.1% | 0.1% | 0.1% | 0.1% |

| SCM20IBC_FF vs | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ISC 1D 1MB | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, TGM, 1080p | -1.5% | -1.7% | -1.7% | -0.9% | -1.2% | -1.1% | -0.6% | -1.0% | -0.9% |
| RGB,TGM, 720p | -1.1% | -1.1% | -1.2% | -1.1% | -1.2% | -1.3% | -0.7% | -0.5% | -0.7% |
| RGB, MC, 1440p | -0.1% | -0.2% | -0.2% | -0.5% | -0.6% | -0.7% | -0.4% | -0.5% | -0.4% |
| RGB, MC, 1080p | -1.1% | -1.5% | -1.3% | -0.8% | -1.1% | -1.0% | -0.9% | -1.0% | -0.4% |
| YUV, TGM, 1080p | -1.2% | -1.7% | -1.5% | -0.8% | -1.7% | -1.5% | -0.5% | -1.2% | -1.0% |
| YUV, TGM, 720p | -0.4% | -0.9% | -1.3% | -0.9% | -2.0% | -2.4% | -0.5% | -1.1% | -1.9% |
| YUV, MC, 1440p | -0.2% | -0.9% | -0.7% | -0.2% | -1.2% | -1.0% | -0.5% | -1.2% | -0.9% |
| YUV, MC, 1080p | -1.0% | -3.1% | -2.6% | -1.0% | -3.3% | -2.8% | -1.1% | -3.3% | -2.5% |

# Introduction

Statistics data of ISC show that the following types of strings occur more frequently than others:

- strings with pos = 1

- strings with pos = cu_width

- strings with pos = cu_width - 1 and pos = cu_width + 1

- strings with pos = multiple of cu_width

- strings with pos = (multiple of cu_width) – 1

- strings with pos = (multiple of cu_width) + 1

By optimizing the scheme to code these special ISC coding parameters pos, len, and NRP, ISC coding performance can be improved.

The proposed ISC coding applies and extends a few schemes used in Palette coding to ISC coding: to do color quantization (CQ) as a pre-processing step and to code the special cases of 1) and 2). The proposed ISC coding also uses a Secondary Reference Buffer to very efficiently code the NRPs that has a high frequency to be referenced.

The proposed ISC coding has almost all benefits and merits of both traditional ISC coding and Palette Coding.

# New scheme to code ISC coding parameters

The syntax element of ISC bit-stream is very simple. It always has the same format:

copy_parameter[0], copy_parameter[1], … … , copy_parameter[i], copy_parameter[i+1], … …

Each copy_parameter[i] is either a pair (pos, len) or a NRP or a combination of both.

The first aspect of the new scheme is to assign very short binary code to code a few special cases of (pos, len) which have high frequency to occur, such as the six cases shown in Introduction section.

A string with pos=1 and len=14 is shown below. The reference string and the current string overlap.

A A A A A A A A A A A A A A

A A A A A A A A A A A A A A

Reference string

Current string

pos=1

In traditional ISC, the first pixel of the reference string is coded as a NRP and the current string is coded as a pair (pos=1, len=14).

To improve the coding performance in this case, the NRP also needs to be coded efficiently.

To code NRPs efficiently, the second aspect of the new scheme is to use NRPs to build a Secondary Reference Buffer (SRB). Then, for any NRP, if it is already in SRB, then it is copied from SRB using pos2 which is the position (address) of SRB. If it is a frequently referenced NRP (known in the CQ pre-processing step) but not in SRB, then it is put into SRB. SRB is illustrated in the following figure. The copy operation from SRB is actually a Point Copy (the reference string is one pixel) with a len to specify the length of the current string at the destination.

Left CTU

= Reconstructed picture buffer = Primary reference buffer (PRB)

Above CTU

Current CTU

The current CU being coded

point copy, and string copy are mixed in a CU

No Reference Pixels (NRP)

Secondary reference buffer (SRB)

NRP is put into SRB for point copy

Ref. string

Ref. string

Reference point

=Pixel of point matching (point copy)

=Pixel of string matching (string copy)

On the other hand, the traditional ISC reference buffer, i.e., the reconstructed picture buffer, is called Primary Reference Buffer (PRB) in ISC coding.

When coding a CU, both PRB and SRB are searched to find either an optimal reference string or an optimal reference point (pixel).

In the current implementation of the new scheme to code ISC parameters, two special cases (the first two mentioned in Introduction section, i.e. pos = 1 and pos = cu_width) are first considered for optimization. SRB size is up to 64 pixels.

# Similarity and difference between Palette coding and ISC coding

Palette coding and ISC coding have many similarities, although they look very different.

Palette coding first converts pixel to index and tries to code indices efficiently by finding reference runs in index domain.

On the other hand, ISC coding tries to directly code pixels efficiently by finding reference strings directly in pixel domain.

All reference runs in index domain in Palette coding have corresponding reference strings in pixel domain in ISC coding.

But not the other way around, because reference runs of index are all in the same CU but reference strings of pixels can be anywhere in the picture.

Therefore, ISC coding can generally find better references than Palette coding because ISC coding treats all pixels equally, no discrimination to any pixel no matter it is inside or outside of the current CU

Finding better references is not enough. Coding the references efficiently is also very important. The traditional ISC coding is designed for general-purpose reference string coding and is not very efficient for SCC.

By applying all useful schemes in Palette coding and extend them to ISC coding in a broader sense, ISC coding performance can be improved to have almost all benefits and merits of both Palette coding and ISC coding.

The followings are a few similarities or counterparts between Palette coding and ISC coding:

| Palette coding | ISC coding |
| --- | --- |
| Runs of index | Strings of pixel |
| COPY_ABOVE runs | String with pos = cu_width |
| COPY_LEFT runs | String with pos = 1 |
| Palette color or Major color | Frequently referenced NRP |
| Escape color | Infrequently referenced NRP |
| Anything new to be proposed | Can always find a counterpart |

# What is applied and extended to ISC doing from palette coding

The current implementation of the proposed ISC coding applies and extends the following schemes in Palette Coding to ISC coding in a broader sense.

- Color quantization (CQ) as a pixel pre-processing step of encoding

- The way to code COPY_ABOVE runs

- The way to code COPY_LEFT runs

- The way to code index (index is really a special type of pos in ISC coding)

- The way to code escape color

- travScan

Almost all schemes used in Palette coding to code “run of index”, palette color, and escape color can easily find a counterpart in ISC coding and be modified and applied to ISC coding to code the special counterpart “sting of pixel” and NRP.

Furthermore, most schemes currently used in Palette Coding probably can play an extended role in ISC coding because ISC coding does not use palette and index, and can use any pixels inside or outside of the current CU.

For example, COPY_ABOVE can be extended to outside of CU because in ISC coding, everything is done in “pixel domain” instead of “index domain”.

On the other hand, SRB is not from Palette coding, but it plays part of the role of palette in the sense that it keeps frequently referenced NRPs.

Another very important advantage of ISC coding is that it has much better lossless coding performance.

# Simulation results

See SCM20_FF_vs_ISC1DSRB_1MB_lossless.xls and SCM20_FF_vs_ISC1DSRB_1MB_lossy.xls

# Conclusion

Unifying ISC coding and Palette coding provides significant coding gain in lossless coding and moderate coding gain in lossy coding. ISC memory access bandwidth is almost the same as IBC if string number is limited. Still have room to improve for lossy and lossless coding.

Recommend to further study in CE:

- Unifying ISC coding and Palette coding,

- 1D vs 2D ISC test,

- Further reduce worst case mem access,

- Reduce cabac context model.

# References

- JCT-VC, “HM-15.0+RExt-8.0+SCM-2.0,” https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/ HM-15.0+RExt-8.0+SCM-2.0/

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common conditions for screen content coding tests,” JCTVC-R1015, Sapporo, Jul. 2014.

# Patent rights declaration(s)

Tongji may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).