| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0254 |
| --- | --- |

| Title: | Unification of colour transforms in ACT |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Zhang Jianle Chen Marta Karczewicz (Qualcomm) Bin Li Jizheng Xu (Microsoft) | Tel: Email: | 1-858-651-6660 lizhang@qti.qualcomm.com 1-858-651-8028 cjianle@qti.qualcomm.com libin@microsoft.com jzxu@microsoft.com |
| Source: | Qualcomm Incorporated, Microsoft Corp. |  |  |

_____________________________

# Abstract

The current design of residual-domain adaptive color transform uses two sets of colour transforms (i.e., YCoCg, and YCoCg-R) for lossy and lossless coding, respectively. In this contribution, it is proposed to use YCoCg-R for both lossy and lossless coding. To keep the bit-depth unchanged for lossy coding mode, the values of Co and Cg components after forward YCoCg-R transform are scaled by a factor of 1/2. Simulation results show that there is almost no coding performance difference compared to current design.

# Introduction

Residual adaptive colour transform (ACT) brings significant coding efficiency improvement, especially for RGB sequences, and it has been adopted into screen content coding extension [1]. In [1], the colour transforms, i.e., YCoCg and YCoCg-R used for lossy and lossless coding, respectively, are defined as follows:

YCoCg:       Forward: YCoCg=12120−2−12−1RGB4 Backward:RGB=11−11011−1−1YCoCg       (1)

YCoCg-R:        Forward: CoR−BtB+(co>>1)CgG−tYt+(cg>>1) Backward:tY−(Cg>>1)GCg+tBt−(Co>>1)RCo+b       (2)

# Proposed Method

As also noticed in [2], using reversible conversion does not harm the coding efficiency of lossy coding. To unify the design, this document proposes to use the YCoCg-R transform for both lossy coding and lossless coding. Different from [2], wherein the bit-depth of all three components are increased, the bit-depth is kept unchanged compared to those coded with ACT disabled in this proposal. To achieve this aim, after the forward YCoCg-R transform, both of Co and Cg components are scaled by a factor of ½. Meanwhile, before invoking the backward YCoCg-R transform, both of Co and Cg components are scaled by a factor of 2. No other changes are required.

# Experimental results

HM-15.0+RExt-8.0+SCM-2.0 default coding structure is used as the anchor. The common test conditions specified in [3] are followed. The summarized experimental results are shown in the following tables. More detailed experimental results could be found in the attached spreadsheets.

There is no performance changing for lossless coding.

Table 1. Experimental results for lossy coding

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | -0.1% |
| RGB, text & graphics with motion,720p | -0.1% | -0.1% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | -0.1% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.1% | -0.1% | 0.2% |
| YUV, text & graphics with motion, 1080p | 0.0% | -0.1% | -0.1% |
| YUV, text & graphics with motion,720p | 0.1% | 0.0% | 0.1% |
| YUV, mixed content, 1440p | 0.0% | -0.1% | -0.1% |
| YUV, mixed content, 1080p | 0.0% | -0.2% | -0.1% |
| YUV, Animation, 720p | 0.1% | -0.2% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 94% |  |  |

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | -0.1% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | -0.2% |
| RGB, camera captured, 1080p | 0.0% | -0.2% | 0.1% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | -0.1% |
| YUV, text & graphics with motion,720p | 0.1% | -0.4% | 0.0% |
| YUV, mixed content, 1440p | 0.1% | -0.1% | -0.2% |
| YUV, mixed content, 1080p | 0.0% | -0.5% | -0.4% |
| YUV, Animation, 720p | 0.1% | -0.2% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | -0.4% | -0.1% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 97% |  |  |

|  | Low delay B |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.0% | -0.1% | 0.0% |
| RGB, text & graphics with motion,720p | -0.2% | -0.1% | 0.1% |
| RGB, mixed content, 1440p | -0.1% | 0.0% | -0.1% |
| RGB, mixed content, 1080p | -0.1% | 0.2% | 0.5% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | -0.3% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | -0.1% | 0.0% |
| YUV, text & graphics with motion,720p | -0.2% | -0.1% | -0.3% |
| YUV, mixed content, 1440p | -0.2% | -0.3% | -0.1% |
| YUV, mixed content, 1080p | 0.0% | -0.3% | 0.0% |
| YUV, Animation, 720p | 0.0% | -1.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | -0.7% | -0.2% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 96% |  |  |

# References

- JCTVC-R0147, “SCCE5 Test 3.2.1: In-loop color-space transform,” Sapporo, JP, 30 June – 9 July 2014.

- JCTVC-S0086, “On residual adaptive colour transform,” Strasbourg, FR, 17–24 Oct. 2014.

- JCTVC-R1015, “Common conditions for screen content coding tests,” Sapporo, JP, 30 June – 9 July 2014.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Text Modifications

The text provided in this section is based on JCTVC-R1005_v3. All the additions are highlighted in yellow and the deletions are strikethrough.

### 8.6.8	Residual modification process for transform blocks using adaptive colour transform

This process is only invoked when ChromaArrayType is equal to 3.

Inputs to this process are:

–	a variable blkSize specifying the block size,

–	an (blkSize)x(blkSize) array of luma residual samples rY with elements rY[ x ][ y ],

–	an (blkSize)x(blkSize) array of chroma residual samples rCb with elements rCb[ x ][ y ],

–	an (blkSize)x(blkSize) array of chroma residual samples rCr with elements rCr[ x ][ y ].

Outputs of this process are:

–	an modified (blkSize)x(blkSize) array rY of luma residual samples,

–	an modified (blkSize)x(blkSize) array rCb of chroma residual samples,

–	an modified (blkSize)x(blkSize) array rCr of chroma residual samples.

The (blkSize)x(blkSize) arrays of residual samples rY, rCb and rCr are modified as follows:

- If cu_transquant_bypass_flag is equal to 1, tThe (blkSize)x(blkSize) arrays of residual samples rY, rCb and rCr with  x = 0..blkSize − 1, y = 0..blkSize − 1 are modified as follows:

- When cu_transquant_bypass_flag is equal to 0, rcb[ x ][ y ] = rcb[ x ][ y ] << 1 and rcr[ x ][ y ] = rcr[ x ][ y ] << 1.

- The following applies:

tmp = rY[ x ][ y ] − ( rCb[ x ][ y ]  >>  1 )
rY[ x ][ y ] = tmp + rCb[ x ][ y ]
rCb[ x ][ y ] = tmp − (  rCr[ x ][ y ]  >>  1 )
rCr[ x ][ y ] = rCb[ x ][ y ] + rCr[ x ][ y ]

- Otherwise (cu_transquant_bypass_flag is equal to 0), the (blkSize)x(blkSize) arrays of residual samples rY, rCb and rCr with  x = 0..blkSize − 1, y = 0..blkSize − 1 are modified as follows:

tmp = rY[ x ][ y ] − rCb[ x ][ y ] 
rY[ x ][ y ] = rY[ x ][ y ] + rCb[ x ][ y ]
rCb[ x ][ y ] = tmp − rCr[ x ][ y ]
rCr[ x ][ y ] = tmp + rCr[ x ][ y ]