| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0256 |
| --- | --- |

| Title: | Non-CE6: Cross-check of removal of parsing dependency in palette-based coding (JCTVC-S0181) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Vadim Seregin 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Email: | vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the cross-verification results of JCTVC-S0181 on removal of parsing dependency in the palette mode. It is confirmed that implementation is consistent with the proposal description and the experiment results are matched with the proponent data.

# Introduction and experimental results

In the current palette mode, a sample can be coded using one of the three modes: INDEX, COPY_ABOVE, or ESCAPE. The escape sample (ESCAPE mode) is coded with INDEX mode having the highest palette index which is equal to the palette size, followed by the escape colour triplet.

In the contribution JCTVC-S0181, it is proposed to include escape coded sample into INDEX or COPY_ABOVE mode with the following modifications:

- Escape sample can be included into the INDEX mode, in this case the run value indicates the number of subsequent escape samples.

- Escape sample can be included into the COPY_ABOVE mode, in this case if the above sample is escape the current sample will be treated as escape too.

- The escape colour triplet values are signalled at the end of the CU, after the whole palette parsing is finished.

- Encoder has been modified to include a rate-distortion optimization in deriving the sample mode and run values. It is similar to the encoder modification proposed in JCTVC-S0156.

The software was received form the proponent and simulation was performed under common test conditions JCTVC-R1015 on heterogeneous Linux cluster, so the encoding and decoding running time can be inaccurate.

The test results are summarized in the below tables, and can be found in more details in the accompanying Excel spreadsheets. The cross-verification results match the results provided by the proponent.

Table1. Proposed method against SCM2.0, lossy configuration

Table2. Proposed method against SCM2.0, lossless configuration