| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0261-v2 |
| --- | --- |

| Title: | Crosscheck of JCTVC-S0065 on IBC encoder improvements for SCM2.0 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chao Pang 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | + 1-858-658-3779 cpang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution provides a crosscheck report of JCTVC-S0065. In the proposal JCTVC-S0065, changes have been made to encoders for Intra block copy. The crosschecked results match those results provided by the proponent.

# Introduction

In the current Intra block copy (Intra BC) mode, sampling-based fast search is used for Intra BC search for local region and hash-based fast search is used for Intra BC search for the other region inside the frame. In JCTVC-S0065, several methods have been proposed to further enhance the Intra BC fast search, including enabling 32x32 Intra BC, different checking order for 2NxN/Nx2N/NxN partition modes, and so on. The software provided by the proponent is compiled and tested on Linux cluster. The simulation is performed under the SCC common test condition.

# Results

In this section, the crosscheck results are presented, and they match the results provided by the proponent.

Table 1 Lossy results

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -5.8% | -5.9% | -5.8% |
| RGB, text & graphics with motion,720p | -2.4% | -2.4% | -2.4% |
| RGB, mixed content, 1440p | -2.1% | -2.0% | -2.1% |
| RGB, mixed content, 1080p | -3.0% | -3.0% | -2.8% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -2.9% | -3.5% | -3.5% |
| YUV, text & graphics with motion,720p | -1.8% | -2.0% | -2.0% |
| YUV, mixed content, 1440p | -1.7% | -1.8% | -1.9% |
| YUV, mixed content, 1080p | -2.5% | -2.4% | -2.8% |
| YUV, Animation, 720p | 0.0% | 0.0% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 81% |  |  |
| Dec Time[%] | 96% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -2.8% | -2.8% | -2.8% |
| RGB, text & graphics with motion,720p | -1.5% | -1.5% | -1.5% |
| RGB, mixed content, 1440p | -1.3% | -1.1% | -1.2% |
| RGB, mixed content, 1080p | -2.0% | -2.0% | -1.8% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.9% | -2.0% | -2.0% |
| YUV, text & graphics with motion,720p | -1.2% | -1.3% | -1.1% |
| YUV, mixed content, 1440p | -0.8% | -0.7% | -0.9% |
| YUV, mixed content, 1080p | -1.6% | -1.5% | -2.0% |
| YUV, Animation, 720p | 0.0% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 97% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -2.0% | -1.9% | -1.9% |
| RGB, text & graphics with motion,720p | -0.7% | -0.6% | -0.6% |
| RGB, mixed content, 1440p | -0.9% | -0.6% | -0.6% |
| RGB, mixed content, 1080p | -0.9% | -0.1% | -0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | -1.5% | -1.4% | -1.4% |
| YUV, text & graphics with motion,720p | -0.5% | -0.1% | -0.6% |
| YUV, mixed content, 1440p | -0.5% | 0.0% | -0.1% |
| YUV, mixed content, 1080p | -0.4% | 0.5% | -0.2% |
| YUV, Animation, 720p | 0.0% | -0.4% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 95% |  |  |

Table 2 Lossless results

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 5.2% | 5.1% | 3.8% | 6.3% |
| RGB, text & graphics with motion,720p | 0.9% | 1.0% | 0.2% | 2.1% |
| RGB, mixed content, 1440p | 1.3% | 1.3% | 1.0% | 1.6% |
| RGB, mixed content, 1080p | 1.5% | 1.5% | 1.5% | 1.5% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 4.1% | 4.0% | 2.2% | 5.0% |
| YUV, text & graphics with motion,720p | 0.9% | 1.1% | 0.2% | 2.2% |
| YUV, mixed content, 1440p | 1.5% | 1.4% | 1.2% | 1.7% |
| YUV, mixed content, 1080p | 1.6% | 1.6% | 1.6% | 1.6% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 86% |  |  |  |
| Dec Time[%] | 100% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 4.0% | 3.3% | 1.6% | 4.4% |
| RGB, text & graphics with motion,720p | 0.2% | 0.6% | 0.1% | 1.7% |
| RGB, mixed content, 1440p | 0.3% | 0.4% | 0.1% | 0.6% |
| RGB, mixed content, 1080p | 0.3% | 0.3% | 0.3% | 0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 3.8% | 3.0% | 1.4% | 4.3% |
| YUV, text & graphics with motion,720p | 0.2% | 0.6% | 0.1% | 1.8% |
| YUV, mixed content, 1440p | 0.4% | 0.4% | 0.1% | 0.7% |
| YUV, mixed content, 1080p | 0.3% | 0.3% | 0.3% | 0.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  |  |
| Dec Time[%] | 100% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 4.3% | 2.9% | 1.3% | 4.8% |
| RGB, text & graphics with motion,720p | 0.2% | 0.3% | 0.1% | 0.9% |
| RGB, mixed content, 1440p | 0.3% | 0.3% | 0.0% | 0.5% |
| RGB, mixed content, 1080p | 0.2% | 0.2% | 0.2% | 0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 3.8% | 2.6% | 1.3% | 4.3% |
| YUV, text & graphics with motion,720p | 0.1% | 0.3% | 0.1% | 1.0% |
| YUV, mixed content, 1440p | 0.2% | 0.3% | 0.0% | 0.5% |
| YUV, mixed content, 1080p | 0.2% | 0.2% | 0.2% | 0.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 100% |  |  |  |