| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0262-v1 |
| --- | --- |

| Title: | Crosscheck of JCTVC-S0172 on unification of IntraBC mode with inter mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chao Pang 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | + 1-858-658-3779 cpang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution provides a crosscheck report of JCTVC-S0172. In the proposal JCTVC-S0172, several changes have been made for the Intra BC and Inter unification. The crosschecked results match those results provided by the proponent.

# Introduction

In JCTVC-S0172, based on CE2 Test 5, the Intra BC merge is changed with different spatial neighbors from HEVC merge candidates. Also, the derived block vector and temporal block vector are added to the merge candidate list. The software provided by the proponent is compiled and tested on Linux cluster. The simulation is performed under the SCC common test condition with one temporal reference frames.

# Results

In this section, the crosscheck results are presented, and they match the results provided by the proponent.

Table 1 Lossy results

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -5.5% | -6.9% | -6.8% |
| RGB, text & graphics with motion,720p | -3.1% | -4.0% | -3.9% |
| RGB, mixed content, 1440p | -2.7% | -2.9% | -3.1% |
| RGB, mixed content, 1080p | -2.4% | -2.9% | -2.9% |
| RGB, Animation, 720p | -0.2% | -0.2% | -0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -3.5% | -5.0% | -4.9% |
| YUV, text & graphics with motion,720p | -2.8% | -3.5% | -3.8% |
| YUV, mixed content, 1440p | -1.9% | -2.6% | -2.8% |
| YUV, mixed content, 1080p | -2.0% | -3.3% | -3.3% |
| YUV, Animation, 720p | -0.4% | -0.5% | -0.6% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 115% |  |  |
| Dec Time[%] | 94% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -5.9% | -7.8% | -7.6% |
| RGB, text & graphics with motion,720p | -2.3% | -3.6% | -3.6% |
| RGB, mixed content, 1440p | -2.0% | -2.9% | -3.1% |
| RGB, mixed content, 1080p | -2.0% | -3.2% | -3.3% |
| RGB, Animation, 720p | 0.0% | -0.2% | -0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -5.3% | -7.1% | -7.2% |
| YUV, text & graphics with motion,720p | -1.8% | -3.3% | -3.4% |
| YUV, mixed content, 1440p | -1.6% | -3.1% | -3.0% |
| YUV, mixed content, 1080p | -1.8% | -4.5% | -4.4% |
| YUV, Animation, 720p | -0.1% | -0.7% | -0.6% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 91% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -5.8% | -7.4% | -7.1% |
| RGB, text & graphics with motion,720p | -1.5% | -2.7% | -2.9% |
| RGB, mixed content, 1440p | -1.6% | -2.4% | -2.6% |
| RGB, mixed content, 1080p | -1.8% | -3.1% | -3.4% |
| RGB, Animation, 720p | 0.0% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -5.5% | -7.1% | -6.9% |
| YUV, text & graphics with motion,720p | -1.6% | -2.5% | -3.3% |
| YUV, mixed content, 1440p | -1.5% | -3.0% | -3.0% |
| YUV, mixed content, 1080p | -1.5% | -4.8% | -6.3% |
| YUV, Animation, 720p | 0.1% | -0.5% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.2% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 93% |  |  |

Table 2 Lossless results

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 3.2% | 3.2% | 2.8% | 3.5% |
| RGB, text & graphics with motion,720p | 0.7% | 0.8% | 0.2% | 1.2% |
| RGB, mixed content, 1440p | 2.0% | 1.7% | 0.4% | 3.0% |
| RGB, mixed content, 1080p | 1.7% | 1.7% | 1.7% | 1.7% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 2.2% | 2.1% | 1.5% | 2.6% |
| YUV, text & graphics with motion,720p | 0.6% | 0.6% | 0.2% | 0.9% |
| YUV, mixed content, 1440p | 1.9% | 1.7% | 0.5% | 2.9% |
| YUV, mixed content, 1080p | 1.4% | 1.4% | 1.4% | 1.4% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 111% |  |  |  |
| Dec Time[%] | 101% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 4.7% | 4.1% | 2.9% | 5.0% |
| RGB, text & graphics with motion,720p | 0.4% | 0.5% | 0.3% | 1.0% |
| RGB, mixed content, 1440p | 0.4% | 0.4% | 0.1% | 0.8% |
| RGB, mixed content, 1080p | 0.4% | 0.4% | 0.4% | 0.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 4.4% | 3.5% | 2.0% | 4.8% |
| YUV, text & graphics with motion,720p | 0.3% | 0.5% | 0.3% | 0.9% |
| YUV, mixed content, 1440p | 0.4% | 0.4% | 0.1% | 0.8% |
| YUV, mixed content, 1080p | 0.4% | 0.4% | 0.4% | 0.4% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 95% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 5.1% | 4.2% | 2.5% | 5.5% |
| RGB, text & graphics with motion,720p | 0.3% | 0.4% | 0.2% | 0.8% |
| RGB, mixed content, 1440p | 0.2% | 0.2% | 0.0% | 0.5% |
| RGB, mixed content, 1080p | 0.2% | 0.2% | 0.2% | 0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 4.5% | 3.8% | 1.9% | 5.0% |
| YUV, text & graphics with motion,720p | 0.2% | 0.4% | 0.2% | 0.9% |
| YUV, mixed content, 1440p | 0.2% | 0.2% | 0.0% | 0.4% |
| YUV, mixed content, 1080p | 0.2% | 0.2% | 0.2% | 0.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 101% |  |  |  |