| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0269 |
| --- | --- |

| Title: | CE6-related: Harmonization of CE6 Tests A4, A5, and A6 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Tzu-Der Chuang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 Rajan Joshi, Wei Pu, Marta Karczewicz, Feng Zou, Vadim Seregin, Joel Sole 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | Shih-Ta Hsiang +886-3-5670766 ext. 24685 {shih-ta.hsiang, peter.chuang, shawmin.lei} @mediatek.com Rajan Joshi 1-858-658-4511 rajanj@qti.qualcomm.com |
| Source: | MediaTek Inc Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes the method for run coding in palette mode after harmonization of related proposals evaluated by CE6 Tests A4, A5, and A6. It reportedly achieves average Luma BD-rate savings of 1.9%, 1.0%, and 0.8% for the AI, RA, LB settings, respectively, with context selection further conditioned on the coded palette index, and achieves average Luma BD-rate savings of 2.0%, 1.0%, and 0.9% for the AI, RA, LB settings, respectively, with context selection further conditioned on the reconstructed palette index for lossy coding YUV, text & graphics with motion, 1080p sequences under the SCC CTCs.

# Introduction

This contribution proposes the method for run coding in palette mode after harmonization of related proposals to run coding evaluated by CE6 Tests A4, A5, and A6 [4-6]. For binarization in Test A. 4 [4], the palette run value in a binary form is represented by its most significant bit (MSB) index in a truncated unary(TU) code followed by its refinement bits in a truncated binary code. In Test A. 6 [6], the palette run value is binarized by using 1-bit TU concatenated with truncated EG-0 code. It turns out that the two methods lead to exactly the same bin string after binarization, as illustrated in Table 1.

For context modeling, both Tests A4 and A5 exploit the current palette index for context selection for bin index 0 in the INDEX_MODE but with different decision thresholds. For the bin index value greater than 0, context modeling in Test A.4 employs a greater bypass threshold with context sharing among two adjacent bins.

The harmonized solution proposed in this contribution is based on CE6 Tests A4 but using the context selection method proposed in Test A.5 for bin index 0 in INDEX_MODE. The proposed method is described in the next section in detail. The proposed modifications to WD can be found in the attached file.

Table 1: Comparison of Methods A.4 and A.5 for binarization

# Proposed method

This contribution proposes the method for run coding in palette mode by using a universal entropy coding scheme. This entropy coding method, previously employed in [3] for coding IBC block vector difference, represents a syntax element in a binary value by sending its most significant bit (MSB) index followed by its refinement bits. The bin string for a binarized syntax value is thus a concatenation of a prefix bin string and a suffix bin string. The prefix string represents the MSB index plus 1, or msb_id_plus1, of a syntax value in a unary code, or a truncated unary code if the maximal syntax value is known. msb_id_plus1 of a unsigned syntax x is given by

msb_id_plus1 =  Floor ( Log2( x ) ) + 1,  	if x > 0; 
                                      0,   	otherwise.

The suffix bin string represents the refinement bits and is present if msb_id_plus1 > 1. The value of refinement bits, denoted by refinement_bits, is coded in a fixed-length binary code with the string length equal to msb_id_plus1 – 1 or truncated binary code if the maximal syntax value is known. The decoded syntax value x is given by

x  =  (1 << ( msb_id_plus1-1)) + refinement_bits,  if msb_id_plus1 > 1;

msb_id_plus1,  otherwise.

For application to run coding in this proposal, we adopt truncated signaling for coding prefix and suffix with the max run value, MaxPaletteRun, equal to the number of the uncoded pixels minus 1. The resulting prefix syntax palette_run_msb_id_plus1 is coded by a truncated unary code with cMax = Floor ( Log2((MaxPaletteRun ) ) + 1. The suffix syntax palette_run_refinement_bits is coded by truncated binary code with cMax = MaxPaletteRun - (1 <<( palette_run_msb_id_plus1-1)).

Context modeling for entropy coding the bin string of binarized msb_id_plus1 of palette_run is conditioned on the bin index and the two separate context sets are employed, corresponding to palette_run_type_flag equal to COPY_ABOVE_MODE and INDEX_MODE, respectively. Inspired by the observation that palette_run is often equal to 0 when palette_run_type_flag is equal to INDEX_MODE and the current palette index is large, context modeling for bin index 0 is further conditioned on the palette index for the INDEX_MODE run type. The context index is assigned to 0 if palette_index =0, 1 if 0 < palette_index <3, or 2 otherwise. Table 2 provides the mapping table for context selection currently employed by our proposal.

Table 2 – Specification of ctxIdx[palette_run_type_flag][ binIdx ]

| binIdx | 0 | 1 | 2 | 3 | 4 | > 4 |
| --- | --- | --- | --- | --- | --- | --- |
| palette_run_type_flag = COPY_ABOVE_MODE | 5 | 6 | 6 | 7 | 7 | bypass |
| palette_run_type_flag = INDEX_MODE | 0,1,2 | 3 | 3 | 4 | 4 | bypass |

# Experimental results

The proposed method has been implemented using reference software HM-15.0+RExt-8.0+SCM-2.0 as the code base. The simulation is conducted according to the common test conditions specified in [2]. The simulation results are summarized in Tables 3 and 4 for lossy and lossless coding, respectively, with context selection further conditioned on the coded index value for the INDEX mode. The additional simulation results are further summarized in Tables 5 and 6 for lossy and lossless coding, respectively, with context selection further conditioned on the reconstructed index value.

The detailed simulation results can be found in the included spread sheets.

Table 3: Lossy BD bitrate results compared with the SCM anchor for lossy coding, conditioned on the coded palette index value

Table 4: Bitrate saving results compared with the SCM anchor for lossless coding, conditioned on the coded palette index value

Table 5: Lossy BD bitrate results compared with the SCM anchor for lossy coding, conditioned on the reconstructed palette index value

Table 6: Bitrate saving results compared with the SCM anchor for lossless coding, conditioned on the coded palette index value

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- S.-T. Hsiang, T.-D. Chuang, S. Lei, “Non-SCCE3:Run coding of the palette index map using a universal entropy coding scheme,” Document of Joint Collaborative Team on Video Coding, JCTVC-R0136, 18th Meeting: Sapporo, Japan, 30 June – 9 July 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” Document of Joint Collaborative Team on Video Coding, JCTVC-R1015, 18th Meeting: Sapporo, Japan, 30 June – 9 July 2014.

- S.-T. Hsiang, T.-D. Chuang, S. Lei, “SCCE1: Test 3.5 - Block vector difference coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-R0133, 18th Meeting: Sapporo, JP, 30 June – 9 July 2014.

- S.-T. Hsiang, T.-D. Chuang, S. Lei, “CE6: Results of Test A.4 on palette run coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-S0163, 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014.

- R. Joshi, M. Karczewicz, W. Pu, V. Seregin, J. Sole , “CE6 subtest A.5: Contexts for run coding in palette mode,” Document of Joint Collaborative Team on Video Coding, JCTVC-S0038, 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014.

- R. Joshi, W. Pu, M. Karczewicz, F. Zou, V. Seregin, J. Sole, “CE6 subtest A.6: Binarization for run coding in palette mode,” Document of Joint Collaborative Team on Video Coding, JCTVC-S0039, 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014.