| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0271 |
| --- | --- |

| Title: | CE5: Cross-verification of JCTVC-S0037 on performance impact of varying maximum palette size |  |  |
| --- | --- | --- | --- |
| Status: | Information |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yuwen He, Yan Ye 9710 Scranton Rd, #250 San Diego, CA 92121, USA | Tel: Email: | + 1-858-210-4830 Xiaoyu.Xiu@InterDigital.com Yan.Ye@InterDigital.com Yuwen.He@InterDigital.com |
| Source: | InterDigital Communications, LLC |  |  |

_____________________________

# Abstract

This document presents the cross-verification results of JCTVC-S0037 on checking the performance impact of varying palette size from Qualcomm. It is verified that all the simulation results are perfectly matched with that provided by the proponent.

# Introduction

In SCM2.0, the maximum palette size is 31. In JCTVC-S0037,  the performance impacts of using maximum palette sizes of 15, 47, and 63 is investigated, while keeping the palette predictor size as the default (=64). The source codes of these methods are provided by proponent. This verification is carried out by using the provided source code.

# Simulation results

The detailed results are presented and bundled in the same archive.

Table 1 BD-rate performance of using maximum palette size 15 and maximum palette predictor size 64

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | -1.1% | -1.1% | -2.2% | -0.5% |
| RGB, text & graphics with motion,720p | -1.9% | -1.5% | -5.1% | -0.1% |
| RGB, mixed content, 1440p | -0.3% | -0.3% | -0.5% | -0.1% |
| RGB, mixed content, 1080p | -0.1% | -0.1% | -0.1% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.7% | -1.9% | -2.8% | -0.6% |
| YUV, text & graphics with motion,720p | -2.1% | -1.7% | -5.4% | -0.1% |
| YUV, mixed content, 1440p | -0.2% | -0.2% | -0.2% | -0.2% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 91% |  |  |  |
| Dec Time[%] | 98% |  |  |  |

Table 2 BD-rate performance of using maximum palette size 47 and maximum palette predictor size 64

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | -0.1% |
| RGB, mixed content, 1080p | -0.1% | -0.1% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | -0.1% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | -0.1% | -0.1% |
| YUV, mixed content, 1080p | 0.0% | -0.1% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 94% |  |  |
| Dec Time[%] | 99% |  |  |

Table 3 BD-rate performance of using maximum palette size 63 and maximum palette predictor size 64

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | -0.1% |
| RGB, mixed content, 1080p | -0.1% | -0.1% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | -0.1% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | -0.1% | -0.1% |
| YUV, mixed content, 1080p | -0.1% | 0.0% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 94% |  |  |
| Dec Time[%] | 99% |  |  |