| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0273 |
| --- | --- |

| Title: | Cross-check of ‘AhG13: Palette and deblocking’ (JCTVC-S0096) by Qualcomm |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Rosewarne | Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au |
|  | V. Kolesnikov Building A, The Park Estate, 5 Talavera Road, Macquarie Park NSW 2113, Australia. | Tel: Email: | +61 2 9805 2973 vladimir.kolesnikov@cisra.canon.com.au |
|  | M. Maeda 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan. | Tel: Email: | +81-3-3758-2111 maeda.mitsuru@canon.co.jp |
| Source: | Canon Information Systems Research Australia, Canon Inc. |  |  |

_____________________________

# Abstract

This contribution provides a cross-check result of contribution JCTVC-S0096 on palette deblocking by Qualcomm. The result reported by the proponent is confirmed in this cross-check.

# Cross check procedure

The source code for JCTVC-S0096 [1] was provided by Qualcomm. Canon compiled the source code using GCC 4.7.1 and the simulation was performed on a Linux cluster under lossy conditions, as defined in [2]. Note that timing results are not comparable due to differences in the simulation environment.

# Analysis

The source code provided by Qualcomm was studied and the following observations were made:

- For a CU configured to use palette mode, the deblocking filter is disabled for samples within the CU for a given edge, regardless of the deblocking filter strength set for samples on the other side of the given edge.  This is the same method as used for lossy CUs.

The normative changes listed above correspond to the description in [1].

# Coding performance measurement

Table 1 shows the lossy simulation result for [1] implemented on top of HM-15.0-RExt8.0-SCM2.0 base software.  The anchor of the provided result is the HM-15.0-RExt8.0-SCM2.0 base software.  The simulation result of the provided source code matches the result lossy reported by the proponent.

Table 1. Lossy simulation result with HM-15.0-RExt8.0-SCM2.0 base software as anchor.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.0% | -0.2% | -0.2% |
| RGB, text & graphics with motion,720p | 0.0% | -0.2% | -0.2% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | -0.1% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | -0.5% | -0.4% |
| YUV, text & graphics with motion,720p | 0.0% | -0.3% | -0.4% |
| YUV, mixed content, 1440p | 0.0% | -0.2% | -0.2% |
| YUV, mixed content, 1080p | 0.0% | -0.1% | -0.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | -0.3% | -0.2% |
| RGB, text & graphics with motion,720p | 0.0% | -0.2% | -0.3% |
| RGB, mixed content, 1440p | 0.0% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | 0.0% | -0.1% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | -0.5% | -0.4% |
| YUV, text & graphics with motion,720p | 0.0% | -0.3% | -0.6% |
| YUV, mixed content, 1440p | 0.0% | -0.3% | -0.3% |
| YUV, mixed content, 1080p | 0.0% | -0.3% | -0.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | -0.2% | -0.2% |
| RGB, text & graphics with motion,720p | -0.2% | -0.2% | -0.3% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | -0.1% | 0.2% | -0.1% |
| RGB, Animation, 720p | -0.1% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.1% | -0.3% | -0.3% |
| YUV, text & graphics with motion,720p | -0.1% | -0.1% | -0.3% |
| YUV, mixed content, 1440p | -0.1% | 0.1% | -0.1% |
| YUV, mixed content, 1080p | -0.2% | -0.8% | -0.5% |
| YUV, Animation, 720p | 0.0% | -0.1% | 0.3% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

# Conclusion

The result provided by the proponent is verified in this cross-check report.

# References

- J. Sole, W. Pu, C. Pang, R. Joshi, V. Seregin, M. Karczewicz, “AhG13: Palette and deblocking” Document of Joint Collaborative Team on Video Coding, JCTVC-S0096, 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” Document of Joint Collaborative Team on Video Coding, JCTVC-R1015, 18th Meeting: Sapporo, Japan, 30 June – 9 July 2014.