| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0285 |
| --- | --- |

| Title: | Cross-verification of JCTVC-S0140: On transform coefficient scaling for adaptive colour transform |  |  |
| --- | --- | --- | --- |
| Status: | Information |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yuwen He, Yan Ye 9710 Scranton Rd, #250 San Diego, CA 92121, USA | Tel: Email: | + 1-858-210-4830 Xiaoyu.Xiu@InterDigital.com Yan.Ye@InterDigital.com Yuwen.He@InterDigital.com |
| Source: | InterDigital Communications, LLC |  |  |

_____________________________

# Abstract

This document presents the cross-verification results of JCTVC-S0140 on transform coefficient scaling for adaptive colour transform by Sharp. It is verified that all the simulation results are perfectly matched with that provided by the proponent.

# Introduction

JCTVC-S0140 proposes to fix the problem of the current adaptive colour transform where negative QP may be used, for which the corresponding transform coefficient scaling operation is undefined. Additionally, two solutions has been proposed to fix this issue: a) disable adaptive colour transform for negative QPs; b) clipping negative QP to zero. The source codes of these methods are provided by proponent. This verification is carried out by using the provided source code.

# Simulation results

The SCM-2.0 and the proposed two solutions are evaluated for the QP parameters {1, 2, 3, 4} and the AI configuration. And the first 16 frames are used in the simulation.

|  | SCM 2.0 |  | Option (a) |  | Option (b) |  |
| --- | --- | --- | --- | --- | --- | --- |
| Quantization parameter | Bitrates | PSNR (G) | Bitrates | PSNR (G) | Bitrates | PSNR (G) |
| 1 | 204865.4700 | 33.6777 | 228095.8500 | 84.9411 | 284879.5200 | 78.8354 |
| 2 | 377669.5800 | 39.7507 | 218379.3000 | 80.4757 | 257264.1600 | 77.0936 |
| 3 | 326218.4100 | 63.2395 | 202108.1400 | 77.1090 | 225076.8900 | 75.0776 |
| 4 | 208964.7000 | 69.4524 | 178521.4200 | 71.6991 | 189562.9800 | 70.6202 |