| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0287 |
| --- | --- |

| Title: | Crosscheck report of JCTVC-S0152 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Keiichi Chono 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 44 431 7659 chono@ct.jp.nec.com |
| Source: | NEC Corp. |  |  |

_____________________________

# Abstract

This contribution provides a software study report on JCTVC-S0152 entitled “Non-CE6: Index Coding Group (ICG) for 8x8 CU of Palette Mode in HEVC SCC”. Through the software study, it was confirmed that software implementation matches with the description.

# Software study results

A patch provided by the proponent was successfully applied to HM-15.0+RExt-8.0+SCM-2.0. It was confirmed that the three macros are defined to enable the features of JCTVC-S0152 as follows.

[Macros]

#define IDX_1D_SEARCH                              1

#define GROUP_2X8                                   1

#define FLAG_COPY_LEFT_ABOVE                     1

[Features]

IDX_1D_SEARCH: Use simplified escape coding proposed in JCTVC-S0150 and a reduced context for index coding mode.

GROUP_2X8: Enable splitting of 8x8 index coding group into 4 subgroups (2x8 size).

FLAG_COPY_LEFT_ABOVE: Introduce flag in each subgroup to indicate whether the RUN equals to 16.

# Reference

- W. Wang et al., "Non-CE6: Index Coding Group (ICG) for 8x8 CU of Palette Mode in HEVC SCC", JCTVC-S0152, JCT-VC of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014.

# Patent rights declaration

NEC Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).