| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0292_r1 |
| --- | --- |

| Title: | BoG report on CE6-related palette mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Robert Cohen Yu-Wen Huang | Emails: | cohen@merl.com yuwen.huang@mediatek.com |
| Source: | CE6-related BoG |  |  |

_____________________________

# Abstract

This document contains the notes from the BoG meeting on CE6-related topics (palette mode improvements).

# Introduction

The mandate of this BoG is to review remaining (approximately 35) CE6-related proposals that were not discussed during the JCT-VC session on Friday October 17.

The BoG met on Saturday October 18, 2014 from 0820 – 19:30 and Sunday October 19, 2014 from 0830 – 2015.

# BoG meeting notes

JCTVC-S0043 Non-CE6: Delta QP signalling for palette [J. Sole, W. Pu, R. Joshi, V. Seregin, M. Karczewicz (Qualcomm)]

The palette adopted at the Sapporo meeting quantizes escape values with the coefficient quantization parameter (QP). However, there is no mechanism to vary this QP for the CUs coded with palette, so an encoder has to work at constant QP as long as CUs are coded in palette mode.

Palette specifies a CU-level escape flag that indicates whether escape pixels exist in the current CU. This document, as in JCTVC-R0237, proposes to use the CU escape flag as entry point for the syntax elements that adjust the QP, i.e., like the cbf flags in residue coding. When CU-level escape flag is on, then delta QP can be signalled. In this way, encoders have the flexibility to use palette as well as adjust the QP. Furthermore, this change allows adjusting the QP for palette deblocking and permits to use palette as a PCM mode with a tunable quantization step.

Source code was provided, but no simulation results were provided because the common test conditions do not use the QP step size adjustment.

There was a question on whether a sufficient number of palette-coded blocks would use escape mode, and if there are only 1 or 2 escape pixels in a CU, this proposal would introduce additional overhead.

There was a comment on propagating the earlier-used QP values in the slice.

There was a suggestion on enabling rate control to show the performance of this proposal.

It was noted that the proposed syntax needs to be modified to check to see whether cu_qp_delta_abs has already been signalled for the CU, and if so, use that cu_qp_delta_abs instead of signalling another one. Therefore, the current proposal also needs to add a check for !IsCuQpDeltaCoded for luma and a check for !IsCuChromaQpOffsetCoded for chroma.

A participant commented that they would support this proposal if the syntax and software were corrected.

The proponents will update their proposal and provide simulation results with a delta QP of zero to determine the impact on overhead. They will also look at the rate control to see if they can generate results with nonzero delta QP.

Recommendation: Revisit when updated syntax and simulation results are available.

JCTVC-S0047 Non-CE6: Modifications of copy-left and copy-above modes in index coding [J.-S. Tu, C.-L. Lin, C.-H. Hung, C.-C. Lin, Y.-J. Chang (ITRI)]

In past meetings, several palette redundancy removal methods are provided. This proposal presents redundancy removal methods for the palette coding; in certain cases, the index value of current pixel can be deduced impossible to have the same index value of its neighbouring pixels. Therefore the index coding bits can be saved. This proposal also presents a copy-above mode modification method. The reported gain is 0.3% for 1080p text and graphics test sequences for full frame intra BC test condition at All Intra lossy case.

Three methods are proposed to reduce the number of runs or palette indices that need to be signalled.

A fourth supplemental result, not yet cross-checked, was also proposed (0.4% gain).

For the three methods, they do not have complete simulation results on their individual performance.

It was asked whether the gains are sufficient to propose adopting at this meeting, or whether further study in a CE should be done. The proponent said that study in a CE would be appropriate, and results for the individual proposed methods could be provided.

This method currently checks if RC ≥ 3. It was commented that JCTVC-S0156 proposes (non-normative) a parameter to select run mode, and that could affect whether the RC ≥ 3 would need to be changed accordingly.

It was asked whether parsing dependencies are introduced, as the neighboring reconstructed pixel values need to be checked. The proponent clarified that the dependency is on parsing the index, not on reconstructing any pixel values. Therefore it is asserted that parsing dependencies are not an issue.

Recommendation: Further study in a CE with Method 3 of JCTVC-S0110, and to study the gains of the individual methods.

JCTVC-S0260 Cross-check of Non-CE6: Modifications of copy-left and copy-above modes in index coding (JCTVC-S0047) [S.H. Kim, A.Segall (Sharp)] [late] [miss]

JCTVC-S0051 Non-CE8: Multi-colour intra mode for screen content coding [Y.-J. Chang, C.-C. Lin, C.-L. Lin, J.-S. Tu, C.-H. Hung (ITRI)]

JCTVC-S0050 proposed a combined single color and bi-color mode, which was a combination of two CE8 tests. This contribution proposes a multi-color intra mode, which is an extension of JCTVC-S0050. The multi-color intra mode includes single color mode, bi-color mode and the color modes with more colors. The colors of the multi-color intra mode are selected from the neighboring reference sources. For the modes with the number of colors over one, an index map is established to represent each pixel in the CU by an index. Compared to the anchor SCM2.0, the proposed mode reportedly shows that 1.0%, 1.0%, 0.8% and 0.8% BD-rate savings under All-Intra Lossy-coding conditions are achieved for the classes of “RGB, text & graphics with motion,1080p”, “RGB, text & graphics with motion,720p”, “YUV, text & graphics with motion,1080p” and “YUV, text & graphics with motion,720p”, respectively.

This was also briefly visited yesterday in the CE8 discussions.

It was clarified that the main part of this proposal is a new mode, as discussed yesterday. The proponent said that this could be viewed as the same as palette mode, but using a different method for generating the major colors. However, yesterday, the decision was that adding a new mode for these amount of gains was not desirable.

The proponents said they are currently working on incorporating this proposed method into the existing palette mode, thus not as an additional mode, and that would be of interest for study in a CE. It was commented that the signalling would be in palette mode, but the encoder would still need to perform an additional R-D check to decide whether to use this mode.

It was mentioned JCTVC-S0098 included a single-color mode incorporated into palette, up to 0.8% gain.

It was commented that because results aren’t available yet for their supplemental results, there isn’t sufficient information to recommend study in a CE. Later in the day, the proponents said that their supplemental results, not yet cross-checked, are available and will be uploaded in a revision of this document shortly.

It was commented that similar gains could be achieved using encoder-only methods, such as including a check for, e.g. one color or two colors in the existing palette mode.

Recommendation: Further study, but not necessarily in a CE.

JCTVC-S0209 CE8-related: Crosscheck for Multi-colour intra mode for screen content coding (JCTVC-S0051) [Y.-W. Chen (MediaTek)] [late] [miss]

JCTVC-S0052 Non-CE6: Escape coded pixel prediction using previous escape coded pixels for palette based coding [J. Ye, J. Zhu (Fujitsu)]

Proposals JCTVC-S0052, JCTVC-S0053, and JCTVC-S0054 were combined into one presentation.

In JCTVC-S0052, escape coded pixels are predicted using escape coded pixels those occurred in previous palette-coded CUs. There is a CU-level flag to indicate existence of escape coded pixel prediction mode in current CU. If the prediction exists, for each escape pixel, there is a flag to indicate current escape coded pixel is predicted or not. For those escape coded pixels which are predicted, there is an index to indicate which element of predictor is its prediction.

There are 2 tests. One is using escape flag to indicate escape mode. Reported test results show gain of 3.4% and 5.1% on TGM RGB and YUV (AI), 2.6% and 3.1% on TGM RGB and YUV (RA), 2.5% and 2.8% on TGM RGB and YUV (LB) for lossless coding on top of SCM2 anchor when maximum escape predictor size is 128;

The other is use max index value to indicate escape mode. Reported test results show gain of 2.0% and 3.4% on TGM RGB and YUV (AI), 1.6% and 2.1% on TGM RGB and YUV (RA), 1.6% and 1.9% on TGM RGB and YUV (LB) for lossless coding on top of SCM2 anchor when maximum escape predictor size is 128.

The size of the predictor array was tested using values of 64 and 128.

In JCTVC-S0053, escape coded pixels are predicted using palette and escape coded pixels those occurred in previous palette-coded CUs. There is a CU-level flag to indicate existence of escape coded pixel prediction mode in current CU. If the prediction exists, for each escape pixel, use two flags to indicate current escape coded pixel is predicted from palette predictor, escape predictor or not predicted. For those escape coded pixels which are predicted, there is an index to indicate which element of predictor is its prediction.

There are 2 tests. One is using escape flag to indicate escape mode. Reported test results show gain of 2.6% and 3.8% on TGM RGB and YUV (AI), 1.8% and 2.1% on TGM RGB and YUV (RA), 1.7% and 1.9% on TGM RGB and YUV (LB) for lossless coding on top of SCM2 anchor when maximum escape predictor size is 64 and maximum palette predictor size is 64;

The other is use max index value to indicate escape mode. Reported test results show gain of 1.3 % and 2.3% on TGM RGB and YUV (AI), 0.9% and 1.3% on TGM RGB and YUV (RA), 0.9% and 1.2% on TGM RGB and YUV (LB) for lossless coding on top of SCM2 anchor when maximum escape predictor size is 64 and maximum palette predictor size is 64.

This proposal doesn’t code the differences, it copies the differences, in contrast to CE6 Test B.5, which codes the residuals.

In JCTVC-S0054, escape coded pixels are predicted using previous palette. There is a CU-level flag to indicate existence of escape coded pixel prediction mode in current CU. If the prediction exists, for each escape pixel, there is a flag to indicate current escape coded pixel is predicted or not. For those escape coded pixels which are predicted, there is an index to indicate which element of predictor is its prediction.

There are 2 tests. One is using escape flag to indicate escape mode. Reported test results show gain of 2.1% and 3.4.% on TGM RGB and YUV (AI), 1.2% and 1.5% on TGM RGB and YUV (RA), 1.0% and 1.2% on TGM RGB and YUV (LB) for lossless coding on top of SCM2 anchor when maximum palette predictor size is 128. And, gain of 1.3% and 1.4.% on TGM RGB and YUV (AI), 0.8% and 0.7% on TGM RGB and YUV (RA), 0.8% and 0.6% on TGM RGB and YUV (LB) for lossless coding on top of SCM2 anchor is reported when maximum palette predictor size is 64.

The other is use max index value to indicate escape mode. Reported test results show gain of 0.8% and 2.1% on TGM RGB and YUV (AI), 0.3% and 0.7% on TGM RGB and YUV (RA), 0.2% and 0.5% on TGM RGB and YUV (LB) for lossless coding on top of SCM2 anchor when maximum palette predictor size is 128. And, small gain is reported when maximum palette predictor size is 64.

Summary of the three proposals:

JCTVC-S0052: Escape coded pixels are predicted by escape coded pixels in previous CUs.

Gains for all-intra RGB for method in which the escape mode uses a flag: Lossy AI gains range from 0.1% to 0.4%; lossless range from 0.4% to 5.1%.

Gains when escape mode uses an index: lossy from 0.0% to 0.2%; lossless from 0.2% to 3.4%.

Additional results using a smaller predictor size (64 as opposed to 128) were presented, lossless gains were up to 3.7%.

JCTVC-S0053: Escape coded pixels are predicted by palette and escape coded pixels in previous CUs

Corresponding gains (MAX_PLT_PRED_SIZE = 128) are:

Using a flag: lossy: 0.0% to 0.4%; lossless: 0.3% to 3.8%

Using index: lossy: 0.0% to 0.1%; lossless: 0.1% to 2.3%

JCTVC-S0054: Escape coded pixels are predicted by palette in previous CUs.

Using a flag: lossy: Not tested; lossless: 0.2% to 3.4%

Using index: lossy: Not tested; lossless: 0.0% to 2.1%

Additional results for MAX_PLT_PRED_SIZE =64 were included in an updated document (lossless gains up to 2.4%).

-----------------------------

Summary of lossless gains for MAX_PLT_PRED_SIZE=64:

JCTVC-S0052: 0.1% to 2.4%

JCTVC-S0053: 0.1% to 2.3%

JCTVC-S0054: 0.0% to 0.2%

-----------------------------

There was a question on why combined palette/escape prediction performs lower than just prediction via escape. Answer is that there are two flags used for the combined palette/escape prediction.

It was commented that JCTVC-S0053 is similar to an escape coding related proposal in CE6 B.5, with some differences, e.g. CE6 B.5 used a predictor size of 32, whereas this proposal uses 64 or 128. CE6 Test B.5 had slightly lower gains. Because the current plans are to include CE6 Test B.5 in a CE, this proposal would be suitable for inclusion in that CE as well.

Recommendation: Include this proposal in the CE that’s related to CE6 Test B.5.

JCTVC-S0169 Non-CE6: Cross check of JCTVC-S0052 on escape coded pixel prediction using previous escape coded pixels for palette based coding [S.-T. Hsiang (MediaTek)] [late] [miss]

JCTVC-S0053 Non-CE6: Escape pixel prediction using previous escape coded pixels and palette for palette based coding [J. Ye, J. Zhu (Fujitsu)]

See notes for JCTVC-S0052.

JCTVC-S0170 Non-CE6: Cross check of JCTVC-S0053 on escape pixel prediction using previous escape coded pixels and palette for palette based coding [S.-T. Hsiang (MediaTek)] [late] [miss]

JCTVC-S0054 Non-CE6: Escape coded pixel prediction using previous palette for palette based coding [J. Ye, J. Zhu (Fujitsu)]

See notes for JCTVC-S0052.

JCTVC-S0246 Non-CE6: Cross-check of S0054 (Escape coded pixel prediction using previous palette for palette based coding) [M. Xu, W. Wang, Z. Ma, H. Yu (Huawei USA R&D)] [late] [miss]

JCTVC-S0055 Non-CE6: Escape colour signalling [C. -C Lin, C.-L. Lin, Y.-J. Chang, J.-S. Tu, C.-H. Hung (ITRI)]

This contribution proposes escape colour signalling methods for the palette based coding. Escape colours are signalled by the copy above run mode. A CU-level flag is also proposed to enable/disable the proposed signalling method. Compared with the SCM 2.0 full frame IBC anchor, the proposed method can achieve total bit-rate savings for lossless coding conditions up to 1.0%, 0.2%, 0.2%, 0.1% for lossless AI for R component excluding categories of animation and camera captured.

Method 1 moves the escape colour signalling from copy left mode to copy above mode (only when escape present flag is on). Pixel-level flag indicates whether it is escape mode or copy above mode.

Method 2 Copy above run value starts from 1, so the pixel-level flag isn’t needed.

Method 3 also uses a CU level flag to indicate escape signalling change (copy left mode/copy above mode)

Summary of results:

Test 1: Method 1 + Method 3: Lossless RGB all-intra gains: 0.1 to 1.0%, for YCbCr: 0.0% to 0.8%

Test 2: Method 2 + Method 3: Lossless RGB 0.1% to 1.0%; YCbCr: 0.1% to 0.8%

The proponent asks for adoption of Test 2 (Method 2 + Method 3). (Note that Method 2 is a modification of Method 1).

It was commented that this is similar to using a flag for escape in JCTVC-S0052/0053/0054. This proposal, however, signals in copy-above mode. The proponent clarified that 52/53/54 is a pixel-level flag, but it signals before the palette mode flag (to choose copy-above or index mode). In this proposal, the escape signalling is only signalling copy above mode, and there’s a CU-level flag to switch to the conventional mode.

It was commented that this proposal deviates from the current design, with respect to indication of the escape color.

Recommendation: A signaling method was recommended under the “Combined discussion of JCTVC-S0115/S0150/S0156/S0157/S0181/S0258” (under JCTVC-S0258), therefore no action.

JCTVC-S0214 Cross check non-CE6: Escape colour signalling [W. Pu (Qualcomm)] [late] [miss]

JCTVC-S0060 Non-CE6: Modified unpredicted palette entries coding in palette based coding [Z. Wang, J. Zhu (Fujitsu)]

Modification of unpredicted palette entry coding is presented. A flag is used to indicate whether the difference between the current palette entry and previous one or original value is coded. The process of encoding and decoding is proposed. Reported test results show gain of 0.5% and 0.7% on TGM RGB and YUV (AI), 0.1% and 0.2% on TGM RGB and YUV (RA), 0.1% and 0.2% on TGM RGB and YUV (LB) for lossless coding on top of SCM2 anchor.

This proposal comprises 3 steps:

- sort (encoder-side only) the palette depend on the value of the first color component

- reorder the palette (encoder side only)

- code the non-predicted palette entries, using one method to code the first component and another method for coding the remaining two components.

Lossy all-intra gains: 0.0% to 0.1%

Lossless all-intra gains: 0.1% to 0.7%

It was asked if this proposal replaces the current CABAC coding table for reuse flags. Answer: This proposal doesn’t change that.

The encoder/decoder run-time ratios were not reliable in the reported results and in the cross-check results. It was asked whether the encoder-side sorting introduces much complexity. Another person commented that the complexity increase is non-trivial. The proponent stated that the encoder implementation was not optimized. The proponent asserts that the complexity of the sorting part decreases.

Recommendation: No action

JCTVC-S0247 Non-CE6: Cross-check of S0060 (Modified unpredicted palette entries coding in palette based coding) [M. Xu, W. Wang, Z. Ma, H. Yu (Huawei USA R&D)] [late] [miss]

JCTVC-S0064 Non-CE6: Last run flag for Palette mode [G. Laroche, T. Poirier, C. Gisquet, P. Onno (Canon)]

This contribution is about a modification of the run coding in the current design of the palette coding mode. The SCM2.0 design of the run coding is modified in order to insert a flag after the palette_run > 2 flag for signalling if the current run is the last run for the processed CU. It is reported that the results of this modification brings an average BDR gain of -0.3% for AI, -0.2 % for RA and -0.2% for LDB for the lossy case on Screen Content sequences.

When the last run flag is 1, then the remainder of the CU uses the same mode (copy above or copy left) (in contrast to signalling the actual number of elements included in the run, as in the current syntax).

RGB all-intra lossy gains: 0.2% to 0.6%. RA: 0.0% to 0.3%; LB: 0.1% to 0.4%

The RA and LB results in the presentation differ from those in the document. The proponent will correct and upload the presentation.

The proponent also tested this method in CE6 A.6 configuration 1. The truncation part of the truncated Golomb code is replaced with this proposed last run flag. Gains averaged to about 0.0% for all-intra.

It was commented that JCTVC-S0111 is a related competing method. It was asserted that the truncated code and the method in JCTVC-S0111 could be combined, and the gains would be additive.

See JCTVC-S0111 for continued discussion of this proposal.

JCTVC-S0267 Cross-verification of JCTVC-S0064 on Last run flag for Palette mode [X. Xiu, Y. He, Y. Ye (InterDigital)] [late] [miss]

JCTVC-S0223 Cross check Non-CE6: Last run flag for Palette mode [W. Pu (Qualcomm)] [late] [miss]

JCTVC-S0066 Non-CE6: Palette encoder improvements for SCM2.0 [C. Gisquet, G. Laroche, P. Onno (Canon)]

This contribution is proposing three encoder improvements related to Palette coding mode in the SCM2.0 without introducing any normative change. It is reported that the proposed encoder changes enable decreasing the encoding runtime ratio to 97% compared to SCM2 while keeping the same BDR performance. It is furthermore asserted that the improvement is bigger in encoders where IBC has reduced runtime.

The three changes are:

- The process determining palette elements is terminated if the proportion of escape-coded pixels exceeds a threshold.

- If the current original pixel is identical to the previous original pixel, then the index associated with the previous pixel is reused.

- Palette sharing is evaluated only if the current best mode for coding the CU is palette mode and if a large proportion of the palette elements (in non-shared mode) are predicted.

The cross-check is still running. It was noted that the cross-check run-time ratios will not be reliable.

This is an encoder-only change.

It was commented that the comparisons for method (2) on average may reduce complexity, but worst case may increase complexity.

Results are not available for the individual methods.

Recommendation: No action

JCTVC-S0067 Combination of several encoder improvements for SCM2.0 [G. Laroche, C. Gisquet, T. Poirier, P. Onno (Canon)]

The palette-related improvements for this proposal are identical to JCTVC-S0066. The other improvements are not related to palette (e.g. IBC-related).

Recommendation: Discuss the other improvements in the JCT-VC track or other BoG given the wider scope of this proposal.

JCTVC-S0282 	Crosscheck of JCTVC-S0067 [F. Zhou (Qualcomm)] [late] [miss]

JCTVC-S0278 Cross check Non-CE6: Palette encoder improvements for SCM2.0 [W. Pu (Qualcomm)] [late] [miss]

JCTVC-S0079 Non-CE6: Cross-CU palette colour index prediction [Y.-C. Sun, J. Kim, T.-D. Chuang, Y.-W. Chen, S. Liu, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-S0114 is included in this discussion.

For JCTVC-S0079: In JCTVC-P0093, it was proposed for palette mode to use the nearest row of the above CU or the nearest column of the left CU to predict colour indexes of the current CU. In this proposal, the method is ported to SCM-2.0 and evaluated again. Experiments show that, compared with SCM-2.0,2.5%, 1.3%, and 1.0% BD-rate savings are shown for “YUV, text & graphics with motion, 1080p sequences” under AI, RA, and LB, respectively.

For JCTVC-S0114: This contribution proposes enabling ‘Copy Above’ prediction mode for the first line of the CU when it is coded as Palette mode. The index of neighbouring pixels of the CU is used as index predictor and the indices are derived with the palette of the current CU. It shows maximum 2.5%, 2.5%, 2.5% gain and average 0.6%, 0.7%, 0.7% gain for All Intra configuration. Maximum 1.3%, 1.5%, 1.4% and average 0.4%, 0.5%, 0.5% gain for Random Access, Maximum 0.9%, 1.2%, 1.2% and average 0.3%, 0.4%, 0.3% gain for Low Delay cases respectively. This contribution presents 4 variations of the method to decide the indices of neighbouring pixels.

A revised version of JCTVC-S0079 document with complete simulation results should be uploaded shortly.

Neighboring palette indices are used to predict the palette indices of the current CU.

JCTVC-S0079: Predicts the colour index from row from CU above or column from CU to the left, and applies redundancy index removal on the first row or column of the current CU. All-intra lossy gains: 0.3% to 2.5%; lossless gains: 0.0% to 1.1%

JCTVC-S0114: Same as JTVC-S0079, except that redundancy index removal is not applied on the first row or first column. All intra lossy gains: 0.3% to 2.5%; lossless gains: 0.0% to 1.1%

Some simplifications for implementing the index conversion (from pixel to index) are also proposed:

- Infer NCP indices as 0 (similar to JCTVC-S0155). Gains of [this method + S0079]: 0.0% to 0.3%

- Reduce number of colors that are searched (from the full 32 to the first 8). Gains of [this method + S0079]: 0.2% to 2.3%

- Search only for the first color component: Gains of [this method + S0079]: 0.3% to 2.4%

- Directly copy the pixel value of NCP for the current pixel, instead of converting the NCP into palette indices. For example, if copy-above mode is used, instead of converting the pixel to index, the pixel from above is copied. Gains of [this method + S0079]: 0.1% to 1.9%

Simulation results (run-time ratios not accurate) for each of the above simplifications on top of JCTVC-S0079 were provided (see above for lossy all-intra gains).

A participant said that it would not be desirable to incur the parsing dependency from the JCTVC-S0079 and JCTVC-S0114 main parts of the proposal, especially if the decoder has corrupted reconstructed pixel values (thus mapping to the wrong index value), e.g. due to loss. Other participants said that this is not a parsing issue, it’s related to error propagation as would be the case for the existing modes. The proponent asserted that there’s no parsing dependency. A participant said that introducing these copy left and copy above methods may limit the ways that the decoder can be implemented, e.g. can’t be parallelized on a block level (e.g., if one has to wait for reconstructed pixels to be available). That participant said more study of this may be needed in a CE.

Another question was what would the gains of simplification (1) be on top of the anchor (without JCTVC-S0079). Participants said that this would likely be similar to JCTVC-S0155).

The proponent of JCTVC-S0120 implemented something similar to JCTVC-S0079 (two lines instead of one line) on top of CE6 Test C.3 (proposes copy previous row).

Recommendation: Further study in a CE, to answer the questions on parallelization and the performance of simplification method (1). Also see the discussion on JCTVC-S0155.

JCTVC-S0099 Non-CE6: Syntax redundancy removal for palette mode with one index value [P. Lai, J. Kim, S. Liu, S. Lei (MediaTek)]

For palette mode, it is possible that for all samples, their palette indices are all the same (e.g. all zeros), such as there is only one colour in the current CU’s palette. This contribution proposes that, for palette mode with one index value (e.g. zero), omit signalling the syntax elements related to the palette index map, and derive the unique palette index value (e.g., zero).

All Intra lossy gains: 0.0% to 0.1%; -0.1 to 0.1 for RA; -0.1% to 0.3% for LB

JCTVC-S0173 Part 1 is a superset of what’s proposed in JCTVC-S0099 (no text was available yet for JCTVC-S0099).

JCTVC-S0110 proposes a similar method.

See the discussion on JCTVC-S0173 for further discussion of this proposal.

JCTVC-S0105 CE6-related: Syntax fixes for zero palette in palette coding [K. Zhang, T.-D. Chuang, S. Liu, J. An, X. Zhang, S. Lei (MediaTek)]

There are two problems in the current syntax design for palette coding. First, the palette of the current CU may be coded in a sharing mode even when the palette size of the previous CU is zero. Second, there is no valid palette index for coding pixels in a CU when the palette size of the current CU is zero and the CU-level escape flag is 0. For solving the first problem of syntax redundancy, it is proposed to infer the palette_sharing_flag as 0 if the palette size of last coded palette is 0. For solving the second problem that can lead to decoder crash, it is proposed to infer palette_escape_val_present_flag as 1 if the palette size of current palette CU is 0. It also includes signalling value of pixels as PCM. The results reportedly show negligible coding performance change.

Fix 1 is more of a clean-up (syntax redundancy removal), whereas Fix 2 is a bug fix, without which the decoder could crash.

It was commented that if there’s no coding performance improvement for Fix 1, then don’t adopt it.

Fix 2 is the same as Modification 1 in JCTVC-S0173 and Method 1.2 of JCTVC-S0110.

Alternative method to Fix 2: If the palette size is zero, then use pcm mode to code the block. Negligible change on coding performance reported. It was commented that the encoder could choose to use pcm when the palette size is zero, instead of modifying the decoder syntax in this proposed alternative method.

Recommendation: Adopt Fix 2 of JCTVC-S0105/Modification 1 of JCTVC-S0173/the part of Method 1.2 of JCTVC-S0110 that is identical to these methods (infer palette_escape_val_present_flag as 1 if the palette size of current palette CU is 0). Offline discussion on whether to include the additional syntax changes of JCTVC-S0110 Method 1.2 is occurring now. For Fix 1, no action.

Continued discussion of S0099, S0110 and S0173:

S0099 is the same as second part of modification 1 in S0173 and is a subset of Method 1.2 of S0110: palette_mode and palette_run will not be signalled when there is only one color in the CU due to one of two possibilities: 1) there is only one color in the palette table and there is no escape color, or 2) the palette table is empty.

Syntax:

Palette_coding( x0, y0, nCbS ) {

…

if(indexMax>0 && scanPos > = nCbS && palette_mode[xC_prev][yC_prev] ! = COPY_ABOVE )

palette_mode[ xC ][ yC ]

…

if( indexMax>0 && palette_mode[xC][yC] ! = ESCAPE ) {

palette_run

runPos = 0

runMode = palette_mode[ xC ][ yC ]

…

Recommendation: Adopt the above syntax changes shown above (From S0099/S0110/subset of S0173). In-meeting offline discussion (expected to be completed by Sunday) will be done on whether to recommend for adoption the additional parts of S0110 (affecting palette_transpose_flag).

JCTVC-S0280 Crosscheck of JCTVC-S0105 [F. Zou (Qualcomm)] [late] [miss]

JCTVC-S0108 Non-CE6: Improvement On Palette Sharing Mode [W. Pu, M. Karczewicz, R. Joshi, F. Zou, V. Seregin, J. Sole (Qualcomm)]

The palette mode adopted at the Sapporo meeting contains a CU level syntax element palette_share_flag which specifies the palette entry signaling methods. If this flag equals to 1, the current CU’s palette is derived from previousPaletteSize and previousPaletteEntries without additional signaling. In this contribution, it is proposed to extend the palette_share_flag to exploit the additional coding gain when all reconstructed pixels within the current CU share the same color. It is reported that the proposed method achieves (0.3%, 0.3%, 0.3%) and (0.3%, 0.2%, 0.2%) BD-rate reduction for AI RGB text & graphics with motion 1080p and AI YUV text & graphics with motion 1080p, respectively, under SCC common test conditions.

The semantics of the palette_share_flag is modified, and the signalling of the palette_transpose_flag is moved to after the palette_share_flag is signalled, so that a check on the value of the palette_share_flag can be done to determine whether to signal the palette_transpose_flag and subsequent syntax.

All-intra lossy gains: 0.0% to 0.3%; lossless: 0.0% to 0.2%.

A participant questioned the benefit of doing this.

It was commented by more than one participant that instead of modifying the palette_share_flag, one could introduce a new flag to indicate that all of the pixels have the same color as previousPaletteEntries[ cIdx ][ 0 ], thus keeping palette_share_flag unmodified. A participant said it would be of interest to study these modifications separately. It was commented that the CE8 test of using single color using the palette entry as a candidate would perform similarly as this method.

It was noted that this proposal introduces an extra R-D check for determining whether the modified palette_share_flag is 0 or not.

Recommendation: Further study, not necessarily in a CE.

JCTVC-S0266 Non-CE6: cross-check of improvement on palette sharing mode (JCTVC-S0108) [B. Li, J. Xu (Microsoft)] [late]

JCTVC-S0110 Non-CE6: Syntax Redundancy Fixes for Palette Mode [W. Pu, M. Karczewicz, V. Seregin, F. Zou, R. Joshi, J. Sole (Qualcomm)]

Note: Continued discussion of this proposal and JCTVC-S0047, S0099, S0105, and S0173 are under JCTVC-S0105.

This document proposes three syntax redundancy fixes related to palette mode.

First, the palette mode adopted at the Sapporo meeting contains a CU level syntax element palette_escape_val_present_flag which specifies whether escape pixel may exist in the current CU or not. If escape pixel does not exist and the palette size equals to 1, it can be implicitly inferred that all of the pixels in the current CU share the same color as the one in the palette while in the current design, redundant syntax elements, such as palette_mode, palette_run, are still required to be coded into the bitstream. This document proposes to simplify the codec by removing the two syntax elements when such case happens.

Second, in the scenario when palette size equals to zero, i.e. all of the pixels in the current CU are escape pixels, the pallet syntax elements except palette_escape_val can be bypassed. Alternatively, a normative restriction can be imposed to ensure that when palette mode is used, the palette size is always greater than zero.

Third, the palette mode contains an implicit restriction to the bitstream to improve coding efficiency. Specifically, an INDEX run mode will always select the longest possible run. In another words, an INDEX run mode cannot be followed by a pixel which has the same index. This document reports a redundancy associated with this restriction and proposes a method to fix it. It is reported that (0.3%, 0.3%, 0.3%) and (0.4%, 0.3%, 0.3%) BD-rate reduction can be achieved for AI RGB text & graphics with motion1080p and AI YUV text & graphics with motion 1080p.

Method 1.1 (note: not section number): Don’t allow palette_size == 0

Method 1.2: When the palette_size == 0, don’t signal the remaining palette-related syntax such as palette_transpose_flag, etc.

Method 2: When palette_size == 1 and palette_escape_val_present_flag == 0, then the whole CU is composed of one color, so some additional palette-related syntax need not be signalled.

Method 3: an INDEX run mode will always select the longest possible run. Because a run mode cannot be followed by a pixel having the same index, some improvement in coding efficiency is possible.

The all-intra lossy gains for Methods 1.1, 1.2, and 2 have essentially no effect on BD-Rate.

For Method 3, all-intra lossy gains are 0.0% to 0.4%.

It was commented that if the palette isn’t allowed to be empty (Method 1), then a coding penalty may be incurred. The proponent said that by disallowing an empty palette, the types of checks that other proposals make to handle palette_size == 0 do not have to be performed. It was commented that if the coding efficiency of the escape color is improved, then having an all-escape block may not incur a significant coding penalty. Another proponent prefers allowing an all-escape block.

It was commented that Section 2.2 in JCTVC-S0047 also addresses the same aspects as Method 3.

Recommendations:

For Method 1.1, no action.

For Method 1.2 (part of this adopt, rest is offline in-meeting discussion), see discussions “Continued discussion of S0099, S0110 and S0173” after JCTVC-S0105.

For Method 2, similar recommendation as Method 1.2 (partial adoption would occur if the syntax shown in the continued discussions is adopted, and the rest would be discussed offline in-meeting). Finish review in JCT-VC session when text is confirmed.

For Method 3, further study in a CE with JCTVC-S0047.

JCTVC-S0217 Non-CE6: cross-check of JCTVC-S0110 on Syntax Redundancy Fixes for Palette Mode [Y. He, X. Xiu, Y. Ye (InterDigital)] [late]

JCTVC-S0111 Non-CE6: Improvement On Palette Run Coding [W. Pu, R. Joshi, M. Karczewicz, F. Zou, V. Seregin, J. Sole (Qualcomm)]

The palette mode adopted at the Sapporo meeting uses coefficient coding method to code the palette_run syntax element, as in SCC draft 1. CE6 Test A.4, A.5, and A.6 were established to study alternative CABAC context design and binarization methods. In this contribution, an improvement on palette_run binarization is proposed, which achieves 1.7% and 2.1% BD-rate redunction for AI RGB text & graphics with motion 1080p and AI YUV text & graphics with motion 1080p, respectively, compared with SCM2.0 anchor.

Proposal: Allocate a shorter codeword for the ‘run-to-the-end’ case.

Method A: Treats palette_run == 3 as a special case, to represent ‘run-to-the-end’ (change in semantics (on top of SCM 2.0 and on top of CE6 Test A.6 configurations)

Method B: If the prefix is truncated, then insert one flag to indicate ‘run-to-the-end’ (change in binarization). Note that this method doesn’t apply to SCM 2.0, it’s on top of CE6 Test A.6 configurations.

Gains for Method A over SCM 2.0: lossy all-intra: 0.1% to 0.6%

Gains for Method B over CE6 Test A.6 configurations:

The method in JCTVC-S0064 inserts a flag after the palette_run > 2 flag for signalling if the current run is the last run for the processed CU.

The proponent of JCTVC-S0064 supports Method A as well.

Recommendation: Adopt Method A, on the condition that in-meeting simulation runs performed with Method A on top of JCTVC-S0269 yield results on the order of those shown for Method B here on top of CE6 Test A.6 configuration 2, compared to an anchor of SCM 2.0 (lossy all-intra 0.3% to 2.1%). For completeness, the proponents will also implement Methods A and B on top of S0269 and run simulations to compare them to an anchor of S0269 as well. A cross-check will also be performed and text will be provided.

JCTVC-S0228 Crosscheck of Non-CE6 on Improvement On Palette Run Coding (JCTVC-S0111) [C. -C Lin (ITRI)] [late]

JCTVC-S0114 CE6-related: Enabling copy above mode prediction at the boundary of CU [J. Kim, Y.-C. Sun, S. Liu, T. -D. Chuang, Y.-W. Chen, Y. -W. Huang, S. Lei (MediaTek)]

See the notes for JCTVC-S0079.

JCTVC-S0255 Non-CE6: Cross-check of enabling copy above mode prediction at the boundary of CU (JCTVC-S0114) [V. Seregin (Qualcomm)] [late] [miss]

JCTVC-S0115 Clarifying decoder’s ambiguous behaviour for escape index in palette mode [J. Kim, S. Liu, T. -D. Chuang, Y. -W Huang, S. Lei (MediaTek)]

This contribution proposes two approaches to clarify decoder’s behaviour for escape index handling in palette mode. The horizontal traverse scan is assumed for describing the approaches as follows. One approach is preventing each escape index from being a reference pixel of ‘copy above’ mode by limiting the number of runs of ‘copy above’ mode at decoder side as it works at encoder side. It also derives prediction mode of a pixel below escape index as ‘index mode’ without signalling the mode. The other approach is allowing each escape index being a reference pixel of ‘copy above’ mode by copying the escape values of the above escape pixel. The two approaches for vertical traverse scan can be easily understood by analogy, and both the approaches can solve the ambiguity issue successfully with very little impacts on coding efficiency.

Method 1: Prevent ESCAPE from being included as a reference index

A) Pixel below ESCAPE is inferred to be INDEX mode; and

B) The run of COPY_ABOVE prediction mode will not include the pixel below the ESCAPE index. (decoder syntax)

Lossy all-intra gains: 0.0% to 0.1%; RA -0.1% to 0.1%; LB -0.4% to 0.2%; Lossless gains 0.0% to 0.2% for all cases.

Method 2: Allow an escape index to be a reference index

Copies the escape value of the three components for the current pixel

Lossy all-intra gains: 0.0%; RA 0.0% to 0.1%; LB -0.1% to 0.25; lossless 0.0% for all cases.

Discussion of this document will continue under JCTVC-S0258

JCTVC-S0270 CE6-related: Cross check of JCTVC-S0115 Test 2.1 [J. Zhao, S. H Kim (Sharp)] [late] [miss]

JCTVC-S0279 Crosscheck of JCTVC-S0115 method 2 [F. Zou (Qualcomm)] [late] [miss]

JCTVC-S0120 Non-CE6: Copy previous mode [J. Ye, J. Kim, S. Liu, P. Lai, S. Lei (MediaTek)]

This proposal presents a “copy previous” mode with the use of neighbour indices for palette coding. The proposed “copy previous” mode is signalled by the “copy above” mode flag in current SCM, followed by an offset which specifies the distance between the previous row and the current row. It is proposed that the “copy previous” mode starts from the first row in the CU. The simulation results report that the proposed method achieves 3.9%, 1.9% and 1.5% BR-rate savings for “YUV 1080p text and graphics” AI, RA and LB configurations, respectively against the SCM 2.0 anchor.

This is similar in concept to JCTVC-S0079 (same proponents) plus CE6 Test C.3 (copy previous row), except that in JCTVC-S0120 two rows are available for copying instead of one.

All-intra lossy gains: 0.7% to 3.9%; lossless: 0.1% to 2.0%

(Gains for JCTVC-S0079 were 0.3% to 2.5% lossy)

Question on if another LCU is visited when going two rows above, would an additional line buffer might be needed. Answer: Currently for deblocking, a 2-line chroma buffer (4-line luma) is already available, so no additional buffers would be needed for this method.

The concerns addressed for JCTVC-S0079 apply to here as well.

Recommendation: Further study in a CE. When defining the CE, keep in mind the outcome of the CE6 Test C.3 decision from this meeting.

JCTVC-S0253 Non-CE6: Crosscheck for Copy Previous Mode (JCTVC-S0120) [W. Zhang, L. Xu, Y. Chiu (Intel)] [late] [miss]

JCTVC-S0134 Non-CE6: Simplified palette size coding [J. Zhao, S. H. Kim, K. Misra, A. Segall (Sharp)]

This contribution proposes to change the binarization of “palette_num_signalled_entries”.  When the palette_num_signalled_entries is less than 16, there is no change to the binarization; it still uses the existing TR code. When palette_num_signalled_entries is equal to or greater than 16, the proposed binarization uses a concatenated TR binarization of a number 16 and a fixed length code. The benefit of the proposed method is that it reduces the worst case coded bins from 31 to 20 bins, which is 35.5% reduction in the number of bins. It has no effect on coding loss for almost all lossy and lossless coding tests.

Proposed change:

| palette_coding( ) | previous_palette_entry_flag[] | FL | cMax = 1 |
| --- | --- | --- | --- |
|  | palette_num_signalled_entries | Section a.b.c.d. |  |

#### a.b.c.d. Binarization process for palette_num_signalled_entries

Inputs to this process are a request for a binarization for the syntax element palette_num_signalled_entries.

Output of this process is the binarization of the syntax element.

Depending on the value of palette_num_signalled_entries, the following applies:

- If palette_num_signalled_entries <16

  - Binaize the palette_num_signalled_entries using using TR binarization with cMax=16 and cRiceParam=0

else

  - Binarize the value 16 using using TR binarization with cMax=16 and cRiceParam=0

  - Binarize the value of palette_num_signalled_entries-16 using 4-bit FLC

There’s almost no effect on coding performance, so the proponent’s stated purpose for doing this is simplification.

Two non-proponents said that this method may reduce the number of bins, but may make the binarization more complex (the existing method is truncated unary coding).

Recommendation: No action

JCTVC-S0213 CE6-related: Crosscheck report of JCTVC-S0134 [J. Kim, S.Liu (MediaTek)] [late] [miss]

JCTVC-S0138 Non-CE6: Exponential Golomb binarization for Palette Run [S. H. Kim, K. Misra, J. Zhao, A. Segall (Sharp)]

This contribution proposes a concatenated binarization based on fixed length code and exponential golomb code for the syntax element ‘palette_run.’ Specifically, greater_than_2_flag was replaced by greater_than_5_flag. Here, if greater_than_5_flag is equal to 0 then fixed length code with length of 2 is used to signal the palette_run information between 2 and 5. If greater_than_5_flag is equal to 1, then exponential-golomb binarization is employed to code ‘palette_run-6’. It is reported that the proposed approach provides bit rate reductions of 0.3 %, 0.4% for All Intra 1080p text and graphics RGB and YUV categories, respectively.

Lossy all-intra gains: 0.0% to 0.4%

This proposes to replace the greater_than_2_flag with a greater_than_5_flag, which means that if palette_run is equal to 2, 3, 4, or 5, then fixed-length coding is used to signal it.

Code significant flag (context coded)

if(significant flag ==1)

Code GR_1 flag (context coded)

if (GR_1 flag ==1)

Code GR_5 flag (context coded)

if(GR_5 flag ==0)

Code Fixed Length Code (‘Run-2’)

else Code Exp-Golomb (‘Run-6’)

The proponent recommends further study in a CE related to what may be adopted after the revisit of CE6 Tests A4,A5,A6 (JCTVC-S0269).

Recommendation: Further study, perhaps in a CE related to what may be adopted from JCTVC-S0269.

JCTVC-S0229 Crosscheck of Non-CE6 on Exponential Golomb binarization for Palette Run(JCTVC-S0138) [C. -C Lin (ITRI)] [late]

JCTVC-S0139 Using flat scaling lists for escape coded palette pixels [K. Misra, S. H. Kim, J. Zhao, A. Segall (Sharp)]

This contribution proposes that the existing HEVC dequantizer along with flat scaling lists be used for palette pixels coded using escape mode. This approach was first proposed in JCTVC-R0234. The contribution demonstrates that the use of the default scaling list can lead to visual artifacts for palette coded blocks.

Artifacts may arise when non-flat scaling lists are used. An experiment demonstrates that when SCM 2.0 is used with the default (non-flat) scaling lists, artifacts are visible.

This proposes that the dequantizer is restricted to using flat scaling lists when processing palette pixels.

There was a question on whether the artifact pixels are in a different CU from the object pixels.

There was a question on whether we’re sure the blocks with artifacts were coded using palette mode or a different mode.

Examination of the JCTVC-R1005 text shows that the dequantization of the palette-coded blocks do not use any scaling list.

We do not know at the moment what QP value was used.

Recommendation: The artifacts are most likely caused by something else, given that palette mode doesn’t use a scaling list. Proponent is encouraged to investigate the cause of the artifacts. Perhaps report results to an AHG between meetings.

JCTVC-S0150 Non-CE6: Simplification on Escape Coding of Palette Mode in HEVC SCC [M. Xu, Z. Ma, W. Wang, H. Yu (Huawei USA R&D)]

This proposal is identical to [JCTVC-S0181 without encoder improvements]. See the discussion under JCTVC-S0181 and the combined discussion of JCTVC-S0115/S0150/S0156/S0157/S0181/S0258 under JCTVC-S0258

JCTVC-S0232 Crosscheck of JCTVC-S0150: Non-CE6: Simplification on Escape Coding of Palette Mode in HEVC SCC [R.-L. Liao, C.-C. Chen, W.-H. Peng, H.-M. Hang (NCTU/ITRI)] [late] [miss]

JCTVC-S0151 Non-CE6: 2-D Index Map Coding of Palette Mode in HEVC SCC [W. Wang, Z. Ma, M. Xu, H. Yu (Huawei USA R&D)]

This contribution presents a 2-D index map coding method designed for improving the performance of the palette mode in HEVC SCC WD and SCM 2.0. The proposed method is available as an additional method for coding palette indices. A flag associated with each run indicates which method to use (1-D or 2-D). The 1-D method is a modified version of the current method for coding palette indices. Its performance reported in this contribution was evaluated under common test conditions with various search range configurations. Experimental results demonstrate BD-Rate gain of 3.3% (IBC-1x4/2D-1x4), 20.2% (IBC-FF/2D-1x4), 14.8% (IBC-2x4/2D-2x4) and 18.4% (IBC-3x5/2D-3x5), over Anchor SCM-2.0+IBC-1x4, for lossy 1080p text graphics content encoded using All intra configuration, respectively.

Proposes using a 2-D string copy method to code the index map, in addition to the RUN-based 1D string match. This proposal also includes the JCTVC-S0150 method for coding the escape pixels.

If 2D_flag = 1, COPY_ABOVE flag is skipped, otherwise, COPY_ABOVE flag is coded using only one context (2 contexts are used in the default RUN-based 1 search)

Gains for 2D strings search range of 1x4: 1080p Text & Graphics all-intra about 2.1% for RGB, 1.7% YUV; 1.4% RGB 1.2% YUV for low delay. Encoder run-time ratio for all-intra: 129%, decoder 106%; LB: encoder 103%, decoder 103%. It was commented in that JCTVC-S0114, the gains of extending copy-above to one line above, higher gains were achieved.

For the case when the IBC search range is 2x4 and the 2D palette coding search range is 2x4, the time ratios are encoder 147% decoder 104% for all intra; encoder 106% decoder 103% for LB, and in this case, the BD-Rate gain as compared to an anchor using full-frame IBC is 0.6% for T&G 1080p RGB, and 0.8% for YUV.

The proponent asserts that their run times may not be accurate.

A crosscheck has not yet been registered.

The proponent clarified that the purpose of this proposal is that one can undo the losses incurred from decreasing the IBC search range by using this 2-D index map coding on palette mode.

A non-proponent recommended that this be further studied in a CE, as there are many questions about what the method is doing and what the relevant tested and anchor conditions are. Another non-proponent was concerned about the encoder run-time ratio increase given the gains. After the cross-check is uploaded, perhaps more accurate run-time ratios will be made available.

Recommendation: Further testing in a CE, with sufficient attention given to defining the appropriate test conditions in the CE description (e.g. IBC search ranges and 2-D search range for palette coding, etc.).

JCTVC-S0152 Non-CE6: Index Coding Group (ICG) for 8x8 CU of Palette Mode in HEVC SCC [W. Wang, Z. Ma, M. Xu, H. Yu (Huawei USA R&D)]

This contribution presents a parallel processing method for index map coding using an index coding group. This method suggests that to improve the overall system performance, coding of color index map may be done on smaller subgroups, instead of on the original color index map. It is suggested that this method might be used for 8x8 CUs, given that 8x8 CUs are a bottleneck for the overall throughput. Experimental results demonstrate performance losses of approximately 0.2% AI/0.1% RA/0.0% LB lossy BD-Rate loss over all content), but with potential 4x throughput improvement for 8x8 index map processing. Small gains are observed for lossless coding scenarios (i.e., 0.5% bit rate reduction for 1080p text/graphics content in RGB format).

This proposes to split an 8x8 index map into four 2x8 groups suitable for parallel processing. An additional flag is proposed for each group, which indicates whether the RUN equals the full size of the current group. COPY_ABOVE is maintained across groups.

Some participants suggested that it may be of interest to see if one could perform this kind of parallelization in the encoder, without any normative changes.

Recommendation: Further testing.

JCTVC-S0104 CE6-related: Cross-check of JCTVC-R0152 on Index Coding Group (ICG) for 8x8 CU of Palette Mode [P. Lai, J. Kim (MediaTek)] [late] [miss]

JCTVC-S0287 Crosscheck report of JCTVC-S0152 [K.Chono (NEC)] [late] [miss]

JCTVC-S0155 Non-CE6: Palette copy above mode for the first row [V. Seregin, M. Karczewicz, W. Pu, R. Joshi, J. Sole (Qualcomm)]

This contribution proposes to extend the use of the palette copy above mode for the very first row in the block. It is achieved by assigning the default palette index for the samples outside of the block. Reportedly, it provides 0.6% and 0.8% luma BD-rate reduction for All Intra 1080p text and graphics RGB and YUV categories in the lossy coding configuration, respectively.

This proposes to enable copy above mode for the first row, as follows:

Signal palette mode for the first row with additional context

Assign a palette index 0 for the samples coded with copy above mode in the first row

Lossy all-intra gains: 0.0% to 0.5%

This method is related to parts of JCTVC-S0079 and S0114.

If, additionally, If left and above indices are equal, then assign palette index 0 for copy above mode; then the gains are 0.0% to 0.6%. This method is related to Section 2.2 of JCTVC-S0047.

Recommendation: As with the related parts of JCTVC-S0079, S0114, and S0047/S0110, further study in a CE (or split across two CE’s if S0047/S0110 is studied under a different CE).

JCTVC-S0219 Cross-verification of JCTVC-S0155 [X. Xiu, Y. He, Y. Ye (InterDigital)] [late]

JCTVC-S0156 Non-CE6: Palette parsing dependency and palette encoder improvement [W. Pu, F. Zou, V. Seregin, R. Joshi, M. Karczewicz, J. Sole (Qualcomm)]

This contribution discusses a parsing dependency issue for the palette prediction mode, and suggests encoder improvements which reportedly provide 0.1%, 0.2% and 0.3%, 0.5% luma BD-rate reduction for All Intra 1080p text and graphics RGB and YUV categories in the lossy coding configuration for two encoder improvements, respectively.

Method 1: Currently, if (copyRun + 2 > indexRun), choose COPY_ABOVE run mode. This document proposes: if (copyRun > indexRun), choose COPY_AbOVE run mode. All-intra lossy gains for this method are 0.0% to 0.2%.

Method 2: The encoder is modified to compare the cost of copy left and copy above modes, and it chooses the mode with the lower cost. Lossy all-intra gains: 0.0% to 0.5%

It was also noted that in the Sapporo meeting, it was asserted that during parsing, the palette index has to be fully reconstructed to determine ESCAPE/INDEX copy modes. Now, it is asserted that the escape pixel’s index is always binarized to all ones ‘111…1”, therefore there is no parsing dependency issue, i.e. the parsing is required, but the reconstruction of the palette index is not required to determine ESCAPE/INDEX copy modes.

Methods 1 and 2 are mutually exclusive.

CE6 Category C, there’s a revisit of Tests C.2+C.3, and the related JCTVC-S0188 includes Method 2 shown here. JCTVC-S0188 is being considered for adoption pending a cross-check.

Recommendation (SW): Include the changes of Method 2 in the encoder software. Note also that JCTVC is going to have a revisit of JCTVC-S0188 under CE6 Test C.2+C.3, and the encoder improvement in S0188 is identical to Method 2 here.

JCTVC-S0157 Non-CE6: Copy mode and escape coded sample [V. Seregin, R. Joshi, M. Karczewicz, W. Pu, J. Sole (Qualcomm)]

This contribution addresses the problem of having escape coded samples in the above row and samples coded with the 'copy from above' mode. In the current software, such cases are not chosen by the encoder.  But from the decoder side, the run value signaled for the 'copy from above' mode may include escape coded samples. However the 'copy from above' mode is not defined for the escape samples.

Method 1: Restrict encoder from generating escape reference sample in copy above mode

Method 2: Restrict the copy above signaling if the above sample is escape:

- Pixel below ESCAPE is inferred to be INDEX mode; and (Same as in S0115)

- The encoder is constrained so that the ESCAPE pixel is not allowed to be included into the COPY_ABOVE mode.

Method 3: Include escape into the copy above mode (This is mutually exclusive to Method 2)

Method 4: Replace the escape sample with palette index 0 for copy above mode

Lossy all-intra gains for Methods 2-3 are within 0.0% to 0.1%.

See the combined discussion of JCTVC-S0115/S0150/S0156/S0157/S0181/S0258 under JCTVC-S0258

JCTVC-S0259 CE6-related: Crosscheck report of JCTVC-S0157 [J. Kim, S. Liu (??)] [late] [miss]

JCTVC-S0173 Non-CE6: Redundancy removal and simplification for Palette coding [Y. He, X. Xiu, Y. Ye (InterDigital)]

This proposal proposes to remove some syntax signalling redundancy and simplify the binarization method of palette_run value for palette coding. Compared to CE6 anchors, the lossy coding with proposed technologies achieves {Y, U, V} BD rate gain of {-0.2%, -0.2%, -0.2%}, {-0.2%, -0.2%, -0.2%} and {-0.2%, -0.2%, -0.3%} for the category (RGB/YUV, text & graphics with motion, 1080p and 720p) for AI, RA and LDB, respectively. For lossless coding,it achieves bitratre saving of 0.1%, 0.1% and 0.1% for the category (RGB, text & graphics with motion, 1080p) for AI, RA and LDB, respectively.

Two modifications are proposed:

Modification1: If palette size is 0, i.e. one color, then the palette_escape_val_present_flag is not signalled and is inferred to be true. A second part of this modification proposes that palette_mode and palette_run not be signalled when there is only one color in the CU due to one of two possibilities: 1) there is only one color in the palette table and there is no escape color, or 2) the palette table is empty.

Note that the above modification is a superset of JCTVC-S0099 (the two checks for indexMax >0 are both used in this proposal and JCTVC-S0099).

Modification 2: Simplifies the binarization method for palette_run: Proposes combining the binarization of Segments 2 and 3 to both use a 2nd-order Exp-Golomb instead of SCM 2.0’s Golomb-Rice for Segment 2 and 0th-order Exp-Golomb for Segment 3. This method thus reduces the length of the binarization for the smaller run values.

JCTVC-R0174 Section 1.1 (description) and Section 2.1(simulation results) propose the exact same modification as Modification 2. The proponents of JCTVC-R0174 decided to propose a different one of their coding methods in CE6 for the current meeting, so this method was not yet formally tested in a CE. The method that the proponents of JCTVC-R0174 decided to use is currently marked as a revisit in the meeting notes.

Gains of modification 1alone are 0.0% to 0.1% for all-intra lossy conditions.

Gains of modification 2 alone are 0.0% to 0.3%

Combined gains: 0.1% to 0.3%

Recommendation for Modification 2: Include in the revisit in the discussion of CE6 Category A (under JCTVC-S0026). For the first part of Modification 1 (adopt), see the recommendation for JCTVC-S0105. For the recommendation on the second part of Modification 1 (adopt) of this proposal and of JCTVC-S0099, see the “Continued discussion of S0099, S0110 and S0173” section in the JCTVC-S0105 notes.

JCTVC-S0200 Cross check Non-CE6: Redundancy removal and simplification for palette coding [W. Pu (Qualcomm)] [late] [miss]

JCTVC-S0178 Non-CE6: Improved binarization and signaling of index coding for transition copy mode [M. Karczewicz, F. Zou, R. Joshi, V. Seregin (Qualcomm)]

A CU level flag use_trans_flag is proposed to indicate the use of transition copy mode for palette coding. When use_trans_flag is 1, for index coding, an equal_to_transition flag is coded in bypass mode to indicate whether the index is equal to the transition index. The simulation results reportedly show that the proposed method achieves 0.2% and 0.3% BD-rate savings for 720p text and graphics RGB and YUV, respectively, when compared against CE6 Test C.2 for All-Intra lossy configuration. The worst-case context coded bins complexity is asserted to be better than CE6 test C.2.

Two flags on top of CE6 Test C.2 are proposed: use_trans_flag and equal_to_transition flag.

Lossy all-intra gains 0.0% to 1.2%. For a few classes of test sequences, these gains were 0.0% to 0.3% better than the gains achieved by CE6 Test C.2.

It was asked if the gains achieved by these two flags were tested separately (no).

It was commented that in the CE6 Test C.2+C.3 (JCTVC-S0188) test, a similar binarization as here was used.

The proponent asserts that using the equal_to_transition flag would be a complexity reduction (one less context-coded bin) than Test C.2+C.3.

Recommendations: Make final decision during Revisit of JCTVC-S0188 in the JCT-VC session.

JCTVC-S0275 Non-CE6: Cross-check of JCTVC-S0178 on improved binarization and signaling of index coding for transition copy mode [P. Onno (Canon)] [late] [miss]

JCTVC-S0181 Non-CE6: Removal of parsing dependency in palette-based coding [X. Xiu, Y. He, Y. Ye (InterDigital)]

This contribution proposes to modify the design of palette-based coding in order to remove the parsing dependency of various syntax elements for CUs coded with palette mode. Two modifications are introduced to achieve this goal. Firstly, it is proposed to treat the palette index used to indicate escape colors as normal palette index and them using either run mode or copy-above mode. Secondly, it is proposed to separate the parsing of escape color values from the parsing of palette indices.

This document proposes:

Code the palette index indicating escape colors using run mode and copy above mode;

Separate the parsing of escape color triplets from the parsing of palette indices.

The encoder used to generate results in this document also includes modifications to the decision between index and run modes, similar as in JCTVC-S0156.

Gains, including the encoder improvements mentioned above: Lossy All-intra gains: 0.1% to 0.5%; RA: 0.1% to 0.2%; 0.1% to 0.4%; Lossless: 0.1% to 0.95; RA 0.0% to 0.5%; 0.0% to 0.5%.

The gains without encoder improvements are identical to the gains reported in JCTVC-S0258. These gains are: Lossy all-intra and RA: -0.1% to 0.1%; LB: 0.0% to 0.3%. Lossless: 0.1% to 0.7%; RA 0.05 to 0.5%; LB 0.0% to 0.4%.

It was noted that JCTVC-S0150 is identical to [JCTVC-S0181 without encoder improvements]

It was noted that to implement the second part of this proposal (separate the parsing of escape color triplets from the parsing of palette indices) in the decoder, there can be several ways to accomplish this: Storing an array up to size 64x64 of a binary indication of whether the position is ESCAPE, or, re-scan all pixel positions in the CU to parse and fill in the escape color values, or, if the architecture chooses to scan twice, then one could merge the escape value decoding with the index-to-pixel value conversion process.

JCTVC-S0256 Non-CE6: Cross-check of removal of parsing dependency in palette-based coding (JCTVC-S0181) [V. Seregin (Qualcomm)] [late]

JCTVC-S0186 Non-CE6: on context modeling of palette_transpose_flag [X. Xiu, Y. He, Y. Ye (InterDigital)]

This contribution proposes to simplify the context modeling of the syntax element palette_transpose_flag for palette-based coding by only using one context model for the CABAC entropy coding. Experimental results show that the proposed change does not introduce any coding performance loss for both lossy coding and lossless coding (0.4% BD-rate saving is observed for the category of RGB mixed content 1080p), while reducing the total number of context models used in palette mode.

The cross-checker said using two contexts should have given more gain over one context, so we don’t have enough information to know why this is not the case.

Recommendation: Further study

JCTVC-S0276 Non-CE6: Cross-check of JCVC-S0186 on context modeling of palette_transpose_flag [P. Onno (Canon)] [late] [miss]

JCTVC-S0188 Non-CE6: A combination of CE6 Test C.2 – transition-copy mode and CE6 Test C.3 (configuration 1) - copy-from-previous-row mode [Y.-C. Sun, T.-D. Chuang, Y.-W. Huang, S. Lei (MediaTek)]

Initially discussed when CE6 summary report was reviewed on October 17, 2014.

Revisit in Track A (Gary)

JCTVC-S0288 Crosscheck of JCTVC-S0188: Non-CE6: A combination of CE6 Test C.2 – transition-copy mode and CE6 Test C.3 (configuration 1) - copy-from-previous-row mode [	R.-L. Liao, C.-C. Chen, W.-H. Peng, H.-M. Hang (NCTU/ITRI)] [late] [miss]

JCTVC-S0201 Non-CE5: CU dependent color palette maximum size [W. Wang, Z. Ma, M. Xu, H. Yu (Huawei USA R&D)] [late]

Presenters were not available

This contribution recommends a CU dependent color palette maximum size to balance the trade-off between coding efficiency and complexity throughput. Simulation results demonstrate averaged 0.5%/0.3%/0.1% lossy BD-Rate loss, and averaged -0.8%/-0.2%/-0.1% lossless bit rate increase for AI/RA/LB encoder configurations, respectively. It provides gain compared with the scenarios that max. palette size and palette predictor size fixed at respectively 15 and 32, but it keeps the same throughput with small CU sizes.

Experimental results were shown for the proposed method using a max palette predictor size of 32 for all block sizes, and a max palette size = 15 for 8x8 and 16x16, and 31 for 32x32 and 64x64. Results were compared to both the SCM 2.0 anchor (max palette size = 31 and max predictor size = 64), and to an anchor using max palette size = 15 and max predictor size = 32. Compared to SCM 2.0, losses of up to 1.2% were observed. Compared to the other anchor tested, gains were up to 0.5%.

It was asked whether this is an encoder or decoder complexity reduction. The cross-checker said that he thinks they are targeting the encoder. It was commented that during the CE5 review in JCT-VC Track A discussions, one of the decisions under CE5 (JCTVC-S0025) was to send the maximum palette size and maximum palette predictor size at the SPS level. It was also commented that the encoder can choose not to construct a palette up to max palette size.

Recommendation: Presenter not available, however, BoG reviewed document and slides. Therefore make final recommendation during BoG summary report review in the JCT-VC session.

JCTVC-S0106 CE5-related: Cross-check of JCTVC-S0201 on CU dependent color palette maximum size [P. Lai, J. Kim (MediaTek)] [late]

JCTVC-S0258 Non-CE6: Unification of coding of escape indices and other palette indices [X. Xiu, Y. He, Y. Ye (InterDigital), V. Seregin, R. Joshi, M. Karczewicz, W. Pu, J. Sole (Qualcomm)] [late]

In SCM2.0, escape coded samples are assigned the highest palette index (after expanding the palette index by 1). But in coding of palette indices, the escape index is treated differently from other palette indices. This contribution proposes a method that is asserted to unify the coding of escape index and other palette indices. The proposed method uses elements from JCTVC-S0157 (solution 3) and JCTVC-S0181. BD-rate results for the proposed method with and without encoder modification are presented. Without encoder modification, for the category of text and graphics with motion 1080p, the BD-rate saving for the Y/G component is 0.1% for the All-Intra lossy configuration, and the bit-rate savings for the Y/G component 0.6% for the All-intra lossless configuration. With encoder modification, the corresponding BD-rate savings and bit-rate savings are 0.4% and 0.8% for lossy and lossless configurations, respectively.

This proposal comprises the common parts of JCTVC-S0157 and JCTVC-S0181:

Escape index is enabled for INDEX mode with a run value to specify the number of subsequent indices that are also escape colors

Escape index allowed to be part of Copy mode run.

Without encoder improvements, gains for all-intra lossy: -0.1 to 0.1%; lossless: 0.1% to 0.7%.

The difference between JCTVC-S0258 and JCTVC-S0181 is that in JCTVC-S0258, escape color values are coded in the same interleaved way as in SCM 2.0, whereas in JCTVC-S0181 the coding of escape color values is moved outside of the coding loop for escape indices.

Combined discussion of JCTVC-S0115/S0150/S0156/S0157/S0181/S0258

Condensed summary of these proposals:

JCTVC-S0115:

Method 1: Prevent ESCAPE from being included as a reference index:

A) Pixel below ESCAPE is inferred to be INDEX mode; and

B) The run of COPY_ABOVE prediction mode will not include the pixel below the ESCAPE index. (decoder syntax)

Method 2: Allow an escape index to be a reference index

Recommendations for both Methods 1 and 2: Given the other recommendations, this is not needed.

JCTVC-S0150 and JCTVC-S0181 and JCTVC-S0258:

All three include this:

Code the palette index indicating escape colors using run mode and copy above mode; Recommendation for this method for coding the palette index: adopt JCTVC-S0258/S0150 subset/S0181 subset/S0157 Method 3.

S0150 and S0181 additionally include: Separate the parsing of escape color triplets from the parsing of palette indices. One non-proponent said that they don’t see any benefit to doing this and therefore this change to the spec should not be made; they see added cost, asserting that one would have perform an additional scan of all pixel positions in the CU to parse and fill in the escape color values. Several proponents and a non-proponent disagreed, saying that there are other ways to implement it that doesn’t require an additional scan (e.g. see notes near end of JCTVC-S0181 discussion). A proponent said that the current spec’s inverleaved way of decoding requires (if done in one loop) that the dequantization occurs inside the loop for decoding escape colors (in contrast to the decoding process of the regular color indices). A non-proponent and proponent said that with the current specs, another implementation is possible: one can first parse indices and escape colors, and then invoke the mapping of indices to colors and perform inverse dequantization of escape colors. The non-proponent asserts that adoption of this method would always require a separate pass for parsing the escape values. The proponents said that this proposed solution is consistent with the HEVC v.1 for residual coding where the signaling of the significance map is separated from that of the individual residual values. A non-proponent disagreed, saying that in palette mode decoding, for the palette index map there are more syntax elements which have to be parsed and decoded first, in contrast to the significance map coding in HEVC v.1 residual coding. The proponent asserts that with this proposal, the current encoding process can be made to be more efficient; a non-proponent disagrees.

Current status: two proponents proposed this, one non-proponent opposes, and one non-proponent supports this. Recommendation: Make final decision in JCT-VC session, and offline discussion is encouraged before this final decision.

JCTVC-S0156: No further discussion needed, recommendation reached.

JCTVC-S0157:

Method 1: Restrict encoder from generating escape reference sample in copy above mode: Recommendation: Given the other recommendations, this is not needed.

Method 2: Restrict the copy above signaling if the above sample is escape:

A)	Pixel below ESCAPE is inferred to be INDEX mode; and (Same as in S0115)

B)	The encoder is constrained so that the ESCAPE pixel is not allowed to be included into the COPY_ABOVE mode.

Recommendation for Method 2: Given the other recommendations, this is not needed.

Method 3: Include escape into the copy above mode (this is also a subset of S0150/S0181/S0258) (see earlier recommendation for its adoption)

Method 4: Replace the escape sample with palette index 0 for copy above mode. Recommendation: Given the other recommendations, this is not needed.

Inconsistency: It was noted that in the SCC text specifications JCTVC-R1005-v3, the definition of palette_mode[xC][yC] in Section 7.4.9.6 uses the term “sample value”, whereas the syntax in Section 7.3.8.8 (which will eventually be moved to the decoder process description as an editorial change) uses the indices paletteMap[xC][yC], not values, in COPY_ABOVE mode. The earlier implementations of palette mode (e.g. earlier versions of SCM, JCT-VC R0348, all CE’s conducted on palette, etc.) copy indices, not pixel values. Recommendation: Edit the semantics for palette_mode[xC][yC] to clarify that COPY_ABOVE mode copies the indices paletteMap[xC][yC], not sample values.

Method 2 of S0157 is the same as part of Method 1 of S0115 (restrict the copy above signaling if the above sample is escape). S0157 uses an encoder constraint, whereas S0115 uses decoder syntax so that the pixel below the ESCAPE index is not included in the COPY_ABOVE run mode.

The method of coding the palette index indicating escape colors using run mode and copy above mode is common to JCTVC-S0150/S0181 and the joint S0258 proposals.

JCTVC-S0277 Non-CE6: Crosscheck of JCTVC-S0258 on unification of coding of escape indices and other palette indices [P. Onno (Canon)] [late] [miss]

JCTVC-S0269 CE6-related: Harmonization of CE6 Tests A4, A5, and A6 [S.-T. Hsiang, T.-D. Chuang, S. Lei (MediaTek), R. Joshi, W. Pu, M. Karczewicz, F. Zou, V. Seregin, J. Sole (Qualcomm)] [late] [miss]

Initially discussed when CE6 summary report was reviewed on October 17, 2014.

Revisit in Track A (Gary)

JCTVC-S0289 Cross check of Harmonization of CE6 Tests A4, A5, and A6 (JCTVC-S0269) [	O. Nakagami (Sony)] [late] [miss]