| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0294 |
| --- | --- |

| Title: | BoG report for complexity assessment of IBC block vector coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chao Pang, Shan Liu | Email: | cpang@qti.qualcomm.com shan.liu@mediatek.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

This document provides a report of the BoG for complexity assessment of IBC block vector coding.

# Introduction

A BoG was held on Saturday October 18, 2014, 9:00 am to 11:15 am to provide more detailed complexity assessment of IBC vector coding. The agreed mandates of the BoG were:

- Analyze the worst case number of context coded bins in IBC for the current SCM and the methods CE1 Test 1.1 (JCTVC-S0142) and Test 1.4 (JCTVC-S0162). Compare against current 4x8 MV coding, including considerations about reference index and mode coding.

- Discuss whether invoking merge at 4x4 blocks could cause some complexity problems.

# List of Proposals

## IBC block vector coding

- JCTVC-S0142, CE1 Test 1.1: CE1: Results of Test 1.1, Test 2.1 and Test 3.1, K. Rapaka, M. Karczewicz, C. Pang (Qualcomm)

- JCTVC-S0162, CE1 Test 1.4: CE1: Results of Tests 1.4, 2.2, & 3.3 on unified BVD & MVD coding, S.-T. Hsiang, T.-D. Chuang, S. Lei (MediaTek)

## IBC merge

- JCTVC-S0080, CE2: Result of Test 1 [B. Li, J. Xu (Microsoft), X. Xu, S. Liu, S. Lei (MediaTek)]

- JCTVC-S0131, CE2: Test3 – IBC with block vector derivation [Y. He, Y. Ye, X. Xiu (InterDigital)]

- JCTVC-S0122, CE2: Test 5 – IBC unified with Inter using intra_bc_flag [X. Xu, S. Liu, S. Lei (MediaTek), B. Li, J. Xu (Microsoft)]

- JCTVC-S0081, CE2: Result of Test 6 [B. Li, J. Xu (Microsoft)]

# Discussed topics

## Complexity assessment of IBC block vector coding

The complexity of IBC block vector coding mainly refers to the number of context coded bins.

### Complexity comparison of IBC block vector coding

|  | Complexity |  |  |
| --- | --- | --- | --- |
|  | CE1 Test 1.1 | CE1 Test 1.4 | SCM 2.0 |
| # regular bins per 4x4 | 4 | 14 | 2 |
| # contexts per 4x4 | 4 | 6 | 2 |

### Complexity comparison with Inter mode

In this section, the methods of CE1 Test 1.1 and CE1 Test 1.4 are compared with HEVC Inter mode in terms of the worst-case number of context coded bins. The total number of regular bins per 8x8 CU in the worst case are summarized in the following table, and the detailed results can be found in the attached excel file.

For IBC block vector coding,

|  | Complexity |  |  |  |
| --- | --- | --- | --- | --- |
|  | CE1 Test 1.1 | CE1 Test 1.4 | SCM 2.0 | Inter |
| # regular bins per 8x8 CU | 490 | 530 | 482 | 488 |
| % compared with SCM 2.0 | +1.6% | +10.0% | x | x |
| % compared with Inter | +0.4% | +8.6% | x | x |

In addition, since the CE1 Test 2.1 has the same binarization as CE1 Test 1.1 and CE1 Test 2.2 has the same binarization as CE1 Test 1.4, the complexity of CE1 Test 2.1 and CE 1 Test 2.2 are analyzed as well by comparing with HEVC Inter mode. The total number of regular bins per 8x8 CU in the worst case are summarized in the following table, and the detailed results can be found in the attached excel file.

|  | Complexity |  |  |
| --- | --- | --- | --- |
|  | CE1 Test 2.1 | CE1 Test 2.1 | Inter |
| # regular bins per 8x8 CU | 488 | 508 | 488 |
| % compared with Inter | The same. | +4.0% | x |

An expert commented that the performance might be dependent on the ME method used for IBC. A couple of experts also ask for additional results with the bits estimation as Inter.

In the following table, the results of Test 3.1 (Test 1.1 + Test 2.1) and Test 3.3 (Test 1.4 + Test 2.2) with local search (no hash-based full frame search) and the bit estimation method as Inter in HM.

*: Use local search instead of full frame search.

**: Use bit estimation method as Inter in HM.

x: The results are not provided by the proponent.

#### Coding performance for block vector coding of CE1 Test 3.1 and Test 3.3 reported in CE 1

|  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 3.1 | 3.3 | 3.1* | 3.3* | 3.1** | 3.3** |
| RGB, text & graphics with motion, 1080p | -0.3% | -0.4% | -0.1% | x | -0.3% | x |
| RGB, text & graphics with motion,720p | -0.2% | -0.4% | -0.1% | x | -0.2% | x |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | x | 0.0% | x |
| RGB, mixed content, 1080p | -0.1% | -0.1% | 0.0% | x | -0.1% | x |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | x | 0.0% | x |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | x | 0.0% | x |
| YUV, text & graphics with motion, 1080p | -0.3% | -0.5% | -0.1% | x | -0.3% | x |
| YUV, text & graphics with motion,720p | -0.2% | -0.4% | -0.1% | x | -0.2% | x |
| YUV, mixed content, 1440p | -0.1% | -0.1% | -0.1% | x | -0.1% | x |
| YUV, mixed content, 1080p | 0.0% | -0.1% | 0.0% | x | 0.0% | x |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | x | 0.0% | x |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | x | 0.0% | x |
| Enc Time[%] | 97% | 99% | 103% | x | 87% | x |
| Dec Time[%] | 100% | 101% | 100% | x | 100% | x |

|  | RA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 3.1 | 3.3 | 3.1* | 3.3* | 3.1** | 3.3** |
| RGB, text & graphics with motion, 1080p | -2.4% | -2.7% | -1.8% | x | -2.2% | x |
| RGB, text & graphics with motion,720p | -0.6% | -0.9% | -0.5% | x | -0.4% | x |
| RGB, mixed content, 1440p | -0.1% | -0.2% | -0.1% | x | 0.0% | x |
| RGB, mixed content, 1080p | -0.1% | -0.1% | -0.2% | x | 0.0% | x |
| RGB, Animation, 720p | -0.1% | -0.3% | -0.1% | x | 0.2% | x |
| RGB, camera captured, 1080p | 0.0% | -0.2% | 0.0% | x | 0.3% | x |
| YUV, text & graphics with motion, 1080p | -2.7% | -3.0% | -2.2% | x | -2.5% | x |
| YUV, text & graphics with motion,720p | -0.7% | -0.9% | -0.5% | x | -0.6% | x |
| YUV, mixed content, 1440p | 0.0% | -0.1% | -0.2% | x | 0.0% | x |
| YUV, mixed content, 1080p | -0.1% | -0.1% | -0.1% | x | 0.0% | x |
| YUV, Animation, 720p | 0.0% | -0.2% | -0.2% | x | 0.3% | x |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.0% | x | 0.3% | x |
| Enc Time[%] | 100% | 95% | 95% | x | 80% | x |
| Dec Time[%] | 100% | 100% | 100% | x | 100% | x |

|  | LD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 3.1 | 3.3 | 3.1* | 3.3* | 3.1** | 3.3** |
| RGB, text & graphics with motion, 1080p | -3.1% | -3.4% | -2.4% | x | -2.7% | x |
| RGB, text & graphics with motion,720p | -1.2% | -1.5% | -1.1% | x | -0.8% | x |
| RGB, mixed content, 1440p | -0.2% | -0.2% | -0.2% | x | 0.1% | x |
| RGB, mixed content, 1080p | -0.6% | -0.3% | -0.7% | x | -0.3% | x |
| RGB, Animation, 720p | 0.0% | -0.1% | -0.1% | x | 0.3% | x |
| RGB, camera captured, 1080p | 0.0% | -0.2% | 0.0% | x | 0.3% | x |
| YUV, text & graphics with motion, 1080p | -3.4% | -3.9% | -2.8% | x | -3.1% | x |
| YUV, text & graphics with motion,720p | -1.4% | -1.8% | -1.6% | x | -1.1% | x |
| YUV, mixed content, 1440p | -0.3% | -0.4% | -0.2% | x | -0.2% | x |
| YUV, mixed content, 1080p | -0.4% | -0.1% | -0.6% | x | -0.2% | x |
| YUV, Animation, 720p | -0.1% | -0.1% | -0.1% | x | 0.4% | x |
| YUV, camera captured, 1080p | 0.1% | -0.1% | 0.1% | x | 0.3% | x |
| Enc Time[%] | 87% | 94% | 90% | x | 83% | x |
| Dec Time[%] | 100% | 99% | 100% | x | 100% | x |

An expert commented that if the Inter MV coding method is changed, it is better to have a unified solution for IBC and Inter.

## IBC merge for 4×4

An expert commented that when using 4x4 merge, parallel processing cannot be performed at encoder and decoder. Need additional merge candidate list construction.

An expert commented that what is the coding performance improvement from encoder only changes instead of using merge. Additional results are requested. This is a general question for whether to bring in merge mode for IBC. In addition, what is the gain from IBC 4x4. Another expert also requests the performance improvement from IBC 4x4 merge.

#### Coding performance of enabling full RD check at the current BVP candidate positions (JCTVC-S0087)

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -2.6% | -2.6% | -2.6% | -2.4% | -2.2% | -2.3% | -2.4% | -2.1% | -2.1% |
| RGB, text & graphics with motion,720p | -0.9% | -1.0% | -0.9% | -0.6% | -0.8% | -0.7% | -0.5% | -0.4% | -0.4% |
| RGB, mixed content, 1440p | -1.3% | -1.2% | -1.4% | -0.8% | -0.9% | -1.1% | -0.7% | -0.7% | -0.8% |
| RGB, mixed content, 1080p | -1.1% | -1.1% | -1.1% | -0.8% | -1.0% | -1.2% | -0.6% | -0.1% | -0.2% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | -0.6% | -0.7% | -0.7% | -2.1% | -2.1% | -2.1% | -2.3% | -2.2% | -2.1% |
| YUV, text & graphics with motion,720p | -0.5% | -0.5% | -0.5% | -0.3% | -0.4% | -0.3% | -0.5% | -0.2% | -0.3% |
| YUV, mixed content, 1440p | -0.7% | -0.8% | -0.9% | -0.7% | -0.7% | -0.8% | -0.7% | -0.3% | -0.3% |
| YUV, mixed content, 1080p | -0.6% | -0.6% | -0.7% | -0.6% | -0.8% | -0.9% | -0.7% | 0.0% | -0.7% |
| YUV, Animation, 720p | -0.2% | -0.2% | -0.2% | -0.1% | -0.2% | 0.0% | 0.0% | -0.2% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | -0.2% | 0.0% |
| Enc Time[%] | 122% |  |  | 109% |  |  | 103% |  |  |
| Dec Time[%] | 101% |  |  | 103% |  |  | 103% |  |  |

#### Coding performance of disabling 4x4

|  | AI (SCM2.0) |  |  | AI (JCTVC-S0065) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 1.0% | 1.0% | 0.9% | 1.8% | 1.8% | 1.7% |
| RGB, text & graphics with motion,720p | 0.6% | 0.4% | 0.4% | 0.9% | 0.8% | 0.8% |
| RGB, mixed content, 1440p | 0.5% | 0.4% | 0.3% | 0.8% | 0.7% | 0.6% |
| RGB, mixed content, 1080p | 0.9% | 0.6% | 0.6% | 1.3% | 1.0% | 1.0% |
| RGB, Animation, 720p | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.9% | 0.8% | 0.8% | 1.8% | 1.6% | 1.6% |
| YUV, text & graphics with motion,720p | 0.6% | 0.5% | 0.5% | 0.8% | 0.7% | 0.6% |
| YUV, mixed content, 1440p | 0.4% | 0.2% | 0.3% | 0.7% | 0.6% | 0.6% |
| YUV, mixed content, 1080p | 0.8% | 0.6% | 0.5% | 1.0% | 0.7% | 0.9% |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 95% |  |  | 96% |  |  |
| Dec Time[%] | 97% |  |  | 100% |  |  |

An expert has concern regarding the memory bandwidth for 4x4. Another experts commented that as there is no interpolation for IBC, there should be no issue for the memory bandwidth for 4x4.

# BoG suggestions

## IBC block vector coding

If the Inter MV coding method is changed, it is better to have a unified solution for IBC and Inter.