| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0295 |
| --- | --- |

| Title: | Cross-check of JCTVC-S0188 |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |  |
| Purpose: | Information |  |  |  |
| Author(s) or Contact(s): | Döne Bugdayci Sansli Jani Lainema | Tel: Email: | +358 7180 08000 firstname.lastname@nokia.com |  |
| Source: | Nokia |  |  |  |

_____________________________

# Abstract

This document is a cross-check report for JCTVC-S0188: “Non-CE6: A combination of CE6 Test C.2 – transition-copy mode and CE6 Test C.3 (configuration 1) - copy-from-previous-row mode”. Simulation results match with the results provided in JCTVC-S0188.

# Introduction

The source code for the combination of CE6 Test C.2 and C.3 (configuration 1) was investigated and it is verified that the software changes match the description of the method.

# Results

The provided source code is evaluated under Common Test Conditions for SCC, lossy coding. The summary of the experimental results are provided in the tables below. The detailed results can be found in the spreadsheet attached. The encoder and decoder runtimes measured are not reliable as the simulations for the anchor and the studied method were run in a different computing cluster.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -2.3% | -2.6% | -2.5% |
| RGB, text & graphics with motion,720p | -1.6% | -1.7% | -1.7% |
| RGB, mixed content, 1440p | -0.2% | -0.3% | -0.3% |
| RGB, mixed content, 1080p | -1.4% | -1.8% | -1.5% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -2.5% | -3.0% | -2.9% |
| YUV, text & graphics with motion,720p | -1.2% | -1.6% | -2.0% |
| YUV, mixed content, 1440p | -0.4% | -1.1% | -1.0% |
| YUV, mixed content, 1080p | -1.5% | -3.8% | -3.2% |
| YUV, Animation, 720p | 0.0% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | N/A |  |  |
| Dec Time[%] | N/A |  |  |

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.1% | -1.3% | -1.3% |
| RGB, text & graphics with motion,720p | -1.3% | -1.4% | -1.4% |
| RGB, mixed content, 1440p | -0.1% | -0.3% | -0.3% |
| RGB, mixed content, 1080p | -1.0% | -1.4% | -1.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | -0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.2% | -1.7% | -1.6% |
| YUV, text & graphics with motion,720p | -1.0% | -1.4% | -1.9% |
| YUV, mixed content, 1440p | -0.3% | -1.2% | -0.8% |
| YUV, mixed content, 1080p | -1.0% | -3.4% | -2.8% |
| YUV, Animation, 720p | 0.0% | -0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | N/A |  |  |
| Dec Time[%] | N/A |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.9% | -1.2% | -1.1% |
| RGB, text & graphics with motion,720p | -1.2% | -1.2% | -1.2% |
| RGB, mixed content, 1440p | -0.4% | -0.5% | -0.4% |
| RGB, mixed content, 1080p | -0.9% | -1.3% | -0.6% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | -1.0% | -1.6% | -1.5% |
| YUV, text & graphics with motion,720p | -0.9% | -1.2% | -1.9% |
| YUV, mixed content, 1440p | -0.4% | -1.6% | -1.2% |
| YUV, mixed content, 1080p | -1.4% | -4.0% | -3.1% |
| YUV, Animation, 720p | 0.0% | -0.3% | 0.2% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | N/A |  |  |
| Dec Time[%] | N/A |  |  |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).