| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0300_r1 |
| --- | --- |

| Title: | Qp derivation and offsets signaling for adaptive color transform |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Krishna Rapaka Li Zhang Rajan Joshi Marta Karczewicz Keiichi Chono Jizheng Xu Rickard Sjöberg S. H. Kim K. Misra, A. Segall (Sharp) | Tel: Email: | 1-858-658-2042 krapaka@qti.qualcomm.com +81 44 431 7659 chono@ct.jp.nec.com jzxu@microsoft.com rickard.sjoberg@ericsson.com kimse@sharplabs.com |
| Source: | Qualcomm Incorporated, NEC, Microsoft Corp, Ericsson, Sharp Labs |  |  |

_____________________________

# Abstract

In the 18th JCTVC meeting, adaptive color transform was adopted into the test model of SCC extension. The tool adaptively transforms prediction residuals of one color space into another and signals a flag to indicate its color space at CU level. Several contribution in the 19th JCTVC meeting have identified various issues in the current test model related to quantization parameter derivation when adaptive color transform is used and proposed solutions to fix them. This contribution combines aspects from proposal JCTVC-S0144, JCTVC-S0040, JCTVC-S0094 and JCTVC-S0086.

# Introduction and Problem statement

In the current working draft the normalization for adaptive color transform is performed as follows:

### 8.6.2 Scaling and transformation process (in the current specification)

The quantization parameter qP is derived as follows in the current test model,

If cIdx is equal to 0,

qP = Qp′Y + (cu_residual_act_flag[ xTbY ][ yTbY ] ? −5 : 0)	(8-1)

–	Otherwise, if cIdx is equal to 1,

qP = Qp′Cb + (cu_residual_act_flag[ xTbY ][ yTbY ] ? −5 : 0)	(8-2)

–	Otherwise (cIdx is equal to 2),

qP = Qp′Cr + (cu_residual_act_flag[ xTbY ][ yTbY ] ? −3 : 0)	(8-3)

where cIdx specifies the colour component of the current block and cu_residual_act_flag specifies whether adaptive colour transform is applied to the residual samples of the current coding unit.

The contribution identifies following problems:

- In the current working draft when adaptive color transform is enabled, during the scaling and transformation process, a QP offset of -5 is added for luma and Cb chroma component, and -3 is added for Cr chroma component. It is asserted that the resultant value of Qp may underflow the allowed Qp range.

- The Qp offset is added to normalize the color transform and it was observed that selection of best Qp offset may depend on the content. It is asserted that flexible Qp offset signalling when adaptive color transform may provide additional flexibility for encoder to choose appropriate Qp offset. It is also asserted that adaptive QP offset signalling would benefit visual quality.

# Proposal

This contribution primarily proposes following aspects

- Clip such that the resulting qP does not underflow the allowed Qp range.

- Signal flexible Qp offsets at picture and slice level so as to allow encoder to choose appropriate normalization offset for different contents. As a result move the signaling of residual_adaptive_colour_transform_enabled_flag from SPS to PPS. Also, clipping such that the resulting qP does not overflow is needed.

# Specification Text

The detailed specification text for the proposed changes are mentioned below

| pps_scc_extension ( ) { | Descriptor |
| --- | --- |
| … |  |
| residual_adaptive_colour_transform_enabled_flag | u(1) |
| if(residual_adaptive_colour_transform_enabled_flag){ |  |
| pps_slice_act_qp_offsets_present_flag | u(1) |
| pps_act_y_qp_offset_plus5 | se(v) |
| pps_act_cb_qp_offset_plus5 | se(v) |
| pps_ act_cr_qp_offset_plus3 | se(v) |
| } |  |
| … |  |
| } |  |

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| …………….. |  |
| slice_qp_delta | se(v) |
| if( pps_slice_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| } |  |
| if( chroma_qp_offset_list_enabled_flag ) |  |
| cu_chroma_qp_offset_enabled_flag | u(1) |
| if(pps_slice_act_qp_offsets_present_flag){ |  |
| slice_act_y_qp_offset | se(v) |
| slice_act_cb_qp_offset | se(v) |
| slice_act_cr_qp_offset | se(v) |
| } |  |
| …… |  |

residual_adaptive_colour_transform_enabled_flag equal to 1 specifies that an adaptive colour transform may be applied to the residual in the decoding process for the pictures referring to the PPS. residual_adaptive_colour_transform_enabled_flag equal to 0 specifies that adaptive colour transform is not applied to the residual for the pictures referring to the PPS. When not present, the value of residual_adaptive_colour_transform_enabled_flag is inferred to be equal to 0.

When chroma_format_idc is not equal to 3, residual_adaptive_colour_transform_enabled_flag shall be equal to 0.

pps_act_y_qp_offset_plus5, pps_act_cb_qp_offset_plus5 and pps_act_cr_qp_offset_plus3 specify offsets to the luma, cb and cr quantization parameter qP derived in section 8.6.2, respectively. The values of pps_act_y_qp_offset, pps_act_cb_qp_offset and pps_act_cr_qp_offset are derived as pps_act_y_qp_offset_plus5 - 5, pps_act_cb_qp_offset_plus5 - 5 and pps_act_cr_qp_offset_plus3 -3 respectively and shall be in the range of −12 to +12, inclusive. When ChromaArrayType is equal to 0, pps_act_cb_qp_offset and pps_act_cr_qp_offset are not used in the decoding process and decoders shall ignore their value. When not present, the values of pps_act_y_qp_offset_plus5, pps_act_cb_qp_offset_plus5 and pps_act_cr_qp_offset_plus3 are inferred to be equal to 0.

pps_slice_act_qp_offsets_present_flag equal to 1 specifies that slice_act_y_qp_offset , slice_ act _cb_qp_offset, slice_ act _cr_qp_offset are present in the slice header. pps_slice_act_qp_offsets_present_flag equal to 0 specifies that slice_act_y_qp_offset , slice_ act _cb_qp_offset, slice_ act _cr_qp_offset are not present in the slice header. When not present, the value of pps_slice_act_qp_offsets_present_flag is inferred to be equal to 0.

slice_act_y_qp_offset, slice_act_cb_qp_offset and slice_act_cr_qp_offset specify offsets to the luma, cb and cr quantization parameter qP derived in section 8.6.2, respectively. The values of slice_act_y_qp_offset, slice_act_cb_qp_offset and slice_act_cr_qp_offset shall be in the range of −12 to +12, inclusive. When ChromaArrayType is equal to 0, slice_act_cb_qp_offset and slice_act_cr_qp_offset are not used in the decoding process and decoders shall ignore their value. When not present, the values of slice_act_y_qp_offset, slice_act_cb_qp_offset and slice_act_cr_qp_offset are inferred to be equal to 0. The value of pps_act_y_qp_offset + slice_act_y_qp_offset shall be in the range of -12 to +12, inclusive. The value of pps_act_cb_qp_offset + slice_act_cb_qp_offset shall be in the range of -12 to +12, inclusive. The value of pps_act_cr_qp_offset + slice_act_cr_qp_offset shall be in the range of -12 to +12, inclusive.

In order to keep the allowed Qp range same to that of HEVC when adaptive color transform is used, it is proposed to clip the range of the Qp values to that of HEVC Qp range. The proposed changes to the test model are highlighted in yellow text.

[In the derivation process for quantization parameters (Section 8.6.1):]

When ChromaArrayType is not equal to 0, the following applies.

–	If cu_residual_act_flag[ xTbY ][ yTbY ] is equal to 0, the variables qPCb and qPCr are derived as follows:

qPiCb = Clip3( −QpBdOffsetC, 57, QpY + pps_cb_qp_offset + slice_cb_qp_offset + CuQpOffsetCb )

qPiCr = Clip3( −QpBdOffsetC, 57, QpY + pps_cr_qp_offset + slice_cr_qp_offset + CuQpOffsetCr )

–	Otherwise (cu_residual_act_flag[ xTbY ][ yTbY ] is equal to 1), the variables qPCb and qPCr are derived as follows:

qPiCb = Clip3( −QpBdOffsetC, 57, QpY + pps_act_cb_qp_offset + slice_act_cb_qp_offset + CuQpOffsetCb )

qPiCr = Clip3( −QpBdOffsetC, 57, QpY + pps_act_cr_qp_offset + slice_act_cr_qp_offset  + CuQpOffsetCr )

[In the scaling and transformation process (Section 8.6.2)]

The quantization parameter qP is derived as follows:

–	If cIdx is equal to 0,

qP = Clip3(0, 51 + QpBdOffsetY, Qp′Y + (cu_residual_act_flag [ xTbY ][ yTbY ] ? pps_act_y_qp_offset + slice_act_y_qp_offset : 0)

–	Otherwise, if cIdx is equal to 1,

qP = Qp′Cb + (cu_residual_act_flag [ xTbY ][ yTbY ] ? −-5 : 0)

–	Otherwise (cIdx is equal to 2),

qP = Qp′Cr + (cu_residual_act_flag [ xTbY ][ yTbY ] ? −-3 : 0)

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

NEC Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

SHARP may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).