| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0302-v1 |
| --- | --- |

| Title: | Non-CE2: Intra block copy and Inter signaling unification |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chao Pang, Krishna Rapaka, Ye-Kui Wang, Vadim Seregin, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA Xiaozhong Xu, Shan Liu, Shawmin Lei MediaTek Bin Li, Jizheng Xu Microsoft Corp. | Tel: Email: | + 1-858-658-3779 cpang@qti.qualcomm.com xiaozhong.xu@mediatek.com shan.liu@mediatek.com shawmin.lei@mediatek.com libin@microsoft.com jzxu@microsoft.com |
| Source: | Qualcomm Inc., MediaTek, Microsoft Corp. |  |  |

_____________________________

# Abstract

In this contribution, the Intra block copy mode is signalled reusing inter signalling by adding the current picture to the reference picture list(s). Comparing to the current design in SCM2.0, Intra BC 4x4 block has been removed. In addition, different block vector prediction methods are tested. The working draft text changes, with change marks, are provided in an attachment of this contribution.

# Introduction

Intra block copy (Intra BC) is an efficient technique to predict repetitive patterns in the frame and leads to significant bit-rate savings for screen content. Currently, the Intra BC mode is signalled at CU-level, and it has different partition modes, including 2Nx2N/2NxN/Nx2N/NxN. To save the memory bandwidth, the block vector (BV) for Intra BC is with integer-pixel precision.

Comparing with CE2 Test1 [1], the main differences of the proposed method lie in the following aspects: (1) Avoid extra conditional check in the parsing process; (2) Different motion vector difference (MVD) coding method when the current picture is selected as the reference picture; (3) Necessary constraints are added for completeness.

# Proposed

In this contribution, the Intra BC mode is signalled reusing inter signalling by adding the current picture (without in-loop filtering) into the reference picture list(s). With the shared signaling, Intra BC can be differentiated from the conventional Inter mode by checking reference index of the reference picture used in the prediction. If the reference picture is the current picture, then the prediction mode is Intra BC mode. Otherwise, it is conventional Inter mode. To avoid having a merge mode with 4x4, contrary to JCTVC-S0113, Intra BC 4x4 block has been removed. The modifications in this contribution lie in the following aspects:

- When Intra BC is enabled, there shall be no I-slice in CVSs.

- The current picture is marked as long-term before decoding the current frame, and marked back to short-term after decoding the current frame.

- When the reference picture is the current picture, MVD is coded in integer-pixel precision with the current Intra BC block vector difference (BVD) coding method.

- When the reference picture is the current picture, the MVs stored in DPB are with quarter-pixel precision. Thus, the motion compensation and deblocking modules for Inter can be transparently used without any changes.

- It is restricted that the collocated picture cannot be the current picture.

- 4x4 block is not used.

- When constrained Intra prediction is enabled, the prediction only uses decoded samples from neighbouring coding blocks coded with Intra mode or Intra BC mode.

- When the reference picture is the current picture and constrained Intra prediction is enabled, the TMVP shall be disallowed.

# Encoder changes

Same as in JCTVC-S0113, two encoder changes are included in the software to be aligned with CE2 Test 1. First, the full RD costs for Intra BC merge mode are checked for 2Nx2N partition mode of all the CU sizes. Second, the full RD costs of 2NxN and Nx2N partition modes are checked additionally for 16x16 CU.

# Results

In this section, the experimental results of the proposed methods are presented. All the simulations are conducted on top of SCM2.0, and SCC common test conditions [3] are used. The BD-rates are provided for the lossy case under different configurations. The following tests are performed:

- Test 1: Intra BC BV prediction with HEVC AMVP.

- Test 2: SCM 2.0 Intra BC BV prediction with BV line buffer.

- Test 3: Test 1 + the BV entropy coding technique from CE1 Test 3.1 [4].

- Test 4: Test 2 + the BV entropy coding technique from CE1 Test 3.1 [4].

- Test 5: Comparison of Intra BC BV prediction with the current BV prediction method with BV line buffer (anchor) and HEVC AMVP (test) on top of SCM 2.0.

Table 1 Lossy results of Test 1

Table 2 Lossy results of Test 2

Table 3 Lossy results of Test 3

Table 4 Lossy results of Test 4

Table 5 Lossy results of Test 5

# Conclusions

In this contribution, Intra BC is signalled by reusing Inter signaling by adding the current picture to the reference picture set, and the Intra BC 4x4 block is removed in this signaling unification.

# References

[1] J. Xu, S. Liu, K. Rapaka, and X. Xiu, “CE2: Summary report for Core Experiment 2 on intra block copy signalling and partitioning”, JCTVC-S0022, Strasbourg, FR, October 2014.

[2] C. Pang, K. Rapaka, Y.-K. Wang, V. Seregin, M. M. Karczewicz, B. Li, and J. Xu, “Non-CE2: Intra block copy with Inter signaling”, JCTVC-S0021, Strasbourg, FR, October 2014.

[3] H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common conditions for screen content tests”, JCTVC-R1015, Sapporo, JP, July 2014.

[4] C. Pang, and X. Xu, “CE1: Summary report for Core Experiment 1 on vector entropy coding”, JCTVC-S0021, Strasbourg, FR, October 2014.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).