| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0303 |
| --- | --- |

| Title: | CE9: Cross-check of supplemental results from JCTVC-S0039, CE6 subtest A.6: Binarization for run coding in palette mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Robert Cohen Mitsubishi Electric Research Laboratories 201 Broadway, Cambridge, MA 02139 U.S.A | Tel: Email: | +1-617-621-7500 cohen@merl.com |
| Source: | Mitsubishi Electric Research Laboratories (MERL) |  |  |

_____________________________

# Abstract

This document provides the results of a crosscheck of supplemental results included with JCTVC-S0039, CE6 subtest A.6: Binarization for run coding in palette mode, for the case with five regular bins. The results of this crosscheck match the results in the corresponding spreadsheets included in JTVC-S0039.

# Cross-check overview

JCTVC-S0039 includes spreadsheets “CE6_A6+A5_MoreCtx_5RegBin(decoded)_lossless.xlsm” and “(CE6_A6+A5_MoreCtx_5RegBin(decoded)_lossy.xlsm” showing the coding performance obtained by performing a test related to CE6 Test A.6, with a combination with five regular bins. The software provided by the authors of JCTVC-S0039 was used for this cross-check, and results are compared to the SCM-2.0 reference using the common test conditions.

# Experimental Results

Note that decoding run-time ratios are inaccurate due to variations in computing platforms.

## Lossy coding results

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.7% | -1.7% | -1.7% |
| RGB, text & graphics with motion,720p | -1.0% | -1.0% | -1.0% |
| RGB, mixed content, 1440p | -0.3% | -0.4% | -0.4% |
| RGB, mixed content, 1080p | -0.5% | -0.5% | -0.5% |
| RGB, Animation, 720p | 0.0% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -2.0% | -2.0% | -2.1% |
| YUV, text & graphics with motion,720p | -1.1% | -1.2% | -1.3% |
| YUV, mixed content, 1440p | -0.5% | -0.8% | -0.8% |
| YUV, mixed content, 1080p | -0.6% | -0.9% | -0.9% |
| YUV, Animation, 720p | 0.0% | -0.3% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 103% (not accurate) |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.9% | -0.9% | -1.0% |
| RGB, text & graphics with motion,720p | -0.7% | -0.8% | -0.7% |
| RGB, mixed content, 1440p | -0.2% | -0.3% | -0.3% |
| RGB, mixed content, 1080p | -0.2% | -0.3% | -0.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.0% | -1.2% | -1.2% |
| YUV, text & graphics with motion,720p | -0.9% | -0.9% | -1.1% |
| YUV, mixed content, 1440p | -0.3% | -0.8% | -0.7% |
| YUV, mixed content, 1080p | -0.3% | -0.8% | -0.6% |
| YUV, Animation, 720p | 0.0% | -0.2% | -0.3% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 130% (not accurate) |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.9% | -1.0% | -0.9% |
| RGB, text & graphics with motion,720p | -0.6% | -0.7% | -0.7% |
| RGB, mixed content, 1440p | 0.0% | -0.3% | -0.3% |
| RGB, mixed content, 1080p | -0.4% | 0.1% | 0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | -0.9% | -1.2% | -1.0% |
| YUV, text & graphics with motion,720p | -0.6% | -0.7% | -1.3% |
| YUV, mixed content, 1440p | -0.1% | -0.6% | -0.2% |
| YUV, mixed content, 1080p | -0.3% | -0.8% | -1.0% |
| YUV, Animation, 720p | 0.0% | -0.5% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 143% (not accurate) |  |  |

## Lossless coding results

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.8% | 0.8% | 0.5% | 1.1% |
| RGB, text & graphics with motion,720p | 0.4% | 0.4% | 0.1% | 0.8% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 1.1% | 1.2% | 0.5% | 1.6% |
| YUV, text & graphics with motion,720p | 0.4% | 0.5% | 0.1% | 0.9% |
| YUV, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.1% |
| YUV, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 103% (not accurate) |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.3% | 0.5% | 0.3% | 0.9% |
| RGB, text & graphics with motion,720p | 0.1% | 0.2% | 0.0% | 0.5% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.4% | 0.8% | 0.3% | 1.0% |
| YUV, text & graphics with motion,720p | 0.1% | 0.3% | 0.0% | 0.6% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  |  |
| Dec Time[%] | 102% (not accurate) |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.3% | 0.5% | 0.2% | 0.8% |
| RGB, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.3% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.4% | 0.7% | 0.3% | 0.9% |
| YUV, text & graphics with motion,720p | 0.0% | 0.2% | 0.0% | 0.4% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  |  |
| Dec Time[%] | 107% (not accurate) |  |  |  |

# Conclusions

The results of this crosscheck match the corresponding results in the spreadsheets “CE6_A6+A5_MoreCtx_5RegBin(decoded)_lossless.xlsm” and “CE6_A6+A5_MoreCtx_5RegBin(decoded)_lossy.xlsm” included in JTVC-S0039.