| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0306_r1 |
| --- | --- |

| Title: | Cross-verification of JCTVC-S0220 on parallel processing capability of intra block copy |  |  |
| --- | --- | --- | --- |
| Status: | Information |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu 9710 Scranton Rd, #250 San Diego, CA 92121, USA | Tel: Email: | + 1-858-210-4830 Xiaoyu.Xiu@InterDigital.com |
| Source: | InterDigital Communications, LLC |  |  |

_____________________________

# Abstract

This document presents cross-verification results of JCTVC-S0220 on parallel processing capability of intra block copy from Qualcomm. It is verified that the software implementation is consistent with the description of the contribution and all the simulation results are perfectly matched with that provided by the proponent.

# Introduction

The intra block copy mode uses previously decoded unfiltered samples within the same picture for its prediction. For full intra block search, the search range is unrestricted and can use any unfiltered decoded samples of the current picture. In such a scenario, some parallel processing configurations (e.g processing multiple CTB’s in wave front) might have synchronization issues when processed in parallel. To resolve the abovementioned parallel processing issue, in JCTVC-S0220, two methods are proposed as stated as below:

- Fixed bitstream restriction on IBC prediction region.

- Flexible IBC prediction region indication to decoders to enhance parallel processing capabilities.

For this cross-check work, the experimental results of method a) are verified.  The software used in this cross-check was provided from the proponents. The software implementation is consistent with the contribution description and all the simulation results are perfectly matching the results provided from the proponents.

# Simulation results

The detailed results are presented and bundled in the same archive.

Table 1 BD-rate performance of IBC processing with one CTB delay to the above CTB row, in lossy coding

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 1.3% | 1.3% | 1.3% |
| RGB, text & graphics with motion,720p | 1.6% | 1.5% | 1.5% |
| RGB, mixed content, 1440p | 0.5% | 0.4% | 0.4% |
| RGB, mixed content, 1080p | 0.2% | 0.2% | 0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 1.5% | 1.4% | 1.4% |
| YUV, text & graphics with motion,720p | 1.8% | 1.7% | 1.7% |
| YUV, mixed content, 1440p | 0.5% | 0.5% | 0.4% |
| YUV, mixed content, 1080p | 0.2% | 0.3% | 0.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 81% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.7% | 0.7% | 0.6% |
| RGB, text & graphics with motion,720p | 0.9% | 0.9% | 0.9% |
| RGB, mixed content, 1440p | 0.1% | 0.0% | 0.1% |
| RGB, mixed content, 1080p | 0.2% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.6% | 0.6% | 0.6% |
| YUV, text & graphics with motion,720p | 1.3% | 1.2% | 1.2% |
| YUV, mixed content, 1440p | 0.3% | 0.2% | 0.2% |
| YUV, mixed content, 1080p | 0.2% | 0.3% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.2% | 0.0% |
| Enc Time[%] | 86% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.5% | 0.5% | 0.5% |
| RGB, text & graphics with motion,720p | 0.2% | 0.3% | 0.3% |
| RGB, mixed content, 1440p | 0.0% | -0.1% | 0.0% |
| RGB, mixed content, 1080p | -0.1% | 0.5% | 0.2% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.5% | 0.4% | 0.5% |
| YUV, text & graphics with motion,720p | 0.4% | 0.5% | 0.1% |
| YUV, mixed content, 1440p | -0.1% | 0.1% | 0.0% |
| YUV, mixed content, 1080p | 0.2% | 0.4% | -0.1% |
| YUV, Animation, 720p | 0.1% | -0.3% | 0.3% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 72% |  |  |
| Dec Time[%] | 100% |  |  |

Table 2 BD-rate performance of IBC processing with two CTB delay to the above CTB row, in lossy coding

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.8% | 0.8% | 0.8% |
| RGB, text & graphics with motion,720p | 1.0% | 0.9% | 0.9% |
| RGB, mixed content, 1440p | 0.3% | 0.2% | 0.2% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.9% | 0.8% | 0.8% |
| YUV, text & graphics with motion,720p | 1.1% | 1.1% | 1.0% |
| YUV, mixed content, 1440p | 0.3% | 0.2% | 0.2% |
| YUV, mixed content, 1080p | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 90% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.4% | 0.4% | 0.4% |
| RGB, text & graphics with motion,720p | 0.7% | 0.6% | 0.6% |
| RGB, mixed content, 1440p | 0.2% | 0.1% | 0.1% |
| RGB, mixed content, 1080p | 0.1% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | -0.1% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.4% | 0.4% | 0.4% |
| YUV, text & graphics with motion,720p | 0.8% | 0.8% | 0.9% |
| YUV, mixed content, 1440p | 0.2% | 0.1% | 0.0% |
| YUV, mixed content, 1080p | 0.1% | -0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | -0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.2% | -0.1% |
| Enc Time[%] | 79% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.2% | 0.2% | 0.2% |
| RGB, text & graphics with motion,720p | 0.2% | 0.1% | 0.2% |
| RGB, mixed content, 1440p | 0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | 0.0% | 0.4% | 0.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.2% | 0.1% | 0.2% |
| YUV, text & graphics with motion,720p | -0.1% | -0.1% | -0.1% |
| YUV, mixed content, 1440p | 0.0% | 0.1% | 0.3% |
| YUV, mixed content, 1080p | -0.2% | -0.2% | -0.2% |
| YUV, Animation, 720p | 0.1% | -0.7% | 0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 83% |  |  |
| Dec Time[%] | 100% |  |  |