| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0308 |
| --- | --- |

| Title: | BoG report on Intra String Copy (CE10) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Ying Chen | Tel: Email: | cheny@qti.qualcomm.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

This document provides a report of the BoG on CE 10 proposals, focusing on the memory bandwidth analysis and CABAC throughput, from 6 pm to 6:45 pm, Oct. 21.

# Mandates of the BoG

Establish BoG to investigate complexity impact, particularly

- the memory bandwidth requirements for the CE10 common software and the test 1 method (which shows interesting coding gain, but could have impact on the memory access due to switching hor/vert scan).

- the worst case number of context coded bins, for the CE10 common software and the test 6 method(s)

Revisit to determine further steps after that report.

# Worst Case Memory Bandwidth

We start with the case of assuming per pixel access, meaning accessing a pixel requires accessing the whole mxn memory pattern.

Consider also various sizes of the memory pattern.

An excel sheet (provided by Y. Chen) has been provided. It contains the memory bandwidth access values for various modes, including Intra BC (4x4), Intra Line Copy (4x1 and 1x4) as well as Inter prediction cases, including 4x8, 8x4 and 8x8 bi-predicted PU.

Roughly speaking, Intra String Copy requires twice of the memory bandwidth as Intra Line Copy and 3~4 times of the memory bandwidth as compared to Intra BC (4x4), even when large memory access pattern is used.

Some typically comparisons are provide below:

Summary of the memory bandwidth when various memory access patterns are considered. Note that the following equation is considered for calculating the memory bandwidth for each component of the pixels within an MxN block, wherein L is the filter length and mxn is the memory access pattern.

Memory pattern 8x2 per pixel

|  | L | M | N | Y/ (U or V) | 3 component |
| --- | --- | --- | --- | --- | --- |
| Intra BC (4x4) | 1 | 4 | 4 | 6 | 18 |
| Intra LC | 1 | 1 | 4 | 12 | 36 |
|  | 1 | 4 | 1 | 8 | 24 |
| Inter 8x8 | 8/6 | 8 | 8 | 12/10.5 | 33 |
| Intra String Copy | 1 | 1 | 1 | 16 | 48 |

Memory pattern 8x4 per pixel

|  | L | M | N | Y/ (U or V) | 3 component |
| --- | --- | --- | --- | --- | --- |
| Intra BC (4x4) | 1 | 4 | 4 | 8 | 24 |
| Intra LC | 1 | 1 | 4 | 16 | 48 |
|  | 1 | 4 | 1 | 16 | 48 |
| Inter 8x8 | 8/6 | 8 | 8 | 16/12 | 39 |
| Intra String Copy | 1 | 1 | 1 | 16/12 | 96 |

Memory pattern 4x2 per pixel

|  | L | M | N | Y/ (U or V) | 3 component |
| --- | --- | --- | --- | --- | --- |
| Intra BC (4x4) | 1 | 4 | 4 | 3 | 9 |
| Intra LC | 1 | 1 | 4 | 6 | 18 |
|  | 1 | 4 | 1 | 4 | 12 |
| Inter 8x8 | 8/6 | 8 | 8 | 10/7 | 24 |
| Intra String Copy | 1 | 1 | 1 | 8 | 24 |

Memory pattern 4x4 per pixel

|  | L | M | N | Y/ (U or V) | 3 component |
| --- | --- | --- | --- | --- | --- |
| Intra BC (4x4) | 1 | 4 | 4 | 4 | 12 |
| Intra LC | 1 | 1 | 4 | 8 | 24 |
|  | 1 | 4 | 1 | 8 | 12 |
| Inter 8x8 | 8/6 | 8 | 8 | 12.5/8 | 28.5 |
| Intra String Copy | 1 | 1 | 1 | 16 | 48 |

Memory pattern 8x8 per pixel

|  | L | M | N | Y/ (U or V) | 3 component |
| --- | --- | --- | --- | --- | --- |
| Intra BC (4x4) | 1 | 4 | 4 | 16 | 48 |
| Intra LC | 1 | 1 | 4 | 16 | 96 |
|  | 1 | 4 | 1 | 32 | 96 |
| Inter 8x8 | 8/6 | 8 | 8 | 18/18 | 54 |
| Intra String Copy | 1 | 1 | 1 | 64 | 192 |

The test 1 method

Even when vertical scan is supported, since each pixel is assumed to consume the whole memory of a pattern, the vertical scan doesn’t require more memory bandwidth as compared with the solution in the common CE software.

Detailed calculations are shown in the associated excel file: JCTVC-S0308_Memory.xlsx.

# CABAC throughput

As discussed in other CEs, 4x8 uni-prediction requires 488 number of context bins and 7.62 bin per pixel which corresponds to the worst case of the current CU.

In the current CE 10 common software, the motion vectors are not CACAC coded, however, two flags indicating the components of a motion vector being zero are context coded. The length of the strings consumes also in the worst case 1 context coded bin per pixel. In addition, a CU level flag is content coded.

The Test 6.1 corresponds to 964 context coded bins for a CU, corresponding to 15.06 bin per pixel.

Detailed calculations are shown in the associated excel file: JCTVC-S0308_CABAC.xlsx.

| Cases | 4x8, Uni Prediction (SCM-2.0) | Intra String Copy (Common CE SW) | Intra String Copy: Test 6.1 | Intra String Copy: Test 6.2 | Intra String Copy: Test 6.3 |
| --- | --- | --- | --- | --- | --- |
| #bins per 8x8 CU | 488 | 196 | 964 | 196 | 964 |
| #bins per pixel | 7.625 | 3.06 | 15.06 | 3.06 | 15.06 |