| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0309 |
| --- | --- |

| Title: | CE9: Summary of informal subjective viewing of effects from enabling or disabling intra boundary filtering for all components |  |  |
| --- | --- | --- | --- |
| Status: | Input Document To JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Robert Cohen PoLin Lai Jizheng Xu | cohen@merl.com polin.lai@mediatek.com jzxu@microsoft.com |  |
| Source: | MERL, MediaTek Inc., Microsoft |  |  |

_____________________________

# Abstract

This document provides the summary of in-meeting informal subjective testing performed for a revisit of Core Experiment 9 (CE9) on IBF/CCP interdependency. Coded videos corresponding to two tests: Test A.1 which enables the intra boundary filter for all components, and Test A.2 which disables intra boundary filtering for all components. A table of observations is included in this report..

# Introduction

On October 14, 2014, the authors of this document conducted in-meeting informal subjective testing of decoded videos corresponding to the results given for Tests A.1 and A.2 in Core Experiment 9 [1]. Test A.1 disables the intra boundary filter for all components, and Test A.2 enables the intra boundary filter for all components.

# Sequences viewed

The list of sequences examined is shown in Table 1.

Table 1. Sequences viewed

| Coding condition | Sequence name | Resolution | fps |
| --- | --- | --- | --- |
| All Intra | Desktop | 1920x1080 | 60 |
|  | WebBrowsing | 1280x720 | 30 |
|  | MissionControlClip2 | 2560x1440 | 60 |
|  | MissionControlClip3 | 1920x1080 | 60 |
|  | Kimono | 1920x1080 | 24 |
| Low Delay | BasketBallScreen | 2560x1440 | 60 |

# Observations from informal subjective viewing

|  | Test A.1 (disable IBF) | Test A.2 (enable IBF) |
| --- | --- | --- |
| Kimono (AI) |  |  |
| Frame 0 | Blurred lapel line |  |
|  | Ear border visible | Ear border somewhat flatter |
|  | More blocking on face | More blocking/verticle strip on neck (vert. strip somewhat in original) |
| Frame 94 |  | More blocking on face |
| WebBrowsing (AI) |  |  |
| Frame 0 |  | "shadow" visible in background below upper left logo/text |
| Frame 90 | subtle differences between both; inconclusive |  |
| Desktop (AI) |  |  |
| Frame 0 |  | Weak ghosting in background near paragraph button |
| Frame 335 | Both are similar |  |
| Moving video | Both have same flckering/ghosting in bkgrnd near buttons (not in original) |  |
| MissionControlClip2 (AI) |  |  |
| Frame 0 | More noise on chin |  |
| Frame 196 | Preserves MATLAB texture better |  |
| MissionControlClip3 (AI) |  |  |
| Frame 0 | Reproduces texture on MATLAB better |  |
|  | Reproduces hair better |  |
| BasketballScreen (LB) |  |  |
| Frame 0 (I-slice) | Different but no conclusion |  |
| Frame 38 (B-slice) | Different but no conclusion |  |

# References

- R. Cohen, S. Liu, J. Xu, L. Zhang, “CE9: Summary report for Core Experiment 9 on IBF/CCP interdependency,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-R1108, 19th Meeting: Strasbourg, FR, 17 – 24 October, 2014.