| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 19th Meeting: Strasbourg, FR, 17 – 24 Oct. 2014 | Document: JCTVC-S1002 |
| --- | --- |

| Title: | High Efficiency Video Coding (HEVC) Test Model 16 (HM 16) Improved Encoder Description |  |  |
| --- | --- | --- | --- |
| Status: | Output Document of JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | K. McCann C. Rosewarne B. Bross M. Naccari K. Sharman G. Sullivan | Email: | ken@zetacast.com chris.rosewarne@cisra.canon.com.au benjamin.bross@hhi.fraunhofer.de matteo.naccari@bbc.co.uk karl.sharman@eu.sony.com garysull@microsoft.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

The JCT-VC established the HEVC Test Model 16.2 at its 19th meeting in Strasbourg from 17 October to 24 October 2014. This document serves as an overview of HEVC Version 1 and the Range Extension of HEVC Version 2 and also provides an encoder-side description of the HM-16.2 software.

CONTENTS

Page

Abstract	1

List of figures	5

List of tables	7

1	Introduction	8

1.1	Overview of coding structures	8

1.2	Obtaining the HEVC test model	9

1.3	Obtaining the HEVC Standard	9

2	Scope	9

3	Encoder control of the HEVC test model 16	9

3.1	Encoder configuration options	9

3.1.1	File, format and profile/level/tier configuration options	9

3.1.2	Coding tool configuration options	9

3.1.3	Slice coding parameters	11

3.1.4	SEI message configuration options	11

4	Overview of tools in HEVC Version 1 and HEVC Version 2 RExt	11

4.1	High level syntax	11

4.2	Picture partitioning	11

4.2.1	Coding tree unit (CTU) partitioning	11

4.2.2	Slice and tile structures	12

4.2.3	Coding unit (CU) and coding tree structure	13

4.2.4	Prediction unit (PU) structure	13

4.2.5	Transform unit (TU) and transform tree structure	14

4.3	Intra prediction	15

4.3.1	Prediction modes	15

4.3.2	Filtering of neighbouring samples	16

4.3.3	Intra boundary filter	17

4.3.4	4:2:2 chroma format mode adjustment	18

4.4	Inter prediction	19

4.4.1	Prediction modes	19

4.4.1.1	Derivation of candidates for merge mode	20

4.4.1.2	Spatial candidates derivation	20

4.4.1.3	Temporal candidates derivation	21

4.4.1.4	Additional candidates insertion	22

4.4.1.5	Motion estimation regions for parallel processing	23

4.4.2	Motion vector prediction	23

4.4.2.1	Derivation of motion vector prediction candidates	23

4.4.2.2	Spatial motion vector candidates	23

4.4.2.3	Temporal motion vector candidates	24

4.4.3	Interpolation filter	24

4.4.4	Weighted Prediction	25

4.4.4.1	High precision offsets	25

4.5	Transform and quantization (scaling)	26

4.5.1	Inverse transforms	26

4.5.2	1D inverse transform matrices	26

4.5.3	Scaling and quantization	26

4.5.4	Scaling lists	27

4.5.5	Scaling lists for transform skipped TUs	27

4.5.6	Transform selection for the 4:2:2 chroma format	27

4.5.7	Chroma QP initialization offset table	28

4.5.8	Extended precision processing	28

4.5.9	CU-adaptive chroma QP offset	29

4.6	Slice QP offset	29

4.7	Residual prediction in case of transquant bypass and transform skip	29

4.8	Entropy coding	29

4.8.1	CABAC alignment	30

4.9	Coefficient Coding	30

4.9.1	Transform skip residual rotation	30

4.9.2	Significance map context modelling	30

4.9.3	Rice parameter adaptation	30

4.9.4	Maximum coeff_abs_level_remaining codeword length restriction	30

4.10	Cross-component prediction	31

4.11	Loop Filtering	31

4.11.1	Overview of Loop filtering	31

4.11.2	Deblocking filter	32

4.11.2.1	Boundary decision	32

4.11.2.2	Boundary strength calculation	32

4.11.2.3	Threshold variables	33

4.11.2.4	Filter on/off decision for 4 lines	34

4.11.2.5	Strong/weak filter selection for 4 lines	34

4.11.2.6	Strong filtering	34

4.11.2.7	Weak filtering	35

4.11.2.8	Chroma filtering	35

4.11.3	Sample adaptive offset filter	36

4.11.3.1	Operation of each SAO type	36

4.12	Wavefront parallel processing	37

5	Profiles, Levels and Tiers	37

6	Description of HM Encoder and encoding methods	39

6.1	Cost Functions	39

6.1.1	Sum of Square Error (SSE)	39

6.1.2	Sum of Absolute Difference (SAD)	39

6.1.3	Hadamard transformed SAD (SATD)	40

6.1.4	RD cost functions	40

6.1.4.1	Lagrangian constant values	40

6.1.4.2	Weighting factor for chroma component	40

6.1.4.3	SAD based cost function for prediction parameter decision	41

6.1.4.4	SATD based cost function for prediction parameter decision	41

6.1.4.5	Cost function for mode decision	41

6.2	Encoder configurations	41

6.2.1	Overview of encoder configurations	41

6.2.2	Intra-only configuration	41

6.2.3	Low-delay configurations	41

6.2.4	Random-access configuration	42

6.3	Slice and tile partitioning operation	42

6.4	Derivation process for slice-level coding parameters	43

6.4.1	Sample Adaptive Offset (SAO) parameters	43

6.4.1.1	Search the SAO type with minimum rate-distortion cost	43

6.4.1.2	Slice level on/off Control	43

6.4.2	Adaptive QP selection	43

6.4.3	Adaptive search range for motion estimation	43

6.5	Derivation process for CU-level and PU-level coding parameters	44

6.5.1	Intra prediction mode and parameters	44

6.5.1.1	Rate-distortion penalty for intra coding	44

6.5.2	Inter prediction mode and parameters	44

6.5.2.1	Derivation of motion parameters	44

6.5.2.2	Motion estimation	45

6.5.2.3	Decision process on AMP mode evaluation procedure	47

6.5.3	Intra/Inter/PCM mode decision	48

6.5.4	Adaptive QP	50

6.6	Derivation process for TU-level coding parameters	50

6.6.1	Residual Quad-tree partitioning	50

6.6.2	Rate-distortion optimized quantization	51

6.6.3	Quantization rounding for residual DPCM	51

6.7	Cross-component prediction	51

6.8	Transform skip selection	51

6.9	Cost mode	52

6.10	Inter-prediction search	52

6.11	Rate control	52

7	References	54

- List of figures

Figure 1-1 – Simplified block diagram of HM encoder	8

Figure 4-1 – Example of a picture divided into CTUs.	12

Figure 4-2 – Example of slices and slice segments	12

Figure 4-3 – Examples of tiles and slices	13

Figure 4-4 – Example of coding tree structure	14

Figure 4-5 – 8 partition modes for inter PU	14

Figure 4-6 – Example of transform tree structure within CU	15

Figure 4-7 – The 33 intra prediction directions	16

Figure 4-8 – Mapping between intra prediction direction and intra prediction mode	16

Figure 4-9 – Intra boundary filter example.	18

Figure 4-10 – Intra prediction angle for 4:2:2 chroma format.	19

Figure 4-11 – Derivation process for merge candidates list construction	20

Figure 4-12 – Positions of spatial merge candidate	21

Figure 4-13 – Candidate pairs considered for redundancy check of spatial merge candidates	21

Figure 4-14 – Positions for the second PU of Nx2N and 2NxN partitions	21

Figure 4-15 – Illustration of motion vector scaling for temporal merge candidate	22

Figure 4-16 – Candidate positions for temporal merge candidate, C0 and C1	22

Figure 4-17 – Example of combined bi-predictive merge candidate	22

Figure 4-18 – Derivation process for motion vector prediction candidates	23

Figure 4-19 – Illustration of motion vector scaling for spatial motion vector candidate	24

Figure 4-20 – Lossless scaling and transformation process.	26

Figure 4-21 – Transform skip scaling and transformation process.	26

Figure 4-22 – Inverse transformation process (quantities illustrated for HEVC version 1 profiles).	26

Figure 4-23 – Scaling lists	27

Figure 4-24 – Square transform arrangement for the 4:2:2 chroma format.	28

Figure 4-25 – Diagram showing magnitude bit depths in HEVC encoding path	28

Figure 4-27 – Overall processing flow of deblocking filter process	32

Figure 4-28 – Flow diagram for Bs calculation	33

Figure 4-29 – Referred information for Bs calculation at CTU boundary	33

Figure 4-30 – Pixels involved in filter on/off decision and strong/weak filter selection	34

Figure 4-31 – Deblocking behaviour in the 4:2:2 chroma format.	35

Figure 4-32 – Four 1-D 3-pixel patterns for the pixel classification in EO	36

Figure 4-33 – Four bands are grouped together and represented by its starting band position	37

Figure 7-1 – Graphical presentation of intra-only configuration	41

Figure 7-2 – Graphical presentation of low-delay configuration	42

Figure 7-3 – Graphical presentation of random-access configuration	42

Figure 7-4 – Three step motion search strategy for integer-pel accuracy	45

Figure 7-5 – Start position selection	46

Figure 7-6 – Search patterns for the first search	46

Figure 7-7 – The schematic of Intra/Inter/PCM mode decision	49

Figure 7-8 – Dead zone uniform quantizer with rounding offset.	51

- List of tables

Table 3-2 – Encoder configuration options for control of coding tools.	10

Table 3-3 – Encoder configuration options for slice coding.	11

Table 4-1 – Mapping between intra prediction direction and intra prediction mode for chroma	17

Table 4-2 – Specification of predefined threshold for various transform block sizes	17

Table 4-3 – Specification of intra prediction mode for 4:2:2 chroma (Proposal 2)	18

Table 4-4 – 8-tap DCT-IF coefficients for 1/4th luma interpolation	24

Table 4-5 – 4-tap DCT-IF coefficients for 1/8th chroma interpolation	25

Table 4-6 – g_maxTrDynamicRange[channel]	28

Table 4-7 – α Mapping Table.	31

Table 4-8 – Derivation of threshold variables from input Q	33

Table 4-9 – Specification of SAO type	36

Table 4-10 – Pixel classification rule for EO	36

Table 6-1 – Bitstream indications for format range extensions profiles.	38

Table 6-2 – Mapping between user configuration and automatically calculated bitstream format range extensions profile indication.	39

Table 7-1 – Derivation of 	40

Table 7-2 – Conditions and actions for fast AMP mode evaluation	47

Table 7-3. Configuration options for rate control algorithm.	53

# Introduction

This document provides an overview of the coding tools defined in HEVC Version 1 and the Range Extension (RExt) of HEVC Version 2, both of which are implemented in the second point release of the 16th HEVC test model (HM-16.2). In addition some of the algorithms that the HM-16.2 encoder uses to control these tools and user-controlled configuration options are also provided.

## Overview of coding structures

HEVC is a block-based hybrid coding architecture, combining inter and intra prediction and transform coding with high-efficiency entropy coding. However, in contrast to previous video coding standards, HEVC employs a quad-tree coding block partitioning structure that enables a flexible use of large and small coding, prediction, and transform blocks. HEVC also allows for improved intra prediction and coding, adaptive motion parameter prediction and coding, a new loop filter and an enhanced version of context-adaptive binary arithmetic coding (CABAC) entropy coding over that defined by previous standards. New high level structures have also been designed to aid parallel processing.

Figure 1-1 shows a general block diagram of the HM encoder.

Figure 1-1 – Simplified block diagram of HM encoder

The picture partitioning structure, which is further described in Section 4.1, divides the input video into blocks called coding tree units (CTUs). These CTUs have a role that is broadly analogous to that of macroblocks in previous standards. A CTU is split using a quad-tree into coding units (CUs), with a leaf coding unit (CU) defining a region sharing the same prediction mode (intra, inter or skip). The leaf CU also defines the shape of prediction units (PUs) present, with each PU detailing the prediction information to be used for the respective picture region. Leaf CUs also define another quad-tree that defines the residual-quad-tree (RQT) containing transform units (TUs), which define a region sharing the same transformation and quantization process. The term ‘unit’ defines a region of an image covering all components; the term ‘block’ is used to define a region covering a particular component (e.g. luma), and may differ in spatial location when considering chroma sub-sampling such as 4:2:0.

The intra prediction processes, which are further described in Section 4.1, provide 35 modes (Planar, DC and 33 angular directions) for the luma of each PU, one of which is selected according to the intra prediction mode of a prediction block. Mode-dependent reference and prediction sample smoothing is applied to increase prediction efficiency and intra prediction mode is coded using either one of the three most probable modes (MPM) or one of the 32 remaining modes.

The inter picture prediction processes, which are further described in Section 4.4, select the motion parameters, which includes the option of a skip mode, a merge mode, with motion vectors being coded relative to predictors and able to describe offsets with ¼ pixel accuracy.

The transform and quantization processes, which are further described in Section 4.5, take the residuals generated by subtracting the prediction from the input and spatially transform and quantize them. In the transform process, matrices which are approximations to the DCT are used. In the case of 4x4 intra predicted residuals, an approximation to DST is used for the luma. For 8-bit video, 52-level quantization steps are permitted (the number of steps increases by 6 for each additional video bit depth). Reconstructed samples are created by inverse quantization and inverse transform.

Entropy coding, which is described in Section 4.9, is applied to the generated symbols and quantized transform coefficients in the encoding process using a Context-based Adaptive Binary Arithmetic Coding (CABAC) process.

Loop filtering is described in Section 4.11, which applies two in-loop filtering processes (namely deblocking filtering and sample adaptive offset (SAO) filtering) after the reconstructed pixel data is formed. The resulting image is stored in the decoded picture buffer (DPB) and may be used for inter coding predictions for future frames in coding order.

## Obtaining the HEVC test model

The current version of the 16th HEVC test model (HM-16.2) is available from the following locations:

https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.2

https://hevc.hhi.fraunhofer.de/trac/hevc/browser/tags/HM-16.2

## Obtaining the HEVC Standard

The HEVC Standard may be obtained from the ISO/IEC and ITU Standards bodies. The document prepared by the JCT-VC for consideration of Version 2 by ISO/IEC/MPEG and ITU/SG16-Q6[VCEG], which includes the Range Extension, can be found at:

http://phenix.int-evry.fr/jct/doc_end_user/documents/18_Sapporo/wg11/JCTVC-R1013-v6.zip

# Scope

This document provides an encoder-side description of the HEVC test model 16 (HM), which serves as a tutorial on the encoding algorithms implemented in the HM software. The purpose of this text is to establish a common understanding on reference encoding methods supported in the HM software, in order to facilitate the assessment of the technical impact of proposed new technologies during the HEVC standardization process. Although brief descriptions of the HEVC design are provided to help understanding of the HM, the corresponding sections of the HEVC draft specification [3] should be referred to for any descriptions regarding normative processes. Document [2] provides a summary of configuration options for the HM encoder, without going into detail of encoder algorithms. Documents [4] and [5] define the common test conditions and software reference configurations that should be used for experimental work.

# Encoder control of the HEVC test model 16

## Encoder configuration options

### File, format and profile/level/tier configuration options

Configuration options for the HEVC test model 16 associated with files, formats and profiles/levels/tiers are described in the JCTVC HM software manual [2].

### Coding tool configuration options

Error: Reference source not found provides a list of encoder configuration options for the HEVC test model 16 associated with coding tools.

Table 3-1 – Encoder configuration options for control of coding tools.

| Configuration option | Section reference |
| --- | --- |
| AlignCABACBeforeBypass | 4.8.1 |
| ChromaFormatIDC | 4.1 |
| CostMode | 6.9 |
| CrossComponentPrediction | 4.10 |
| ExplicitResidualDPCM | 4.7 |
| ExtendedPrecision | 4.5.8 |
| GolombRiceParameterAdaptation | 4.9.3 |
| HighPrecisionPredictionWeighting | 4.4.4.1 |
| ImplicitResidualDPCM | 4.7 |
| IntraReferenceSmoothing | 4.3.2 |
| LowerBitRateConstraintFlag | 5 |
| MaxBitDepthConstraint | 5 |
| MaxCUChromaQpAdjustmentDepth | 4.5.9 |
| MaxCUWidth | 4.2.3 |
| MaxCUHeight | 4.2.3 |
| MaxCUSize | 4.2.3 |
| MaxPartitionDepth | 4.2.3 |
| QuadtreeTULog2MaxSize | 4.2.5 |
| QuadtreeTULog2MinSize | 4.2.5 |
| QuadtreeTUMaxDepthIntra | 4.2.5 |
| QuadtreeTUMaxDepthInter | 4.2.5 |
| ReconBasedCrossCPredictionEstimate | 4.10 |
| ResidualRotation | 4.9.1 |
| SaoLumaOffsetBitShift | 4.11.3 |
| SaoChromaOffsetBitShift | 4.11.3 |
| SingleComponentLoopInterSearch | 6.10 |
| SingleSignificanceMapContext | 4.9.2 |
| TransformSkipLog2MaxSize | 4.5 |

### Slice coding parameters

Table 3-2 – Encoder configuration options for slice coding.

| SliceMode | 6.3 |
| --- | --- |
| SliceArgument | 6.3 |
| SliceSegmentMode | 6.3 |
| SliceSegmentArgument | 6.3 |
| WaveFrontSynchro | 4.12 |
| TileUniformSpacing | 6.3 |
| NumTileColumnsMinus1 | 6.3 |
| NumTileRowsMinus1 | 6.3 |
| TileColumnWidthArray | 6.3 |
| TileRowHeightArray | 6.3 |

### SEI message configuration options

SEI message configuration options are described in [2].

# Overview of tools in HEVC Version 1 and HEVC Version 2 RExt

## High level syntax

An overview of the HEVC high level syntax in provided in a tutorial paper [7].

## Picture partitioning

### Coding tree unit (CTU) partitioning

Pictures are divided into a sequence of coding tree units (CTUs), all being the same size, and each covering a square pixel region of the picture. An example of a picture divided into CTUs is shown in Figure 4-2. The size of a CTU is specified with respect to the luma channel, to prevent ambiguity when considering chroma formats.

The size of the CTU is configured as one of 16x16, 32x32 or 64x64 luma samples.

Figure 4-2 – Example of a picture divided into CTUs.

### Slice and tile structures

A slice is a data structure that can be decoded independently from other slices of the same picture, in terms of entropy coding, signal prediction, and residual signal reconstruction. A slice can either be the entire picture or a region of a picture, which is not necessarily rectangular. A slice consists of a sequence of one or more slice segments starting with an independent slice segment and containing all subsequent dependent slice segments (if any) that precede the next independent slice segment (if any) within the same access unit.

A slice segment consists of a sequence of CTUs. An independent slice segment is a slice segment for which the values of the syntax elements of the slice segment header are not inferred from the values for a preceding slice segment. A dependent slice segment is a slice segment for which the values of some syntax elements of the slice segment header are inferred from the values for the preceding independent slice segment in decoding order. For dependent slice segments, prediction can be performed across dependent slice segment boundaries, and entropy coding is not initialized at the starting of the dependent slice segment parsing process.

An example of picture with 11 by 9 CTUs that is partitioned into two slices is shown in Figure 4-3, below. In this example, the first slice is composed of an independent slice segment containing 4 CTUs, a dependent slice segment containing 32 CTUs, and another dependent slice segment containing 24 CTUs. The second slice consists of a single independent slice segment containing the remaining 39 CTUs of the picture.

Figure 4-3 – Example of slices and slice segments

A tile is a rectangular region containing an integer number of CTUs. CTUs are ordered in raster scan within a tile, and tiles in a picture are ordered consecutively in a raster scan of the tiles of the picture. This defines the coding order of CTUs, which is referred to as the tile scan order.

A tile may consist of CTUs contained in more than one slice. Similarly, a slice may consist of CTUs contained in more than one tile.  Note that within the same picture, there may be both slices that contain multiple tiles and tiles that contain multiple slices. However, one or both of the following conditions must be fulfilled for each slice and tile:

–	All coding tree units in a slice belong to the same tile.

–	All coding tree units in a tile belong to the same slice.

In addition, one or both of the following conditions must be fulfilled for each slice segment and tile:

–	All coding tree units in a slice segment belong to the same tile.

–	All coding tree units in a tile belong to the same slice segment.

Two examples of possible slice and tile structures for a picture with 11 by 9 coding tree units are shown in Figure 4-4, below. In both examples, the picture is partitioned into two tiles, separated by a vertical tile boundary. The left-hand example shows a case in which the picture only contains one slice, starting with an independent slice segment and followed by four dependent slice segments. The right-hand example illustrates an alternative case in which the picture contains two slices in the first tile and one slice in the second tile.

Figure 4-4 – Examples of tiles and slices

### Coding unit (CU) and coding tree structure

The coding unit (CU) is a square region (in terms of pixels/luma-samples), and is a node of the quad-tree partitioning of the CTU. The quad-tree partitioning structure allows recursive splitting into four equally sized nodes, starting from the CTU and stopping when no further splitting is signalled in the bit stream (as determined by an encoder) or when the minimum CU size is reached. The minimum CU size is configured in the SPS  to be 32x32, 16x16, or 8x8 luma samples; 8x8 is used in the common test conditions [4] and [5] for HEVC development. Each leaf node CU is configured to use a particular prediction mode, that being either intra prediction or inter-prediction. Figure 4-5 shows a CTU divided into multiple CUs.

Figure 4-5 – Example of coding tree structure

The quad-tree partitioning thus allows a content-adaptive coding tree structure comprised of CUs, each of which may be as large as the CTU or as small as 8x8.

The CTU size and minimum CU size are configured using the syntax elements log2_min_luma_coding_block_size_minus3 and log2_diff_max_min_luma_coding_block_size. The HM Encoder sets these syntax elements according to  MaxCUWidth, MaxCUHeight, MaxCUSize and MaxPartitionDepth. The maximum partition depth is defined as the maximum number of splits in the CU quad-tree and one split resulting from dividing a CU into partitions for prediction (to be described below). Thus, a CTU size of 64x64 and a maximum partition depth of 4 implies a minimum CU size of 8x8.

### Prediction unit (PU) structure

Each leaf CU is associated with one or more prediction units (PU) according to a partition mode and all PUs associated with a given CU have the same prediction mode. Each CU includes one, two or four PUs, depending on the partition mode of the CU. Figure 4-6 shows the eight partition modes that may be used to define the PUs for a CU.

Figure 4-6 – 8 partition modes for inter PU

For a CU configured to use intra prediction, only square PUs are available and thus only partition modes PART_NxN and PART_2Nx2N are available; the latter is only available when the CU size is equal to the configured minimum CU size.

For a CU configured to use inter-prediction, all eight partition modes are available. The availability of non-square PU sizes permits improved matching of boundaries of real objects in the picture. To reduce the worst case memory bandwidth of motion compensation, the 4x4 PU is prohibited and 4x8 and 8x4 PU sizes may only reference one reference picture. Other PU sizes can reference one or two reference picture.

A PU spans all colour channels and is generally associated with one prediction block (PB) for each colour channel. The exception being for a 4x4 PU where there may or may not be associated chroma PBs: for 4:2:0 chroma formats, an 8x8 CU that is split into four 4x4 PUs will have four luma PBs, but only one 4x4 PB per chroma channel for intra coded blocks. Fortunately for this exception, it will be seen that the transform block (TB) term is used instead.

### Transform unit (TU) and transform tree structure

The transform unit (TU) is a square region of size 8x8, 16x16 or 32x32 luma samples/pixels defined by a quadtree partitioning of a leaf CU. The quadtree partitioning of the CU into one or more TUs is known as a ‘residual quadtree’ (RQT). In general, each TU is associated with one transform block (TB) per colour channel. However, for the 4:2:2 chroma format (permitted in some RExt profiles) there must always be two TBs per chroma channel in order to ensure that the TBs are square (to match the defined transforms). In addition, in all chroma formats, the 8x8 TU may be split into four 4x4 luma TBs, but for the 4:2:0 format, the 8x8 TU will continue to have just one 4x4 chroma-sample (8x8 pixels) TB per chroma channel; similarly two 4x4 chroma-sample (8x4 pixels) TBs per chroma channel for 4:2:2. For 4:4:4, there will be four 4x4 chroma-sample (4x4 pixels) for each chroma channel. Figure 4-7 shows an example RQT.

Figure 4-7 – Example of transform tree structure within CU

The range of supported transform sizes is signalled in the bitstream using log2_min_luma_transform_block_size_minus2 and log2_diff_max_min_luma_transform_block_size. These values are specified using QuadtreeTULog2MaxSize and QuadtreeTULog2MinSize.

The maximum depth of the RQT is signalled in the bitstream independently for inter-predicted CUs and intra-predicted CUs using max_transform_hierarchy_depth_inter and max_transform_hierarchy_depth_intra, respectively in the SPS. These values are controlled using QuadtreeTUMaxDepthInter and QuadtreeTUMaxDepthIntra, respectively, in the HM Encoder.

For a CU configured to use inter-prediction, PU boundaries may occur within a given TU. For a CU configured to use intra prediction, PU boundaries cannot occur within a given TU, except for when an 8x8 TU is split into four 4x4 luma TBs (which effectively makes a virtual TU of 4x4 luma samples, with 4x4 luma samples also being the smallest PU size).

## Intra prediction

### Prediction modes

Intra prediction involves producing samples for a given TB using samples previously reconstructed in the considered colour channel. The intra prediction mode is separately signalled for the luma and chroma channels, with the chroma channel intra prediction mode optionally dependant on the luma channel intra prediction mode via the ‘DM_CHROMA’ mode. Although the intra prediction mode is signalled at the PB level, the intra prediction process is applied at the TB level, in accordance with the residual quad-tree hierarchy for the CU, thereby allowing the coding of one TB to have an effect on the coding of the next TB within the CU, and therefore reducing the distance to the samples used as reference values.

HEVC includes 35 intra prediction modes – a DC mode, a planar mode and 33 directional, or ‘angular’ intra prediction modes. The 33 angular intra prediction modes are illustrated in Figure 4-8 below.

Figure 4-8 – The 33 intra prediction directions

The mapping between the direction of each of the angular intra prediction modes and the intra prediction mode number is specified in Figure 4-9, below.

Figure 4-9 – Mapping between intra prediction direction and intra prediction mode

For PBs associated with chroma colour channels, the intra prediction mode is specified as either planar, DC, horizontal, vertical, ‘DM_CHROMA’ mode or sometimes diagonal mode ‘34’. Table 4-3 shows the rule specifying the chroma colour channel PB intra prediction mode given the luma colour channel PB intra prediction mode and the ‘intra_chroma_pred_mode’ syntax element.

Note for chroma formats 4:2:2 and 4:2:0, the chroma PB may overlap two or four (respectively) luma PBs; in this case the luma direction for DM_CHROMA is taken from the top left of these luma PBs.

The DM_CHROMA mode indicates that the intra prediction mode of the luma colour channel PB is applied to the chroma colour channel PBs. Since this is relatively common, the most-probable-mode coding scheme of the intra_chroma_pred_mode is biased in favour of this mode being selected.

Table 4-3 – Mapping between intra prediction direction and intra prediction mode for chroma

| intra_chroma_pred_mode | Luma intra prediction direction, X |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 26 | 10 | 1 | Otherwise ( 0 <= X <= 34 ) |
| 0 | 34 | 0 | 0 | 0 | 0 |
| 1 | 26 | 34 | 26 | 26 | 26 |
| 2 | 10 | 10 | 34 | 10 | 10 |
| 3 | 1 | 1 | 1 | 34 | 1 |
| 4 (DM_CHROMA) | 0 | 26 | 10 | 1 | X |

### Filtering of neighbouring samples

The neighbouring samples filtering process for intra prediction is skipped when intra_smoothing_disabled_flag is set to 1 (as configured by the IntraReferenceSmoothing enable flag). The intra reference smoothing filter is disabled in common test conditions [5] only when sequence-level lossless coding is used.

If the intra reference smoothing filter is enabled, then for the luma component, the neighbouring samples used for generation of intra-predicted samples are filtered. The filtering further is controlled by the given intra prediction mode and transform block size. If the intra prediction mode is DC or the transform block size is equal to 4x4, neighbouring samples are not filtered. If the distance between the given intra prediction mode and vertical mode (or horizontal mode) is larger than predefined threshold, the filtering process remains enabled (otherwise the filtering process becomes disabled). The predefined threshold is specified in Table 4-4, where nT represents the TB size.

Table 4-4 – Specification of predefined threshold for various transform block sizes

|  | nT = 8 | nT = 16 | nT = 32 |
| --- | --- | --- | --- |
| Threshold | 7 | 1 | 0 |

If filtering remains enabled, then either a neighbouring sample filtering, [1, 2, 1] or a bi-linear filter are used. The bi-linear filtering is used if all of the following conditions are true (otherwise the neighbouring sample filtering is used):

–	strong_intra_smoothing_enabled_flag is equal to 1

–	luma channel under consideration

–	transform block size is equal to 32

–	Abs( p[ −1 ][ −1 ] + p[ nT*2−1 ][ −1 ] – 2*p[ nT−1 ][ −1 ] ) < (1 << ( BitDepthY − 5 ))

–	Abs( p[ −1 ][ −1 ] + p[ −1 ][ nT*2−1 ] – 2*p[ −1 ][ nT−1 ] ) < (1 << ( BitDepthY − 5 ))

### Intra boundary filter

When reconstructing intra-predicted TBs an intra-boundary filter (IBF) may be used when predicting samples along the left and/or top edges of the TB for PBs using horizontal, vertical and DC intra prediction modes, as shown in Figure 4-10. For horizontal and vertical intra prediction modes, the IBF is disabled when implicit RDPCM and transquant bypass are enabled. For the DC intra prediction mode, the IBF is applied to the luma channel of TBs smaller than 32x32.

Figure 4-10 – Intra boundary filter example.

The intra boundary filter is defined with respect to an array of predicted samples p as input and predSamples as output as follows:

–	For horizontal intra-prediction applied to luma transform blocks of size less than 32x32, and disableIntraBoundaryFilter is equal to 0, the following filtering applies with x = 0..nTbS − 1, y = 0:

predSamples[ x ][ y ] = Clip1Y( p[ −1 ][ y ] + ( ( p[ x ][ −1 ] − p[ −1 ][ −1 ] )  >>  1 ) )

–	For vertical intra-prediction applied to luma transform blocks of size less than 32x32, and disableIntraBoundaryFilter is equal to 0, the following filtering applies with x = 0..nTbS − 1, y = 0:

predSamples[ x ][ y ] = Clip1Y( p[ x ][ −1 ] + ( ( p[ −1 ][ y ] − p[ −1 ][ −1 ] )  >>  1 ) )

–	For DC intra-prediction applied to luma transform blocks of size less than 32x32 the following filtering applies with x = 0..nTbS − 1, y = 0 (where dcVal is the DC predictor):

predSamples[ 0 ][ 0 ] = ( p[ −1 ][ 0 ] + 2 * dcVal + p[ 0 ][ −1 ] + 2 )  >>  2

predSamples[ x ][ 0 ] = ( p[ x ][ −1 ] + 3 * dcVal + 2 )  >>  2, with x = 1..nTbS − 1

predSamples[ 0 ][ y ] = ( p[ −1 ][ y ] + 3 * dcVal + 2 )  >>  2, with y = 1..nTbS − 1

### 4:2:2 chroma format mode adjustment

When the DM_CHROMA mode is selected (i.e. intra_chroma_pred_mode is equal to 4) and the 4:2:2 chroma format is in use, the intra prediction mode for a chroma PB is derived from intra prediction mode for the corresponding luma PB and 4:2:0/4:4:4 chroma as specified in Table 4-5.

Table 4-5 – Specification of intra prediction mode for 4:2:2 chroma (Proposal 2)

| intra pred mode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| intra pred mode for 4:2:2 chroma | 0 | 1 | 2 | 2 | 2 | 2 | 2 | 4 | 6 | 8 | 10 | 12 | 14 | 16 | 18 | 18 | 18 | 18 |
| intra pred mode |  | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| intra pred mode for 4:2:2 chroma |  | 22 | 22 | 23 | 23 | 24 | 24 | 25 | 25 | 26 | 27 | 27 | 28 | 28 | 29 | 29 | 30 | 30 |

The result of this mapping table is illustrated in Figure 4-11, which shows the intra prediction angles for the 4:2:2 chroma format.

Figure 4-11 – Intra prediction angle for 4:2:2 chroma format.

## Inter prediction

### Prediction modes

Each inter-predicted PU has a set of motion parameters consisting of one or two motion vectors and reference picture indices and reference picture list usage index to be used for inter-predicted sample generation and signalled in an explicit or implicit manner. When a CU is coded with skip mode, the CU is associated with one PU and has no significant residual coefficients, no coded motion vector delta or reference picture index. A merge mode is specified whereby the motion parameters for the current PU are obtained from neighbouring PUs, including spatial and temporal candidates. The merge mode can be applied to any inter-predicted PU, not only for skip mode. The alternative to merge mode is the explicit transmission of motion parameters, where motion vector, corresponding reference picture index for each reference picture list and reference picture list usage flag are signalled explicitly per each PU. When the reference picture list usage index signals that either one of two reference picture lists is to be used, the PU is produced from one reference block of samples. This mode is referred to as ‘uni-prediction’. For P-slices, this is the only supported mode of operation.  When the reference picture list usage index signals both reference picture lists are to be used, the PU is produced from two reference blocks of samples. This mode is referred to as ‘bi-prediction’. The following text provides the details on the inter prediction modes specified in HEVC. The description will start with the merge mode.

#### Derivation of candidates for merge mode

When a PU is predicted using merge mode, an index pointing to an entry in the merge candidates list is parsed from the bitstream and used to retrieve the motion information. The construction of this list is specified in the HEVC standard and can be summarized according to the following sequence of steps:

- Step 1: Initial candidates derivation

  - Step 1.1: Spatial candidates derivation

  - Step 1.2: Redundancy check for spatial candidates

  - Step 1.3: Temporal candidates derivation

- Step 2: Additional candidates insertion

  - Step 2.1: Creation of bi-predictive candidates

  - Step 2.2: Insertion of zero motion candidates

These steps are also schematically depicted in Figure 4-12. For spatial merge candidate derivation, a maximum of four merge candidates are selected among candidates that are located in five different positions. For temporal merge candidate derivation, a maximum of one merge candidate is selected among two candidates. Since constant number of candidates for each PU is assumed at decoder, additional candidates are generated when the number of candidates does not reach to maximum number of merge candidate (MaxNumMergeCand) which is signalled in slice header. Since the number of candidates is constant, index of best merge candidate is encoded using truncated unary binarization (TU). If the size of CU is equal to 8, all the PUs of the current CU share a single merge candidate list, which is identical to the merge candidate list of the 2Nx2N prediction unit.

In the following, the operations associated with the aforementioned steps are detaild.

Figure 4-12 – Derivation process for merge candidates list construction

#### Spatial candidates derivation

In the derivation of spatial merge candidates, a maximum of four merge candidates are selected among candidates located in the positions depicted in Figure 4-13. The order of derivation is A1, B1, B0, A0 and B2. Position B2 is considered only when any PU of position A1, B1, B0, A0 is not available (e.g. because it belongs to another slice or tile) or is intra coded. After candidate at position A1 is added, the addition of the remaining candidates is subject to a redundancy check which ensures that candidates with same motion information are excluded from the list so that coding efficiency is improved. To reduce computational complexity, not all possible candidate pairs are considered in the mentioned redundancy check. Instead only the pairs linked with an arrow in Figure 4-14 are considered and a candidate is only added to the list if the corresponding candidate used for redundancy check has not the same motion information. Another source of duplicate motion information is the “second PU” associated with partitions different from 2Nx2N. As an example, Figure 4-15 depicts the second PU for the case of Nx2N and 2NxN, respectively. When the current PU is partitioned as Nx2N, candidate at position A1 is not considered for list construction. In fact, by adding this candidate will lead to two prediction units having the same motion information, which is redundant to just have one PU in a coding unit. Similarly, position B1 is not considered when the current PU is partitioned as 2NxN.

Figure 4-13 – Positions of spatial merge candidate

Figure 4-14 – Candidate pairs considered for redundancy check of spatial merge candidates

Figure 4-15 – Positions for the second PU of Nx2N and 2NxN partitions

#### Temporal candidates derivation

In this step, only one candidate is added to the list. Particularly, in the derivation of this temporal merge candidate, a scaled motion vector is derived based on co-located PU belonging to the picture which has the smallest POC difference with current picture within the given reference picture list. The reference picture list to be used for derivation of the co-located PU is explicitly signalled in the slice header. The scaled motion vector for temporal merge candidate is obtained as illustrated by the dotted line in Figure 4-16, which is scaled from the motion vector of the co-located PU using the POC distances, tb and td, where tb is defined to be the POC difference between the reference picture of the current picture and the current picture and td is defined to be the POC difference between the reference picture of the co-located picture and the co-located picture. The reference picture index of temporal merge candidate is set equal to zero. A practical realization of the scaling process is described in the HEVC specification [3]. For a B-slice, two motion vectors, one is for reference picture list 0 and the other is for reference picture list 1, are obtained and combined to make the bi-predictive merge candidate.

Figure 4-16 – Illustration of motion vector scaling for temporal merge candidate

In the co-located PU (Y) belonging to the reference frame, the position for the temporal candidate is selected between candidates C0 and C1, as depicted in Figure 4-17. If PU at position C0 is not available, is intra coded, or is outside of the current CTU, position C1 is used. Otherwise, position C0 is used in the derivation of the temporal merge candidate.

Figure 4-17 – Candidate positions for temporal merge candidate, C0 and C1

#### Additional candidates insertion

Besides spatio-temporal merge candidates, there are two additional types of merge candidates: combined bi-predictive merge candidate and zero merge candidate. Combined bi-predictive merge candidates are generated by utilizing spatio-temporal merge candidates. Combined bi-predictive merge candidate is used for B-Slice only. The combined bi-predictive candidates are generated by combining the first reference picture list motion parameters of an initial candidate with the second reference picture list motion parameters of another. If these two tuples provide different motion hypotheses, they will form a new bi-predictive candidate. As an example, Figure 4-18 depicts the case when two candidates in the original list (on the left), which have mvL0 and refIdxL0 or mvL1 and refIdxL1, are used to create a combined bi-predictive merge candidate added to the final list (on the right). There are numerous rules regarding the combinations which are considered to generate these additional merge candidates,  defined in [3].

Figure 4-18 – Example of combined bi-predictive merge candidate

Zero motion candidates are inserted to fill the remaining entries in the merge candidates list and therefore hit the MaxNumMergeCand capacity. These candidates have zero spatial displacement and a reference picture index which starts from zero and increases every time a new zero motion candidate is added to the list. The number of reference frames used by these candidates is one and two for uni and bi-directional prediction, respectively. Finally, no redundancy check is performed on these candidates.

#### Motion estimation regions for parallel processing

To speed up the encoding process, motion estimation can be performed in parallel whereby the motion vectors for all prediction units inside a given region are derived in simultaneously. The derivation of merge candidates from spatial neighbourhood may interfere with parallel processing as one prediction unit cannot derive the motion parameters from an adjacent PU until its associated motion estimation is completed. To mitigate the trade-off between coding efficiency and processing latency, HEVC defines the Motion Estimation Region (MER) whose size is signalled in the picture parameter set using the “log2_parallel_merge_level_minus2” syntax element [3]. When a MER is defined, merge candidates falling in the same region are marked as unavailable and therefore not considered in the list construction.

### Motion vector prediction

Motion vector prediction exploits spatio-temporal correlation of motion vector with neighbouring PUs, which is used for explicit transmission of motion parameters. It constructs a motion vector candidate list by firstly checking availability of left, above temporally neighbouring PU positions, removing redundant candidates and adding zero vector to make the candidate list to be constant length. Then, the encoder can select the best predictor from the candidate list and transmit the corresponding index indicating the chosen candidate. Similarly with merge index signalling, the index of the best motion vector candidate is encoded using truncated unary. The maximum value to be encoded in this case is 2 (see Figure 4-19). In the following sections, details about derivation process of motion vector prediction candidate are provided.

#### Derivation of motion vector prediction candidates

Figure 4-19 summarizes derivation process for motion vector prediction candidate.

Figure 4-19 – Derivation process for motion vector prediction candidates

In motion vector prediction, two types of motion vector candidates are considered: spatial motion vector candidate and temporal motion vector candidate. For spatial motion vector candidate derivation, two motion vector candidates are eventually derived based on motion vectors of each PU located in five different positions as depicted in Figure 4-15..

For temporal motion vector candidate derivation, one motion vector candidate is selected from two candidates, which are derived based on two different co-located positions. After the first list of spatio-temporal candidates is made, duplicated motion vector candidates in the list are removed. If the number of potential candidates is larger than two, motion vector candidates whose index is larger than 1 are removed from the list. If the number of spatio-temporal motion vector candidates is smaller than two, additional zero motion vector candidates is added to the list.

#### Spatial motion vector candidates

In the derivation of spatial motion vector candidates, a maximum of two candidates are considered among five potential candidates, which are derived from PUs located in positions as depicted in Figure 4-13, those positions being the same as those of motion merge. The order of derivation for the left side of the current PU is defined as A0, A1,and  scaled A0,scaled A1. The order of derivation for the above side of the current PU is defined as B0, B1, B2, scaled B0, scaled B1, scaled B2. For each side there are therefore four cases that can be used as motion vector candidate, with two cases not required to use spatial scaling, and two cases where spatial scaling is used. The four different cases are summarized as follows.

- No spatial scaling

  - (1) Same reference picture list, and same reference picture index (same POC)

  - (2) Different reference picture list, but same reference picture (same POC)

- Spatial scaling

  - (3) Same reference picture list, but different reference picture (different POC)

  - (4) Different reference picture list, and different reference picture  (different POC)

The no-spatial-scaling cases are checked first followed by the spatial scaling. Spatial scaling is considered when the POC is different between the reference picture of the neighbouring PU and that of the current PU regardless of reference picture list. If all PUs of left candidates are not available or are intra coded, scaling for the above motion vector is allowed to help parallel derivation of left and above MV candidates. Otherwise, spatial scaling is not allowed for the above motion vector.

Figure 4-20 – Illustration of motion vector scaling for spatial motion vector candidate

In a spatial scaling process, the motion vector of the neighbouring PU is scaled in a similar manner as for temporal scaling, as depicted as Figure 4-20. The main difference is that the reference picture list and index of current PU is given as input; the actual scaling process is the same as that of temporal scaling.

#### Temporal motion vector candidates

Apart for the reference picture index derivation, all processes for the derivation of temporal merge candidates are the same as for the derivation of spatial motion vector candidates (see Figure 4-17). The reference picture index is signalled to the decoder.

### Interpolation filter

For the luma interpolation filtering, an 8-tap separable DCT-based interpolation filter is used for 2/4 precision samples and a 7-tap separable DCT-based interpolation filter is used for 1/4 precisions samples, as shown in Table 4-6.

Table 4-6 – 8-tap DCT-IF coefficients for 1/4th luma interpolation

| Position | Filter coefficients |
| --- | --- |
| 1/4 | { -1, 4, -10, 58, 17, -5, 1 } |
| 2/4 | { -1, 4, -11, 40, 40, -11, 4, -1 } |
| 3/4 | { 1, -5, 17, 58, -10, 4, -1 } |

Similarly, a 4-tap separable DCT-based interpolation filter is used for the chroma interpolation filter, as shown in Table 4-7.

Table 4-7 – 4-tap DCT-IF coefficients for 1/8th chroma interpolation

| Position | Filter coefficients |
| --- | --- |
| 1/8 | { -2, 58, 10, -2 } |
| 2/8 | { -4, 54, 16, -2 } |
| 3/8 | { -6, 46, 28, -4 } |
| 4/8 | { -4, 36, 36, -4 } |
| 5/8 | { -4, 28, 46, -6 } |
| 6/8 | { -2, 16, 54, -4 } |
| 7/8 | { -2, 10, 58, -2 } |

For the vertical interpolation for 4:2:2 and the horizontal and vertical interpolation for 4:4:4 chroma channels, the odd positions in Table 4-7 are not used, resulting in 1/4th chroma interpolation.

For the bi-directional prediction, the bit-depth of the output of the interpolation filter is maintained to 14-bit accuracy, regardless of the source bit-depth, before the averaging of the two prediction signals. The actual averaging process is done implicitly with the bit-depth reduction process as:

predSamples[ x, y ] = ( predSamplesL0[ x, y ] + predSamplesL1[ x, y ] + offset ) >> shift

where

shift = ( 15 – BitDepth ) and offset = 1 << ( shift – 1 )

### Weighted Prediction

A weighted prediction (WP) tool is provided by HEVC. WP corresponds to the equivalent tool present in AVC and is intended to improve the performance of inter prediction when the source material is subject to illumination variations, e.g. when using fading or cross-fading. It should be noted that WP is not enabled in the HM common test conditions [4] and [5].

The principle of WP is to replace the inter prediction signal P by a linear weighted prediction signal P’ (with weight w and offset o):

Uni-prediction: P’ = w × P + o

Bi-prediction: P’ = (w0 × P0 + o0 + w1 × P1 + o1) / 2

The applicable weights and offsets are selected by the encoder and are conveyed within the bitstream. L0 and L1 suffixes define List0 and List1 of the reference pictures list, respectively. Bit depth is maintained to 14 bit accuracy (in HEVC Version 1) before averaging the prediction signals, as for interpolation filters.

In the case of bi-prediction with at least one reference picture available in each list L0 and L1, the following formula applies to the explicit signalling of weighted prediction parameters relating to the luma channel:

predSamples[ x ][ y ] =

Clip3( 0, ( 1 << bitDepth ) − 1, ( predSamplesL0 [ x ][ y ] * w0 + predSamplesL1[ x ][ y ] * w1 + ( (o0 + o1 + 1) << log2WD) ) >> (log2WD + 1) )

where

log2WD = luma_log2_weight_denom + 14 - bitDepth

w0 = LumaWeightL0[ refIdxL0 ],  w1 = LumaWeightL1[ refIdxL1 ]

o0 = luma_offset_l0[ refIdxL0 ] * highPrecisionScaleFactor

o1 = luma_offset_l1[ refIdxL1 ] * highPrecisionScaleFactor

highPrecisionScaleFactor = (1 << ( bitDepth – 8 ) )		(except see Section 4.4.4.1)

A corresponding formula applies to the chroma channel and to the case of uni-prediction.

#### High precision offsets

A highPrecisionScaleFactor of ( 1 << ( bitDepth – 8 ) ) is applied to the weighted prediction offsets o0 and o1 when high_precision_offsets_enabled_flag is equal to zero. At higher bit depths, this factor increases in magnitude, reducing the performance of weighted prediction.

When the high_precision_offsets_enabled_flag is equal to one (permitted in RExt profiles), the offsets (o0 and o1) have the same precision as the input (i.e. the factor above is removed) in order to provide enough precision for the weighted prediction process.

The high_precision_offsets_enabled_flag is configured by HighPrecisionPredictionWeighting in the encoder.

## Transform and quantization (scaling)

In HEVC, each TB (which are 4x4, 8x8, 16x16 or 32x32 samples) is transformed, transform skipped or coded losslessly via a trans-quant-bypass mode.

The scaling and transformation processes at the decoder side for a transformed block are shown in Figure 4-21.

Figure 4-21 – Lossless scaling and transformation process (quantities illustrated for HEVC version 1 profiles)..

When transform skip (TS) is used, a bit-shift is applied instead of a transform. TS may be applied to 4x4 TBs for Main/Main10 profiles and TBs of any size for format range extensions profiles. In this case, the scaling and transformation processes at the decoder side are as shown in Figure 4-22.

Figure 4-22 – Transform skip scaling and transformation process (quantities illustrated for HEVC version 1 profiles)..

For the format range extensions profiles, transform skip is supported on all TU sizes. The maximum sized TU for which transform skip is available is signaled by log2_max_transform_skip_block_size_minus2 (as configured by TransformSkipLog2MaxSize)

If lossless mode is used, scaling and transformation process at the decoder side are as follows.

r[ x ][ y ] = TransCoeffLevel[ x ][ y ]

### Inverse transforms

The inverse transform is implemented as a vertical 1D transformation step operating on each column of residual coefficients (i.e. d[ x ][ y ]), following by a clipping step operating on the output of the vertical 1D transformation step (i.e. e[ x ][ y ]) and finally a horizontal 1D transformation step operating on each row of the output of the clipping step (i.e. g[ x ][ y ]). This process is illustrated in Figure 4-23 for HEVC version 1 profiles. The clipping of intermediate sample values g[ x ][ y ] ensures that these values can be represented with 16 bits.

Figure 4-23 – Inverse transformation process (quantities illustrated for HEVC version 1 profiles).

### 1D inverse transform matrices

The transform matrices are an approximation of mathematical DCT matrices with 8-bit integers (including sign), utilizing 6 bits of fractional accuracy. The matrices have been optimized for maximizing orthogonality. Smaller size transform matrices are embedded in larger size transform matrices, enabling reuse of a 32x32 matrix when performing 4x4, 8x8, 16x16, and 32x32 transforms.

A 4x4 DST is also provided and is used only for residuals of all 4x4 intra-predicted luma TBs.

In the HM implementation, the transform is applied using a partial butterfly structure for low computational complexity.

### Scaling and quantization

The quantized transform coefficients qij (i, j=0..nS-1) are derived from the transform coefficients dij (i, j=0..nS-1) as

qij = ( dij * f[QP%6] + offset  ) >> ( 29 + QP/6 – nS – BitDepth), with i,j = 0,...,nS-1

where

f[x] = {26214,23302,20560,18396,16384,14564}, x=0,…,5

228+QP/6–nS-BitDepth  < offset < 229+QP/6–nS-BitDepth

### Scaling lists

Scaling lists (c.f. quantization matrices) can be applied during the (inverse) quantization process. The scaling values are signalled in the PPS, and each possible TB size, colour component and prediction type (intra/inter) can have its own scaling list, except for 32x32 chroma blocks, which is used only for the 4:4:4 RExt chroma format.  For 16x16 and 32x32 scaling lists, the scaling lists are specified with an 8x8 grid of values which is value-repeated to the required size, along with a value used for the entry corresponding to the DC frequency location.

The scaling list for 32x32 chroma blocks is derived from the 16x16 chroma scaling list. This derivation is shown in Error: Reference source not found.

Figure 4-24 – Scaling lists

### Scaling lists for transform skipped TUs

Scaling lists are not used for any transform-skipped TUs, other than 4x4.

### Transform selection for the 4:2:2 chroma format

When the 4:2:2 chroma format is in use, a TU has a rectangular chroma block. In this case, the rectangular chroma blocks are divided into two square TBs per channel and existing square transforms are used for the TBs. A separate coded block flag is signalled for each TB. Intra prediction reconstruction occurs separately for the two square blocks within a rectangular chroma block, enabling the lower block to be predicted from the reconstructed upper block. Deblocking of these new boundaries is described in Section 4.11.

Figure 4-25 – Square transform arrangement for the 4:2:2 chroma format.

### Chroma QP initialization offset table

When the chroma format is set to 4:2:2 or 4:4:4, the chroma QPc is initialized according to the luma qPi using the formula Min( qPi, 51 ). In particular, the mapping relationship of Table 8-10 of [2] is not used.

### Extended precision processing

An extended_precision_processing_flag (as configured by ExtendedPrecision) is provided to allow increased internal precision, particularly for use at higher bit depths. When this flag is set to one, the internal width of the transform and the entropy coder (g_maxTrDynamicRange[channel]in the software model) are increased according to the selected bit depth. Figure 4-26 shows the bit depths in the HEVC encoding path.

Figure 4-26 – Diagram showing magnitude bit depths in HEVC encoding path

Table 4-8 shows the relationship between the internal precisions and the bit depth.

Table 4-8 – g_maxTrDynamicRange[channel]

| extended_precision_processing_flag | Bit depth[channel] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 |
| 1 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 15 |  |
| 0 | 15 |  |  |  |  |  |  |  |  |

### CU-adaptive chroma QP offset

A chroma QP offset adjustment may be signalled at the CU level, for CUs down to a particular depth (signalled via diff_cu_chroma_qp_adjustment_depth). The same QP offset may also be applied to subsequent sibling or child CUs within the CTU tree if the maximum depth is reached. This is similar in principle to the operation of delta QP signalling in the current HEVC specification. To provide additional flexibility in terms of number of offsets, each CU that invokes the mode may signal an index into an offset table. This offset table contains up to five pairs of chroma QP offset values (cb_qp_adjustment, cr_qp_adjustment). The offset table is signalled in the PPS, limiting the rate cost of providing the chroma QP offsets. If the table contains one offset then no index is signalled. The chroma QP adjustment values are restricted such that the total deviation from the luma QP is limited to ±12.

For a CU where the offset is applied, each chroma QP adjustment value in a pair is applied to the corresponding chroma component. The feature is globally enabled through use of a picture parameter set flag chroma_qp_adjustment_enabled_flag, and locally through a slice header flag slice_chroma_qp_adjustment_enabled_flag.

## Slice QP offset

The configuration  option dQPFile allows specification of a QP offset to be applied to all slices belonging to a picture. When this option is specified with a valid file name, the encoder reads a text file with as many lines as the pictures to be encoded and stores these values in the array m_aidQP in class TAppEncCfg where the i-th entry in this array contains the QP offset applied to all slice belonging to the picture with picture order count equal to i.

## Residual prediction in case of transquant bypass and transform skip

When lossless coding is used (i.e. cu_transquant_bypass_flag is equal to one) and implicit_rdpcm_enabled_flag is equal to one, the residues obtained from intra prediction are further predicted using DPCM. Residual DPCM is only applied when the intra prediction direction is either horizontal or vertical. For residues obtained from inter prediction, RDPCM is applied if explicit_rdpcm_enabled_flag is equal to one. In this case the encoder selects whether to apply RDPCM on the residues and if it is applied, whether to perform RDPCM along the horizontal or vertical direction. This decision is based on the sum of absolute difference (SAD) computed over the residues. The decision which minimises the SAD is selected as the best and signalled to the decoder using two binary flags: one to signal whether RDPCM is applied and one to signal the direction in which RDPCM is applied.

When lossy coding is used (i.e. cu_transquant_bypass_flag is equal to zero), RDPCM may be applied at TU level and only for those TUs which are transform skipped (e.g. 4×4 transform units). For intra TUs, RDPCM is always applied when implicit_rdpcm_enabled flag is equal to one for intra coded transform-skip TUs with horizontal or vertical intra prediction mode, so there is no additional signalling. For inter TUs, when explicit_rdpcm_enabled_flag is equal to one, the same mechanism used for lossless coding is followed. For intra and inter coding, RDPCM is computed using the reconstructed residues (i.e. after inverse quantization) in order to avoid any drift between the encoder and the decoder. In particular the process can be formalized as follows. Let rhor and rver be the residues obtained by the RDPCM application. Let r(i,j) be the residue obtained after inverse quantization and r(i, j) the original residue (i.e. obtained by either intra or inter prediction). Over an N×N block, the residues  and  are therefore defined as follows:

rhor(i,j)=Q(r(i,j))j=0 and 0≤i≤N−1Q(r(i,j)−r(i,j−1))0<j≤N−1 and 0≤i≤N−1

rver(i,j)=Q(r(i,j))i=0 and 0≤j≤N−1Q(r(i,j)−r(i−1,j))0<i≤N−1 and 0≤j≤N−1,

where Q denotes the forward quantization operation.

The implicit_rdpcm_enabled_flag and explicit_rdpcm_enabled_flag are controlled using the configuration options ImplicitResidualDPCM and ExplicitResidualDPCM respectively.

## Entropy coding

HEVC uses Context Adaptive Binary Arithmetic Coding (CABAC) and variable length codes. Each syntax element is binarised using a combination of context coded bins and bypass bins. Context coded bins have an associated context, indicating the probable symbol value. The cost of coding a context coded bin depends on the probability and whether the symbol to be coded is equal to the likely symbol value. Bypass coded bins have no associated context and have an equal cost for coding each of a 0 or 1 symbol. Context coding allows adaptation to the probability distribution of symbol values for a given bin. Further adaptation is provided by context selection. A given context coded bin may use one of several contexts, the context being selected based on information previously included in the bitstream.

Variable length codes are used above the slice layer and in the slice header.

### CABAC alignment

When cabac_bypass_alignment_enabled_flag is equal to one (as configured by AlignCABACBeforeBypass), the CABAC engine is bit-aligned (i.e. range is set to 256) prior to the coding of sign bits if there are any coefficients that require coeff_abs_level_remaining syntax elements. Note that once the CABAC engine is bit-aligned, it will remain bit-aligned until a context coded bin is encountered. Alignment prior to decoding equi-probable CABAC bins allows those bins to be read directly from the bit-stream.

Bit alignment results in bypass bins being aligned in the bitstream, i.e. a decoder can read a given bypass bin directly from the bitstream.

## Coefficient Coding

### Transform skip residual rotation

When transform_skip_rotation_enabled_flag is equal to one (as configured by ResidualRotation), the residual of a 4x4 transform-skipped block or transquant-bypass block is rotated by 180 degrees. Due to the symmetry of the scans used in HEVC, this is equivalent to reversing the scan orders.

### Significance map context modelling

When transform_skip_context_enabled_flag is equal to one (as configured by SingleSignificanceMapContext), a separate single context is used for the sig_coeff_flag for TUs that are transform-skipped or transquant-bypassed

### Rice parameter adaptation

When persistent_rice_adaptation_enabled_flag is equal to one (as configured by GolombRiceParameterAdaptation), an alternative mechanism for initializing the Rice parameter used for coding coeff_abs_level_remaining is available.

In this scheme, the 4×4 sub-blocks are divided into different categories (“sbType”). For each sub-block, the initial Rice parameter is derived based on previously coded sub-blocks in the same category. The categorization is based on whether the block is a transform-skip block (“isTSFlag”) or in trans-quant bypass (isTQBFlag) and whether it is the luma component

sbType = isLuma * 2 + (isTSFlag | | isTQBFlag)

Stats statCoeff are maintained for each sub-block type (sbType) depending on the absolute coefficient value (uiLevel):

if (uiLevel >= ( 3 << ( statCoeff[ sbType ] / 4 ) ) )		statCoeff[ sbType ] ++;

else	if ( ( 2 * uiLevel ) < ( 1 << ( statCoeff[ sbType ] / 4 ) ) )	statCoeff[ sbType ] --;

This variable is updated at most once per 4×4 sub-block using the value of the first coded coeff_abs_level_remaining of the sub-block. The entries of statCoeff are reset to 0 at the beginning of the slice (like the CABAC context variables).

The value of statCoeff is used to initialize the Rice parameter at the beginning of each 4×4 sub-block as

cRiceParam = Min( maxRicePara, statCoeff/4 ).

When this mechanism for initialization is enabled, the maximum Rice parameter value is unrestricted (i.e. limited only by the maximum transform dynamic range).

### Maximum coeff_abs_level_remaining codeword length restriction

When extended_precision_processing_flag is enabled, the maximum codeword length of the coeff_abs_level_remaining syntax element is limited to 32-bits.  The maximum codeword length is achieved with the Rice parameter is equal to zero.  The maximum codeword length is dependent on the dynamic range of transform and quantizer stages, referred to as ‘MAX_TR_DYNAMIC_RANGE’ in the HEVC Test Model 16.2 software.  When extended_precision_processing_flag is enabled, MAX_TR_DYNAMIC_RANGE is set equal to the bit-depth plus six bits.

Binarization of coeff_abs_level_remaining is modified such that the maximum prefix length is given by:

maximumPrefixLength = 32 – (3 + MAX_TR_DYNAMIC_RANGE)

When this prefix length is reached, the corresponding suffix length is then given by:

suffixLength = MAX_TR_DYNAMIC_RANGE – rParam

This results in a maximum codeword length for coeff_abs_level_remaining of 32-bits.

## Cross-component prediction

An adaptive cross-component residual prediction scheme (i.e., between colour channels) is provided, where a prediction is performed between the luma residual signal and the chroma residual signals. The chroma residual signal is predicted from the luma residual signal at the encoder side as:

ΔrC(x,y)=rC(x,y)−α×rL'(x,y)>>3					 (1)

and it is compensated at the decoder side as:

rC'(x,y)=ΔrC'(x,y)+α×rL'(x,y)>>3	 				(2)

where rC denotes the chroma residual sample at a position , rL' denotes the reconstructed residual sample of the luma component, ΔrC denotes the predicted signal using inter-colour prediction, ΔrC'denotes the reconstructed signal after coding and decoding , and rC' denotes the reconstructed chroma residual.

The variable α is chosen from −8, −4, −2, −1, 0, 1, 2, 4, 8. This set of values allows good utilization of the correlation between luma and chroma residual signals, including when they are negatively correlated. The absolute value of α, abs(α) is mapped to M(α) according to Table 4-9 before binarization. M(α) is binarized using truncated unary (TU) code and coded using CABAC, with a separate context for each bin in the TU code. If α is not zero, another bin is used to code the sign of α. At the decoder side, after CABAC decoding, M(α) is inversely mapped back to abs(α).

Table 4-9 – α Mapping Table.

| abs(α) | M(α) |
| --- | --- |
| 0 | 0 |
| 1 | 1 |
| 2 | 2 |
| 4 | 3 |
| 8 | 4 |

This prediction is performed both for intra- and inter-coded blocks. However, in case of intra-coded blocks, only those with DM chroma mode are allowed to use this prediction.

For each TU, if the coded block flag of the luma component is zero, α is not signaled and no prediction is performed. Otherwise, α is signalled separately for each chroma component.

When encoding RGB source material, the G component should be encoded as the luma component.

The use of cross-component prediction is signalled using cross_component_prediction_enabled flag and is configured using CrossComponentPrediction.

When reconstruction based cross-component prediction estimate is enabled, use the decoded residual rather than the pre-transform encoder-side residual for determining the alpha value. This is configured using ReconBasedCrossCPredictionEstimate.

If the luma bit-depth differs from the chroma bit-depth, for cross component prediction the luma residual is scaled to align with the chroma bit-depth by either a right-shift or a left-shift operation.

## Loop Filtering

### Overview of Loop filtering

HEVC includes two processing stages in the in-loop filter: a deblocking filter and then a Sample Adaptive Offset (SAO) filter. The deblocking filter aims to reduce the visibility of blocking artefacts and is applied only to samples located at block boundaries. The SAO filter aims to improve the accuracy of the reconstruction of the original signal amplitudes and is applied adaptively to all samples, by conditionally adding an offset value to each sample based on values in look-up tables defined by the encoder.

### Deblocking filter

A deblocking filter process is performed for each CU in the same order as the decoding process. First vertical edges are filtered (horizontal filtering) then horizontal edges are filtered (vertical filtering). Filtering is applied to 8x8 block boundaries which are determined to be filtered, both for luma and chroma components. 4x4 block boundaries are not processed in order to reduce the complexity.

Figure 4-27 illustrates the overall flow of deblocking filter processes. A boundary can have three filtering status values: no filtering, weak filtering and strong filtering. Each filtering decision is based on boundary strength, Bs, and threshold values, β and tC.

Figure 4-27 – Overall processing flow of deblocking filter process

#### Boundary decision

Two kinds of boundaries are involved in the deblocking filter process: TU boundaries and PU boundaries. CU boundaries are also considered, since CU boundaries are necessarily also TU and PU boundaries. When PU shape is 2NxN (N > 4) and RQT depth is equal to 1, TU boundaries at 8x8 block grid and PU boundaries between each PU inside the CU are also involved in the filtering.

#### Boundary strength calculation

The boundary strength (Bs) reflects how strong a filtering process may be needed for the boundary. A value of 2 for Bs indicates strong filtering, 1 indicates weak filtering and 0 indicates no deblocking filtering.,

Let P and Q be defined as blocks which are involved in the filtering, where P represents the block located to the left (vertical edge case) or above (horizontal edge case) the boundary and Q represents the block located to the right (vertical edge case) or above (horizontal edge case) the boundary. Figure 4-28 illustrates how the Bs value is calculated based on the intra coding mode, the existence of non-zero transform coefficients, reference picture, number of motion vectors and motion vector difference.

Figure 4-28 – Flow diagram for Bs calculation

Bs is calculated on a 4x4 block basis, but it is re-mapped to an 8x8 grid. The maximum of the two values of Bs which correspond to 8 pixels consisting of a line in the 4x4 grid is selected as the Bs for boundaries in the 8x8 grid.

At the CTU boundary, information on every second block (on a 4x4 grid) to the left or above is re-used as depicted in Figure 4-29, in order to reduce line buffer memory requirement.

Figure 4-29 – Referred information for Bs calculation at CTU boundary

#### Threshold variables

Threshold values β′ and tC′ are involved in the filter on/off decision, strong and weak filter selection and weak filtering process.  These are derived from the value of the luma quantization parameter Q as shown in Table 4-10 .

Table 4-10 – Derivation of threshold variables from input Q

| Q | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| β′ | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 6 | 7 | 8 |
| tC′ | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| Q | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 |
| β′ | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 20 | 22 | 24 | 26 | 28 | 30 | 32 | 34 | 36 |
| tC′ | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 4 | 4 | 4 |
| Q | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 |  |  |  |
| β′ | 38 | 40 | 42 | 44 | 46 | 48 | 50 | 52 | 54 | 56 | 58 | 60 | 62 | 64 | - | - |  |  |  |
| tC′ | 5 | 5 | 6 | 6 | 7 | 8 | 9 | 10 | 11 | 13 | 14 | 16 | 18 | 20 | 22 | 24 |  |  |  |

The variable β is derived from β′ as follows:

β = β′ * ( 1  <<  ( BitDepthY − 8 ) )

The variable tC is derived from tC′ as follows:

tC = tC′ * ( 1  <<  ( BitDepthY − 8 ) )

#### Filter on/off decision for 4 lines

The filter on/off decision is made using 4 lines grouped as a unit, to reduce computational complexity. Figure 4-30 illustrates the pixels involving in the decision. The 6 pixels in the two red boxes in the first 4 lines are used to determine whether the filter is on or off for those 4 lines. The 6 pixels in the two red boxes in the second group of 4 lines are used to determine whether the filter is on or off for the second group of 4 lines.

Figure 4-30 – Pixels involved in filter on/off decision and strong/weak filter selection

The following variables are defined:

dp0 = | p2,0 – 2*p1,0 + p0,0 |

dp3 = | p2,3 – 2*p1,3 + p0,3 |

dq0 = | q2,0 – 2*q1,0 + q0,0 |

dq3 = | q2,3 – 2*q1,3 + q0,3 |

If dp0+dq0+dp3+dq3 < β, filtering for the first four lines is turned on and the strong/weak filter selection process is applied. If this condition is not met, no filtering is done for the first 4 lines.

Additionally, if the condition is met, the variables dE, dEp1 and dEp2 are set as follows:

dE is set equal to 1

If dp0 + dp3 < (β + ( β >> 1 )) >> 3, the variable dEp1 is set equal to 1

If dq0 + dq3 < (β + ( β >> 1 )) >> 3, the variable dEq1 is set equal to 1

A filter on/off decision is made in a similar manner as described above for the second group of 4 lines.

#### Strong/weak filter selection for 4 lines

If filtering is turned on, a decision is made between strong and weak filtering. The pixels involved are the same as those used for the filter on/off decision, as depicted in Figure 4-30. If the following two sets of conditions are met, a strong filter is used for filtering of the first 4 lines. Otherwise, a weak filter is used.

1) 2*(dp0+dq0) < ( β >> 2 ), | p30 – p00 | + | q00 – q30 | < ( β >> 3 ) and | p00 – q00 | < ( 5* tC  + 1 ) >> 1

2) 2*(dp3+dq3) < ( β >> 2 ), | p33 – p03 | + | q03 – q33 | < ( β >> 3 ) and | p03 – q03 | < ( 5* tC  + 1 ) >> 1

The decision on whether to select strong or weak filtering for the second group of 4 lines is made in a similar manner.

#### Strong filtering

For strong filtering, the filtered pixel values are obtained by the following equations. Note that three pixels are modified using four pixels as an input for each P and Q block, respectively.

p0’ = ( p2 + 2*p1 + 2*p0 + 2*q0 + q1 + 4 ) >> 3

q0’ = ( p1 + 2*p0 + 2*q0 + 2*q1 + q2 + 4 ) >> 3

p1’ = ( p2 + p1 + p0 + q0 + 2 ) >> 2

q1’ = ( p0 + q0 + q1 + q2 + 2 ) >> 2

p2’ = ( 2*p3 + 3*p2 + p1 + p0 + q0 + 4 ) >> 3

q2’ = ( p0 + q0 + q1 + 3*q2 + 2*q3 + 4 ) >> 3

#### Weak filtering

D is defined as follows.

D = ( 9 * ( q0 –  p0 ) – 3 * ( q1 – p1 ) + 8 ) >> 4

When abs(D) is less than tC *10,

D = Clip3( - tC , tC , D )

p0’ = Clip1Y( p0 + D )

q0’ = Clip1Y( q0 - D )

If dEp1 is equal to 1,

Dp = Clip3( -( tC  >> 1), tC  >> 1, ( ( ( p2 + p0 + 1 ) >> 1 ) – p1 + D ) >>1 )

p1’ = Clip1Y( p1 + Dp )

If dEq1 is equal to 1,

Dq = Clip3( -( tC  >> 1), tC  >> 1, ( ( ( q2 + q0 + 1 ) >> 1 ) – q1 – D ) >>1 )

q1’ = Clip1Y( q1 + Dq )

Note that a maximum of two pixels are modified using three pixels as an input for each P and Q block, respectively.

#### Chroma filtering

The boundary strength Bs for chroma filtering is inherited from luma. If Bs > 1, chroma filtering is performed. No filter selection process is performed for chroma, since only one filter can be applied. The filtered sample values p0’ and q0’ are derived as follows.

D = Clip3( -tC, tC, ( ( ( ( q0 – p0 ) << 2 ) + p1 – q1 + 4 ) >> 3 ) )

p0’ = Clip1C( p0 + D )

q0’ = Clip1C( q0 - D )

When the 4:2:2 chroma format is in use, each chroma block has a rectangular shape and is coded using up to two square transforms. This process introduces additional boundaries between the transform blocks in chroma. These boundaries are not deblocked (blue dotted lines in Figure 4-31.)

Figure 4-31 – Deblocking behaviour in the 4:2:2 chroma format.

### Sample adaptive offset filter

Sample adaptive offset (SAO) is applied to the reconstructed signal after the deblocking filter by using offsets specified for each CTB by the encoder. The HM encoder first makes the decision on whether or not the SAO process is to be applied for current slice. If SAO is applied for the slice, each CTB is classified as one of five SAO types as shown in Table 4-11. The concept of SAO is to classify pixels into categories and reduces the distortion by adding an offset to pixels of each category. SAO operation includes Edge Offset (EO) which uses edge properties for pixel classification in SAO type 1-4 and Band Offset (BO) which uses pixel intensity for pixel classification in SAO type 5. Each applicable CTB has SAO parameters including sao_merge_left_flag, sao_merge_up_flag, SAO type and four offsets. If sao_merge_left_flag is equal to 1, the current CTB will reuse the SAO type and offsets of the CTB to the left. If sao_merge_up_flag is equal to 1, the current CTB will reuse SAO type and offsets of the CTB above.

Table 4-11 – Specification of SAO type

| - SAO type | - sample adaptive offset type to be used | - Number of categories |
| --- | --- | --- |
| - 0 | - None | - 0 |
| - 1 | - 1-D 0-degree pattern edge offset | - 4 |
| - 2 | - 1-D 90-degree pattern edge offset | - 4 |
| - 3 | - 1-D 135-degree pattern edge offset | - 4 |
| - 4 | - 1-D 45-degree pattern edge offset | - 4 |
| - 5 | - band offset | - 4 |

#### Operation of each SAO type

Edge offset uses four 1-D 3-pixel patterns for classification of the current pixel p by consideration of edge directional information, as shown in Figure 4-32. From left to right these are: 0-degree, 90-degree, 135-degree and 45-degree.

|  | p |  |  |  | p |  |  |  | p |  |  |  | p |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Figure 4-32 – Four 1-D 3-pixel patterns for the pixel classification in EO

Each CTB is classified into one of five categories according to Table 3-9.

Table 4-12 – Pixel classification rule for EO

| - Category | - Condition | - Meaning |
| --- | --- | --- |
| - 0 | - None of the below | - Largely monotonic |
| - 1 | - p < 2 neighbours | - Local minimum |
| - 2 | - p < 1 neighbour && p == 1 neighbour | - Edge |
| - 3 | - p > 1 neighbour && p == 1 neighbour | - Edge |
| - 4 | - p > 2 neighbours | - Local maximum |

Band offset (BO) classifies all pixels in one CTB region into 32 uniform bands by using the five most significant bits of the pixel value as the band index. In other words, the pixel intensity range is divided into 32 equal segments from zero to the maximum intensity value (e.g. 255 for 8-bit pixels). Four adjacent bands are grouped together and each group is indicated by its most left-hand position as shown in Figure 4-33. The encoder searches all position to get the group with the maximum distortion reduction by compensating offset of each band.

Figure 4-33 – Four bands are grouped together and represented by its starting band position

##### Format range extensions options for SAO

Shift values for luma and chroma for the offset value are included in the PPS. There is no change to classification and the shift values are in the range of 0 to Max(BitDepth – 10, 0). The shift values for luma and chroma are configured in the encoder using SaoLumaOffsetBitShift and SaoChromaOffsetBitShift, respectively.

## Wavefront parallel processing

Wavefront Parallel Processing (WPP) produces a bitstream that can be processed using one or more cores running in parallel. When WPP is used, a slice is divided into rows of CTUs (per tile). The first row is processed in an ordinary way, the second row can begin to be processed after only two CTUs have been processed in the first row, the third row can begin to be processed after only two CTUs have been processed in the second row, and so on. The context models of the entropy coder in each row are inferred from those in the preceding row with a two-CTU processing lag. WPP provides a form of processing parallelism within a slice, without the loss of compression performance that might be expected by using tiles within a slice.

The following operations are performed by the HM encoder.

- When starting the encoding of the first CTU in a CTU row, the following process is applied:

- if the last CU of the second CTU of the row above is available, the CABAC probabilities are set to the values stored in the buffer.

- if not, the CABAC probabilities are reset to the default values

- When the encoding of the second CTU in a CTU row is finished, the CABAC probabilities are stored in a buffer

- If the encoding of the last CTU in a CTU row is finished and the end of a slice has not been reached, CABAC is flushed and a byte alignment is performed.

Entry point offsets are written in the slice header. Each CTU row in the slice has an entry point offset, in byte units, that indicates where the corresponding data starts in the slice data. When WPP is used, a slice that does not start at the beginning of a CTU row does not finish after the last CTU in the same row. When a slice starts at the beginning of a CTB row, there is no constraint on where it finishes.

In all but the high throughput RExt profiles, WPP and tiles cannot be used simultaneously.

WPP is enabled in the HM Encoder by setting WaveFrontSynchro to 1.

# Profiles, Levels and Tiers

The HEVC test model 16 uses particular tools in accordance with the specified profile. To enable the use of tools available in format range extensions profiles, the Profile encoder option is set to the value main-RExt or high-RExt.

For the main or main10 profiles, set the Profile encoder option to the value main or main10, respectively.

Further control of the profile of the format range extensions profiles is provided as shown in Table 6-13.

Table 6-13 – Bitstream indications for format range extensions profiles.

| Configuration option | Description |
| --- | --- |
| IntraConstraintFlag | When the encoder is configured with IntraConstraintFlag, the general_intra_constraint_flag is set accordingly. If this flag is set to 1, only intra slices may be used. |
| LowerBitRateConstraintFlag | When the encoder is configured with LowerBitRateConstraintFlag, the general_lower_bit_rate_constraint_flag is set accordingly. Note that LowerBitRateConstraintFlag cannot be false if IntraConstraintFlag is false. |
| MaxBitDepthConstraint | Specifies the maximum allowed luma and chroma internal bit-depths. If no value (or 0) is specified, the MaxBitDepthConstraint is assumed to be the internal bit depth. The value of MaxBitDepthConstraint controls the setting of the following syntax elements: - general_max_12bit_constraint_flag - general_max_10bit_constraint_flag - general_max_8bit_constraint_flag |
| MaxChromaFormatConstraint | Specifies the maximum allowed chroma format. If no value (or 0) is specified, the MaxChromaFormatConstraint is assumed to be that of the internal chroma format. The value of MaxChromaFormatConstraint controls the setting of the following syntax elements: - general_max_422chroma_constraint_flag - general_max_420chroma_constraint_flag - general_max_monochrome_constraint_flag |

The following syntax elements specify the profile being used:

- general_max_12bit_constraint_flag

- general_max_10bit_constraint_flag

- general_max_8bit_constraint_flag

- general_max_422chroma_constraint_flag

- general_max_420chroma_constraint_flag

- general_max_monochrome_constraint_flag

- general_intra_constraint_flag

- general_one_picture_only_constraint_flag (HM always writes a ‘0’ for this flag)

- general_lower_bit_rate_constraint_flag

- general_profile_idc (set to 4 for all profiles currently specified for the format range extensions profiles).

The encoder does not currently verify that the combination of constraints flags describe a format range extensions profile. However, it is likely that automatic generation of the constraint flags will follow the rules defined in Table 6-14.

Table 6-14 – Mapping between user configuration and automatically calculated bitstream format range extensions profile indication.

| Mode | Chroma format | General RExt tools | Extended precision | Chroma QP tool | Maximum bit depth of all components |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 8 | 9, 10 |  | 11, 12 |  | 13, 14, 15, 16 |
| Intra | 4:0:0 | None | Off | n/a | Main Intra | Main 10 Intra |  | Main 12 Intra |  | Main 4:4:4 16 Intra |
|  | 4:2:0 | None | Off | Off |  |  |  |  |  |  |
|  |  |  |  | On | Main 4:2:2 10 Intra |  |  | Main 4:2:2 12 Intra |  |  |
|  | 4:2:2 | None | Off | - |  |  |  |  |  |  |
|  | 4:4:4 | None | Off | - | Main 4:4:4 Intra | Main 4:4:4 10 Intra |  | Main 4:4:4 12 Intra |  |  |
|  | - | Some/All | Off | - |  |  |  |  |  |  |
|  | - | - | On | - |  |  |  |  |  |  |
| Inter | 4:0:0 | None | Off | n/a | Monochrome 12 |  |  |  |  | Monochrome 16 |
|  |  | - | On | n/a |  |  |  |  |  |  |
|  |  | Some/All | Off | n/a |  |  |  |  |  |  |
|  | 4:2:0 | None | Off | Off | Main 12 |  |  |  |  |  |
|  |  | - | On | - |  |  |  |  |  |  |
|  |  | Some/All | Off | - | Main 4:4:4 | Main 4:4:4 10 |  | Main 4:4:4 12 |  |  |
|  |  | None | Off | On | Main 4:2:2 10 |  |  | Main 4:2:2 12 |  |  |
|  | 4:2:2 | None | Off | - |  |  |  |  |  | n/a |
|  |  | - | On | - |  |  |  |  |  |  |
|  |  | Some/All | Off | - | Main 4:4:4 | Main 4:4:4 10 |  | Main 4:4:4 12 |  |  |
|  | 4:4:4 | - | Off | - |  |  |  |  |  |  |
|  |  | - | On | - |  |  |  |  |  |  |

# Description of HM Encoder and encoding methods

## Cost Functions

Various cost functions are used in the HM software encoder to determine costs used in making encoder decisions. This section documents the cost functions used in the encoding process of the HM software.

### Sum of Square Error (SSE)

The difference between two blocks with the same block size is produced using

Diff(i,j) = BlockA(i,j) - BlockB(i,j)		(7-)

SEE is computed using the following equation:

SSE=i,jDiff(i,j)2		(7-)

### Sum of Absolute Difference (SAD)

SAD is computed using the following equation:

SAD=i,jDiff(i,j)		(7-)

### Hadamard transformed SAD (SATD)

Since the transformed coefficients are coded, an improved estimation of the cost of each mode can be obtained by estimating DCT with the Hadamard transform.

SATD is computed using:

SATD=(i,jDiffT(i,j))2		(7-)

The Hadamard transform flag can be turned on or off. SA(T)D refers to either SAD or SATD depending on the status of the Hadamard transform flag.

SAD is used when computing full-pel motion estimation while SA(T)D is used for sub-pel motion estimation.

### RD cost functions

#### Lagrangian constant values

In the HM encoder, lambda values that are used for cost computation are defined as

λmode=α∗Wk∗2((QP−12)3.0)		(7-)

λpred  =λmode																		(7-)

α=1.0−Clip3(0.0,0.5,0.05∗numberof)1.0						 		(7-)

, for referenced pictures

, for non-referenced pictures

Wkrepresents weighting factor dependent to encoding configuration and QP offset hierarchy level of current picture within a GOP, as specified in Table 7-15. Note that the value of Wkderived from Table 7-15 is further modified by multiplying 0.95 when SATD based motion estimation is used.

Table 7-15 – Derivation of

| k | QP offset hierarchy level | Slice type | Referenced | Wk |
| --- | --- | --- | --- | --- |
| 0 | 0 | I | - | 0.57 |
| 1 | 0 | P or B | 1 | RA: 0.442 LD: 0.578 |
| 2 | 1, 2 | P or B | 1 | RA: 0.3536 * Clip3( 2.0, 4.0, (QP-12)/6.0 ) LD: 0.4624 * Clip3( 2.0, 4.0, (QP-12)/6.0 ) |
| 4 | 3 | B | 0 | RA: 0.68 * Clip3( 2.0, 4.0, (QP-12)/6.0 ) |

#### Weighting factor for chroma component

The following weighting parameter wchroma is used to derive lambda value λchromato be used for chroma-specific decisions in RDOQ and SAO processes.

wchroma=2QP−QPchroma3														(7-)

With this parameter, λchromais obtained by

λchroma=λmodewchroma														(7-)

Note that the parameter wchroma is also used to define the cost function used for mode decisions in order to weight the chroma part of SSE.

#### SAD based cost function for prediction parameter decision

The cost for prediction parameter decision Jpred,SAD is specified by the following formula.

Jpred,SAD =SAD + λpred * Bpred, 														(7-)

where Bpred specifies bit cost to be considered for making decision, which depends on each decision case. λpred and SAD are defined in the section 6.1.4.1 and 6.1.1, respectively.

#### SATD based cost function for prediction parameter decision

The cost for motion parameter decision Jpred,SATD is specified by the following formula.

Jpred,SATD =SATD + λpred * Bpred, 													(7-)

where Bpred specifies bit cost to be considered for making decision, which depends on each decision case. λpred and SATD are defined in the section 6.1.4.1 and 6.1.3, respectively.

#### Cost function for mode decision

The cost for mode decision Jmode is specified by the following formula.

Jmode =(SSEluma+ wchroma *SSEchroma)+ λmode * Bmode,									(7-)

where Bmode specifies bit cost to be considered for mode decision, which depends on each decision case. λmode and SSE are defined in the section 6.1.4.1 and Error: Reference source not found, respectively.

## Encoder configurations

### Overview of encoder configurations

The HM encoder supports three kinds of prediction structures, as used in the common test conditions [4] and [5].  These prediction structures are: intra-only, low-delay and random access. The reference picture list management depends on the temporal configuration.

### Intra-only configuration

For intra-only coding, each picture in the source material is encoded as an IDR picture. No temporal reference pictures are used. QP does not change during a picture. Figure 7-34 gives graphical presentation of an intra-only configuration, where the number associated with each picture represents the encoding order.

Figure 7-34 – Graphical presentation of intra-only configuration

### Low-delay configurations

Two coding configurations have been defined for testing low-delay coding performance, referred to as ‘low-delay P’ and ‘low-delay B’. For low-delay coding conditions, only the first picture in a video sequence is encoded as an IDR picture. Subsequent pictures are each encoded using a P-slice for low-delay P mode or a B-slice for low-delay B mode. For both modes, the P or B slices may only reference pictures preceding the current picture in display order. For low-delay B mode, both reference lists RefPicList0 and RefPicList1 are identical.

Figure 7-35 shows a graphical presentation of this low-delay. The number associated with each picture represents the encoding order. The QP of each inter coded picture is derived by adding an offset to the QP of the intra coded picture depending on the temporal layer.

Figure 7-35 – Graphical presentation of low-delay configuration

### Random-access configuration

For the random-access test condition, a hierarchical B structure is used for encoding. Figure 7-36 shows a graphical representation of a random-access configuration, where the number associated with each picture represents the encoding order. An intra-picture is encoded at approximately one second intervals in accordance with the IntraPeriod configuration option, configured based on the frame rate of the source material. The first intra-picture of a video sequence is encoded as an IDR picture and the other intra pictures are encoded as non-IDR intra pictures (“Open GOP”). The pictures located between successive intra pictures in display order are encoded as B-pictures. The inter-predicted picture (referred to as a ‘Generalised P and B picture) in Figure 7-36 below) is used as the lowest temporal layer that can refer to intra- or inter-pictures for inter prediction. The second and third temporal layers consist of referenced B pictures, while the highest temporal layer contains non-referenced B picture only. The QP of each inter-coded picture is derived by adding an offset to the QP of the intra-coded picture depending on the temporal layer. The reference picture list combination is used for management and entropy coding of the reference picture index.

Figure 7-36 – Graphical presentation of random-access configuration

## Slice and tile partitioning operation

The HM encoder can partition a picture into several slices and tiles. Slices have spatial granularity equal to CTU. The HM encoder has three ways of determining slice size: by specifying the maximum number of CTUs in a slice, by specifying the number of bytes in a slice, and by specifying the number of tiles. The mode is controlled by SliceMode, with SliceArgument used to control the maximum number of CTUs, bytes or tiles in a slice, depending on the SliceMode setting. In addition, the controls for a slice segment are specified separately to those of a slice (using SliceSegmentMode and SliceSegmentArgument), and so allowing streams to contain a mixture of independent and dependent slice segments. Note that if the mode is set to limit the CTUs by the maximum number of bytes per slice or by the maximum number of CTUs, then slices will always terminate at the end of every tile and in addition, if a slice starts mid-way along a CTU row of a tile then it is terminated within the same row when wavefront parallel processing is enabled; similarly for slice segments.

Tiles are configured by the number of columns and rows (NumTileColumnsMinus1 and NumTileRowsMinus1 respectively), and are either specified as having a uniform spacing (TileUnformSpacing equal to 1), or the individual tile column widths (using TileColumnWidthArray) and tile row heights (using TileRowHeightArray) are used. Tiles do not need to be enabled: they are automatically enabled if more than one tile row or column is specified. The five encoder parameters correspond to syntax elements num_tile_colums_minus1, num_tile_rows_minus1, uniform_spacing_flag, col_width_minus1[i], and row_height_minus1[i].

## Derivation process for slice-level coding parameters

### Sample Adaptive Offset (SAO) parameters

#### Search the SAO type with minimum rate-distortion cost

In the HM encoder, the following process is performed to determine the SAO parameters:

- Loop three colour components in a CTB as following

- Collect the statistical information for all SAO type as following

  - Set sao_type_idx = 0

  - Classified pixels into categories according to sao_type_idx.

    - Calculate sum of difference between original signal and reconstructed signal in each category.

    - Calculate number of pixels in each category.

    - Calculate offsets using step 2.2.1 and step 2.2.2.

    - Calculate RD-cost according to section 3.7.2.2

  - Set sao_type_idx = sao_type_idx+1; if sao_type_idx <= 5, run step 2.2; otherwise, end.

- Determine the SAO parameters with lowest rate-distortion (RD) cost among the following three items.

  - If left CTB is available, calculate the RD cost by reusing the SAO parameters of left CTB.

  - If upper CTB is available, calculate the RD cost by reusing SAO parameters of upper CTB.

  - Five SAO types with minimum RD-cost in step 2.

- Update pixels in DPB according to selected SAO type by adding offset.

- Run step1, 2, 3, and 4 for next CTB until all CTB is processed.

#### Slice level on/off Control

A hierarchical coding of pictures is used for both low delay and random access configurations which allows the encoder to enable or disable SAO for picture with higher QP according to the percentage of CTBs to use SAO from the previous picture with lower QP. If previous picture with lower QP had more than 25% of CTBs using SAO type from 1-5, SAO will be enabled for the current picture, otherwise SAO will be disabled for the current picture.

### Adaptive QP selection

When this tool is used, the quantization parameter (QP) for each slice is changed based on the distribution of quantized coefficients in previous pictures. More specifically, for the current slice, the QP used is given as the one which minimizes the following cost measure:

minqlicl,i−l⋅q2

where q denotes the quantization step associated to QP and cl,i denotes the i-th coefficient which is quantised to the level l. As stated above, the optimal QP derivation is computed using data from the previously coded picture to avoid two-pass encoding. The optimal quantization step q is then translated into the corresponding QP according to the mapping described in [1] and set as QP for the slice being encoded.

### Adaptive search range for motion estimation

HM provides an Adaptive Search Range (ASR) algorithm which varies the extent of the region used to search the best motion vector during motion estimation. The variation of the search range depends on the temporal distance between the current and the reference picture. This tool is enabled with the --ASR option and computes the new search range when the encoding of each slice starts.

More precisely, the search range for the current picture (SRcurr) is modified as follows:

SRcurr=SRori⋅(POCcurr−POCref)+θgopSize,

where · denotes the floor operation, SRori denotes the original search range set from command line (64 in common test conditions [4], [5]), POCcurr and POCref are the picture order count for the current and reference picture, respectively, gopSize is the size of the group of picture and θ is a rounding offset equal to gopSize/2. The value of SRcurr is then clipped in the range [8, SRori].

## Derivation process for CU-level and PU-level coding parameters

### Intra prediction mode and parameters

The encoder selects an intra prediction mode for a PU as follows:

- A candidate mode derivation step tests all possible prediction modes for the luma PB with an approximate prediction cost Jpred,SATD specified in the section 6.1.4.4. A pre-determined number of intermediate candidates are found for each PU size (8 for 4x4 and 8x8 PUs, 3 for other PU sizes). In this step, the number of coded bits for an intra prediction mode is set to Bpred.

- An RD optimization step, using the coding cost Jmode specified in the section 6.1.4.5, is applied to the previously determined candidate modes. During this step, prediction parameters and coefficients for luma component of the PU are accumulated into Bmode. Regarding the chroma PB mode decision, all possible intra chroma prediction modes are evaluated through RD decision process, where coded bits for intra chroma prediction mode and chroma coefficient are used as Bmode.

#### Rate-distortion penalty for intra coding

This tool is enabled with the configuration option RDpenalty and provides a fast mode decision for intra coding. Configuration option RDpenalty can take three values: 0 (i.e. disabled), 1 and 2. When the value is equal to one the encoder avoids splitting a transform unit when its size is smaller than 16 × 16 and its associated slice is not intra. When RDpenalty is set to 2, transform units with size 32 × 32 in non intra slices are not checked and the RQT search moves to the next level of recursion.

### Inter prediction mode and parameters

#### Derivation of motion parameters

In the HM encoder, an inter-coded CU can be segmented into multiple inter-predicted PUs, each of which has a set of motion parameters consisting of one or more motion vectors (per RefPicListX), corresponding reference picture indices (ref_idx_lX) and prediction direction index (inter_pred_flag). Note that the current common test conditions those includes inter prediction coding are adopting reference picture list combination process, which is the case “X=c”. An inter-coded CU can be encoded with one of the following coding modes (“PredMode”): MODE_SKIP, MODE_INTER. For MODE_SKIP case, any sub-partitioning to smaller PUs is not allowed and its motion parameters are assigned to the CU itself, where the PU size is PART_2Nx2N. On the contrary, up to eight types of further partitioning to smaller PUs can be allowed for a CU coded with MODE_INTER. The PredMode and the CU partitioning shape (“PartMode”) are signaled by a CU level syntax element “part_type” as specified in Table 7-10 of the WD. For a MODE_INTER CU other than those having maximum depth, seven PU partitioning patterns (PART_2Nx2N, PART_2NxN, PART_Nx2N, PART_2NxnU, PART_2NxnD, PART_nLx2N and PART_nRx2N) can be selected. PART_NxN can only be chosen at maximum CU depth level but permission to set N to 4 is controlled by a specific flag in SPS (“inter_4x4_enabled_flag”). For each PU, PU-based Motion Merging (merge mode) or normal inter prediction with actually estimated motion parameters (inter mode) can be used. This section describes how luma motion parameters are obtained for each PU. It is noted that chroma motion vector are derived from luma motion vector of corresponding PU according to the normative process specified in section 8.4.2.1.10 of the WD, and the same reference picture index and prediction direction index as luma’s one are used in chroma components.

##### Motion Vector Prediction

For each PU, the best motion vector predictor is computed with the process specified as follows. Firstly, a set of motion vector predictor candidates for RefPicListX are derived with normative process specified in section 8.4.2.1.7 of the WD, by referring to motion parameters of neighbouring PUs. Then, the best one from the candidate set is determined by a criterion that selects a motion vector predictor candidate that minimizes the cost Jpred,SAD specified in the section 6.1.4.3, with setting the bits for an index specifying each motion vector predictor candidate to Bpred. The index corresponding to the selected best candidate is assigned to the mvp_idx_lX.

##### CU coding with MODE_SKIP

In the case of skip mode (i.e., PredMode == “MODE_SKIP”), motion parameters for the current CU(i.e., PART_2Nx2N PU) are derived by using merge mode. In this case, the motion parameters are determined by checking all possible merge candidates derived by the normative process specified in section 8.4.2.1.1 to 8.4.2.1.5 of the WD, and selecting the best set of motion parameters that minimizes the cost Jmode specified in the section 6.1.4.5. In this case, Bmode includes coded bits for skip_flag and merge_idx that signals position of the PU having the best motion parameters to be used for the current PU. Since prediction residual is not transmitted for skip mode, SSE is obtained by inter prediction samples.

##### CU coding with MODE_INTER

When a CU is coded with MODE_INTER, motion parameter decision for each PU is performed first based on the ME cost Jpred,SATD specified in the section 6.1.4.4.

For merge mode case, the motion parameter decision starts with checking availabilities of all neighbouring PUs to form merge candidates according to the normative process specified in the section 8.4.2.1.1 to 8.4.2.1.5 of the WD. If there is no available merge candidate, the HM encoder simply skips cost computation for merge mode and does not choose merge mode for the current PU. Otherwise(i.e., if there is at least one merge candidate), the ME cost Jpred,SATD specified in the section 6.1.4.4 is computed for all possible PUs as merge candidate and the best one is selected as the best motion parameters for the PU predicted with merge mode. SATD between source and prediction samples is used as distortion factor, and bits for merge_idx is set to Bpred.

For inter mode case, the best motion parameters are derived by invoking motion estimation process specified in the section 6.5.2.2. During the motion estimation process, the best motion parameters are obtained based on the cost function Jpred,SATD specified in the section 6.1.4.4, which is comparable with the cost of motion parameter derivation for merge mode. SATD between source and prediction samples is used as distortion factor, and bits for inter_pred_flag, ref_idx_lX, mvd_lX and mvp_idx_lX are set to Bpred.

After both of the best motion parameters are obtained, the best motion parameters are determined by comparing them and taking the better one that results in lower cost.

#### Motion estimation

In order to get motion vector for each PU, block matching algorithm (BMA) is performed at encoder. Motion vector accuracy supported in HEVC is quarter-pel. To generate half-pel and quarter-pel accuracy samples, interpolation filtering is performed for reference picture samples. Instead of searching all the positions for quarter-pel accuracy motion, integer-pel accuracy motion vector is obtained at first. For half-pel search, only 8 sample points around the motion vector which has the minimum cost are searched. Similarly, for quarter-pel search, 8 sample points around the motion which has the minimum cost so far are searched. The motion vector which has the minimum cost is selected as the motion vector of the PU. To get the cost, SAD is used for integer-pel motion search and SA(T)D is used for half-pel and quarter-pel motion search. The rate for motion vector is obtained by utilizing pre-calculated rate table. In the following sub-sections, algorithms for integer-pel motion search is provided in detail.

##### Integer-pel accuracy motion search

To reduce search points for integer-pel motion, 3 step motion search strategy is used. Figure 7-37 illustrates the 3 step approach for integer-pel accuracy motion search.

Figure 7-37 – Three step motion search strategy for integer-pel accuracy

At first, start position of the search is selected. As a default, motion vector predictor (PMV) obtained by motion vector predictor derivation process is used. Optionally, motion vectors of neighbouring positions (A, B, and C), and zero motion can be checked. In the common test condition, only PMV is used as the start position of integer-pel search.

Figure 7-38 – Start position selection

As a second step, the first search is done using diamond search pattern or square search pattern. Currently, diamond search pattern is default, and square search pattern is used by changing input configuration. Additional raster search is performed when the difference between obtained motion vector and start position is too big. Currently, search range is set by 64 in integer-pel accuracy. Figure 7-39 illustrates the 3 search patterns used for the first search. Red circles represent current position and coloured squares represent candidate search positions for each pattern. Same colour means positions having same distance from the start position.

Figure 7-39 – Search patterns for the first search

Last step is refinement search. In this step, refinement search is performed by changing the start position to the best position from the second step. Also, diamond or square search is utilized, and refinement is stopped when 2 rounds are passed after best match.

##### Bi-predictive search strategy

In principle, bi-predictive motion search means to search two motion vectors which produce minimum error between original block (O) and predicted block with two prediction (P=P0+P1). In HM, practical strategy is implemented by utilizing iterative uni-predictive search. Bi-predictive search steps are as follows.

- Search P1 which produces minimum error with (2O - P0), where O represents original block and P0 means predictor produced by the first motion vector. P0 is fixed in this step. To get motion vector for P1, uni-predictive motion search is utilized after setting (2O - P0) as reference samples.

- Search P0 which produces minimum error with (2O – P1), where O represents original block and P1 means predictor produced by the second motion vector. P1 is the predictor obtained in step 1) and fixed in this step. To get P0, uni-predictive search is utilized after setting (2O – P1) as reference samples.

- Iterate 1) and 2) until maximum number of iterations is reached. The maximum number of iteration is set by 4 unless the fast search option is enabled.

##### Fast search options

There are two options to accelerate motion estimation. The first one is using sub-sampled SAD for integer motion search. This option is only used for blocks which have larger number of rows than 8. In this method, only samples in odd-number rows are involved for SAD calculations. The second option is reducing number of iterations for bi-predictive motion search. The number of iteration is set by 4 unless the fast search option is enabled.

#### Decision process on AMP mode evaluation procedure

For encoder speed up, additional conditions are checked before doing motion estimation for AMP. If certain conditions are met, additional motion estimation for AMP can be skipped. Conditions of mode skipping are based on two values: the best partition mode (PartMode) before AMP modes are evaluated and the PartMode and prediction mode (PredMode) at the lower level in the CU quad-tree, the so called parent CU, which contains the current PU. The conditions and actions are specified in Table 7-16.

Table 7-16 – Conditions and actions for fast AMP mode evaluation

| Conditions | Actions |
| --- | --- |
| The best PartMode is SIZE_2NxN | Try SIZE_2NxnU and SIZE_2NxnD |
| The best PartMode is SIZE_Nx2N | Try SIZE_nLx2N and SIZE_nRx2N |
| The best PartMode is 2Nx2N && !merge mode && ! skip mode | Try all AMP modes |
| PartMode of parent CU is AMP mode | Try merge mode only for all AMP modes |
| PartMode of parent CU is PART_2Nx2N && parent CU is not skipped | Try merge mode only for all AMP modes |
| PredMode of parent CU is intra && the best PartMode is SIZE_2NxN | Try merge mode only for SIZE_2NxnU and SIZE_2NxnD |
| PredMode of parent CU is intra && the best PartMode is SIZE_Nx2N | Try merge mode only for SIZE_nLx2N and SIZE_nRx2N |
| Size of current CU is 64x64 | No AMP modes are evaluated |

### Intra/Inter/PCM mode decision

For inter coded CUs, the following mode decision process is conducted in the HM encoder. Its schematic is also shown in Figure 7-40.  Please refer to contributions about early termination for Early_CU condition, CBF_Fast condition, and Early_SKIP condition.

- Coding costs (Jmode) for MODE_INTER with PART_2Nx2N is computed and Jmode is set to minimum CU coding cost J.

- Check if motion vector difference of MODE_INTER with PART_2Nx2N is equal to (0, 0) and MODE_INTER with PART_2Nx2N contains no non-zero transform coefficients (Early_SKIP condition). If both are true, proceed to 17 with setting the best interim coding mode as MODE_SKIP. Otherwise, proceed to 3.

- Check if MODE_INTER with PART_2Nx2N contains no non-zero transform coefficients (CBF_Fast condition). If the condition is true, proceed to 17 with setting the best interim coding mode as MODE_INTER with PART_2Nx2N. Otherwise, proceed to 4.

- Jmode for MODE_SKIP is evaluated and J is set equal to Jmode if Jmode < J.

- Check if the current CU depth is maximum and the current CU size is not 8x8 when inter_4x4_enabled_flag is zero. If the conditions are true, proceed to 6. Otherwise, proceed to 7.

- Jmode for MODE_INTER with PART_NxN is evaluated and J is set equal to Jmode if Jmode < J. After that, check if MODE_INTER with PART_NxN contains no non-zero transform coefficients (CBF_Fast condition). If the condition is true, proceed to 17 with setting the best interim coding mode as MODE_INTER with PART_NxN. Otherwise, proceed to 7.

- Jmode for MODE_INTER with PART_Nx2N is evaluated and J is set equal to Jmode if Jmode < J. After that, check if MODE_INTER with PART_Nx2N contains no non-zero transform coefficients (CBF_Fast condition). If the condition is true, proceed to 17 with setting the best interim coding mode as MODE_INTER with PART_Nx2N. Otherwise, proceed to 8.

- Jmode  for MODE_INTER with PART_2NxN is evaluated and J is set equal to Jmode if Jmode < J. After that, check if MODE_INTER with PART_2NxN contains no non-zero transform coefficients (CBF_Fast condition). If the condition is true, proceed to 17 with setting the best interim coding mode as MODE_INTER with PART_2NxN. Otherwise, proceed to 9.

- Invoke a process to determine AMP mode evaluation procedure specified in 6.5.2.3. Output of this process is assigned to TestAMP_Hor and TestAMP_Ver. TestAMP_Hor specifies whether horizontal AMP modes are tested with specific ME or tested with merge mode or not tested. TestAMP_Ver specifies whether vertical AMP modes are tested with specific ME or tested with merge mode or not tested.

- If TestAMP_Hor indicates that horizontal AMP modes are tested, MODE_INTER with PART_2NxnU is evaluated with procedure suggested by TestAMP_Hor and J is set equal to the resulting coding cost Jmode if Jmode < J. After that, check if MODE_INTER with PART_2NxnU contains no non-zero transform coefficients (CBF_Fast condition). If the condition is true, proceed to 17 with setting the best interim coding mode as MODE_INTER with PART_2NxnU. Otherwise, MODE_INTER with PART_2NxnD is evaluated with procedure suggested by TestAMP_Hor and J is set equal to the resulting coding cost Jmode if Jmode < J. After that, check if MODE_INTER with PART_2NxnD contains no non-zero transform coefficients (CBF_Fast condition). If the condition is true, proceed to 17 with setting the best interim coding mode as MODE_INTER with PART_2NxnD. Otherwise, proceed to 11.

- If TestAMP_Ver indicates that vertical AMP modes are tested, MODE_INTER with PART_nLx2N is evaluated with procedure suggested by TestAMP_Ver and J is set equal to the resulting coding cost Jmode if Jmode < J. After that, check if MODE_INTER with PART_nLx2N contains no non-zero transform coefficients (CBF_Fast condition). If the condition is true, proceed to 17 with setting the best interim coding mode as MODE_INTER with PART_nLx2N. Otherwise, MODE_INTER with PART_nRx2N is evaluated with procedure suggested by TestAMP_Ver and J is set equal to the resulting coding cost Jmode if Jmode < J. After that, check if MODE_INTER with PART_nRx2N contains no non-zero transform coefficients (CBF_Fast condition). If the condition is true, proceed to 17 with setting the best interim coding mode as MODE_INTER with PART_nRx2N. Otherwise, proceed to 12.

- MODE_INTRA with PART_2Nx2N is evaluated by invoking the process specified in 6.5.1, only when at least one or more non-zero transform coefficients can be found by using the best interim coding mode. J is set equal to the resulting coding cost Jmode if Jmode < J.

- Check if the current CU depth is maximum, If the condition is true, proceed to 14. Otherwise, proceed to 15.

- MODE_INTRA with PART_NxN is evaluated by invoking the process specified in 6.5.1, only when the current CU size is larger than minimum TU size. The resulting coding cost Jmode is set to J if Jmode < J.

- Check if the current CU size is greater than or equal to the minimum PCM mode size specified by the log2_min_pcm_coding_block_size_minus3 value of SPS parameter. If the condition is true, proceed to 16. Otherwise, proceed to 17.

- Check if any of the following conditions are true. If the condition is true, PCM mode is evaluated and J is set equal to the resulting coding cost Jmode if Jmode < J.

- Bit cost of J is greater than that of the PCM sample data of the input image block.

- J is greater than bit cost of the PCM sample data of the input image block multiplied by λmode.

- Update bit cost Bmode by adding bits for CU split flag and re-compute minimum coding cost J.

- Check if the best interim coding mode is MODE_SKIP (Early_CU condition). If the condition is true, do not proceed to the recursive mode decision at next CU level. Otherwise, go to next CU level of recursive mode decision if the current CU depth is not maximum.

Figure 7-40 – The schematic of Intra/Inter/PCM mode decision

For the computation of Jmode except for PCM mode, residual signal is obtained by subtracting intra or inter prediction samples from source samples and is coded with transform and quantization with quad-tree TU partitioning as specified in the section 6.6. Bits for side information (skip_flag, merge_flag, merge_idx, pred_type, pcm_flag, inter_pred_flag, reference picture indices, motion vector(s), mvp_idx, intra prediction mode signaling) and residual coded data are considered as Bmode. SSEluma and SSEchroma are obtained by using local decoded samples, except for MODE_SKIP case where prediction sample is used as local decoded samples.

For the computation of Jmode for PCM mode, bits for side information (skip_flag, pred_type, pcm_flag, pcm_alignment_zero_bit) and PCM sample data are considered as Bmode. SSEluma and SSEchroma are set to 0. (Note that in current test conditions, the PCM mode decision processes in (15) and (16) are skipped since the minimum PCM mode size is 128.)

This CU level mode decision is recursively performed for each CU depth and final distribution of CU coding modes is determined at CTU level.

### Adaptive QP

This tool varies the quantization parameter for each coding unit to provide improved perceived image quality. QP variation is performed using the same technique originally implemented in the MPEG-2 TM5 which works according to the following rationale: lower QP values are used on smooth image areas while higher values on highly active blocks. The activity of each CU is measured by the variance of its luma samples. More precisely, given a CU with size 2N × 2N, the luma variance of its four sub blocks with size N × N is computed first. Let σ2(i) denote the luma sample variance for sub block i. The CU activity (actCU) is then computed as:

actCU=1+mini=1,..,4σ2(i),

In order to increase QP values on highly active image areas and decrease in smooth ones, the quantity actCU is then normalized with respect to the average activity measured over all coding units with size 2N × 2N inside one picture. Let actf denote the average activity for all CUs with size 2N × 2N belonging to picture f. The normalized CU activity norm_actCU is then given by:

normactCU=s⋅actCU+actfactCU+s⋅actf,

where s denotes the scaling factor associated to the QP adaptation parameter (QPA) and computed as:

s=2QPA6.

The value for QPA is provided as input using the configuration option MaxQPAdaptationRange and has default value of 6. Finally, the coding unit QP is adjusted according to:

QP=QPbase+6⋅log2(normactCU)+0.5,

where QPbase denotes the QP value for the slice where the coding unit belongs to and  operator returns the largest integer smaller than or equal to the argument. The adaptive QP tool is enabled by the configuration option AdaptiveQP. The minimum CU size at which QP can be adapted according to its normalized activity is specified by the configuration option MaxCuDQPDepth whose value should always be less than the maximum CU depth.

## Derivation process for TU-level coding parameters

### Residual Quad-tree partitioning

The residual quadtree is a recursive representation of the partitioning of a coding unit into transform units.

The encoding process for intra-coded coding units can be summarized as follows.

–	The luma intra prediction mode (or modes for intra_split_flag equal to 1) is determined using the residual coding with the largest applicable transform size.

–	Given the determined luma intra prediction mode (or modes for intra_split_flag equal to 1), the transform tree and the corresponding luma transform coefficient levels are determined using an exhaustive subdivision process, taking into account the maximum allowed transform hierarchy depth and considering only the luma component.

–	The chroma intra prediction mode and the corresponding chroma transform coefficient levels are determined given the determined transform tree.

The encoding process for inter-coded coding units can be summarized as follows.

–	The transform tree and the corresponding luma and chroma transform coefficient levels are determined using an exhaustive subdivision process, taking into account the maximum allowed transform hierarchy depth and considering both the luma component and the chroma components.

### Rate-distortion optimized quantization

Rate-distortion optimized quantization (RDOQ) is enabled by the encoder option --RDOQ and performs soft decision quantization for each transform coefficient by minimizing a rate-distortion Lagrangian cost function. More specifically, over a transform unit with size B each coefficient ci (for i = 0,…,B2 – 1) is quantized to the following three level values: 0, lfloor and lceil. The value for lfloor is the one computed for the selected QP and according to [1]. Conversely, lceil = lfloor + 1. For each level value l, the following Lagrangian cost function J is computed:

J(ci,l)=D(ci,l)+λ⋅R(l)

where λ denotes the Lagrangian multiplier computed as described in Section 6.1.4.1, D(ci, l) is the distortion measured when ci is quantized to l and R(l) is the (estimated) coding rate associated to l. The value for l (i.e. 0, lfloor or lceil) which minimizes J is selected as the best one and the process moves to the next coefficient. The coefficients ci are processed according to the scanning order used for the transform unit where they belong to and starting from the last significant coefficient (i.e. from the one associated to i = B2 – 1). This particular processing order is used to mimic actual entropy encoding. In order to keep low computational complexity during RDOQ, the coding rate R(l) is estimated by tabularized values of entropy of the probabilities corresponding to states in CABAC coding engine. RDOQ can be used also for those transform units where the transform skip is selected. In this case the processing is identical and this option can enabled or disabled by the encoder option --RDOQTS.

### Quantization rounding for residual DPCM

Over blocks where both transform skip and Residual DPCM (RDPCM, see Section 4.7) are applied, the quantization rounding offset is different from the one used in spatially transformed blocks. More precisely, the HM-RExt 7 software applies a dead zone uniform quantizer with quantization step Q and rounding offset α as shown in Figure 7-41Error: Reference source not found. For blocks where spatial transformation is applied, α is equal to 1/3 and 1/6 for intra and inter coding modes, respectively. Conversely, over blocks where transform skip and RDPCM are used, the offset α is set equal to1/2.

Figure 7-41 – Dead zone uniform quantizer with rounding offset.

## Cross-component prediction

When calculating the α, the luma residual before scaling and transformation (i.e. the difference between original luma sample and its prediction) is used rather than the reconstructed luma residual signal as in Experiment 1. Specifically, at the encoder side instead of (1) below equation is used.

ΔrC(x,y)=rC(x,y)−α×rL(x,y)>>3

This can reduce the implementation complexity by facilitating pipelining and parallel processing as the prediction can be performed without waiting until reconstruction of luma residual signal. This requires only encoder side modification, and the decoding part is the same as Experiment 1.

## Transform skip selection

The transform skip is allowed for large TUs by a RDO with the transform mode. The maximum size allowed is specified by TransformSkipLog2MaxSize. There is one fast mode decision specified by setting TransformSkipFast to 1, which skips the transform skip on luminance intra coding when the PartMode is not PART_NxN and on chrominance intra coding when all corresponding luminance TUs are not coded in transform skip mode. Therefore, to enable the transform skip on large TUs when the PartMode is PART_2Nx2N for intra coding, the TransformSkipFast option should be set to 0. Another fast method for transform skip is to disable the RDOQ when a TU chooses the transform skip mode, which is achieved by setting RDOQTS to 0.

## Cost mode

The cost equation used by the encoder can be configured using CostMode. This can take one of the following four strings:

- lossy – this is the standard cost equation, where cost = distortion + (bits * lambda).

- sequence_level_lossless – this is equivalent to the HM macro cost equation, although includes distortion, which may be present in mixed lossless and lossy scenarios. Cost = (distortion / lambda) + bits.

- lossless – this is equivalent to sequence_level_lossless, but also sets QP to the value of RExt__LOSSLESS_AND_MIXED_LOSSLESS_RD_COST_TEST_QP (by default 0) (since the QP is used during the encoder search for testing intra modes). This may be deprecated in future versions in favour of the user setting the QP manually.

- mixed_lossless_lossy – this uses the same cost equation as sequence level lossless, but also uses a lambda evaluated at the value of the macro RExt__LOSSLESS_AND_MIXED_LOSSLESS_RD_COST_TEST_QP_PRIME (by default 4) to derive lambdas so that lossless coded blocks are not affected by QP during their encoder search. This affects the intra search during fast evaluation of intra directions and inter search during evaluation of motion vector cost.

## Inter-prediction search

The inter-prediction search function in the encoder includes two modes of operation, a pre-existing search derived from the HM software inter search function and a ‘single component loop inter search’ mode. The single component loop inter search is enabled using SingleComponentLoopInterSearch

The pre-existing inter-prediction search function includes three loops over the colour channels, as follows:

- In the first component loop, the entropy coder is reset, blocks are transformed and quantized, then the entropy coding cost for the block is estimated.

- In the second component loop, the entropy coder is reset, then if the absolute sum of the quantized coefficients is non-zero, the block is dequantized and inverse-transformed and a cost is derived using the result of the first loop. Also, a cost is derived for the case where all residual coefficients are zero. The costs are compared and the lowest is selected. If the absolute sum of the quantized coefficients is zero, only the cost for this case is derived.

- In the third component loop, for each tested mode of transform skip and cross-component prediction the following occurs. Firstly, the entropy coder is restored and reset if the component is not Cr (i.e. cIdx < 2). Then the block is transform skipped and quantized, the block is then entropy code estimated and a cost is derived, and selected if it is an improvement upon the previous best cost.

After the three component loops, the entropy coder is restored and then reset and the decisions are encoded.

When the encoder is configured to use the single component loop inter search mode, the following occurs:

- A loop over each component occurs. For each tested mode of transform skip and (optionally) cross-component prediction, the following occurs. The entropy coder is restored and then reset. The block is then transformed and quantized. The entropy coding cost of the block is then estimated. If the absolute sum of the quantized coefficients is non-zero, the block is dequantized and inverse-transformed, otherwise the inverse-transformed block is set to all zeros. Then a cost is derived. If all optional tools are disabled, then a cost is derived for the tested mode for the case of all coefficients being zero. Then, the best cost is selected.

After the component loop, the entropy coder is restored and then reset and the decisions are encoded.

## Rate control

The HM software implements a single pass rate control algorithm which provides a constant bit rate (CBR) mode. The algorithm is controlled using the RateControl configuration option and is based on a R-λ model as presented in document JCTVC-K0103 [6]. This model assumes an exponential relationship between the coding rate and the Lagrangian multiplier λ:

R=α1⋅λβ1

where α1 and β1 are constants dependent on the video source. The rate control process can be summarised into two main steps:

- Bit allocation,

- Quantization parameter derivation according to the used R-λ model.

Bit allocation is performed at the following levels of granularity: GOP, picture and CTU level. Initially, the rate controller computes the average bits per picture (RPicAvg) from the target bitrate (Rtar) and the video frame rate (f):

RPicAvg=Rtarf

Then, bit allocation at the GOP level is performed using a smoothing window (SW) of 40 frame lengths and taking into account the number of pictures already coded (Ncoded) and the bits spent on these pictures (Rcoded). For the current GOP the target bits (TGOP) are then given as:

TGOP=RPicAvg⋅(Ncoded+SW)−RcodedSW⋅NGOP

where NGOP denotes the number of pictures in the current GOP. At the picture level bit allocation is performed taking into account TGOP and the bits spent over the pictures inside the current GOP and already coded (CODEDGOP). The number of bits allocated for each picture also considers a weight. The weight models the relative number of bits that should be allocated to one picture with respect to the others. As an example, in the random access coding configuration, when a hierarchical GOP structure is used, pictures belonging to lower levels of the hierarchy will receive a larger number of bits since they will serve as reference for subsequent coded pictures (i.e. at higher levels of the hierarchy). Conversely those located to the higher level of the hierarchy will receive fewer bits. The weights used in HM are selected differently depending on whether random access or low delay GOP type is used. The target coding rate for current picture (TCurrPic) is then given as:

TCurrPic=TGOP−CODEDGOPi∈not coded picturesωi⋅ωCurrPic

Finally, for each CTU the bit-rate is allocated taking into account the number of bits allocated for the picture of which the CTU belongs, the number of bits spent while encoding previous CTUs (CODEDCTU) and bits spent while coding high level syntax meta-data such as the slice header, SPS, PPS. As for picture bit allocation, CTUs are differently weighted depending on their complexity. The complexity metric measures the mean of absolute differences (MAD) between the original and the predictor used to encode the current CTU for inter-predicted pictures and the sum of absolute transform differences (SATD) for intra-predicted pictures. Therefore the target bits allocated for the current CTU (TCurrCTU) is given by:

TCurrCTU=TCurrPic−Bitheader−CODEDCTUi∈not coded CTUωi⋅ωCurrCTU

where BitHEADER denotes the bits spent while encoding high level syntax meta data and ωi denotes the weighting factor for each CTU.

Once the available bits have been allocated for the current CTU, the quantization parameter QP can be obtained according to the used R-λ model. More precisely, the Lagrangian multiplier is derived from TCurrCTU as follows:

λ=α⋅TCurrCTUNβ

where N denotes the number of pixels contained in the current CTU. From λ, QP is finally obtained as:

QP=4.2005⋅lnλ+13.7122

where the  operator returns the largest integer smaller than or equal to the argument and ln(·) is the natural logarithm. Parameters α and β are initialized to average values of (3.2003 and -1.367, respectively) and then updated by least square regression once a coded picture is available. Moreover, different values are used with respect to level of hierarchy of the GOP structure.

The HM rate control algorithm allows to manually set the initial QP value used for the first coded picture using option --InitialQP=value. If value is equal to zero the algorithm automatically derives the value using the R-λ model otherwise uses the value provided by the user. The remaining parameters associated to the rate control algorithm are listed in Table 6-17 along with a brief description.

Table 6-17. Configuration options for rate control algorithm.

| Parameter name | Description |
| --- | --- |
| TargetBitrate | Target bitrate for the whole sequence measured in bit per second (bps) |
| KeepHierarchicalBit | Determines how bit allocation is done across different pictures. Allowed values are: 0 = uniform, 1 fixed ratio according to the used GOP structure (i.e. random access or low delay) and 2 = adaptive with respect to the source content |
| LCULevelRateControl | Switch between CTU-based or picture-based rate control |
| RCLCUSeparateModel | Selects whether to use α and β parameters on a CTU or picture basis |
| RCForceIntraQP | Force QP value for intra-pictures to be equal to the value specified with InitialQP |

# References

- K. McCann, C. Rosewarne, B. Bross, M. Naccari, K. Sharman, G. Sullivan, “High Efficiency Video Coding (HEVC) Test Model 16 (HM 16) Encoder Description”, JCTVC-R1002, October 2014.

- F. Bossen, D. Flynn, K. Sharman, K. Suhring, “HM Software Manual”, September 2014.

- D. Flynn, M. Naccari, K. Sharman, C. Rosewarne, J. Sole, G. J. Sullivan, T. Suzuki, “High Efficiency Video Coding (HEVC) Range Extensions text specification: Draft 7”, JCTVC-R1013, July 2014.

- F. Bossen, “Common HM test conditions and software reference configurations”, JCTVC-L1100, February 2013.

- C. Rosewarne, K. Sharman, D. Flynn, “Common test conditions and software reference configurations for HEVC range extensions”, JCTVC-P1006, January 2014.

- B. Li, H. Li, L. Li and J. Zhang, “Rate control by R-lambda model for HEVC”, JCTVC-K0103, October 2012.

- R. Sjoberg, Y. Chen;A. Fujibayashi; M. M. Hannuksela, J.  Samuelsson, T. K. Tan; Y.-K. Wang and S. Wenger, “Overview of HEVC High-Level Syntax and Reference Picture Management”, IEEE Transactions on Circuits and Systems for Video Technology, vol. 22, no. 12, pp. 1858-1870, December 2012.