| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S1004 |
| --- | --- |

| Title: | HEVC Version 1 Conformance Testing Defect Report |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | AHG Report |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki Wade Wan Gary Sullivan | Tel: Email: | teruhikos@jp.sony.com wwan@broadcom.com garysull@microsoft.com |
| Source: | Editors |  |  |

_____________________________

# Introduction

This document reports the known defects of the HEVC v.1 conformance testing specification. Revised bitstreams are available at

http://wftp3.itu.int/av-arch/jctvc-site/bitstream_exchange/under_test/

# Defect report list

### general_profile_idc

In the following bitstreams, general_profile_idc has the reserved value 0 (subclause 7.4.4):

AMP_A_Samsung_6

AMP_B_Samsung_6

AMVP_C_Samsung_6

STRUCT_A_Samsung_6

STRUCT_B_Samsung_6

Those bitstreams were revised after the Sapporo meeting to fix the value of general_profile_compatibility_flag. (See JCTVC-R0004.) However, the value of the general_profile_idc was set to 0 in the revised bitstreams. The value of this general_profile_idc needs to be 1.

### End of slice segment

The specification requires that after decoding end_of_slice_segment_flag with value 1, the remaining bytes in the slice segment data must not be different from zero. (Trac #1283)

The following bitstream violates this constraint.

DBLK_A_MAIN10_VIXS_3

### Max bins

In the following bitstreams, read_bits(1) is called 5145 times, which is more than the allowed maximum of 5*RawCtuBits / 3=5120 times (subclause A.3.2 Main profile specification).

MAXBINS_A_TI_4

MAXBINS_B_TI_4

MAXBINS_C_TI_4

The revised bitstreams are available on the above-identified ftp site.

### general_profile_compatibility_flag

In the following bitstreams, the value of general_profile_compatibility_flag[0] is not equal to 0.

SLIST_A_Sony_4.bin

SLIST_B_Sony_8.bin

SLIST_C_Sony_3.bin

### New bitstream

The following new bitstream to test SAO corner case was provided:

SAO_H_Parabola_1

This was provided in response to an identified weakness in the current bitstream set. The new feature of the bitstream is related to the treatment of CTU corner samples during SAO filtering of the 45 and 135 degree modes when slice_loop_filter_across_slices_enabled_flag = 0. The new bistream checks all four such "corner" cases.