| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S1008 |
| --- | --- |

| Title: | SHVC Conformance Testing Draft 1 |  |  |
| --- | --- | --- | --- |
| Status: | Output Document Approved by JCT-VC |  |  |
| Purpose: | Draft |  |  |
| Author(s) or Contact(s): | Jill Boyce Adarsh K. Ramasubramonian | Email: | jill@vidyo.com aramasub@qti.qualcomm.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

The first version of the SHVC Conformance Testing Draft is provided in this document, which contains both guidelines for bitstream generation as well as a preliminary listing of conformance bitstream features. The names of volunteers are provided for most conformance bitstream features, and additional volunteers are requested for the unassigned bitstream features.

The final version of the SHVC conformance testing draft will eventually be incorporated into a revision of the HEVC conformance specification and the Guideline for conformance test bitstream preparation document.

# Bitstream generation instructions

Volunteers should upload the bitstream and associated files in a zip archive.  Files should be uploaded to the following ftp site.

ftp://ftp3.itu.ch/jctvc-site/dropbox/

Please contact with teruhikos@jp.sony.com or jill@vidyo.com to get the access information to dropbox.

Once a bitstream is uploaded to the dropbox, send an email to teruhikos@jp.sony.com or jill@vidyo.com.

Then it will be moved to the downloading area of the ftp site:

http://wftp3.itu.int/av-arch/jctvc-site/bitstream_exchange/draft_SHVC_conformance/

All files inside the zip archive (name.zip) should have the same base file name, with extensions as described in the list below. Each of the files is further described in the following sections.

- Bitstream file (name.bit)

- Description of the bitstream (name.txt)

- MD5 check sum of decoded YUV file for each layer (name-L#. md5)

- Output picture log for each output layer set (name-OLS#.opl)

The following naming convention should be used for the base name.

name = Feature_BistreamID_Source_Version

Feature is summarized in Section 2.

BitstreamID is a single capital letter, used when more than one bitstream is used for a particular feature.

Source is the name of the company who provided the bitstream.

Version is the version number of the file, in case a bitstream must be replaced because of an identified problem.

For example, if the archive is named "feature_A_company_1.zip", and the bitstream contains two layer sets and three layers, the files included in the zip archive would be named:

feature_A_company_1.bit

feature_A_company_1.txt

feature_A_company_1-OLS0.opl

feature_A_company_1-OLS1.opl

feature_A_company_1-L0.md5

feature_A_company_1-L1.md5

feature_A_company_1-L2.md5

## Bitstream file (name.bit)

Except for the hybrid scalability case, all layers are included within the same bitstream file.

Hybrid scalability case TBD.

## Description of the bitstream (name.txt)

In a short text file which describes the bitstream, the following information should be provided.

- Bitstream file name

- Explanation of bitstream features

- Number of output layer sets in the bitstream

- Number of layers present in the bitstream

- Profile, tier, and level for each layer in each output layer set in the bitstream

- Picture size of each layer

- Frame rate of each layer (if available)

- SHM version number used to generate the bitstream, if appropriate

Picture size and frame rate is necessary to check HRD conformance of the bitstreams.

If the SHM was used to generate the bitstream, the SHM version number used should be noted.

## MD5 file per layer containing the MD5 check sum of decoded YUV file for each layer (name-L#.md5)

An md5 file should be provided for each layer present in the bitstream, regardless of whether or not the layer is a target output layer of an output layer set. The name of the md5 file should contain the layer number of the layer, as represented by the nuh_layer_id value.

Each md5 file should contain the hash value for the decoder’s output YUV file for that layer (which contains all decoded pictures of the layer, in output order).  The md5 file should contain only the hash value, and should not contain any text remarks, which, if needed, can be provided in the bitstream description file.

The recommended md5sum tool on UNIX is described at this link, http://en.wikipedia.org/wiki/Md5sum.

## Output picture log file for each output layer set (name-OLS#.opl)

An output picture log file should be provided for each output layer set present in the bitstream. The name of the output picture log file should contain the output layer set number.

The SHM decoder can be configured to create output picture log files for a bitstream, by using a –confMode command line option. This feature is expected to be included in the SHM-8.0 release, and is now available in the SHM-dev branch.  For example, to decode the n-th output layer set of the bitstream bitstream.bit, along with generating the relevant log files, the following command line could be used:

TAppDecoder.exe –b bitstream.bit –confMode 1 –olsidx n

Additionally, the command line option “-confPrefix” could be used to prefix the names of the log files generated. The default name used would be "decodedBitstream", e.g. decodedBitstream-L#.yuv, decodedBitstream-OLS#.opl.

Each output picture log file contains one row for each output picture in the bitstream, in output order. If a picture is not a target output picture for a particular output layer set, or the value of the PicOutputFlag variable for a picture is equal to 0, the information for the picture should not be included in the output picture log file.

The following information is included in a row for each output picture:

- nuh_layer_id

- PicOrderCntVal

- MD5 checksum for the Y component

- MD5 checksum for the U component

- MD5 checksum for the V component

The first few lines of an example output picture log file for a 2-layer bitstream is shown below.

LayerId      POC    MD5

------------------------

0        0    MD5:d4a80b3da02ce37a0f9c74b84eebd191,4cccea85a761256502c2a0dd50e243d7,dc72e4796ab321b27a16a7fd316b9ddd

1        0    MD5:c48fc168d2052062a5370fd3d75ab510,f6b1bb462b32eadb9b70d98dc8caa40f,0cff1459777fb5ddfda6371eb14208a3

0        1    MD5:a7e7eff85f2773ff19a43e0ddcfa2aa3,2b083d69271697af4a780eb5ae70bd5a,70e1bca8703d5a8dc6a5be8dd5aed5d6

1        1    MD5:31db80f40e5d56743c8f87ae97954d39,20ddc453e7a9168f5d2cb48e545d9c2c,df53c5e1e6e88188a88a8feb13bcab2a

0        2    MD5:e2351d657bb1bfaf38bab96ecce6530e,6fc36992be946374bad85f6dba719dbb,10652af9aa60dad51700a5f760201419

1        2    MD5:221f765c2f80c5c6bfe91628b17e593f,7e89a200590e35391b74aff5073a1b64,69a1b29df26c7c3cfc861ba493cbdb19

0        3    MD5:f70d9bee7e314afb20581a7ab6d6481b,5808ba66486b1b96caea908353b38033,4c0c25773eed62547b320357adcd8323

1        3    MD5:6546b65ad49626c0e7380d2f9c71c4f3,fb3e56671740f88125c53af27ebb6ff2,28a943b8f4d0c84693a91f7da2d1f21c

0        4    MD5:e63fd13ce2708fcf9e3d9f1addd622e2,c60fcf34bd0dd5c449354e2b6faf2940,b0f236ad816d535037cb2fe5ffa6b72a

1        4    MD5:3b580b825e65d0fb4c4f8fd1812036c6,94eca9606fbc2f0529739475d5ba5a5f,7a37aa0e162b3fc988ef50c5187533cc

0        5    MD5:6c4e657076424cc390b0e5829157bef4,4e77418d2db800bcfd8ed26f042c6f72,c75cbf596dcbdeb8903d74d2d12e7ba2

1        5    MD5:b4e28798dfdf50b9012b9f9fa433b91a,632424ddb5f1ce9e80de6f1e58cd0d1f,ae63094bb9abef4f7204b0c51a283ce0

0        6    MD5:32e9c63a8bcc9fd8d4803d34427609a1,301dfae4b86d17651f4f165c0f361b53,21bbc1d163c8a6c57ae1b07ccb8dfe4f

1        6    MD5:b27a004c9190c627415a14c582820cbb,e28dac76acb8e91a371eab142664b95e,95c38cb60f3a98efbf855d8846852a57

0        7    MD5:b59b6f712251fb146de05236f3d62d69,5133f701cd84278c3a5000428ffc06db,7e925a90f137dab5552e4498026d6b6c

1        7    MD5:e2d75b80f2b4b30b5b534d3cc2e4af4c,24a413e09de9807e8180fdb9db5dc6d7,05c931b9051b8dc58374f2fbe3e0aa71

0        8    MD5:6abfa89ea2ab076668aae713ca8ad252,1fcf393e5648540e8f2abc7a5056f5c4,f6360d3839e4896266467b281c37f252

1        8    MD5:b3b01379ba08916ef6b1b35f7d9ad51c,7fa38083093d5a57cfca3680f35c9c93,3c4aa75b2f95d5ae40f24c14f677336f

0        9    MD5:992cd6d0574bf0026b477ae35062f20e,b820cd9b8874684dfb1b6c7f01b8142e,df535187e3be8eeb22e54ac43718af72

1        9    MD5:b3b01379ba08916ef6b1b35f7d9ad51c,c5e8f694e90712a783b09a7d4bd4dd1e,9899e7f2a7079f16637ac778be73a930

# Conformance description

In an SHVC bitstream, conformance is expressed for each output layer set, through the signalling of a profile_tier_level( ) syntax structure associated with each necessary layer in the output layer set.  A decoder that conforms to a list of profile, tier, level, and INBLD capabilities adequate for each necessary layer of the output layer set (as described in subclause F.11.2 Decoder capabilities) must correctly decode all pictures of the necessary layers of the output layer set in order to correctly output all output pictures of target output layers of the output layer set.

To test correct decoding, per layer MD5 sum files are provided with each conformance bitstream, regardless of whether or not a layer is included within an output layer set. However, decoder conformance is only required for layers included within an output layer set, for which an associated profile_tier_level( ) syntax structure is present in the bitstream.

Correct output of individual pictures from only the target output layers in an output layer set, in the correct output order, is also a requirement of conformance. The output picture log files provided for each output layer set can be used to verify the presence and order of the appropriate output pictures of an output layer set.

## Conformance bitstream listing

The table below lists the bitstream features, and the bitstream feature name to be used in the base file name. Some bitstream features will be represented using multiple bitstreams.

This section will be updated with bitstream descriptions once they are available.

| Volunteer Company | Volunteer Name | Bitstream Feature Name | Brief Description |
| --- | --- | --- | --- |
|  |  |  | LAYER DEPENDENCIES |
| Nokia | Antti Hallapuro | LAYERID | gaps in layer id |
| InterDigital | Yan Ye | MVD | no MV dependency |
| Nokia | Antti Hallapuro | MVD | varying sample, MV dependencies |
| ETRI | Hahyun Lee | MAXTID | max_tid |
| Qualcomm | Adarsh Ramasubramonian | INACTIVE | inactive ref layers |
| Vidyo | Jill Boyce | REFLAYER | multiple active ref layers |
| Vidyo | Jill Boyce | REFLAYER | per pic inter layer dependency |
| Vidyo | Jill Boyce | REFLAYER | various inter layer predictors |
|  |  |  | VPS SYNTAX |
| HHI | Robert Skupin | SPLITFLAG | splitting_flag |
| Qualcomm | Hendry | VUI | non VUI extension |
| Qualcomm | Hendry | VUI | VUI present |
|  |  |  | DPB |
| NEEDED | NEEDED | DPB | sub layer info |
| NEEDED | NEEDED | DPB | sub layer dpb info |
|  |  |  | RESOLUTIONS |
| Samsung | Elena Alshina | SRATIOS | 2 spatial layers w/ unusual ratios |
| InterDigital | Yan Ye | SNR | 2 SNR layers |
| InterDigital | Yan Ye | SNR | 3 SNR layers |
| Vidyo | Jill Boyce | REPFMT | VPS rep formats |
| Vidyo | Jill Boyce | RESCHANGE | resolution change in SPS |
| Ericsson | Rickard Sjoberg | ADAPTRES | adaptive resolution |
| Sony | Ohji Nakagami | SPSREPFMT | SPS rep format idx |
| Vidyo | Jill Boyce | CONFCROP | conformance cropping window in VPS |
|  |  |  | OFFSETS AND PHASE ADJUSTMENTS |
| Qualcomm | Adarsh Ramasubramonian | SCREFOFF | scaled ref layer offsets |
| Sharp | Tomoyuki Yamamoto | REFREGOFF | ref region offsets |
| Samsung | Elena Alshina | RESPHASE | resample phase |
|  |  |  | OUTPUT LAYERS, PICS |
| Nokia | Antti Hallapuro | OLS | add output layer sets |
| Nokia | Antti Hallapuro | OLS | non-default target output layer sets |
| Nokia | Antti Hallapuro | OLS | unnecessary layers |
| Qualcomm | Hendry | DISFLAG | discardable flag |
|  |  |  | SCALING LIST |
| Sony | Ohji Nakagami | SLIST | SPS scaling list infer |
| Sony | Ohji Nakagami | SLIST | PPS scaling list infer |
|  |  |  | COLOR GAMUT SCALABILITY |
| Technicolor | Pierre Andrivon | CGS | Colour gamut scalability 8-to-10 bits |
| Technicolor | Pierre Andrivon | CGS | Colour gamut scalability 10-to-10 bits |
| Technicolor | Pierre Andrivon | CGS | spatial ratio change with CGS |
|  |  |  | ADDITIONAL EXTENSIBILITY |
| Vidyo | Jill Boyce | PSEXT | SPS, PPS additional extension |
| HHI | Karsten Suehring | NUHEXT | NUHs with layer_id of 63 |
|  |  |  | POC |
| Ericsson | Rickard Sjoberg | POC | unaligned POC |
|  |  |  | BASE LAYER TYPE |
| Qualcomm | Hendry | HYBRID | hybrid scalability |
| Nokia | Antti Hallapuro | INBLD | base layer not available (INBLD) |
| InterDigital | Yan Ye | SIM | simulcast |
|  |  |  | PROFILE SIGNALING |
| Vidyo | Jill Boyce | VPSPROF | VPS profile signaling |
| Technicolor | Pierre Andrivon | SLPROF | sub_layer profile signaling (Scalable Main 8 and Scalable Main 10) |
| Vidyo | Jill Boyce | SLLEV | sub_layer level signaling |
|  |  |  | AUX PICS |
| BBC | Matteo Naccari | ALPHA | Alpha |
| Nokia | Antti Hallapuro | DEPTH | Depth |
| Disney | Nikolce Stefanoski | OVERLAY | Overlay |
|  |  |  | LAYERS |
| NEEDED | NEEDED | LAYERS | 3 layers |
| NEEDED | NEEDED | LAYERS | 8 layers |