| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S1101 |
| --- | --- |

| Title: | Description of Core Experiment 1 (CE1): Improvements of palette mode |  |  |
| --- | --- | --- | --- |
| Status: | Output Document of JCT-VC |  |  |
| Purpose: | CE Description |  |  |
| Author(s) or Contact(s): | PoLin Lai Patrice Onno Robert Cohen Vadim Seregin Xiaoyu Xiu Zhan Ma | polin.lai@mediatek.com patrice.onno@crf.canon.fr cohen@merl.com vseregin@qti.qualcomm.com xiaoyu.xiu@interdigital.com Zhan.Ma@huawei.com |  |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document provides the description of Core Experiment 1 (CE1) on improvements of palette mode.

# Introduction

The goal of this Core Experiment (CE) is to further investigate improvements of the existing palette mode in SCM-3.0.

# Participant list

| Company/Organization | Participant Name | Email Address |
| --- | --- | --- |
| Broadcom | Minhua Zhou | minhua@broadcom.com |
| Canon | Chris Rosewarne Christophe Gisquet Guillaume Laroche Patrice Onno | Chris.Rosewarne@cisra.canon.com.au christophe.gisquet@crf.canon.fr guillaume.laroche@crf.canon.fr patrice.onno@crf.canon.fr |
| Fujitsu | Jianqing Zhu Jiangli Ye Kimihiko Kazui | zhujianqing@cn.fujitsu.com threerabbit@cn.fujitsu.com kazui.kimihiko@jp.fujitsu.com |
| Huawei | Haoping Yu Meng Xu Wei (Rick) Wang Zhan Ma | haoping.yu@huawei.com m.xu@huawei.com rick.wang.101@gmail.com Wei.Wang.sc@huawei.com Zhan.Ma@huawei.com Zhan.ma@outlook.com |
| Intel | Lidong Xu Wenhao Zhang Yi-Jen Chiu | lidong.xu@intel.com wenhao.zhang@intel.com yi-jen.chiu@intel.com |
| InterDigital | Jiun-Yu Kao Xiaoyu Xiu Yan Ye Yuwen He | jiun-yu.kao@interdigital.com xiaoyu.xiu@interdigital.com yan.ye@interdigital.com yuwen.he@interdigital.com |
| ITRI International | Ching-Chieh Lin Chun-Lung Lin Yao-Jen Chang | JackLin@itri.org.tw JackLin.itri@gmail.com Chunlung@itri.org.tw britpablo@itri.org.tw |
| KDDI | Kei Kawamura | ki-kawamura@kddi.com |
| LG Electronics | Sehoon Yea Sunmi Yoo | sehoon.yea@lge.com sunmi.yoo@lge.com |
| MediaTek | Jing Ye Jungsun Kim PoLin Lai Xiaozhong Xu Shan Liu Shih-Ta Hsiang Tzu-Der (Peter) Chuang Yi-Wen Chen Yu-Chen Sun Yu-Wen Huang | jing.ye@mediatek.com jungsun.kim@mediatek.com polin.lai@mediatek.com xiaozhong.xu@mediatek.com shan.liu@mediatek.com shih-ta.hsiang@mediatek.com peter.chuang@mediatek.com yiwen.chen@mediatek.com yc.sun@mediatek.com yuwen.huang@mediatek.com |
| Mitsubishi Electric | Akira Minezawa Kazuyuki Miyazawa | Minezawa.Akira@ds.MitsubishiElectric.co.jp Miyazawa.Kazuyuki@cw.MitsubishiElectric.co.jp |
| MERL | Robert Cohen Xingyu Zhang | cohen@merl.com xzhang@merl.com |
| Microsoft | Jizheng Xu Xun Guo | jzxu@microsoft.com xunguo@microsoft.com |
| NCTU | Chun-Chi Chen Tsui-Shan Chang Wen-Hsiao Peng | cheerchen.cs98g@g2.nctu.edu.tw abaibai1122.cs01g@nctu.edu.tw wpeng@cs.nctu.edu.tw |
| Nokia | Jani Lainema Döne Bugdayci | jani.lainema@nokia.com ext-done.bugdayci@nokia.com |
| Qualcomm | Feng Zou Joel Sole Krishna Rapaka Marta Karczewicz Rajan Joshi Vadim Seregin Wei Pu | fzou@qti.qualcomm.com joels@qti.qualcomm.com krapaka@qti.qualcomm.com martak@qti.qualcomm.com rajanj@qti.qualcomm.com vseregin@qti.qualcomm.com wpu@qti.qualcomm.com |
| Samsung | Ankur Saxena Madhukar Budagavi | a.saxena1@samsung.com saxena.ankur0@gmail.com m.budagavi@samsung.com |
| Sharp | Jane Zhao Kiran Misra Seung-Hwan Kim | jzhao@sharplabs.com misrak@sharplabs.com kimse@sharplabs.com |
| Sony | Ohji Nakagami Cheung Auyeung | ohji.nakagami@jp.sony.com Cheung.Auyeung@am.sony.com |
| University of Hanover | Thorsten Laude | laude@tnt.uni-hannover.de |

# Software

All tests are to be built on top of SCM-3.0.

# Test condition

All to-be-tested methods shall be tested under SCM-3.0 common test condition [1].

# To-be-tested methods

## Category A – Generalized copy above mode for palette Index coding

### Test A.1 – Extended copy above mode to the first line

- Proponent: MediaTek, JCTVC-S0079/S0114; Qualcomm, JCTVC-S0155

- Crosschecker:Fujitsu

- Description:

This test will evaluate enabling ‘Copy Above’ mode for coding palette indices in the first line of a palette-mode CU. The indices of the neighbouring CU pixels are used as reference indices for ‘Copy Above’ mode, and the indices are derived with the palette of the current CU. Simplifications for the index conversion (from pixel to index) will be tested. Specifically, the following tests will be conducted.

- Index conversion with search over all palette colours

  - A.1.1 with SCM3.0 palette index redundancy removal on all indices coded.

  - A.1.2 with SCM3.0 palette index redundancy removal on applicable indices.

- Index conversion with reduced number of colours that are searched (from 32 to 8)

  - A.1.3 with SCM3.0 palette index redundancy removal on all indices coded.

  - A.1.4 with SCM3.0 palette index redundancy removal on applicable indices.

- A.1.5: Directly copy the pixel values, instead of converting the pixel values into palette indices.

- Index conversion with search using only the primary colour component, with or without reduced number of search

  - A.1.6 with SCM3.0 palette index redundancy removal on all indices coded.

  - A.1.7 with SCM3.0 palette index redundancy removal on applicable indices.

- A.1.8: Infer neighbouring CU pixels indices as 0 (similar to JCTVC-S0155).

- Infer indices as 0 (similar to JCTVC-S0155).

- Reduce number of colors that are searched (from 32 to 8).

- Search using only the primary color component.

- Directly copy the pixel value, instead of converting the pixel values into palette indices.

- Constraints [REVISE, precise descriptions] on the redundancy index removal will be tested, to address the implementation limitation. [Revised version here]: When the previous pixel is copied using the proposed copy_above methods, applying or not applying the SCM3.0 palette index redundancy removal, will be tested.

### Test A.2 – Generalized copy above mode

- Proponent: MediaTek, JCTVC- S0120; Qualcomm, JCTVC-S0174

- Crosschecker: InterDigital

- Description:

With the extended copy mode techniques described in Test A.1, this test will evaluate a “generalized copy above” mode, where the converted indices or pixel values can be copied. The proposed “generalized copy above” mode is signalled by a “copy above” mode flag, followed by an offset which specifies the distance between the row being copied from and the current row being coded. Constraints on the maximum offset for each row will also be tested, and results for the mode constrained within the CU will be provided as well.

## Category B – Colour index coding

    - Test B.1 – Modified palette run coding (S0111)

- Proponent: Qualcomm, JCTVC-S0111

- Crosschecker: ITRI

- Description:

This test will evaluate a modification of the binarization for run coding. A shorter codeword is assigned to the run to the end of the block. Two methods will be tested. In one method, a symbol (such as 3) is reserved for the run to the end of the block. The reserve value may be varied depending on the maximum possible run. Different reserve values will be tested. In the second method, sending an additional flag to indicate the run to the end of the block will be tested. Context coding of the flag will be tested. Related redundancy removals such as conditional signalling, CU level enabling flag, will also be tested in combination with the proposed methods.

## Category C – 2-D index map coding of palette mode

### Test C.1 – 2-D index map coding of palette mode

- Proponent: Huawei, JCTVC-S0151

- Crosschecker: Qualcomm

- Description:

This test will evaluate 2-D index map coding methods designed for improving the performance of the palette mode. The proposed method is available as an additional method for coding palette indices. A flag associated with each run indicates which method to use (1-D or 2-D). The 1-D method is a modified version of the current method for coding palette indices. If 2D_flag = 1, COPY_ABOVE flag is skipped, otherwise, COPY_ABOVE flag is coded using only one context (2 contexts are used in the default RUN-based 1 search)

## Category D – Encoder modification of palette coding for escape pixels

### Test D.1 – Encoder modification of palette coding for escape pixels

- Proponent: ITRI, JCTVC-S0048

- Crosschecker: Qualcomm

- Description:

This test evaluates a fast algorithm that adaptively determines the size of major color table. The algorithm may change a number of major colour pixels to escape pixels or change a number of escape pixels to major colour pixels.

# Timeline

T1: Release of SCM-3.0, Nov. 17, 2014 (US Pacific Time)

T2: Finalization of CE description, T2 = T1 + 1 week, Nov. 24, 2014

T3: Release of to-be-tested software packages and start crosschecking, T3 = T1 + 3 weeks, Dec. 8, 2014

T4: Release of text specifications, T4 = T1 + 5 weeks, Dec. 22, 2014

T5: Release of all simulation results and finish crosschecking, T5 = T1 + 6 weeks, Dec. 29, 2014

T6: Document upload deadline, Friday, Jan. 30, 2015

The 20th JCT-VC meeting will be held in Geneva during February 10-18, 2015.

# References

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common Test Conditions for Screen Content Coding,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S1015, 19th Meeting: Strasbourg, FR, Oct. 2014.