| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S1102 |
| --- | --- |

| Title: | Description of Core Experiment 2 (CE2): Intra block copy relationship to inter coding |  |  |
| --- | --- | --- | --- |
| Status: | Output Document of JCT-VC |  |  |
| Purpose: | CE Description |  |  |
| Author(s) or Contact(s): | Shan Liu Chao Pang Ji-Zheng Xu | shan.liu@mediatek.com cpang@qti.qualcomm.com jzxu@microsoft.com |  |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document provides the description of Core Experiment 2 (CE2) on intra block copy relationship to inter coding.

# Introduction

At the 19th JCT-VC meeting, the harmonization of intra block copy (IBC) with Inter mode was discussed. The simplified text was presented and coding performance of the harmonized solution was reported. At the same time it was also asserted that some of the coding gains may come from features which are enabled by ref_idx signalling; or may be achieved by some encoder side improvements.

The purpose of this core experiment is to evaluate the performance and complexity of different aspects with regard to IBC and inter harmonization.

# Participant list

| Company/Organization | Participant Name | Email Address |
| --- | --- | --- |
| Canon | Patrice Onno Guillaume Laroche Christophe Gisquet | Patrice.Onno@crf.canon.fr Guillaume.LAROCHE@crf.canon.fr Christophe.Gisquet@crf.canon.fr |
| Canon (Australia) | Chris Rosewarne | Chris.Rosewarne@cisra.canon.com.au |
| Intel | Wenhao Zhang Lidong Xu Yi-Jen Chiu | Wenhao.zhang@intel.com lidong.xu@intel.com yi-jen.chiu@intel.com |
| InterDigital | Yuwen He Xiaoyu Xiu Yan Ye | yuwen.he@interdigital.com xiaoyu.xiu@interdigital.com yan.ye@interdigital.com |
| ITRI | Chun-Lung Lin Ching-Chieh Lin | Chunlung@itri.org.tw JackLin@itri.org.tw |
| MediaTek | Xiaozhong Xu Polin Lai Tzu-Der Chuang Shan Liu | xiaozhong.xu@mediatek.com polin.lai@mediatek.com peter.chuang@mediatek.com shan.liu@mediatek.com |
| Microsoft | Bin Li Jizheng Xu | libin@microsoft.com jzxu@microsoft.com |
| Mitsubishi Electric | Akira Minezawa Kazuyuki Miyazawa | Minezawa.Akira@ds.MitsubishiElectric.co.jp Miyazawa.Kazuyuki@cw.MitsubishiElectric.co.jp |
| NCTU | Wen-Hsiao Peng Chun-Chi Chen Ru-Ling Liao | wpeng@cs.nctu.edu.tw cheerchen.cs98g@g2.nctu.edu.tw jlliao.cs02g@nctu.edu.tw |
| Nokia | Jani Lainema | jani.lainema@nokia.com |
| Qualcomm | Chao Pang Krishna Rapaka Vadim Seregin | cpang@qti.qualcomm.com krapaka@qti.qualcomm.com vseregin@qti.qualcomm.com |
| Samsung | Madhukar Budagavi Ankur Saxena | m.budagavi@samsung.com a.saxena1@samsung.com |
| Sharp | Kiran Misra | misrak@sharplabs.com |

# Software

The software to be used in this CE is SCM-3.0.

# Test conditions

The common conditions for screen content coding tests from JCTVC-S1015 shall be used for these experiments.

# Tests

## On Harmonization of IBC with inter

### Test 1: Intra block copy and Inter signalling unification (JCTVC-S0113, JCTVC-S0302)

In JCTVC-S0113 and JCTVC-S0302, the IBC mode is signalled without intra_bc_flag by reusing Inter signalling by adding the current picture to the reference picture list(s). With the shared signalling, IBC can be differentiated from the conventional Inter by checking the selected reference picture.

  - On PU IBC on/off

### Test 2: Intra block copy signalled at PU level (JCTVC-S0121)

In JCTVC-S0121, the result for PU level IBC on/off on top of SCM-2.0 was provided. In this test, the performance of PU level IBC on/off will be tested on SCM-3.0.

  - On IBC merge/skip mode

### Test 3.1: IBC merge mode using HEVC merge candidates (JCTVC-S0081)

In JCTVC-S0081, merge candidates for IBC are constructed according to inter mode. In this test, the performance of this method will be tested.

### Test 3.2: IBC merge mode using default candidates (JCTVC-S0123)

In JCTVC-S0123, some default IBC merge candidates are added to the merge list when the merge list is not full. In this test, the performance of this method will be tested on SCM-3.0.

## On IBC deblocking

### Test 4.1: On deblocking for screen content coding (JCTVC-S0045)

The boundary strength rule is altered as follows:

         If either side of an edge is IntraBC, BS = 1,

         else, if either side of an edge is Intra, BS = 2,

         else, apply inter rules to select between BS = 1 or BS = 0.

### Test 4.2: Inter deblocking for IBC (JCTVC-S0112)

In JCTVC-S0112, IBC is treated as Inter during the deblocking process instead. Moreover, in order to keep the deblocking module unchanged, after an IBC block is decoded and before the deblocking process, it is converted to be a uni-directionally predictive Inter block with the BV being its MV and the current picture being the reference picture.

## On Vector coding

### Test 5.1: Vector binarization (JCTVC-S0142)

In this test, first a context coded flag is coded to indicate MVD is zero. When MVD is not zero, another flag may be coded to indicate if absolute level of MVD is greater than N. If MVD is greater than N, the remaining absolute level is coded using bypass Exp-Golomb codes. One flag is coded to indicate sign of the vector. Additionally, other Golomb codes would be tested and restrictions may be tested to reduce number of regular bins.

Further this test will be tested with other related tests described in this document.

### Test 5.2: IBC BV coding for small PUs (JCTVC-S0033)

JCTVC-S0033 proposes to limit the number of full "unrestricted" intra block copy vectors signalled for a CU to one. In the case the CU is split to multiple PUs, the first PU will have a traditional block vector associated with it, while the rest of the block vectors are restricted to either use the vertical component of the primary block vector or have a zero vertical component. The choice between primary and zero vertical block vector components is bypass coded in order to improve CABAC throughput. Effects of different search areas and encoder side parallel processing approaches may be reported as part of the experiment.

### Test 5.3: Block vector coding for IBC (JCTVC-S0143)

### In this test following aspects will be tested separately and jointly. a) One flag is added to specify whether the vector is predicted or not. If not predicted, vector is coded without prediction. b) Reducing redundancy of vectors with respect to their sign and absolute values.

Further this test will be tested with other related tests described in this document.

## On BV Prediction

### Test 6.1: On BV predictor (JCTVC-S0087)

In JCTVC-S0087, IBC merge mode is simulated using the current BV predictors and BVD=0. In this test, the performance will be evaluated in the context of SCM3.0.

### Test 6.2: On IBC BV prediction method (JCTVC-S0112)

In JCTVC-S0112, the line buffer for HEVC AMVP is shared with IBC such that the above spatial neighbours can be used for BV predictor as well with no additional buffer requirement.

Further this test will be tested with other related tests described in this document.

## Combination Tests

### Test 7: combination of Test 1 and Test 3.2

In this test, the software in test 1 is used as the platform. Default IBC merge candidates are added to the merge list when the list is not full.

### Test 8: combination of Test 1 and Test 5.1, Test 5.3

The binarization methods detailed in test 5.1 and test 5.3 are tested under the unification framework of Test 1 and studied.

- Cross-check

|  | Proponent | Cross-checker |
| --- | --- | --- |
| Test 1 | Qualcomm, MediaTek, Microsoft |  |
| Test 2 | MediaTek | Intel |
| Test 3.1 | Microsoft | MediaTek |
| Test 3.2 | MediaTek | Microsoft |
| Test 4.1 | Canon | Qualcomm |
| Test 4.2 | Qualcomm | Canon |
| Test 5.1 | Qualcomm | Canon |
| Test 5.2 | Nokia | Qualcomm |
| Test 5.3 | Qualcomm, Mitsubishi |  |
| Test 6.1 | Microsoft | MediaTek |
| Test 6.2 | Qualcomm |  |
| Test 7 | MediaTek | Microsoft |
| Test 8 | Qualcomm |  |

# Timeline

T1: Release of SCM-3.0, November 14, 2014 (US Pacific Time)

T2: Finalization of CE description, T2 = T1 + 1 week, November 21, 2014

T3: Release of CE software packages and start crosschecking, T3 = T1 + 3 weeks, December 5, 2014

T4: Release of CE software packages for combined tests, T4 = T3 + 2 weeks, December 19, 2014

T5: Release of text specifications, T5 = T4 + 3 weeks, January 9, 2014

T6: Release of simulation results, T6 = T3 + 5 weeks, January 9, 2015

T6: Document upload deadline, January, 30, 2015

The 20th JCT-VC meeting will be held in Geneva during February 10-18, 2015.