| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0002 |
| --- | --- |

| Title: | JCT-VC AHG report: HEVC test model editing and errata reporting (AHG2) |  |  |
| --- | --- | --- | --- |
| Status: | AHG report input to JCT-VC |  |  |
| Purpose: | AHG report |  |  |
| Author(s) or Contact(s): | Benjamin Bross (Fraunhofer HHI) Ken McCann (Samsung/Zetacast) Chris Rosewarne (Canon) Matteo Naccari (BBC) Jens-Rainer Ohm (RWTH Aachen) Karl Sharman (Sony) Gary J. Sullivan (Microsoft) Ye-Kui Wang (Qualcomm) | Email: | benjamin.bross@hhi.fraunhofer.de ken@zetacast.com Chris.Rosewarne@cisra.canon.com.au Matteo.Naccari@bbc.co.uk ohm@ient.rwth-aachen.de Karl.Sharman@eu.sony.com garysull@microsoft.com yekuiw@qti.qualcomm.com |
| Source: | AHG |  |  |

_____________________________

# Abstract

This document reports the work of the JCT-VC ad hoc group on HEVC test model editing and errata reporting (AHG2) between the 19th meeting in Strasbourg, FR (October 2014) and the 20th meeting in Geneva, CH (February 2015).

# Mandate

| Title and Email Reflector | Chairs | Meeting |
| --- | --- | --- |
| HEVC test model editing and errata reporting (AHG2) (jct-vc@lists.rwth-aachen.de) - Produce and finalize JCTVC-S1002 HEVC Test Model 16 (HM 16) Improved Encoder Description, including merging of the RExt and prior HM test model descriptions - Collect reports of errata for HEVC - Gather and address comments for refinement of these documents. - Coordinate with AHG3 on software development and HM software technical evaluation to address issues relating to mismatches between software and text. | B. Bross K. McCann C. Rosewarne (co-chairs) M. Naccari J.-R. Ohm K. Sharman G. J. Sullivan Y.-K. Wang (vice-chairs) | No |

# Status of AHG2 Work

An issue tracker (https://hevc.hhi.fraunhofer.de/trac/hevc) was used in order to facilitate the reporting of errata with the HEVC documents.

The ‘High Efficiency Video Coding (HEVC) Test Model 16 (HM 16) Improved Encoder Description’ was published as JCTVC-S1002. This document represented a refinement of the previous HM16 Encoder Description document (JCTVC-R1002). In particular, numerous tool descriptions were improved and command-line description separated out, due to the availability of the HM software manual. The resultant document provides a source of general tutorial information on HEVC Edition 1 and Range Extensions, together with an encoder-side description of the HM-16 software.

A list of bug fixes for HEVC Edition 2 provided by some of the editors of HEVC Edition 2 was collected and included in the attachment.

The following discussion occurred, some text clarifications seems beneficial:

Alexis Tourapis:

We were wondering if there might be an omission in Table A.1 of the HEVC specification. In particular, the table does not seem to mention Cross-Component prediction (cross_component_prediction_enabled_flag). This flag is also not mentioned explicitly in the formal profile definition unlike other flags (e.g.  cabac_bypass_alignment_enabled_flag). Given the naming of this table, i.e. "Allowed values for syntax elements in the format range extensions profiles", we found that it is possible that someone could misinterpret this to mean that this tool is not supported by the existing HEVC profiles, even though it is available. Wouldn't it therefore be better if it was either included in this table or explicitly mentioned in section A.3.5 and wherever else it is appropriate? I wonder if also the same applies to other flags such as high_precision_offsets_enabled_flag among others.

Gary Sullivan and Karl Sharman:

> But I think it should be obvious that there are lots of syntax

> elements that are not discussed in the profile specifications, and the

> values of those are not constrained unless some constraint is

> expressed somewhere else in the spec.

This is related (actually, the exact opposite) to bug #1366, where someone was producing 4:2:0 streams with CCP enabled, because, unlike other new RExt syntax elements, CCP was not constrained in Annex A. However, I indicated to that reporter that CCP was sufficiently constrained elsewhere in the Spec, due to it only being permitted for 4:4:4 chroma formats.

> Perhaps the table title for Table A.1 should say "Constraints on

> values for syntax elements ..." rather than "Allowed values for syntax

> elements ..."

Yes, I think that "Constraints on ..." looks more consistent.

> Is that syntax element the only one in a certain category that is not

> listed in the table?

There are 4 SE's from the SPS/PPS not listed in Table A.1 or the profile definition:

Syntax Element                            Constraint

cross_component_prediction_enabled_flag   via chroma format only.

high_precision_offsets_enabled_flag       in main/main10/etc

log2_sao_offset_scale_luma                mathematically (bit depth > 10)

log2_sao_offset_scale_chroma              mathematically (bit depth > 10)

> If so, then maybe it does merit a NOTE or something.

Although I agree it is not necessary, a NOTE may help advertise the availability of those tools, and it could also be used to highlight that there may be constraints expressed elsewhere.

This also affects Table A.3, and a similar note should be added to section A.3.6 as well.

Alexis Tourapis:

I heard that there is also some confusion with the dash “-“ in table A.1 as well. Although it would seem obvious to us, the interpretation that I have heard is that if there is a dash, that feature is not supported. A tick mark might be a bit better. Unfortunately, a dash can symbolize a variety of things and (as David pointed out to me) it can even symbolize an ambiguity or a mixture of options supported in a document. Using a better symbol or being explicit in terms of what we support (e.g. for Main 4:4:4 it is clear that chroma_format_idc takes values 0..3) would be preferred.

To be honest, I personally also find the table on Wikipedia a bit cleaner (http://en.wikipedia.org/wiki/High_Efficiency_Video_Coding). This places the profiles in the column headings instead of the other way around. I understand that given that we have also the intra profiles that would be difficult in a single table, but could it look a bit neater if we split table A.1 into two subtables, with the extra one containing only intra (and still?) profiles? But I guess that is unlikely given also the format of tables A.2 and A.3 (and less important I guess. This is maybe more of a personal preference).

A second (unrelated) point was raised:

1 - There is a problem in paragraph 3.b. on page 174, as shown in the following excerpt :

"... clause 8.7.2.5.6 is invoked with sample values pi,0, qi,0 with i = 0..3 ..."

I suspect the "i = 0..3" is wrong as it would imply 8 samples while clause 8.7.2.5.6 requires only 4 samples. Instead, as in the sibbling paragraph 3.b. on page 175, it should probably read something like the following :

"... clause 8.7.2.5.6 is invoked with sample values p0,0, p3,0, q0,0 and q3,0 ..."

2 - There is a problem in paragraph 3.d. on page 174, as shown in the following excerpt :

"... clause 8.7.2.5.6 is invoked with sample values pi,3, qi,3 with i = 0..3 ..."

I suspect the "i = 0..3" is wrong as it would imply 8 samples while clause 8.7.2.5.6 requires only 4 samples. Instead, as in the sibbling paragraph 3.d. on page 175, it should probably read something like the following :

"... clause 8.7.2.5.6 is invoked with sample values p0,3, p3,3, q0,3 and q3,3 ..."

In discussion amongst Karl Sharman, Matteo Naccari and Chris Rosewarne it was agreed that this does not cause ambiguous behaviour, however it would be desirable to have sub-clause 8.7.2.5.6 invoked with 4 samples, as per other invocations.

# Recommendations

The recommendations of the HEVC test model editing and errata reporting AHG are for JCT-VC to:

- Encourage the use of the issue tracker to report issues with the text of both the HEVC specification and the Encoder Description

- Identify and resolve any residual issues relating to mismatches between software and text

- Review the above reported discussions and conclude whether and what changes should be made to the HEVC version 2 specification

- Review the list of bug fixes collected for HEVC Edition 2, and include all confirmed bug fixes, including the outcome of the above items, if any, into a JCT-VC output document for the purpose of HEVC Edition 2 defect reporting

# References

- JCTVC-R1002, “High Efficiency Video Coding (HEVC) Test Model 16 (HM 16) Encoder Description”, 18th JCT-VC meeting, Sapporo, JP, 30 Jun. –11 Jul. 2014.

- JCTVC-R1003, “High Efficiency Video Coding (HEVC) Edition 1 Defect Report for Single-Layer Coding”, 17th JCT-VC meeting, Sapporo, JP, 30 Jun. –11 Jul. 2014.

- JCTVC-S1002, “High Efficiency Video Coding (HEVC) Test Model 16 (HM 16) Improved Encoder Description”, 19th JCT-VC meeting, Strasbourg, FR, 17 Oct. –24 Oct. 2014.