| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0022 |
| --- | --- |

| Title: | CE2: Summary report for Core Experiment 2 on Intra block copy relationship to inter coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Shan Liu Chao Pang Ji-Zheng Xu |  | shan.liu@mediatek.com cpang@qti.qualcomm.com jzxu@microsoft.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document summarizes the activities in Core Experiment 2 on Intra block copy relationship to inter coding. A set of individual coding tools and their combinations were evaluated. The evaluated tools and the test conditions are described in document JCTVC-S1102 [1].

# Introduction

At the 19th JCT-VC meeting, the harmonization of Intra block copy (IBC) with inter mode was discussed. The simplified text was presented and coding performance of the harmonized solution was reported. At the same time it was also asserted that some of the coding gains may come from features which are enabled by ref_idx signalling; or may be achieved by some encoder side improvements.

This core experiment was therefore established to further evaluate the performance and complexity of different aspects with regard to IBC and inter harmonization.

# List of participants

| Company/ Organization | Participants | Email Address | Proponent | Cross-checker |
| --- | --- | --- | --- | --- |
| Canon | Patrice Onno Guillaume Laroche Christophe Gisquet | Patrice.Onno@crf.canon.fr Guillaume.LAROCHE@crf.canon.fr Christophe.Gisquet@crf.canon.fr |  | X |
| Canon (Australia) | Chris Rosewarne | Chris.Rosewarne@cisra.canon.com.au | X | X |
| Intel | Wenhao Zhang Lidong Xu Yi-Jen Chiu | Wenhao.zhang@intel.com lidong.xu@intel.com yi-jen.chiu@intel.com |  | X |
| InterDigital | Yuwen He Xiaoyu Xiu Yan Ye | yuwen.he@interdigital.com xiaoyu.xiu@interdigital.com yan.ye@interdigital.com |  |  |
| ITRI | Chun-Lung Lin Ching-Chieh Lin | Chunlung@itri.org.tw JackLin@itri.org.tw |  |  |
| MediaTek | Xiaozhong Xu Polin Lai Tzu-Der Chuang Shan Liu | xiaozhong.xu@mediatek.com polin.lai@mediatek.com peter.chuang@mediatek.com shan.liu@mediatek.com | X | X |
| Microsoft | Bin Li Jizheng Xu | libin@microsoft.com jzxu@microsoft.com | X | X |
| Mitsubishi Electric | Akira Minezawa Kazuyuki Miyazawa | Minezawa.Akira@ds.MitsubishiElectric.co.jp Miyazawa.Kazuyuki@cw.MitsubishiElectric.co.jp | X |  |
| NCTU | Wen-Hsiao Peng Chun-Chi Chen Ru-Ling Liao | wpeng@cs.nctu.edu.tw cheerchen.cs98g@g2.nctu.edu.tw jlliao.cs02g@nctu.edu.tw |  |  |
| Nokia | Jani Lainema | jani.lainema@nokia.com |  |  |
| Qualcomm | Chao Pang Krishna Rapaka Vadim Seregin | cpang@qti.qualcomm.com krapaka@qti.qualcomm.com vseregin@qti.qualcomm.com | X | X |
| Samsung | Madhukar Budagavi Ankur Saxena | m.budagavi@samsung.com a.saxena1@samsung.com |  |  |
| Sharp | Kiran Misra Seung-Hwan Kim | misrak@sharplabs.com kimse@sharplabs.com |  |  |

# List of CE contributions

| Doc No. | Title | Participants | Type |
| --- | --- | --- | --- |
| JCTVC-T0094 | CE2 Test1: Intra block copy and inter signalling unification | Qualcomm, MediaTek, Microsoft | Proposal |
| JCTVC-T0071 | CE2: Test 2 – Intra BC signalled at PU level | MediaTek | Proposal |
| JCTVC-T0040 | CE2: results of test 3.1 and 6.1 | Microsoft | Proposal |
| JCTVC-T0073 | CE2: Test 3.2 – Intra BC merge mode with default candidates | MediaTek | Proposal |
| JCTVC-T0039 | CE2 Test 4.1: On deblocking for screen content coding | Canon | Proposal |
| JCTVC-T0095 | CE2 Test 4.2: Inter deblocking for intra block copy | Qualcomm | Proposal |
| JCTVC-T0089 | CE2: Test 5.1 and Test 8.1 on vector binarization | Qualcomm | Proposal |
| JCTVC-T0090 | CE2: Test 5.3 and Test 8.2 on block vector (BV) coding for IBC | Qualcomm, Mitsubishi | Proposal |
| JCTVC-T0096 | CE2 Test 6.2: Intra block copy block vector prediction with line buffer | Qualcomm | Proposal |
| JCTVC-T0075 | CE2: Test 7 – Combination of Test 1 and Test 3.2 | MediaTek | Proposal |
| JCTVC-T0046 | CE2: Cross-check of Test 1 on intra block copy and inter signalling unification | Mitsubishi | Crosscheck |
| JCTVC-T0143 | CE2 Test2: Crosscheck of Intra Block Copy Signalled at PU Level (JCTVC-T0071) | Intel | Crosscheck |
| JCTVC-T0083 | Crosscheck of CE2 Test 3.1 (JCTVC-T0040) | MediaTek | Crosscheck |
| JCTVC-T0158 | CE2: Cross-check of test 3.2 (JCTVC-T0073) | Microsoft | Crosscheck |
| JCTVC-T0100 | Crosscheck of CE2 Test 4.1 on intra block copy deblocking | Qualcomm | Crosscheck |
| JCTVC-T0155 | Cross-check of ‘CE2 Test 4.2: Inter deblocking for intra block copy’ (JCTVC-T0095) by Qualcomm | Canon | Crosscheck |
| JCTVC-T0156 | Cross-check of ‘CE2: Test 5.1 and Test 8.1 on vector binarization’ (JCTVC-T0089) by Qualcomm | Canon | Crosscheck |
| JCTVC-T0144 | CE2 Test5.3: Crosscheck of Block Vector Coding for IBC (JCTVC-T0090) | Intel | Crosscheck |
| JCTVC-T0084 | Crosscheck of CE2 Test 6.1 (JCTVC-T0040) | MediaTek | Crosscheck |
| JCTVC-T0085 | Crosscheck of CE2 Test 6.2 (JCTVC-T0085) | MediaTek | Crosscheck |
| JCTVC-T0159 | CE2: Cross-check of test 7 (JCTVC-T0075) | Microsoft | Crosscheck |

# Summary of CE proposals

## On Harmonization of IBC with inter

### Test 1: Intra block copy and Inter signalling unification (JCTVC-T0094)

In JCTVC-S0113 and JCTVC-S0302, the IBC mode is signalled without intra_bc_flag but reusing inter signalling by adding the current picture to the reference picture list(s) instead. With the shared signalling, IBC can be differentiated from the conventional inter mode by checking the selected reference picture.

## On PU IBC on/off

### Test 2: Intra block copy signalled at PU level (JCTVC-T0071)

In JCTVC-S0121, the result for PU level IBC on/off on top of SCM-2.0 was provided. In this test, the performance of PU level IBC on/off is tested on SCM-3.0.

## On IBC merge/skip mode

### Test 3.1: IBC merge mode using HEVC merge candidates (JCTVC-T0040)

In JCTVC-S0081, merge candidates for IBC are constructed according to inter mode. In this test, the performance of this method is tested on SCM-3.0.

### Test 3.2: IBC merge mode using default candidates (JCTVC-T0073)

In JCTVC-S0123, some default IBC merge candidates are added to the merge list when the merge list is not full. In this test, the performance of this method is tested on SCM-3.0.

## On IBC deblocking

### Test 4.1: On deblocking for screen content coding (JCTVC-T0039)

In this test, the boundary strength rule is altered as follows:

         if either side of an edge is IntraBC, BS = 1,

         else, if either side of an edge is Intra, BS = 2,

         else, apply inter rules to select between BS = 1 or BS = 0.

### Test 4.2: Inter deblocking for IBC (JCTVC-T0095)

In JCTVC-S0112, IBC is treated as inter mode during the deblocking process. Moreover, in order to keep the deblocking module unchanged, after an IBC block is decoded and before the deblocking process, it is converted to be a uni-directionally predictive inter block with the BV being its MV and the current picture being the reference picture.

## On Vector coding

### Test 5.1: Vector binarization (JCTVC-T0089)

In this test, first a context coded flag is coded to indicate MVD is zero. When MVD is not zero, another flag may be coded to indicate if absolute level of MVD is greater than N. If MVD is greater than N, the remaining absolute level is coded using bypass Exp-Golomb codes. One flag is coded to indicate sign of the vector.

### Test 5.2: IBC BV coding for small PUs (withdrawn)

### Test 5.3: Block vector coding for IBC (JCTVC-T0090)

### In this test following aspects will be tested separately and jointly. a) One flag is added to specify whether the vector is predicted or not. If not predicted, vector is coded without prediction. b) Reducing redundancy of vectors with respect to their sign and absolute values.

The combination of 5.1+5.3 is tested and results are provided.

## On BV Prediction

### Test 6.1: On BV predictor (JCTVC-T0040)

In JCTVC-S0087, IBC merge mode is simulated using the current BV predictors and BVD=0. In this test, the performance is evaluated in the context of SCM-3.0.

### Test 6.2: On IBC BV prediction method (JCTVC-T0096)

In JCTVC-S0112, the line buffer for HEVC AMVP is shared with IBC such that the above spatial neighbours can be used for BV predictor as well with no additional buffer requirement.

## Combination Tests

### Test 7: combination of Test 1 and Test 3.2 (JCTVC-T0075)

In this test, the software in test 1 is used as the platform. Default IBC merge candidates are added to the merge list when the list is not full.

### Test 8: combination of Test 1 and Test 5.1, Test 5.3 (JCTVC-T0089, JCTVC-T0090)

The binarization methods described in test 5.1 and test 5.3 are tested under the unification framework of Test 1.

# Summary of CE test results

The proposed methods were implemented on top of SCM-3.0 [2] and tested under the SCC common test conditions defined in JCTVC-S1015 [3].

## 444 lossy results

The 444 lossy results are summarized as follows:

|  | 444 Lossy All Intra |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tests | Test 1 | Test 2a | Test 2b | Test 3.1 | Test 3.2 | Test 4.1 | Test 4.2 | Test 5.1 | Test 5.3 | Test 5.1 +5.3 | Test 6.1 T1 | Test 6.1 T2 | Test 6.2 | Test 7 | Test 8 (5.1) | Test 8 (5.1+ 5.3) |
| RGB, TGM | -0.6% | 0.0% | -0.1% | -0.8% | -2.2% | 0.0% | -0.1% | -0.2% | -0.5% | -0.7% | -1.3% | -0.5% | -0.5% | -1.2% | -0.9% | -1.1% |
| RGB, MC | -0.7% | 0.0% | -0.1% | -1.0% | -1.3% | 0.0% | -0.1% | -0.1% | -0.3% | -0.4% | -1.2% | -0.8% | -0.3% | -0.8% | -0.8% | -1.0% |
| RGB, Ani. | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, CC | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, TGM | -0.7% | -0.1% | -0.1% | -1.3% | -2.4% | 0.0% | 0.0% | -0.3% | -0.5% | -0.8% | -1.4% | -0.5% | -0.5% | -1.5% | -1.0% | -1.2% |
| YUV, MC | -0.6% | 0.0% | -0.1% | -1.0% | -1.5% | 0.0% | 0.0% | -0.1% | -0.4% | -0.5% | -1.1% | -0.6% | -0.3% | -1.2% | -0.7% | -0.9% |
| YUV, Ani. | 0.0% | 0.0% | 0.0% | -0.1% | -0.3% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | -0.2% | 0.0% | -0.1% | 0.0% | 0.0% |
| YUV, CC | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| Enc. | 117% | 100% | 99% | 129% | 113% | 100% | 97% | 99% | 104% | 101% | 131% | 126% | 97% | 106% | 112% | 111% |
| Dec. | 102% | 100% | 100% | 101% | 96% | 97% | 96% | 88% | 91% | 91% | 100% | 100% | 96% | 95% | 85% | 86% |

|  | 444 Lossy Random Access |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tests | Test 1 | Test 2a | Test 2b | Test 3.1 | Test 3.2 | Test 4.1 | Test 4.2 | Test 5.1 | Test 5.3 | Test 5.1 +5.3 | Test 6.1 T1 | Test 6.1 T2 | Test 6.2 | Test 7 | Test 8 (5.1) | Test 8 (5.1+ 5.3) |
| RGB, TGM | -2.2% | -1.0% | -1.3% | -0.8% | -1.8% | -0.1% | -0.2% | -1.1% | -0.5% | -1.4% | -1.3% | -0.8% | -0.2% | -2.5% | -3.1% | -3.4% |
| RGB, MC | -0.7% | -0.3% | -0.5% | -0.5% | -0.9% | -0.1% | -0.1% | 0.0% | -0.2% | -0.3% | -0.8% | -0.4% | -0.1% | -1.0% | -0.9% | -0.9% |
| RGB, Ani. | -0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% | 0.0% | -0.1% |
| RGB, CC | -0.2% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | -0.2% |
| YUV, TGM | -2.4% | -1.0% | -1.2% | -1.3% | -2.0% | -0.1% | -0.1% | -1.3% | -0.5% | -1.6% | -1.5% | -0.9% | -0.2% | -2.8% | -3.5% | -3.7% |
| YUV, MC | -0.9% | -0.3% | -0.4% | -0.8% | -1.3% | 0.0% | 0.0% | -0.1% | -0.3% | -0.5% | -1.0% | -0.6% | -0.2% | -1.4% | -1.0% | -1.2% |
| YUV, Ani. | -0.2% | 0.0% | -0.1% | -0.2% | -0.2% | 0.0% | 0.0% | -0.1% | -0.2% | -0.1% | -0.1% | -0.2% | 0.1% | -0.2% | -0.1% | -0.2% |
| YUV, CC | -0.2% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | -0.2% | -0.2% |
| Enc. | 103% | 102% | 102% | 102% | 102% | 100% | 96% | 97% | 101% | 100% | 102% | 103% | 96% | 101% | 95% | 103% |
| Dec. | 96% | 99% | 98% | 102% | 98% | 100% | 100% | 96% | 96% | 98% | 97% | 98% | 97% | 93% | 86% | 84% |

|  | 444 Lossy Low-delay B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tests | Test 1 | Test 2a | Test 2b | Test 3.1 | Test 3.2 | Test 4.1 | Test 4.2 | Test 5.1 | Test 5.3 | Test 5.1 +5.3 | Test 6.1 T1 | Test 6.1 T2 | Test 6.2 | Test 7 | Test 8 (5.1) | Test 8 (5.1+ 5.3) |
| RGB, TGM | -2.3% | -1.4% | -1.7% | -0.7% | -1.2% | -0.2% | -0.2% | -1.6% | -0.5% | -1.8% | -1.1% | -0.9% | -0.1% | -2.4% | -3.6% | -3.8% |
| RGB, MC | -0.9% | -0.6% | -0.8% | -0.4% | -0.8% | -0.2% | -0.1% | -0.3% | -0.4% | -0.3% | -0.6% | -0.4% | -0.1% | -1.3% | -1.2% | -1.2% |
| RGB, Ani. | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | 0.1% | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, CC | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% |
| YUV, TGM | -2.5% | -1.3% | -1.6% | -1.1% | -1.5% | -0.1% | -0.1% | -1.8% | -0.5% | -1.9% | -1.2% | -0.9% | -0.1% | -2.6% | -3.9% | -4.1% |
| YUV, MC | -1.4% | -0.6% | -0.7% | -0.6% | -1.2% | -0.2% | -0.2% | -0.3% | -0.2% | -0.5% | -1.0% | -0.7% | -0.2% | -1.6% | -1.4% | -1.6% |
| YUV, Ani. | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| YUV, CC | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% | 0.1% | -0.1% | 0.1% | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | 0.0% |
| Enc. | 102% | 102% | 102% | 101% | 102% | 100% | 97% | 91% | 97% | 93% | 101% | 101% | 96% | 101% | 91% | 88% |
| Dec. | 102% | 102% | 99% | 101% | 98% | 100% | 100% | 92% | 92% | 93% | 99% | 97% | 98% | 92% | 86% | 83% |

## 444 lossless results

The 444 lossless results are summarized as follows:

|  | 444 Lossless All Intra |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tests | Test 1 | Test 2a | Test 2b | Test 3.1 | Test 3.2 | Test 4.1 | Test 4.2 | Test 5.1 | Test 5.3 | Test 5.1 +5.3 | Test 6.1 T1 | Test 6.1 T2 | Test 6.2 | Test 7 | Test 8 (5.1) | Test 8 (5.1+ 5.3) |
| RGB, TGM | 0.4% | 0.0% | 0.0% | -0.6% | -1.1% | N/A | N/A | -0.1% | -0.2% | -0.2% | -0.9% | -0.6% | -0.1% | 0.2% | 0.4% | 0.3% |
| RGB, MC | -0.9% | 0.0% | 0.0% | -1.4% | -0.6% | N/A | N/A | 0.0% | -0.1% | -0.1% | -1.5% | -1.3% | 0.0% | 0.0% | -0.9% | -1.0% |
| RGB, Ani. | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | N/A | N/A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, CC | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | N/A | N/A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, TGM | 1.2% | 0.0% | 0.0% | -0.5% | -0.8% | N/A | N/A | -0.1% | -0.2% | -0.3% | -0.4% | -0.2% | -0.1% | 0.9% | 1.1% | 1.0% |
| YUV, MC | -0.5% | 0.0% | 0.0% | -1.2% | -0.5% | N/A | N/A | 0.0% | -0.1% | -0.1% | -1.2% | -1.0% | 0.0% | 0.2% | -0.5% | -0.5% |
| YUV, Ani. | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | N/A | N/A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, CC | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | N/A | N/A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc. | 102% | 101% | 100% | 120% | 117% | N/A | N/A | 109% | 105% | 117% | 123% | 120% | 101% | 103% | 100% | 104% |
| Dec. | 100% | 99% | 99% | 100% | 97% | N/A | N/A | 106% | 102% | 112% | 101% | 99% | 100% | 98% | 96% | 99% |

|  | 444 Lossless Random Access |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tests | Test 1 | Test 2a | Test 2b | Test 3.1 | Test 3.2 | Test 4.1 | Test 4.2 | Test 5.1 | Test 5.3 | Test 5.1 +5.3 | Test 6.1 T1 | Test 6.1 T2 | Test 6.2 | Test 7 | Test 8 (5.1) | Test 8 (5.1+ 5.3) |
| RGB, TGM | -0.4% | -1.0% | -1.0% | -0.4% | -0.8% | N/A | N/A | -0.3% | -0.2% | -0.4% | -0.7% | -0.5% | -0.1% | -0.4% | -0.6% | -0.7% |
| RGB, MC | -0.2% | -0.1% | -0.1% | -0.2% | -0.1% | N/A | N/A | 0.0% | 0.0% | 0.0% | -0.2% | -0.2% | 0.0% | -0.1% | -0.2% | -0.2% |
| RGB, Ani. | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | N/A | N/A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, CC | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | N/A | N/A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, TGM | 0.1% | -0.7% | -0.7% | -0.5% | -0.7% | N/A | N/A | -0.2% | -0.2% | -0.4% | -0.5% | -0.3% | -0.1% | 0.0% | -0.1% | -0.2% |
| YUV, MC | -0.1% | -0.1% | -0.1% | -0.2% | -0.1% | N/A | N/A | 0.0% | 0.0% | 0.0% | -0.2% | -0.2% | 0.0% | -0.1% | -0.1% | -0.1% |
| YUV, Ani. | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | N/A | N/A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, CC | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | N/A | N/A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc. | 97% | 103% | 103% | 103% | 103% | N/A | N/A | 107% | 113% | 114% | 102% | 102% | 101% | 100% | 99% | 98% |
| Dec. | 95% | 98% | 98% | 100% | 96% | N/A | N/A | 102% | 104% | 108% | 98% | 99% | 98% | 94% | 94% | 93% |

|  | 444 Lossless Low-delay B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tests | Test 1 | Test 2a | Test 2b | Test 3.1 | Test 3.2 | Test 4.1 | Test 4.2 | Test 5.1 | Test 5.3 | Test 5.1 +5.3 | Test 6.1 T1 | Test 6.1 T2 | Test 6.2 | Test 7 | Test 8 (5.1) | Test 8 (5.1+ 5.3) |
| RGB, TGM | -0.4% | -1.2% | -1.2% | -0.3% | -0.7% | N/A | N/A | -0.3% | -0.2% | -0.4% | -0.6% | -0.5% | -0.1% | -0.4% | -0.7% | -0.7% |
| RGB, MC | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | N/A | N/A | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% |
| RGB, Ani. | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | N/A | N/A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, CC | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | N/A | N/A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, TGM | 0.0% | -0.9% | -0.9% | -0.5% | -0.7% | N/A | N/A | -0.2% | -0.2% | -0.4% | -0.5% | -0.3% | -0.1% | 0.0% | -0.2% | -0.3% |
| YUV, MC | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | N/A | N/A | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% |
| YUV, Ani. | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | N/A | N/A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, CC | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | N/A | N/A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc. | 96% | 101% | 102% | 101% | 102% | N/A | N/A | 100% | 101% | 96% | 101% | 101% | 99% | 99% | 90% | 88% |
| Dec. | 97% | 102% | 99% | 102% | 98% | N/A | N/A | 91% | 90% | 90% | 102% | 98% | 97% | 97% | 84% | 83% |

## 420 lossy results

The 420 lossy results are summarized as follows:

| Tests | Test 1 | Test 2a | Test 2b | Test 3.1 | Test 3.2 | Test 4.1 | Test 4.2 | Test 5.1 | Test 5.3 | Test 5.1 +5.3 | Test 6.1 T1 | Test 6.1 T2 | Test 6.2 | Test 7 | Test 8 (5.1) | Test 8 (5.1+ 5.3) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 420 Lossy All Intra |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TGM, 720p | -2.8% | 0.0% | -0.1% | -2.5% | -2.4% | 0.0% | 0.0% | -0.2% | -0.4% | -0.6% | -2.3% | -1.0% | -0.3% | -3.0% | -3.1% | -3.4% |
| MC, 480p | -0.9% | 0.0% | -0.1% | -0.3% | -0.5% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | -0.4% | -0.2% | -0.1% | -1.3% | -0.9% | -1.0% |
| Ani., 768p | -0.6% | 0.0% | -0.1% | -0.2% | -0.5% | 0.0% | 0.0% | -0.1% | -0.1% | -0.2% | -0.2% | -0.1% | 0.0% | -1.0% | -0.6% | -0.8% |
| Enc. | 94% | 99% | 99% | 107% | 105% | 100% | 97% | 99% | 97% | 97% | 111% | 109% | 99% | 93% | 81% | 81% |
| Dec. | 98% | 98% | 100% | 95% | 94% | 99% | 97% | 97% | 94% | 94% | 96% | 95% | 97% | 95% | 84% | 84% |
| 420 Lossy Random Access |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TGM, 720p | -2.1% | -0.2% | -0.3% | -1.4% | -1.8% | 0.1% | 0.0% | -0.3% | -0.4% | -0.7% | -1.4% | -0.7% | -0.2% | -2.7% | -2.6% | -2.9% |
| MC, 480p | -0.5% | -0.1% | -0.1% | -0.1% | -0.3% | 0.0% | 0.0% | -0.2% | 0.0% | -0.2% | -0.2% | -0.1% | 0.0% | -0.5% | -0.7% | -0.7% |
| Ani., 768p | -0.6% | -0.3% | -0.3% | -0.1% | -0.3% | 0.0% | 0.0% | -0.8% | -0.3% | -0.8% | -0.1% | -0.1% | 0.0% | -0.8% | -1.3% | -1.4% |
| Enc. | 101% | 100% | 101% | 100% | 99% | 100% | 97% | 102% | 112% | 104% | 100% | 100% | 100% | 101% | 92% | 93% |
| Dec. | 93% | 98% | 99% | 97% | 98% | 101% | 101% | 99% | 105% | 100% | 97% | 97% | 99% | 97% | 88% | 86% |
| 420 Lossy Low-delay B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TGM, 720p | -1.3% | -0.9% | -0.8% | -0.6% | -1.5% | 0.0% | -0.2% | -0.9% | -0.5% | -1.1% | -0.7% | -0.5% | 0.1% | -2.0% | -1.9% | -2.3% |
| MC, 480p | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | -0.1% | 0.0% | 0.0% | 0.2% | 0.2% | 0.1% | 0.3% | -0.2% | -0.2% |
| Ani., 768p | -0.4% | -0.4% | -0.3% | 0.0% | -0.2% | 0.1% | 0.0% | -0.8% | -0.2% | -0.9% | -0.1% | 0.0% | 0.0% | -0.2% | -1.1% | -1.2% |
| Enc. | 100% | 100% | 99% | 98% | 99% | 102% | 98% | 103% | 107% | 96% | 99% | 98% | 100% | 99% | 92% | 93% |
| Dec. | 99% | 97% | 97% | 93% | 96% | 101% | 100% | 100% | 99% | 93% | 92% | 94% | 96% | 93% | 87% | 91% |

## 420 lossless results

The 420 lossless results are summarized as follows:

| Tests | Test 1 | Test 2a | Test 2b | Test 3.1 | Test 3.2 | Test 4.1 | Test 4.2 | Test 5.1 | Test 5.3 | Test 5.1 +5.3 | Test 6.1 T1 | Test 6.1 T2 | Test 6.2 | Test 7 | Test 8 (5.1) | Test 8 (5.1+ 5.3) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 420 Lossless All Intra |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TGM, 720p | -0.8% | 0.0% | 0.0% | -1.0% | -0.7% | N/A | N/A | N/A | N/A | N/A | -0.9% | -0.7% | 0.0% | -0.7% | -0.8% | -0.9% |
| MC, 480p | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% | N/A | N/A | N/A | N/A | N/A | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% | -0.1% |
| Ani., 768p | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | N/A | N/A | N/A | N/A | N/A | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% |
| Enc. | 76% | 99% | 99% | 105% | 108% | N/A | N/A | N/A | N/A | N/A | 110% | 107% | 101% | 80% | 67% | 67% |
| Dec. | 98% | 99% | 99% | 98% | 96% | N/A | N/A | N/A | N/A | N/A | 98% | 100% | 99% | 98% | 89% | 91% |
| 420 Lossless Random Access |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TGM, 720p | -0.6% | -0.3% | -0.3% | -0.4% | -0.2% | N/A | N/A | N/A | N/A | N/A | -0.3% | -0.2% | 0.0% | -0.5% | -0.7% | -0.8% |
| MC, 480p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | N/A | N/A | N/A | N/A | N/A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Ani., 768p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | N/A | N/A | N/A | N/A | N/A | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| Enc. | 100% | 100% | 101% | 98% | 99% | N/A | N/A | N/A | N/A | N/A | 99% | 100% | 99% | 98% | 87% | 88% |
| Dec. | 96% | 101% | 99% | 100% | 101% | N/A | N/A | N/A | N/A | N/A | 98% | 99% | 99% | 99% | 88% | 88% |
| 420 Lossless Low-delay B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TGM, 720p | -0.1% | -0.4% | -0.4% | 0.0% | 0.0% | N/A | N/A | N/A | N/A | N/A | 0.0% | -0.1% | 0.0% | -0.2% | -0.2% | -0.2% |
| MC, 480p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | N/A | N/A | N/A | N/A | N/A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Ani., 768p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | N/A | N/A | N/A | N/A | N/A | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| Enc. | 102% | 100% | 100% | 98% | 97% | N/A | N/A | N/A | N/A | N/A | 98% | 97% | 99% | 96% | 86% | 86% |
| Dec. | 101% | 100% | 100% | 98% | 105% | N/A | N/A | N/A | N/A | N/A | 97% | 97% | 89% | 97% | 88% | 87% |

# Complexity analysis

Number of context coded bins per 8x8 block

AI configuration

| Test | SCM-3.0 | Test 1 | Test 5.1 | Test 5.3 | Test 5.1+5.3 | Test 8 (1+5.1) | Test 8 (1+5.1+5.3) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| # of context coded bins | 482 | 484 | 490 | 490 | 490 | 488 | 488 |

LD/RA configuration

| Test | SCM-3.0 | Test 1 | Test 5.1 | Test 5.3 | Test 5.1+5.3 | Test 8 (5.1) | Test 8 (5.1+5.3) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| # of context coded bins | 488 | 488 | 490 | 490 | 490 | 488 | 488 |

# List of CE-related proposals

| Doc No. | Title | Proponents | Type |
| --- | --- | --- | --- |
| JCTVC-T0050 | Non-CE2: IBC encoder improvements and combinations with CE2 | Canon | Proposal |
| JCTVC-T0055 | CE2-related: IntraBC constraint for multiple slices | Mediatek | Proposal |
| JCTVC-T0056 | CE2-related: IntraBC constraint for multiple tiles | Mediatek | Proposal |
| JCTVC-T0057 | CE2-related: Temporal BV merge mode on CE2 Test-1 and Test-7 | Mediatek | Proposal |
| JCTVC-T0081 | CE2-related: on the initialization of block vector prediction for intra block copy | Mediatek | Proposal |
| JCTVC-T0097 | Non-CE2: Block vector prediction method for intra block copy | Qualcomm | Proposal |
| JCTVC-T0098 | Non-CE2: Zero merging candidates derivation for Intra BC/Inter signaling unification | Qualcomm | Proposal |
| JCTVC-T0114 | Non-CE2: Adding a context for significance flag in BVD | Sharp | Proposal |
| JCTVC-T0116 | Non-CE2: encoder improvements on IBC search | InterDigital | Proposal |
| JCTVC-T0117 | Non-CE2: improved inter merge for unified IBC and inter framework | InterDigital | Proposal |
| JCTVC-T0127 | CE2-related: Improved method for entropy coding MVD/BVD | Mediatek | Proposal |

# Other IBC related proposals

| Doc No. | Title | Proponents | Type |
| --- | --- | --- | --- |
| JCTVC-T0043 | On the constraint for Intra BC mode | Microsoft | Proposal |
| JCTVC-T0045 | AHG10: Memory bandwidth reduction for intra block copy | Nokia | Proposal |
| JCTVC-T0049 | On intra mode MPM derivation for SCC | Canon | Proposal |
| JCTVC-T0051 | AHG10: On IBC memory reduction | Canon | Proposal |
| JCTVC-T0114 | Adding a context for significance flag in BVD | Sharp | Proposal |
| JCTVC-T0122 | Implicit transform quadtree partition for intra block copy | InterDigital | Proposal |

# References

- S. Liu, C. Pang, J. Xu, “Description of Core Experiment 2 (CE2): Intra block copy relationship to inter coding”, JCTVC-S1102, Strasbourg, FR, Oct. 2014.

- SCM-3.0, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.2+SCM-3.0/

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” JCTVC-S1015, Strasbourg, FR, Oct. 2014.