| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0034 |
| --- | --- |

| Title: | CE1 Test B.1: Modified Palette Run Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Pu, Rajan Joshi, Marta Karczewicz, Vadim Seregin, Feng Zou 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-651-7749 Wei.Pu@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

A modification of the binarization method for the variable paletteRun in HEVC screen content coding draft text 2 is proposed. A bypass coded flag is used to indicates whether the current palette run reaches the end of the CU. Compared with SCM3.0 anchor under SCC common test condition, the proposed method achieves BD-rates of -0.2% for text and graphics sequences in all intra lossy case.

# Introduction

In [2], the variable paletteRun’s binarization method is specified in Section 7.3.8.8 as:

| if( maxPaletteRun > 0 ) { |  |
| --- | --- |
| palette_run_msb_id_plus1 | ae(v) |
| if( palette_run_msb_id_plus1 > 1 ) |  |
| palette_run_refinement_bits | ae(v) |
| } |  |

In the proposed method, a single bit syntax element named palette_run_to_end_flag is added to indicate whether the current paletteRun reaches the end of the current CU. Specifically, the syntax table can be updated as below:

| if( maxPaletteRun > 0 ) { |  |
| --- | --- |
| palette_run_msb_id_plus1 | ae(v) |
| if( palette_run_msb_id_plus1 > 1 ) { |  |
| if( scanPos && (1<< palette_run _msb_id_plus1) > MaxPaletteRun ) |  |
| palette_run_to_end_flag | ae(v) |
| if( !palette_last_run_flag ) |  |
| palette_run_refinement_bits | ae(v) |
| } |  |
| } |  |

In the case of scanPos equal to 0, the current paletteRun cannot reach to the end due to the adoption of JCTVC-S0110 [3].  In the case of (1<< palette_run _msb_id_plus1) ≤ MaxPaletteRun, the current paletteRun cannot reach to the end due to the SCC semantics specifications in [2]. In these two cases, the redundant palette_run_to_end_flag is not signaled.

# Simulation Results

Simulation are carried out under the test condition specified in CE1 description [5] and SCC common test condition [4]. Only AI lossy results are presented in this section. The complete test results and the source code can be found in the proposal package.

Table 1. Proposed Method v.s. SCM3.0 Anchor

# Conclusion

A modification to the binarization method of paletteRun variable is proposed. The proposed method uses a bypass coded bin, which does not affect the worst case CABAC throughput, while achieving 0.2% BD-rate reduction compared with SCM3.0 under SCC common test condition for text & graphics with motion sequences in AI lossy case.

# References

- W. Pu, R. Joshi, M. Karczewicz, F. Zou, V. Seregin, and J. sole, “Non-CE6: Improvement on palette run coding,” in Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting, JCTVC-S0111, Strasbourg, FR, 17–24 Oct. 2014.

- R. Joshi and J. Xu, “HEVC screen content coding draft text 2,” in Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting, JCTVC-S1005, Strasbourg, FR, 17–24 Oct. 2014.

- W. Pu, M. Karczewicz, F. Zou, V. Seregin, R. Joshi, and J. sole, “Non-CE6: Syntax redundancy fixes for palette mode,” in Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting, JCTVC-S0110, Strasbourg, FR, 17–24 Oct. 2014.

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common test conditions for screen content coding,” in Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting, JCTVC-S1015, Strasbourg, FR, 17–24 Oct. 2014.

- P. Lai, P. Onno, R. Cohen, V. Seregin, X. Xiu, and Z. Ma, “Description of core experiment 1 (CE1): Improvements of palette mode,” in Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting, JCTVC-S1101, Strasbourg, FR, 17–24 Oct. 2014

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).