#### Palette syntax

| palette_coding( x0, y0, nCbS ) { | Descriptor |
| --- | --- |
| palette_share_flag[ x0 ][ y0 ] | ae(v) |
| … | ae(v) |
| scanPos = 0 |  |
| while( scanPos < nCbS * nCbS ) { |  |
| xC = x0 + travScan[ scanPos ][ 0 ] |  |
| yC = y0 + travScan[ scanPos ][ 1 ] |  |
| if( scanPos > 0) { |  |
| xcPrev = x0 + travScan[ scanPos − 1 ][ 0 ] |  |
| ycPrev = y0 + travScan[ scanPos − 1 ][ 1 ] |  |
| } |  |
| if(indexMax > 0 && (scanPos == 0 ¦¦ palette_run_type_flag[ xcPrev ][ ycPrev ] ! = COPY_ABOVE_MODE))) { |  |
| palette_run_type_flag[ xC ][ yC ] | ae(v) |
| } |  |
| if( palette_run_type_flag[ xC ][ yC ] = = COPY_INDEX_MODE && adjustedIndexMax > 0) |  |
| palette_index_idc | ae(v) |
| if( indexMax > 0 ) { |  |
| maxPaletteRun = nCbS * nCbS – scanPos – 1 |  |
| if( maxPaletteRun > 0 ) { |  |
| palette_run_msb_id_plus1 | ae(v) |
| if( palette_run_msb_id_plus1 > 1 ) |  |
| palette_run_refinement_bits | ae(v) |
| } |  |
| } else |  |
| paletteRun = nCbS * nCbS − 1 |  |
| runPos = 0 |  |
| while ( runPos < = paletteRun ) { |  |
| xR = x0 + travScan[ scanPos ][ 0 ] |  |
| yR = y0 + travScan[ scanPos ][ 1 ] |  |
| if( palette_run_type_flag[ xC ][ yC ] = = COPY_INDEX_MODE && paletteIndex = = indexMax ) { |  |
| PaletteSampleMode[ xR ][ yR ] = ESCAPE_MODE |  |
| PaletteIndexMap[ xR ][ yR ] = paletteIndex |  |
| for( cIdx = 0; cIdx < 3; cIdx++ ) { |  |
| palette_escape_val | ae(v) |
| PaletteEscapeVal[ cIdx ][ xR ][ yR ] = palette_escape_val |  |
| } |  |
| } else if(palette_run_type_flag[ xC ][ yC ] = = COPY_INDEX_MODE ) { |  |
| PaletteSampleMode[ xR ][ yR ] = COPY_INDEX_MODE |  |
| if(runPos == 0 ) PaletteIndexMap[ xR ][ yR ] = paletteIndex |  |
| else PaletteIndexMap[ xR ][ yR ] = NCP_INDEX |  |
| } else { |  |
| PaletteSampleMode[ xR ][ yR ] = COPY_ABOVE_MODE |  |
| PaletteIndexMap[ xR ][ yR ] = NCP_INDEX PaletteIndexMap[ xR ][ yR − 1 ] |  |
| } |  |
| runPos++ |  |
| scanPos++ |  |
| } |  |
| } |  |
| } |  |

#### Palette mode semantics

The variable bNeighbour is derived as follows: (General sample prediction 8.4.5.2.1)

- The neighbouring locations ( xNbA, yNbA ) and ( xNbB, yNbB ) are set equal to ( xCb − 1, yCb ) and ( xCb, yCb − 1 ), respectively.

- For X being replaced by either A or B,

- If CuPredMode[ xNbX ][ yNbX ] is not equal to MODE_INTRA and constrained_intra_pred_flag is equal to 1, bNeighbour is marked as false.

- Otherwise, The availability derivation process for a block in z-scan order as specified in subclause Error: Reference source not found is invoked with the location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbX, yNbX ) as inputs, and the output is assigned to bNeighbourX. If  palette_transpose_flag is true, bNeighbourB is assigned to bNeighbour, otherwise bNeighbourA is assigned to bNeighbour.

The variable adjustedIndexMax is derived as follows:

adjustedIndexMax = indexMax
if( scanPos > 0 ) 
	adjustedIndexMax − = 1

palette_index_idc is an indication of an index to the array represented by currentPaletteEntries. The value of palette_index_idc shall be in the range of 0 to adjustedIndexMax, inclusive.

When palette_index_idc is not present, it is inferred to be equal to 0.

The variable PaletteIndexMap[ xC ][ yC ] specifes a palette index, which is an index to the array represented by CurrentPaletteEntries. The array indices xC, yC specify the location ( xC , yC ) of the sample relative to the top-left luma sample of the picture. The value of PaletteIndexMap[ xC ][ yC ] shall be in the range of 0 to indexMax, inclusive.

The variable adjustedRefIndexMax is derived as follows:

adjustedRefIndex = indexMax + 1

if( scanPos > 0 ) { 
	if( palette_run_type_flag[xcPrev][ycPrev]  !=  COPY_ABOVE_MODE ) 
		adjustedRefIndex = PaletteIndexMap[xcPrev][ycPrev]	(7-1)
	else 
		adjustedRefIndex = PaletteIndexMap[xC][yC − 1]

When palette_run_type_flag[ xC ][ yC ] is equal to COPY_INDEX_MODE, the variable paletteIndex is derived as follows:

paletteIndex = palette_index_idc 
if( palette_index_idc >= adjustedRefIndex ) 
	paletteIndex = palette_index_idc + 1

The variable PaletteSampleMode[ xC ][ yC ] specifies whether the palette index is copied from the palette index in the row above or explicitly coded in the bitstream. The array indices xC, yC specify the location ( xC , yC ) of the sample relative to the top-left luma sample of the picture. The value of PaletteSampleMode[ xC ][ yC ] shall be one of COPY_ABOVE_MODE, COPY_INDEX_MODE, and ESCAPE_MODE.

If palette_run_type_flag is equal to COPY_INDEX_MODE and paletteIndexMap[ xC ][ yC ] is equal to indexMax, the sample is coded in escape mode and paletteSampleMode[ xC ][ yC ] is set equal to ESCAPE_MODE.

The variable paletteRun specifies the number of consecutive locations minus 1 with the same palette index as the position in the above row when palette_run_type_flag is equal to COPY_ABOVE_MODE or specifies the number of consecutive locations minus 1 with the same palette index when palette_run_type_flag is equal to COPY_INDEX_MODE.

# Decoding process

8.4.5.2.8

##### Decoding process for palette mode

Inputs to this process are:

–	a location ( xCb, yCb ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

–	a variable cIdx specifying the colour component of the current block,

–	a variable nCbS specifying the size of the current block,

–	An array paletteSampleMode[ xCb + x ][ yCb + y ], with x = 0..nCbS − 1 and  y = 0..nCbS − 1, specifying the method of palette index derivation for each sample in the current block

–	The array paletteIndexMap[ xCb + x ][ yCb + y ], with x = 0..nCbS − 1,  y = 0..nCbS − 1, specifying the palette index for each sample in the current block, and [Ed. (GJS): Check "is not" versus "is not equal to" (generally).] [Ed. (GJS): Check/fix use of "the palette mode".]

–	The array paletteEscapeVal[cIdx][ xCb + x ][ yCb + y ], with x = 0..nCbS − 1 and  y = 0..nCbS − 1, specifying the escape value (possibly quantized) for the cIdx-th component of each sample in the current block for which the paletteSampleMode[ xCb + x ][ yCb + y ] is equal to ESCAPE_MODE.

–	The neighbouring samples p[ c ][ x ][ y ] that are constructed samples of component c prior to the deblocking filter process, with x = −1, y = −1..nCbS – 1, c=0..2 and x = 0..nCbS − 1, y = −1, c=0..2.

Output of this process is an array recSamples[ x ][ y ], with x = 0..nCbS − 1,  y = 0..nCbS − 1, specifying reconstructed sample values for the block.

Depending on the value of the colour component cIdx, the following assignments are made:

–	If cIdx is equal to 0, the function clipCidx1 corresponds to Clip1Y.

–	Otherwise, if cIdx is equal to 1, the function clipCidx1 corresponds to Clip1C.

–	Otherwise (cIdx is equal to 2), the function clipCidx1 corresponds to Clip1C.

The array NCPpaletteIndexMap[x] , with x = 0..nCbS – 1, are derived as follows:

–  bNeighbour is false, NCPpaletteIndexMap[i] = 0

–  Otherwise,

– If palette_transpose_flag is true,

for( i = 0; i < nCbS ; i++ ) 
     {

NCPpaletteIndexMap[i] = 0 ;

Distortion = abs(p[0][ -1 ][ y ] - currentPaletteEntries[ 0 ][ 0 ]) +
                            abs(p[1][ -1 ][ y ] - currentPaletteEntries[ 1 ][ 0 ]) + abs(p[2][ -1 ][ y ] - currentPaletteEntries[ 2 ][ 0 ])

for( j = 1; j < currentPaletteSize; j++ ) 
        {

Distortiontmp = abs(p[0][ -1 ][ y ] - currentPaletteEntries[ 0 ][ j ]) +
                                 abs(p[1][ -1 ][ y ] - currentPaletteEntries[ 1 ][ j ]) + abs(p[2][ -1 ][ y ] - currentPaletteEntries[ 2 ][ j ])

if(Distortiontmp < Distortion )

{

NCPpaletteIndexMap[i] = j ;

Distortion = Distortiontmp;

}

}

}

– If palette_transpose_flag is false,

for( i = 0; i < nCbS ; i++ ) 
      {

NCPpaletteIndexMap[i] = 0 ;

Distortion = abs(p[0][ x ][ -1 ] - currentPaletteEntries[ 0 ][ 0 ]) +
                            abs(p[1][ x ][ -1 ] - currentPaletteEntries[ 1 ][ 0 ]) + abs(p[2][ x ][ -1 ] - currentPaletteEntries[ 2 ][ 0 ])

for( j = 1; j < currentPaletteSize; j++ ) 
        {

Distortiontmp = abs(p[0][ x ][ -1 ] - currentPaletteEntries[ 0 ][ j ]) +
                                abs(p[1][ x ][ -1 ] - currentPaletteEntries[ 1 ][ j ]) + abs(p[2][ x ][ -1 ] - currentPaletteEntries[ 2 ][ j ]

if(Distortiontmp < Distortion )

{

NCPpaletteIndexMap[i] = j ;

Distortion = Distortiontmp;

}

}

}

The variables PredictorPaletteSize, PreviousPaletteSize and PredictorPaletteEntries are derived as follows:

–	The initialization process as specified in subclause 9.3.2 is invoked when starting the parsing of one or more of the following:

–	the slice segment data syntax specified in subclause 7.3.8.1

–	the coding tree unit syntax specified in subclause 7.3.8.2 and the coding tree unit is the first coding tree unit in a tile

–	the coding tree unit syntax specified in subclause 7.3.8.2, entropy_coding_sync_enabled_flag is equal to 1, and the associated luma coding tree block is the first luma coding tree block in a coding tree unit row

–	Otherwise, PredictorPaletteSize, PreviousPaletteSize, and PredictorPaletteEntries updated after decoding the previous coding unit with palette_mode_flag[ x0 ][ y0 ] equal to 1 are used.

The variable PaletteIndexMap[ xR ][ yR ] , with xR = 0…..nCbS – 1, yR = −1..nCbS – 1,  is derived as follows :

for(scanPos = 0; scanPos < nCbS * nCbS ; scanPos++)

{

xR = xCb + travScan[ scanPos ][ 0 ]

yR = yCb + travScan[ scanPos ][ 1 ]

if( scanPos > 0) {

xcPrev = xCb + travScan[ scanPos - 1 ][ 0 ]

ycPrev = yCb + travScan[ scanPos - 1 ][ 1 ]

}

adjustedRefIndex = indexMax + 1

if( xR != 0 && yR != 0 ) {

if( palette_run_type_flag[ xR ][ yR ]  ==  COPY_INDEX_MODE && PaltteIndexMap[ xR ][ yR ] == NCP_INDEX )

PaletteIndxMap[ xR ][ yR ] = PalettIndexMap[ xcPrev ][ ycPrev ]

else if( palette_run_type_flag[ xR ][ yR ]  ==  COPY_INDEX_MODE )

{

if(palette_run_type_flag[ xcPrev ][ ycPrev ]  ==  COPY_INDEX_MODE)

adjustedRefIndex = PaltteIndexMap [xcPrev][ycPrev]

else

{

if(scanPos >=  nCbS )

adjustedRefIndex = PaletteIndexMap[ xR ][ yR  – 1 ]

else if( bNeighbour )

adjustedRefIndex = palette_transpose_flag ? NCPpaletteIndexMap [ yR] :NCPpaletteIndexMap [ xR]

else

adjustedRefIndex = 0

}

if(PaltteIndexMapParse[xR][yR] >= adjustedRefIndex ) PaltteIndexMap[xR][yR]++

}

else if( palette_run_type_flag[ xR ][ yR ]  ==  COPY_ABOVE_MODE) {

if ( yR – 1 < 0 )

PaletteIndexMap[ xR ][ yR ] = bNeighbour ? NCPpaletteIndexMap [ X ] : 0

else

PaletteIndexMap[ xR ][ yR ] = PaletteIndexMap[ xR ][ yR − 1 ]

}

}

}

The (nCbS x nCbS) block of the reconstructed sample array recSamples at location ( xCb, yCb ) is derived as follows:

For x = 0..nCbS − 1,  y = 0..nCbS − 1, recSample[ cIdx ][ yCb + y ][ xCb + x ] is set as follows:

–	If paletteSampleMode[ xCb + x ][ yCb + y ] is not equal to ESCAPE_MODE, the following applies:

recSample[ cIdx ][ xCb + x ][ yCb + y ] = currentPaletteEntries[ cIdx ][ paletteIndexMap[ xCb + x ][ yCb + y ] ],

–	Otherwise, if cu_transquant_bypass_flag is true, the following applies:

recSample[ cIdx ][ xCb + x ][ yCb + y ] = paletteEscapeVal[ cIdx ][ xCb + x ][ yCb + y ],

–	Otherwise (paletteSampleMode[ xCb + x ][ yCb + y ] is equal to ESCAPE_MODE and cu_transquant_bypass_flag is false), the following ordered steps apply: [Ed. (GJS): Indentation problem.] [Ed. (RJ): fixed]

- The derivation process for quantization parameters as specified in subclause Error: Reference source not found is invoked with the location ( xCb, yCb ) specifying the top-left sample of the current block relative to the top-left sample of the current picture.

- The quantization parameter qP is derived as follows:

–	If cIdx is equal to 0,

qP = max( 0, Qp′Y )		(8-2)

–	Otherwise, if cIdx is equal to 1,

qP = max( 0, Qp′Cb )		(8-3)

–	Otherwise (cIdx is equal to 2),

qP = max(0, Qp′Cr )		(8-4)

- The variables bitDepth and bdShift1 are derived as follows:

bitDepth = ( cIdx  = =  0 ) ? BitDepthY : BitDepthC		(8-5)

bdShift1 = Max( 20 − bitDepth, extended_precision_processing_flag ? 11 : 0 )	(8-6)

- The variables log2TransformRange, bdShift2, coeffMin and coeffMax are derived as follows:

–	If cIdx is equal to 0,

log2TransformRange = extended_precision_processing_flag ? Max( 15, BitDepthY + 6 ) : 15	(8-7)

bdShift2 = BitDepthY + Log2( nTbS ) + 10 − log2TransformRange	(8-8)

coeffMin = CoeffMinY		(8-9)

coeffMax = CoeffMaxY		(8-10)

–	Otherwise,

log2TransformRange = extended_precision_processing_flag ? Max( 15, BitDepthC + 6 ) : 15	(8-11)

bdShift2 = BitDepthC + Log2( nTbS ) + 10 − log2TransformRange	(8-12)

coeffMin = CoeffMinC		(8-13)

coeffMax = CoeffMaxC		(8-14)

[Ed. (GJS): Correct equation numbering.]

- The list levelScale[ ] is specified as levelScale[ k ] = { 40, 45, 51, 57, 64, 72 } with k = 0..5.

- The following applies

recSamples[ xCb + x ][ yCb + y ] =
Clip3( coeffMin, coeffMax, ( ( palette_escape_val[ cIdx ][ xCb + x ][ yCb + y ]* 16 * 
			levelScale[ qP%6 ]  <<  (qP / 6 ) ) + ( 1  <<  ( bdShift2 − 1 ) ) )  >>  bdShift2 )	(8-15)

[Ed. (GJS): Indentation problem.]

[Ed. (GJS): Reformulate as sentences or add equation numbers.]

[Ed. (RJ): Added equation numbers.]

- The reconstructed sample value recSamples[ xCb + x ][ yCb + y ] is modified as follows:

recSamples[ xCb + x ][ yCb + y ] = ( recSamples[ xCb + x ][ yCb + y ] 
	+ ( 1  <<  ( bdShift2 − 1 ) ) )  >>  bdShift2	(8-16)

The variable PreviousPaletteSize is set equal to the CurrentPaletteSize. Variables PredictorPaletteSize and PredictorPaletteEntries are modified as follows:

for( i = 0; i < currentPaletteSize; i++ ) 
	for( cIdx = 0; cIdx < 3; cIdx++ ) 
		newPredictorPaletteEntries[ cIdx ][ i ] = CurrentPaletteEntries[ cIdx ][ i ]
newPredictorPaletteSize = currentPaletteSize 
for( i = 0; i < PredictorPaletteSize  &&  newPredictorPaletteSize < palette_max_predictor_size; i++ )
	if( PalettePredictorEntryReuseFlag[ i ]  ! =  1 ) {
		for( cIdx = 0; cIdx < 3; cIdx++ ) 		(8-17)
			newPredictorPaletteEntries[ cIdx ][ newPredictorPaletteSize ] = PredictorPaletteEntries[ cIdx ][ i ]
		newPredictorPaletteSize ++ 
	}
for( cIdx = 0; cIdx < 3; cIdx++ ) 
	for( i = 0; i < newPredictorPaletteSize; i++ ) 
		PredictorPaletteEntries[ cIdx ][ i ] = newPredictorPaletteEntries[ cIdx ][ i ]
PredictorPaletteSize = newPredictorPaletteSize

| Table 9-1 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Syntax element | binIdx |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| palette_run_type_flag | 0, 1, 2 (subclause 1.1.1.1.2) | na | na | na | na | na |
| palette_index_idc | bypass | bypass | bypass | bypass | bypass | bypass |

##### Derivation process of ctxInc using left and above syntax elements

Input to this process is the luma location ( x0, y0 ) specifying the top-left luma sample of the current luma block relative to the top-left sample of the current picture.

Output of this process is ctxInc.

The location ( xNbL, yNbL ) is set equal to ( x0 − 1, y0 ) and the variable availableL, specifying the availability of the block located directly to the left of the current block, is derived by invoking the availability derivation process for a block in z-scan order as specified in subclause Error: Reference source not found with the location ( xCurr, yCurr ) set equal to ( x0, y0 ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbL, yNbL ) as inputs, and the output is assigned to availableL.

The location ( xNbA, yNbA ) is set equal to ( x0, y0 − 1 ) and the variable availableA specifying the availability of the coding block located directly above the current block, is derived by invoking the availability derivation process for a block in z-scan order as specified in subclause Error: Reference source not found with the location ( xCurr, yCurr ) set equal to ( x0, y0 ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbA, yNbA ) as inputs, and the output is assigned to availableA.

The assignment of ctxInc for the syntax elements split_cu_flag[ x0 ][ y0 ] and cu_skip_flag[ x0 ][ y0 ] is specified in Table 9-2.

Table 9-2 – Specification of ctxInc using left and above syntax elements

| Syntax element | condL | condA | ctxInc |
| --- | --- | --- | --- |
| split_cu_flag[ x0 ][ y0 ] | CtDepth[ xNbL ][ yNbL ] > cqtDepth | CtDepth[ xNbA ][ yNbA ] > cqtDepth | ( condL && availableL ) + ( condA && availableA ) |
| cu_skip_flag[ x0 ][ y0 ] | cu_skip_flag[ xNbL ][ yNbL ] | cu_skip_flag[ xNbA ][ yNbA ] | ( condL && availableL ) + ( condA && availableA ) |
| palette_transpose_flag [ x0 ][ y0 ] | palette_transpose_flag [ xNbL ][ yNbL ] | na | ( condL && availableL ) |

##### 9.3.4.2.x Derivation process of ctxInc the syntax element palette_run_type_flag

Inputs to this process are the luma location ( x0, y0 ) specifying the top-left sample of the current prediction block relative to the top-left sample of the current picture, the current sample location ( xC, yC ) specifying the sample relative to the top-left luma sample of the picture, and previously decoded syntax element palette_run_type_flag.

Output of this process is the variable ctxInc.

The variable ctxInc is derived as follows:

- If yC is equal to y0, ctxInc is derived as follows:

ctxInc = 2	(9-18)

- Otherwise, ctxInc is derived as follows:

ctxInc = (palette_run_type_flag[xC][yC-1] == COPY_ABOVE_MODE) ? 1: 0	(9-x)