| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0037 |
| --- | --- |

| Title: | CE1 Tests A.1 : Extended copy above mode to the first line (1.6-1.8) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jungsun Kim, Yu-Chen Sun, Shan Liu, Tzu-Der Chuang, Yi-Wen Chen, Yu-Wen Huang, Shawmin Lei 2860 Junction Ave, San Jose, CA, USA Vadim Seregin, Feng Zou, Wei Pu, Rajan Joshi, Marta Karczewicz 5775 Morehouse Drive San Diego, CA, USA | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 { jungsun.kim,yc.sun, shan.liu, peter.chuang, yiwen.chen, yuwen.huang, shawmin.lei} @mediatek.com vseregin@qti.qualcomm.com |
| Source: | MediaTek Inc. Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In CE1 Test A.1, it was proposed for palette mode to enable copy_above run mode for the first row of the current block by using neighbouring CU’s nearest row. In this contribution several simplifications were reported. Experiment results show that maximum 1.5%, 0.9%, 0.6% and average 0.4%, 0.3%, 0.2% gain for AI, RA and LB configuration respectively in A.1.6, maximum 1.4%, 0.8% and 0.6% and average 0.4%, 0.2%, 0.2% gain for AI, RA and LB configuration respectively in A.1.7, and maximum 0.1%, 0.1% and 0.1% gain for AI, RA and LB configuration respectively in A.1.8.

# Problem statement

The palette mode in SCM-3.0 has two run modes, copy index and copy above. In copy above mode, palette indices are predicted from decoded indices of the above pixel (decoded indices when palette_transpose_flag is on). In SCM-3.0, the above-prediction run is disabled for first row [1] since the reference pixels are not present within the current CU.

CE1 Test A.1 proposed to enable copy above mode for every pixels in the CU and to use pixels in the nearest above row or the nearest left column in neighboring CUs to generate palette index prediction sources for the pixels in the first row or column of the current CU. This cross-CU prediction method and several simplified variations are evaluated on top of SCM-3.0. Test A.1.1 - A.1.5 are presented in JCTVC-T0036, and Test A.1.6 - A.1.8 are presented in JCTVC-T0037.

# Proposed methods

- Index conversion by luma components

## Test A.1.6: with palette index redundancy removal for all coded indices

In Test A.1.6, neighbouring pixel values are mapped to palette indices as follows :

Idxi=argmin0≤n<NNCPi[0]−Colourn[0]

where Idxi indicates index of i-th neighbouring pixel, NCPi[0] indicates luma value of i-th neighbouring pixel and Colourn[0] indicates luma value of n-th entry of the palette of the current CU.

A signalled index cannot be equal to above pixel palette index if the previous pixel in the scanning order has copy above mode and cannot be equal to previous pixel palette index if the previous pixel is coded with copy index mode. With this rule, coded palette index can be reduced by one if the palette index to be coded is greater than the palette index of the reference pixel.

Since ESCAPE values are not assigned to neighbouring pixels, the number of index candidate for the first line can be reduced by one always. But the reconstruction of the true palette index cannot be finished at parsing stage since the reconstructed samples of the neigbhouring row or column and hence palette indices of the reference pixels (orange pixels on Figure 1) are not available. As a result, other palette indices that use outside of the current CU reference pixels cannot be reconstructed at parsing stage as well (grey pixels on Figure1).

Figure 1 indices inherit from neighbouring pixels cannot be reconstructed in parsing stage

In Test A.1.6, redundancy removal is always applied including pixels whose palette index is copied from the neighbouring pixel, and full palette index reconstruction is differed to the decoding stage when neighbouring reconstructed sample become available.

## Test A.1.7: with palette index redundancy removal for partial coded indices

In Test A.1.7, neighbouring pixel values are mapped to index as Test A.1.6 as follows:

Idxi=argmin0≤n<NNCPi[0]−Colourn[0]

where Idxi indicates index of i-th neighbouring pixe,l NCPi[0] indicates luma value of i-th neighbouring pixel and Colourn[0] indicates luma value of n-th entry of the palette of the current CU.

In Test A.1.7, index redundancy removal is not applied to the pixels whose palette index is copied from the palette indices of the neighbouring pixels (grey pixels in Figure1). Therefore, actual palette indices can be reconstructed at parsing stage.

- Index conversion to fixed value

## Test A.1.8: with mapping every indices to 0

In Test A.1.8, neighbouring pixel values are always mapped to palette index 0, i.e

Idxi=0

where Idxi indicates index of i-th neighbouring pixel.

In Test A.1.8, index redundancy removal is always applied to every pixels and actual indices can be reconstructed since the index of reference pixels (orange pixels in Figure 1) are already known as 0 even at parsing stage.

# Experimental results

The proposed methods were implemented on top of SCM-3.0 [2]. All tests were conducted under the common test condition (CTC) [3] for SCC with full frame IBC search range. Table 1 shows the summary of the performance A1.6~A1.8 under AI configuration and Table 2 shows the summary of the performance of A1.1~A1.5 under AI configuration. Tables 3~5 show the results of CE1 Test A.1.6~A.1.8 lossy case in detail and Table 6~8 show the results of CE1 Test 1.6-A1.8 lossless case in detail. The best performance results of each case is 1.5%, 0.9%, 0.6% under AI, RA and LB configuration respectively in A1.6, 1.4%, 0.8%, 0.6% under AI, RA and LB configuration respectively in A1.7, 0.1%, 0.1%, 0.1% under AI, RA and LB configuration respectively in A1.8

Table 1 Test 1.6~1.8 AI lossy cases

|  | Test 1.6 |  |  | Test 1.7 |  |  | Test 1.8 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -1.2% | -1.2% | -1.2% | -1.0% | -1.1% | -1.1% | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1440p & 1080p | -0.3% | -0.3% | -0.4% | -0.3% | -0.3% | -0.3% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -1.5% | -1.7% | -1.7% | -1.4% | -1.5% | -1.5% | -0.1% | -0.1% | -0.1% |
| YUV, mixed content, 1440p & 1080p | -0.6% | -0.9% | -0.9% | -0.5% | -0.8% | -0.8% | -0.1% | -0.1% | -0.1% |
| YUV, Animation, 720p | 0.0% | -0.2% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |

Table 2 Test 1.1~1.5 AI lossy cases

|  | Test 1.1 |  |  | Test 1.2 |  |  | Test 1.3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -1.4% | -1.5% | -1.5% | -1.3% | -1.4% | -1.4% | -1.3% | -1.3% | -1.3% |
| RGB, mixed content, 1440p & 1080p | -0.3% | -0.4% | -0.4% | -0.3% | -0.4% | -0.4% | -0.3% | -0.3% | -0.4% |
| RGB, Animation, 720p | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -1.7% | -1.9% | -1.8% | -1.5% | -1.6% | -1.7% | -1.6% | -1.7% | -1.7% |
| YUV, mixed content, 1440p & 1080p | -0.6% | -1.0% | -1.0% | -0.5% | -0.9% | -0.9% | -0.5% | -0.9% | -0.9% |
| YUV, Animation, 720p | 0.0% | -0.3% | -0.2% | 0.0% | -0.2% | -0.2% | 0.0% | -0.3% | -0.3% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Test 1.4 |  |  | Test 1.5 |  |  |  |  |  |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V |  |  |  |
| RGB, text & graphics with motion, 1080p & 720p | -1.2% | -1.2% | -1.2% | -1.2% | -1.3% | -1.3% |  |  |  |
| RGB, mixed content, 1440p & 1080p | -0.2% | -0.3% | -0.3% | -0.3% | -0.4% | -0.3% |  |  |  |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YUV, text & graphics with motion, 1080p & 720p | -1.4% | -1.5% | -1.4% | -1.5% | -1.6% | -1.6% |  |  |  |
| YUV, mixed content, 1440p & 1080p | -0.4% | -0.8% | -0.8% | -0.5% | -0.9% | -0.7% |  |  |  |
| YUV, Animation, 720p | 0.0% | -0.2% | -0.2% | 0.0% | 0.0% | -0.1% |  |  |  |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |

Table 3 Result of Test 1.6 lossy case

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -1.2% | -1.2% | -1.2% |
| RGB, mixed content, 1440p & 1080p | -0.3% | -0.3% | -0.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -1.5% | -1.7% | -1.7% |
| YUV, mixed content, 1440p & 1080p | -0.6% | -0.9% | -0.9% |
| YUV, Animation, 720p | 0.0% | -0.2% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 103% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.7% | -0.7% | -0.7% |
| RGB, mixed content, 1440p & 1080p | -0.2% | -0.2% | -0.2% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.9% | -1.1% | -1.2% |
| YUV, mixed content, 1440p & 1080p | -0.3% | -0.5% | -0.5% |
| YUV, Animation, 720p | -0.1% | -0.3% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.6% | -0.6% | -0.7% |
| RGB, mixed content, 1440p & 1080p | -0.1% | -0.2% | -0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | -0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.6% | -0.9% | -1.1% |
| YUV, mixed content, 1440p & 1080p | -0.2% | -0.7% | -0.5% |
| YUV, Animation, 720p | 0.0% | 0.0% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 98% |  |  |

Table 4 Result of Test 1.7 lossy case

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -1.0% | -1.1% | -1.1% |
| RGB, mixed content, 1440p & 1080p | -0.3% | -0.3% | -0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -1.4% | -1.5% | -1.5% |
| YUV, mixed content, 1440p & 1080p | -0.5% | -0.8% | -0.8% |
| YUV, Animation, 720p | 0.0% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.6% | -0.7% | -0.6% |
| RGB, mixed content, 1440p & 1080p | 0.0% | -0.2% | -0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.8% | -0.9% | -1.1% |
| YUV, mixed content, 1440p & 1080p | -0.2% | -0.5% | -0.5% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 93% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.5% | -0.5% | -0.5% |
| RGB, mixed content, 1440p & 1080p | -0.2% | -0.1% | -0.3% |
| RGB, Animation, 720p | 0.0% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.6% | -0.8% | -1.1% |
| YUV, mixed content, 1440p & 1080p | -0.1% | -0.5% | -0.4% |
| YUV, Animation, 720p | 0.0% | -0.1% | 0.0% |
| YUV, camera captured, 1080p | -0.1% | 0.1% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |

Table 5 Result of Test 1.8 lossy case

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | -0.1% |
| YUV, mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.1% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.1% | -0.2% | -0.2% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.1% | -0.1% |
| RGB, Animation, 720p | 0.1% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.1% | -0.3% | -0.4% |
| YUV, mixed content, 1440p & 1080p | 0.0% | -0.1% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.1% | 0.3% |
| YUV, camera captured, 1080p | -0.1% | 0.1% | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

Table 6 Result of Test 1.6 lossless case

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -0.4% | -0.5% | -1.4% | -0.1% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | -0.1% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.5% | -0.7% | -1.8% | -0.1% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | -0.1% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  |  |
| Dec Time[%] | 101% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -0.2% | -0.3% | -1.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.2% | -0.4% | -1.1% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 99% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -0.2% | -0.2% | -0.6% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.2% | -0.3% | -0.7% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 99% |  |  |  |

Table 7 Result of Test 1.7 lossless case

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -0.3% | -0.4% | -1.3% | -0.1% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | -0.1% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.5% | -0.6% | -1.7% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | -0.1% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 102% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -0.2% | -0.3% | -0.9% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.2% | -0.4% | -1.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 98% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -0.2% | -0.2% | -0.5% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.2% | -0.3% | -0.6% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 102% |  |  |  |

Table 8 Result of Test 1.8 lossless case

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | -0.1% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | -0.3% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  |  |
| Dec Time[%] | 101% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | -0.1% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | -0.1% | -0.2% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 96% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | -0.1% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | -0.1% | -0.2% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 98% |  |  |  |

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- R. Joshi, J. Xu, “HEVC Screen Content Coding Draft Text 2,” Document of Joint Collaborative Team on Video Coding, JCTVC-S1005, Oct. 2014.

- SCM-3.0, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-14.0+RExt-7.0+SCM-3.0/

- P. Lai, P. Onno, R. Cohen, V. Seregin, X. Xiu, Z. Ma, “Description of Core Experiment 1 (CE1): Improvements of palette mode,” JCTVC-S1101, Oct. 2014.