| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0040 |
| --- | --- |

| Title: | CE2: results of test 3.1 and 6.1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bin Li Jizheng Xu | Tel: Email: | [phone contact number] [email address(es)] |
| Source: | Microsoft Corp. |  |  |

_____________________________

# Abstract

This document reports the experimental results of test 3.1 and 6.1 in CE2. In test 3.1, the merge mode is enabled for Intra BC mode. The BV could be signalled by merging to a spatial neighbouring block. In test 6.1, the process to obtain BVP is modified. The experimental results show that for RGB TMG sequences, test 3.1 improves the coding efficiency by 0.8%. Test 6.1 improves the coding efficiency by 1.3%.

# Introduction

This document reports the experimental results of test 3.1 and 6.1 in CE2. In test 3.1, the merge mode is enabled for Intra BC mode. The BV could be signalled by merging to a spatial neighbouring block. In test 6.1, the process to obtain BVP is modified. The experimental results show that for RGB TMG sequences, test 3.1 improves the coding efficiency by 0.8%. Test 6.1 improves the coding efficiency by 1.3%.

# Solution

## Intra BC merge mode

The IBC merge mode is added to signal the block vectors. The same positions to obtain inter merge candidates are used to construct IBC merge candidates.

## BVP modification

In the current design, the BVP comes from left block and the above block, together the two last BVs. In the proposed method, the BVP construction process checks the similar position as the merge candidate construction process. The two last BVs are also used in the BVP construction process. The number of BVPs is still two and there is no syntax changes introduced by the proposed method.

The BVPs are constructed in the following order:

- Left

- Above

- Above right

- Left bottom

- Above left

- MvLast[0]

- MvLast[1]

- (−2*CUWidth, 0)

- (−CUWidth, 0)

For (1) to (8), if there already exists a same BVP, it is not added into the BVP list. For (1) to (7), if the BVP is not valid for the current PU (e.g., the BVP may result in predicting from unconstructed region), it is also not added into the BVP list.

# Experimental results

HM-16.2+SCM-3.0 default coding structure is used as the anchor. The common test conditions specified in [1] are followed. The summarized experimental results are shown in the following tables. More detailed experimental results could be found in the attached spreadsheets.

## Intra BC merge mode

Table 1 Experimental results for lossy coding (444 sequences)

Table 2 Experimental results for lossy coding (420 sequences)

Table 3 Experimental results for lossless coding (444 sequences)

Table 4 Experimental results for lossless coding (420 sequences)

## BVP modification

Results of the proposed BVP construction process.

Table 5 Experimental results for lossy coding (444 sequences)

Table 6 Experimental results for lossy coding (420 sequences)

Table 7 Experimental results for lossless coding (444 sequences)

Table 8 Experimental results for lossless coding (420 sequences)

Results of non-normative method, i.e., adding using BVP and final BV in the RDO process.

Table 9 Experimental results for lossy coding (444 sequences)

Table 10 Experimental results for lossy coding (420 sequences)

Table 11 Experimental results for lossless coding (444 sequences)

Table 12 Experimental results for lossless coding (420 sequences)

# References

- JCTVC-S1015, “Common conditions for screen content coding tests,” Strasbourg, FR, 17–24 Oct. 2014.

# Patent rights declaration(s)

Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Text modifications

The text provided in this section is based on JCTVC-S1005.

## Intra BC merge mode

#### 7.3.6.1	General slice segment header syntax

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| …… |  |
| if( ( weighted_pred_flag && slice_type = = P ) ¦ ¦ ( weighted_bipred_flag && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ¦ ¦ intra_block_copy_enabled_flag ) { |  |
| five_minus_max_num_merge_cand | ue(v) |
| } |  |
| slice_qp_delta | se(v) |
| …… |  |

#### Prediction unit syntax

| prediction_unit( x0, y0, nPbW, nPbH ) { | Descriptor |
| --- | --- |
| if( cu_skip_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else if( intra_bc_flag[ x0 ][ y0 ] ) /* Intra BC*/ { |  |
| merge_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| bvd_coding( x0, y0, 2 ) |  |
| bvp_flag[ x0 ][ y0 ] | ae(v) |
| } | ae(v) |
| } | ae(v) |
| else { /* MODE_INTER */ |  |
| merge_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |

### Derivation process for block vector components in intra block copying prediction mode

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable log2CbSize specifying the size of the current luma coding block.

Output of this process is the (nCbS)x(nCbX) array [Ed. some callees still call this a single vector] of block vectors bvIntra.

The variables nCbS, nPbSw, and nPbSh are derived as follows:

nCbS = 1  <<  log2CbSize		(8-1)

nPbSw = nCbS / ( PartMode = = PART_2Nx2N  | |  PartMode = = PART_2NxN ? 1 : 2 )	(8-2)

nPbSh = nCbS / ( PartMode = = PART_2Nx2N  | |  PartMode = = PART_Nx2N ? 1 : 2 )	(8-3)

The variable BvpIntra[ compIdx ] specifies a block vector predictor. The horizontal block vector component is assigned compIdx = 0 and the vertical block vector component is assigned compIdx = 1.

Depending upon PartMode, the variable numPartitions is derived as follows:

–	If PartMode is equal to PART_2Nx2N, numPartitions is set equal to 1.

–	Otherwise, if PartMode is equal to either PART_2NxN or PART_Nx2N, numPartitions is set equal to 2.

–	Otherwise (PartMode is equal to PART_NxN), numPartitions is set equal to 4.

The array of block vectors bvIntra is derived by the following ordered steps, for the variable blkIdx proceeding over the values 0..( numPartitions − 1 ):

- The variable blkInc is set equal to ( PartMode = = PART_2NxN ? 2 : 1 ).

- The variable xPb is set equal to xCb + nPbSw * ( blkIdx * blkInc % 2 ).

- The variable yPb is set equal to yCb + nPbSh * ( blkIdx * blkInc / 2 )

- The following ordered steps apply, for the variable compIdx proceeding over the values 0..1:

- The variable LastBvIntra[ 0 ][ compIdx ] and LastBvIntra[ 1 ][ compIdx ]specifies the last two block vector predictor. If this process is invoked for the first time for the current coding tree unit, LastBvIntra[ compIdx ] is derived as follows:

LastBvIntra[ 0 ][ 0 ] = −2* nCbS; LastBvIntra[ 0 ][ 1 ] = 0

LastBvIntra[ 1 ][ 0 ] = −nCbS; LastBvIntra[ 1 ][ 1 ] = 0

If merge_flag[ xPb ][ yPb ] is equal to 1, subclause 8.4.4.3 is invoded with luma coding block location ( xCb, yCb ), the coding block size nCbS, the luma prediction block location ( xPb, yPb ), the luma prediction block width nPbSw, the luma prediction block height nPbSh, and the partition index blkIdx as inputs, and the block vector BvMergeListIntra[ xPb ][ yPb ] as the output, and bvIntra[ xPb ][ yPb ][ compIdx ] is set equal to BvMergeListIntra[ xPb ][ yPb ][ merge_idx[ xPb ][ yPb ] ][ compIdx ].

Otherwise (merge_flag[ xPb ][ yPb ] is equal to 0), Depending upon the number of times this process has been invoked for the current coding tree unit, subclause Error: Reference source not found is invoked with the luma coding block location ( xCb, yCb ), the coding block size nCbS, the luma prediction block location ( xPb, yPb ), the luma prediction block width nPbSw, the luma prediction block height nPbSh, the last block vectors LastBvIntra, and the partition index blkIdx as inputs, and the block vector predictor BvpIntra[ xPb ][ yPb ] as the output, and bvIntra[ xPb ][ yPb ][ compIdx ] is set equal to BvdIntra[ xPb ][ yPb ][ compIdx ] + BvpIntra[ xPb ][ yPb ][ compIdx ] [Ed. (GJS): Needs further formatting cleanup.]

When bvIntra[ xPb ][ yPb ][ 0 ] is not equal to LastBvIntra[ 0 ][ 0 ] or bvIntra[ xPb ][ yPb ][ 1 ] is not equal to LastBvIntra[ 0 ][ 1 ], the value of LastBvIntra[ 1 ][ compIdx ] is updated to be LastBvIntra[ 0 ][ compIdx ], and the value of LastBvIntra[ 0 ][ compIdx ] is updated to be bvIntra[ xPb ][ yPb ][ compIdx ].

- For use in derivation processes of variables invoked later in the decoding process, the following assignment is made for x = 0..nPbSw − 1 and y = 0..nPbSh − 1:

bvIntra[ xPb + x ][ yPb + y ][ compIdx ] = bvIntra[ xPb ][ yPb ][ compIdx ]	(8-4)

–	When the derivation process for z-scan order block availability as specified in subclause Error: Reference source not found is invoked with ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbY, yNbY ) set equal to ( xPb + bvIntra[ xPb ][ yPb ][ 0 ], yPb + bvIntra[ xPb ][ yPb ][ 1 ] ) as inputs, the output is set equal to TRUE.

–	When the derivation process for z-scan order block availability as specified in subclause Error: Reference source not found is invoked with ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbY, yNbY ) set equal to ( xPb + bvIntra[ xPb ][ yPb ][ 0 ] + nPbSw − 1, yPb + bvIntra[ xPb ][ yPb ][ 1 ] + nPbSh – 1 ) as inputs, the output is set equal to TRUE.

–	One or both of the following conditions shall be true: [Ed. (GJS): Clarify that this is a bitstream constraint (if that is the correct interpretation).]

–	bvIntra[ xPb ][ yPb ][ 0 ] + xPb − xCb is less than or equal to 0

–	bvIntra[ xPb ][ yPb ][ 1 ] + yPb − yCb is less than or equal to 0

–	It is a requirement of bitstream conformance that the following condition shall be true:

( xPb + bvIntra[ xPb ][ yPb ][ 0 ] + nPbSw − 1 ) / CtbSizeY − xCurr / CtbSizeY  <=  yCurr/CtbSizeY − ( yPb + bvIntra[ xPb ][ yPb ][ 1 ] + nPbSh − 1 ) / CtbSizeY	(8-5)

#### 8.4.4.3	Derivation process for intra block copy merge candidate

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable nCbS specifying the size of the current luma coding block,

- a luma location ( xPb, yPb ) specifying the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- two variables nPbW and nPbH specifying the width and the height of the luma prediction block,

- a variable partIdx specifying the index of the current prediction unit within the current coding unit.

Output of this process is the block vector merge candidates list BvMergeListIntra[ xPb ][ yPb ].

The block vector merge candidates list BvMergeListIntra[ xPb ][ yPb ] is derived in the following ordered steps:

numMergeCandidates is set to 0.

- The derivation process for block vector predictor candidates from neighbouring prediction unit partitions in subclause Error: Reference source not found is invoked with the luma coding block location ( xCb, yCb ), the coding block size nCbS, the luma prediction block location ( xPb, yPb ), the luma prediction block width nPbW, the luma prediction block height nPbH, the neighbouring block location ( xPb − 1 , yPb + nPbH − 1 ) and the partition index partIdx as inputs, and the availability flags availableFlagN and the block vectors bvIntraN as output. If availableFlagN is equal to TRUE, the following steps apply:

  - When one or more of the following conditions are true, availableFlagN is set equal to FALSE:

    - xPb  >>  Log2ParMrgLevel is equal to xNbA1  >>  Log2ParMrgLevel and  yPb  >>  Log2ParMrgLevel is equal to yNbA1  >>  Log2ParMrgLevel.

    - PartMode of the current prediction unit is equal to PART_Nx2N, PART_nLx2N, or PART_nRx2N, and partIdx is equal to 1.

  - If availableFlagN is equal to TRUE, BvMergeListIntra[ xPb ][ yPb ][numMergeCandidates] is set to bvIntraN and numMergeCandidates is increased by one.

- The derivation process for block vector predictor candidates from neighbouring prediction unit partitions in subclause Error: Reference source not found is invoked with the luma coding block location ( xCb, yCb ), the coding block size nCbS, the luma prediction block location ( xPb, yPb ), the luma prediction block width nPbW, the luma prediction block height nPbH, the neighbouring block location ( xPb + nPbW − 1,  yPb − 1 ) and the partition index partIdx as inputs, and the availability flags availableFlagN and the block vectors bvIntraN as output. If availableFlagN is equal to TRUE, the following steps apply:

  - When one or more of the following conditions are true, availableFlagN is set equal to FALSE:

    - xPb  >>  Log2ParMrgLevel is equal to xNbA1  >>  Log2ParMrgLevel and  yPb  >>  Log2ParMrgLevel is equal to yNbA1  >>  Log2ParMrgLevel.

    - PartMode of the current prediction unit is equal to PART_Nx2N, PART_nLx2N, or PART_nRx2N, and partIdx is equal to 1.

  - If availableFlagN is equal to TRUE, BvMergeListIntra[ xPb ][ yPb ][numMergeCandidates] is set to bvIntraN and numMergeCandidates is increased by one.

- The derivation process for block vector predictor candidates from neighbouring prediction unit partitions in subclause Error: Reference source not found is invoked with the luma coding block location ( xCb, yCb ), the coding block size nCbS, the luma prediction block location ( xPb, yPb ), the luma prediction block width nPbW, the luma prediction block height nPbH, the neighbouring block location ( xPb + nPbW,  yPb − 1 ) and the partition index partIdx as inputs, and the availability flags availableFlagN and the block vectors bvIntraN as output. If availableFlagN is equal to TRUE, the following steps apply:

  - When one or more of the following conditions are true, availableFlagN is set equal to FALSE:

    - xPb  >>  Log2ParMrgLevel is equal to xNbA1  >>  Log2ParMrgLevel and  yPb  >>  Log2ParMrgLevel is equal to yNbA1  >>  Log2ParMrgLevel.

    - PartMode of the current prediction unit is equal to PART_Nx2N, PART_nLx2N, or PART_nRx2N, and partIdx is equal to 1.

  - If availableFlagN is equal to TRUE, BvMergeListIntra[ xPb ][ yPb ][numMergeCandidates] is set to bvIntraN and numMergeCandidates is increased by one.

- The derivation process for block vector predictor candidates from neighbouring prediction unit partitions in subclause Error: Reference source not found is invoked with the luma coding block location ( xCb, yCb ), the coding block size nCbS, the luma prediction block location ( xPb, yPb ), the luma prediction block width nPbW, the luma prediction block height nPbH, the neighbouring block location ( xPb − 1,  yPb + nPbH ) and the partition index partIdx as inputs, and the availability flags availableFlagN and the block vectors bvIntraN as output. If availableFlagN is equal to TRUE, the following steps apply:

  - When one or more of the following conditions are true, availableFlagN is set equal to FALSE:

    - xPb  >>  Log2ParMrgLevel is equal to xNbA1  >>  Log2ParMrgLevel and  yPb  >>  Log2ParMrgLevel is equal to yNbA1  >>  Log2ParMrgLevel.

    - PartMode of the current prediction unit is equal to PART_Nx2N, PART_nLx2N, or PART_nRx2N, and partIdx is equal to 1.

  - If availableFlagN is equal to TRUE, BvMergeListIntra[ xPb ][ yPb ][numMergeCandidates] is set to bvIntraN and numMergeCandidates is increased by one.

- The derivation process for block vector predictor candidates from neighbouring prediction unit partitions in subclause Error: Reference source not found is invoked with the luma coding block location ( xCb, yCb ), the coding block size nCbS, the luma prediction block location ( xPb, yPb ), the luma prediction block width nPbW, the luma prediction block height nPbH, the neighbouring block location ( xPb − 1, yPb − 1 ) and the partition index partIdx as inputs, and the availability flags availableFlagN and the block vectors bvIntraN as output. If availableFlagN is equal to TRUE, the following steps apply:

  - When one or more of the following conditions are true, availableFlagN is set equal to FALSE:

    - xPb  >>  Log2ParMrgLevel is equal to xNbA1  >>  Log2ParMrgLevel and  yPb  >>  Log2ParMrgLevel is equal to yNbA1  >>  Log2ParMrgLevel.

    - PartMode of the current prediction unit is equal to PART_Nx2N, PART_nLx2N, or PART_nRx2N, and partIdx is equal to 1.

  - If availableFlagN is equal to TRUE, BvMergeListIntra[ xPb ][ yPb ][numMergeCandidates] is set to bvIntraN and numMergeCandidates is increased by one.

- BvMergeListIntra[ xPb ][ yPb ][numMergeCandidates] is set to (-nPbW, 0), and numMergeCandidates is increased by one.

- BvMergeListIntra[ xPb ][ yPb ][numMergeCandidates] is set to  (-2*nPbW, 0), and numMergeCandidates is increased by one.

- BvMergeListIntra[ xPb ][ yPb ][numMergeCandidates] is set to (0, -nPbH), and numMergeCandidates is increased by one.

- BvMergeListIntra[ xPb ][ yPb ][numMergeCandidates] is set to (0, -2*nPbH), and numMergeCandidates is increased by one.

- BvMergeListIntra[ xPb ][ yPb ][numMergeCandidates] is set to (-nPbW, -nPbH), and numMergeCandidates is increased by one.

## BVP modification

### 8.4.4	Derivation process for block vector components in intra block copying prediction mode

……

The array of block vectors bvIntra is derived by the following ordered steps, for the variable blkIdx proceeding over the values 0..( numPartitions − 1 ):

- The variable blkInc is set equal to ( PartMode = = PART_2NxN ? 2 : 1 ).

- The variable xPb is set equal to xCb + nPbSw * ( blkIdx * blkInc % 2 ).

- The variable yPb is set equal to yCb + nPbSh * ( blkIdx / 2 )

- The following ordered steps apply, for the variable compIdx proceeding over the values 0..1:

- The variable LastBvIntra[ 0 ][ compIdx ] and LastBvIntra[ 1 ][ compIdx ]specifies the last two block vector predictor. If this process is invoked for the first time for the current coding tree unit, LastBvIntra[ compIdx ] is derived as follows:

LastBvIntra[ 0 ][ 0 ] = −2* nCbS; LastBvIntra[ 0 ][ 1 ] = 0

LastBvIntra[ 1 ][ 0 ] = −nCbS; LastBvIntra[ 1 ][ 1 ] = 0

Depending upon the number of times this process has been invoked for the current coding tree unit, subclause Error: Reference source not found is invoked with the luma coding block location ( xCb, yCb ), the coding block size nCbS, the luma prediction block location ( xPb, yPb ), the luma prediction block width nPbSw, the luma prediction block height nPbSh, the last block vectors LastBvIntra, and the partition index blkIdx as inputs, and the block vector predictor BvpIntra[ xPb ][ yPb ] as the output, and bvIntra[ xPb ][ yPb ][ compIdx ] is set equal to BvdIntra[ xPb ][ yPb ][ compIdx ] + BvpIntra[ xPb ][ yPb ][ compIdx ] [Ed. (GJS): Needs further formatting cleanup.]

When bvIntra[ xPb ][ yPb ][ 0 ] is not equal to LastBvIntra[ 0 ][ 0 ] or bvIntra[ xPb ][ yPb ][ 1 ] is not equal to LastBvIntra[ 0 ][ 1 ], the value of LastBvIntra[ 1 ][ compIdx ] is updated to be LastBvIntra[ 0 ][ compIdx ], and the value of LastBvIntra[ 0 ][ compIdx ] is updated to be bvIntra[ xPb ][ yPb ][ compIdx ].

…

#### 8.4.4.1	Derivation process for intra block copy block vector prediction

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable nCbS specifying the size of the current luma coding block,

- a luma location ( xPb, yPb ) specifying the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- two variables nPbW and nPbH specifying the width and the height of the luma prediction block,

- the reference index of the current prediction unit partition refIdxL2

- the last block vector LastBvIntra

- a variable partIdx specifying the index of the current prediction unit within the current coding unit.

Output of this process is the block vector prediction BvpIntra[ xPb ][ yPb ].

The block vector predictor BvpIntra is derived in the following ordered steps:

numBVP is set to 0.

- The derivation process for block vector predictor candidates from neighbouring prediction unit partitions in subclause Error: Reference source not found is invoked with the luma coding block location ( xCb, yCb ), the coding block size nCbS, the luma prediction block location ( xPb, yPb ), the luma prediction block width nPbW, the luma prediction block height nPbH, the neighbouring block location ( xPb − 1 , yPb + nPbH − 1 ) and the partition index partIdx as inputs, and the availability flags availableFlagN and the block vectors bvIntraN as output. If availableFlagN is equal to TRUE, the following steps apply:

  - If numBVP is equal to 0, bvpIntraList[0] is set to bvIntraN and numBVP is set to 1.

  - Otherwise, if numBVP is equal to 1, if bvpIntraList[0] != bvIntraN, bvpIntraList[1] is set to bvIntraN and numBVP is set to 2.

- The derivation process for block vector predictor candidates from neighbouring prediction unit partitions in subclause Error: Reference source not found is invoked with the luma coding block location ( xCb, yCb ), the coding block size nCbS, the luma prediction block location ( xPb, yPb ), the luma prediction block width nPbW, the luma prediction block height nPbH, the neighbouring block location ( xPb + nPbW − 1,  yPb − 1 ) and the partition index partIdx as inputs, and the availability flags availableFlagN and the block vectors bvIntraN as output. If availableFlagN is equal to TRUE, the following steps apply:

  - If numBVP is equal to 0, bvpIntraList[0] is set to bvIntraN and numBVP is set to 1.

  - Otherwise, if numBVP is equal to 1, if bvpIntraList[0] != bvIntraN, bvpIntraList[1] is set to bvIntraN and numBVP is set to 2.

- The derivation process for block vector predictor candidates from neighbouring prediction unit partitions in subclause Error: Reference source not found is invoked with the luma coding block location ( xCb, yCb ), the coding block size nCbS, the luma prediction block location ( xPb, yPb ), the luma prediction block width nPbW, the luma prediction block height nPbH, the neighbouring block location ( xPb + nPbW,  yPb − 1 ) and the partition index partIdx as inputs, and the availability flags availableFlagN and the block vectors bvIntraN as output. If availableFlagN is equal to TRUE, the following steps apply:

  - If numBVP is equal to 0, bvpIntraList[0] is set to bvIntraN and numBVP is set to 1.

  - Otherwise, if numBVP is equal to 1, if bvpIntraList[0] != bvIntraN, bvpIntraList[1] is set to bvIntraN and numBVP is set to 2.

- The derivation process for block vector predictor candidates from neighbouring prediction unit partitions in subclause Error: Reference source not found is invoked with the luma coding block location ( xCb, yCb ), the coding block size nCbS, the luma prediction block location ( xPb, yPb ), the luma prediction block width nPbW, the luma prediction block height nPbH, the neighbouring block location ( xPb − 1,  yPb + nPbH ) and the partition index partIdx as inputs, and the availability flags availableFlagN and the block vectors bvIntraN as output. If availableFlagN is equal to TRUE, the following steps apply:

  - If numBVP is equal to 0, bvpIntraList[0] is set to bvIntraN and numBVP is set to 1.

  - Otherwise, if numBVP is equal to 1, if bvpIntraList[0] != bvIntraN, bvpIntraList[1] is set to bvIntraN and numBVP is set to 2.

- The derivation process for block vector predictor candidates from neighbouring prediction unit partitions in subclause Error: Reference source not found is invoked with the luma coding block location ( xCb, yCb ), the coding block size nCbS, the luma prediction block location ( xPb, yPb ), the luma prediction block width nPbW, the luma prediction block height nPbH, the neighbouring block location ( xPb − 1, yPb − 1 ) and the partition index partIdx as inputs, and the availability flags availableFlagN and the block vectors bvIntraN as output. If availableFlagN is equal to TRUE, the following steps apply:

  - If numBVP is equal to 0, bvpIntraList[0] is set to bvIntraN and numBVP is set to 1.

  - Otherwise, if numBVP is equal to 1, if bvpIntraList[0] != bvIntraN, bvpIntraList[1] is set to bvIntraN and numBVP is set to 2.

- The availability derivation process for a prediction block as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ), the current luma coding block size nCbS, the luma prediction block location ( xPb, yPb ), the luma prediction block width nPbW, the luma prediction block height nPbH, the luma location ( xPb+ LastBvIntra[0][0]+nPbW-1 , yPb+ LastBvIntra[0][1]+nPbH-1 ), and the partition index partIdx as inputs, if the output is equal to TRUE, the following steps apply:

  - If numBVP is equal to 0, bvpIntraList[0] is set to LastBvIntra[0] and numBVP is set to 1.

  - Otherwise, if numBVP is equal to 1, if bvpIntraList[0] != LastBvIntra[0], bvpIntraList[1] is set to LastBvIntra[0] and numBVP is set to 2.

- The availability derivation process for a prediction block as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ), the current luma coding block size nCbS, the luma prediction block location ( xPb, yPb ), the luma prediction block width nPbW, the luma prediction block height nPbH, the luma location ( xPb+ LastBvIntra[1][0]+nPbW-1 , yPb+ LastBvIntra[1][1]+nPbH-1 ), and the partition index partIdx as inputs, if the output is equal to TRUE, the following steps apply:

  - If numBVP is equal to 0, bvpIntraList[0] is set to LastBvIntra[1] and numBVP is set to 1.

  - Otherwise, if numBVP is equal to 1, if bvpIntraList[0] != LastBvIntra[1], bvpIntraList[1] is set to LastBvIntra[1] and numBVP is set to 2.

- If numBVP is less than 2, the following steps apply:

  - If numBVP is equal to 0, bvpIntraList[0] is set to (−2*nCbS, 0) and bvpIntraList[1] is set to (nCbS, 0),

  - Otherwise, if numBVP is equal to 1 and bvpIntraList[0] != (−2*nCbS, 0), bvpIntraList[1] is set to (−2*nCbS, 0),

  - Otherwise (numBVP is equal to 1 and bvpIntraList[0] == (−2*nCbS, 0)), bvpIntraList[1] is set to (−nCbS, 0).

- The derivation process for block vector predictor candidates from neighbouring prediction unit partitions in subclause Error: Reference source not found is invoked with the luma coding block location ( xCb, yCb ), the coding block size nCbS, the luma prediction block location ( xPb, yPb ), the luma prediction block width nPbW, the luma prediction block height nPbH, and the partition index partIdx as inputs, and the availability flags availableFlagN and the block vectors bvIntraN, with N being replaced by A or B, as output.

- The variables bvpIntraVirtual[ i ][ j ] (with i and j being equal to 0 or 1) specify two virtual block vector predictors, and are derived as follows:

bvpIntraVirtual[ 0 ][ 0 ] = LastBvIntra[ 0 ][ 0 ], bvpIntraVirtual[ 0 ][ 1 ] = LastBvIntra[ 0 ][ 1 ];

bvpIntraVirtual[ 1 ][ 0 ] = LastBvIntra[ 1 ][ 0 ], bvpIntraVirtual[ 1 ][ 1 ] = LastBvIntra[ 1 ][ 1 ];

[Ed. (GJS): Correct equation formatting (e.g., adding equation numbers).]

- The block vector predictor candidate list, bvpIntraList, is constructed as follows:

i = 0	 [Ed. (GJS): Correct equation formatting (e.g., adding equation numbers).]
if(availableFlagA )
		bvpIntraList[ i++ ] = bvIntraA
if( availableFlagB  &&  bvIntraA  !=  bvIntraB )
		bvpIntraList[ i++ ] = bvIntraB
for( j=0; j < 2  &&  i < 2; j++)
		if( ( j = =1 | | bvpIntraList[ 0 ]  !=  bvpIntraVirtual[ j ] )
			bvpIntraList[ i++ ] = bvpIntraVirtual[ j ]

[Ed. (GJS): Check/fix usage of bvpIntraVirtual with two indices versus with one index and the assumption of being a two-dimensional vector. What do we do elsewhere for similar uses?]

- The block vector bvIntra[ xPb ][ yPb ] is derived as follows.

for (i =0; i< 2; i++)
	BvpIntra[ xPb ][ yPb ][ i ]= bvpIntraList[ i ]

[Ed. (GJS): Correct equation formatting (e.g., adding equation numbers). Is this supposed to say bvIntra or BvpIntra? Check/fix substring problem.]

#### 8.4.4.2	Derivation process for intra block copy block vector prediction candidates

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable nCbS specifying the size of the current luma coding block,

- a luma location ( xPb, yPb ) specifying the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- two variables nPbW and nPbH specifying the width and the height of the luma prediction block,

- a luma location ( xN, yN ) specifying the neighbouring sample to obtain the block vector predictor relative to the top-left luma sample of the current picture,

- a variable partIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are (with N being replaced by A or B):

- the block vectors bvIntraN of the neighbouring prediction units.

- the availability flags availableFlagN of the neighbouring prediction units.

The variables bvIntraNA[ compIdx ] specify the left neighbouring block vector predictor with compIdx being 0 or 1. The horizontal block vector component is assigned compIdx = 0 and the vertical block vector component is assigned compIdx = 1. The variable availableFlagN specifies the availability flags of the left and above neighbouring blocks, with N being equal to A or B. bvIntraN[ compIdx ] and availableFlagN are derived as follows:

bvIntraN[ compIdx ] is set equal to 0 for compIdx being equal to 0 and 1 and N being equal to A and B;

availableFlagN is set equal to FALSE for N being equal to A and B.

The availability derivation process for a prediction block as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ), the current luma coding block size nCbS, the luma prediction block location ( xPb, yPb ), the luma prediction block width nPbW, the luma prediction block height nPbH, the luma location ( xPb − 1 , yPb + nPbH − 1 ) ( xN , yN ), and the partition index partIdx as inputs, if the output is equal to TRUE, the following step applies:

The availability derivation process for a prediction block as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ), the current luma coding block size nCbS, the luma prediction block location ( xPb, yPb ), the luma prediction block width nPbW, the luma prediction block height nPbH, the luma location ( xPb+ bvIntra[ xN ][ yN ][0]+nPbW-1 , yPb+ bvIntra[ xN ][ yN ][1]+nPbH-1 ), and the partition index partIdx as inputs, if the output is equal to TRUE, availableFlagNA is set equal to TRUE, and bvIntraNA is set equal to bvIntra[ xPb − 1 ][ yPb + nPbH − 1 ] bvIntra[ xN ][ yN ]

[Ed. (GJS): Check usage of TRUE vs. true (generally).]

[Ed. (GJS): Check "is set to" versus "is set equal to" (generall).]

The availability derivation process for a prediction block as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ), the current luma coding block size nCbS, the luma prediction block location ( xPb, yPb ), the luma prediction block width nPbW, the luma prediction block height nPbH, the luma location ( xPb − nPbW − 1 , yPb − 1 ), and the partition index partIdx as inputs. If the output is equal to TRUE, and (yPb/CtbSizeY) is equal to ( ( yPb − 1 ) / CtbSizeY ), availableFlagB is set equal to TRUE, and bvIntraB is set equal to bvIntra[ xPb − nPbW − 1 ][ yPb − 1 ]