| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0042 |
| --- | --- |

| Title: | Comparison of Compression Performance of HEVC Test Model 16.2 and HEVC Screen Content Coding Extensions Test Model 3 with AVC High 4:4:4 Predictive profile |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Bin Li Jizheng Xu Gary J. Sullivan | Email: | libin@microsoft.com jzxu@microsoft.com garysull@microsoft.com |
| Source: | Microsoft Corp. |  |  |

_____________________________

# Abstract

This contribution is a study of the relative objective (i.e. PSNR-based) compression performance of HEVC RExt Main 4:4:4 12 Test Model 8 (using 8, 10, and 12 bit configurations), HEVC SCC Test Model 1 (SCM 2) and AVC High 4:4:4 Predictive Profile. It builds upon the prior work reported in JCTVC-G399, JCTVC-H0360, JCTVC-I0409, JCTVC-J0236, JCTVC-K0279, JCTVC-L0322, JCTVC-M0329, JCTVC-O0184, JCTVC-P0213, JCTVC-R0101, and JCTVC-S0084 – updating the results by using the latest available reference software (JM-18.6, HM-16.2, HM-16.2+SCM-3.0), profile and test model designs, and the RExt and SCC test sequences. The overall results indicate that for RExt CTC sequences, RExt Main 4:4:4 12 Profile saves 25%, 33%, and 38% bit rate for AI-MT, RA-MT and LB-MT, respectively, relative to AVC High 4:4:4 Predictive Profile. The overall results also indicate that for screen content sequences, the HEVC SCC Test Model 3 also improves quite substantially over JM-18.6. For example, for RGB text and graphics with motion (TGM) 1080p&720 sequences, the HEVC SCC Test Model 3 saves 86%, 80%, and 77% bits for AI, RA and LB lossy coding over JM-18.6.

# Performance Comparison

We use HM-16.2, HM-16.2+SCM-3.0 and JM-18.6 in the tests. The JM software was configured with the same coding structure and referencing structure as HM using the “HM-like” configurations. High 4:4:4 Predictive profile is used in JM. Both the RExt CTC [12] and SCC CTC [13] test conditions (e.g. internal bit depth, intra period, QP setting, etc.) are followed in the comparison.

It should be noted that, for screen test sequences, SCM-3.0 improves quite a lot over JM-18.6. If we still use the same QP values to generate bitstreams and calculate BD-Rate, the PSNR overlapping range may be small (or even not overlapped). So we adjust the QP values to generate JM-18.6 bitstreams. For Desktop, Console, and Webbrowsing, we use QP 12, 17, 22, 27 to generate JM-18.6 bitstreams. For FlyingGraphics, Map, Programming, SlideShow, BasketballScreen, MissionControl2, and MissionControl3, we use QP 17, 22, 27, 32 to generate JM-18.6 bitstreams. For other sequences, we use the QP 22, 27, 32, 37 to generate JM-18.6 bitstreams.

# Experimental result

It should be noted that the encoding/decoding times, especially decoding times, are not so reliable – since the experiments were performed using a computing "cluster".

The results are shown in Tables 1–5 below. For the lossy coding results, the numbers are BD-Rate, and for lossless coding result, the numbers are bitrate change percentages. For both lossy and lossless codng, a negative number means the codec listed on the right is better.

Table 1 – JM18.6 VS. HM-16.2 for RExt CTC sequences

Table 2 – JM18.6 VS HM-16.2+SCM-3.0 for SCC CTC sequences, lossy coding

Table 3 – JM18.6 VS HM-16.2+SCM-3.0 for SCC CTC 4:2:0 sequences, lossy coding

Table 4 – JM18.6 VS HM-16.2+SCM-3.0 for SCC CTC 4:4:4 sequences, lossless coding

Table 5 – JM18.6 VS HM-16.2+SCM-3.0 for SCC CTC 4:2:0 sequences, lossless coding

We also did some experiments of disabling individual coding tools, including ACT, Intra BC and Palette in HM-16.2+SCM-3.0. The summarized experimental results are shown in Table 6–9 below.

Table 6 – HM-16.2+SCM-3.0 tool test for SCC CTC 4:4:4 sequences, lossy coding (Y or B BD-Rate)

|  | ACT disabled | Intra BC disabled | Palette disabled |
| --- | --- | --- | --- |
| All Intra |  |  |  |
| RGB TGM | 10.9% | 55.6% | 19.4% |
| RGB Mixed | 20.0% | 41.1% | 3.9% |
| RGB Animation | 32.4% | 1.1% | 0.0% |
| RGB Camera Captured | 32.8% | 0.1% | 0.0% |
| YUV TGM | 0.4% | 59.7% | 20.3% |
| YUV Mixed | −0.1% | 42.5% | 6.3% |
| YUV Animation | −0.1% | 1.6% | −0.1% |
| YUV Camera Captured | −0.1% | 0.2% | 0.0% |
| Encoding Time | 77% | 77% | 96% |
| Decoding Time | 101% | 110% | 107% |
| Random Access |  |  |  |
| RGB TGM | 13.6% | 28.4% | 12.4% |
| RGB Mixed | 30.3% | 23.9% | 2.6% |
| RGB Animation | 33.1% | 0.5% | 0.1% |
| RGB Camera Captured | 38.2% | 0.0% | 0.0% |
| YUV TGM | 0.7% | 29.2% | 13.0% |
| YUV Mixed | −0.4% | 25.0% | 4.6% |
| YUV Animation | 0.1% | 0.5% | −0.2% |
| YUV Camera Captured | −0.5% | 0.0% | −0.1% |
| Encoding Time | 94% | 97% | 101% |
| Decoding Time | 102% | 103% | 102% |
| Low Delay |  |  |  |
| RGB TGM | 13.1% | 13.0% | 7.5% |
| RGB Mixed | 31.6% | 9.9% | 1.5% |
| RGB Animation | 31.6% | 0.1% | 0.0% |
| RGB Camera Captured | 35.7% | 0.0% | 0.0% |
| YUV TGM | 1.1% | 11.7% | 7.5% |
| YUV Mixed | −0.4% | 10.0% | 2.8% |
| YUV Animation | 0.0% | −0.1% | −0.1% |
| YUV Camera Captured | −0.3% | −0.1% | −0.1% |
| Encoding Time | 98% | 97% | 100% |
| Decoding Time | 102% | 103% | 103% |

Table 7 – HM-16.2+SCM-3.0 Intra BC disabled for SCC CTC 4:2:0 sequences, lossy coding

Table 8 – HM-16.2+SCM-3.0 tool test for SCC CTC 4:4:4 sequences, lossless coding (Bitrate change Average)

|  | ACT disabled | Intra BC disabled | Palette disabled |
| --- | --- | --- | --- |
| All Intra |  |  |  |
| RGB TGM | 0.1% | 32.5% | 14.0% |
| RGB Mixed | 1.8% | 20.3% | 1.3% |
| RGB Animation | 4.4% | 0.1% | 0.0% |
| RGB Camera Captured | 0.2% | 0.0% | 0.0% |
| YUV TGM | 0.1% | 34.3% | 14.7% |
| YUV Mixed | 0.0% | 23.4% | 1.0% |
| YUV Animation | 1.6% | 0.1% | 0.0% |
| YUV Camera Captured | 0.0% | 0.0% | 0.0% |
| Encoding Time | 76% | 71% | 90% |
| Decoding Time | 103% | 107% | 106% |
| Random Access |  |  |  |
| RGB TGM | 0.1% | 20.2% | 6.8% |
| RGB Mixed | 0.5% | 3.9% | 0.2% |
| RGB Animation | 1.3% | 0.0% | 0.0% |
| RGB Camera Captured | −0.3% | 0.0% | 0.0% |
| YUV TGM | 0.1% | 20.8% | 7.6% |
| YUV Mixed | 0.0% | 4.6% | 0.1% |
| YUV Animation | 0.3% | 0.0% | 0.0% |
| YUV Camera Captured | 0.0% | 0.0% | 0.0% |
| Encoding Time | 89% | 93% | 98% |
| Decoding Time | 102% | 104% | 102% |
| Low Delay |  |  |  |
| RGB TGM | 0.1% | 15.3% | 4.6% |
| RGB Mixed | 0.5% | 2.3% | 0.0% |
| RGB Animation | 1.3% | 0.0% | 0.0% |
| RGB Camera Captured | −0.3% | 0.0% | 0.0% |
| YUV TGM | 0.1% | 15.3% | 5.0% |
| YUV Mixed | 0.0% | 2.6% | 0.0% |
| YUV Animation | 0.3% | 0.0% | 0.0% |
| YUV Camera Captured | 0.0% | 0.0% | 0.0% |
| Encoding Time | 91% | 96% | 99% |
| Decoding Time | 103% | 104% | 104% |

Table 9 – HM-16.2+SCM-3.0 Intra BC disabled for SCC CTC 4:2:0 sequences, lossless coding

# References

- JCTVC-G399, “Comparison of Compression Performance of HEVC Working Draft 4 with AVC High Profile,” Geneva, CH, Nov. 2011.

- JCTVC-H0360, “Comparison of Compression Performance of HEVC Working Draft 5 with AVC High Profile,” San José, CA, USA, Feb. 2012.

- JCTVC-I0409, “Comparison of Compression Performance of HEVC Draft 6 with AVC High Profile,” Geneva, CH, 27 April – 7 May 2012.

- JCTVC-J0236, “Comparison of Compression Performance of HEVC Draft 7 with AVC High Profile,” Stockholm, SE, 11–20 July 2012.

- JCTVC-K0279, “Comparison of Compression Performance of HEVC Draft 8 with AVC High Profile and Performance of HM8.0 with Different Delay Characteristics,” Shanghai, CN, 10–19 Oct. 2012.

- JCTVC-L0322, “Comparison of Compression Performance of HEVC Draft 9 with AVC High Profile and Performance of HM9.0 with Temporal Scalability Characteristics,” Geneva, CH, 14–23 Jan. 2013.

- JCTVC-N1005, “High Efficiency Video Coding (HEVC) Range Extensions text specification: Draft 4,” Vienna, Austria, 25 Jul. – 2 Aug. 2013.

- JCTVC-O0184, “Comparison of Compression Performance of HEVC Range Extensions Draft 4 with AVC High 4:4:4 Predictive profile,” Geneva, CH, 23 Oct. – 1 Nov. 2013.

- JCTVC-P0213, “Comparison of Compression Performance of HEVC Range Extensions Draft 5 with AVC High 4:4:4 Predictive profile,” Valencia, ES, 27 Mar. – 4 Apr. 2014.

- JCTVC-R0101, “Comparison of Compression Performance of HEVC 4:4:4 Range Extensions Test Model 7 and HEVC Screen Content Coding Extensions Test Model 1 with AVC High 4:4:4 Predictive profile,” Sapporo, JP, 30 June – 9 July 2014.

- JCTVC-S0084, “Comparison of Compression Performance of HEVC 4:4:4 Range Extensions Test Model 8 and HEVC Screen Content Coding Extensions Test Model 2 with AVC High 4:4:4 Predictive profile,” Strasbourg, FR, 17–24 Oct. 2014.

- JCTVC-P1006, “Common test conditions and software reference configurations for HEVC range extensions,” San José, US, 9–17 Jan. 2014.

- JCTVC-S1015, “Common conditions for screen content coding tests,” Strasbourg, FR, 17–24 Oct. 2014.