| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0043 |
| --- | --- |

| Title: | On the constraint for Intra BC mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bin Li Jizheng Xu | Tel: Email: | [phone contact number] [email address(es)] |
| Source: | Microsoft Corp. |  |  |

_____________________________

# Abstract

This document proposes to add additional constraint on block vectors of the Intra BC mode to avoid the invalid region to be used for Intra BC prediction for non-444 content.

# Problem statement and solution

There have already been many constraint on the block vectors to guarantee the invalid region is not used for Intra BC prediction. For non-444 content, when small CU (e.g., 8x8 CU) and non-2Nx2N PU is used, the chroma blocks may be merged to a larger block and be predicted together using a unified block vector. There are several problems in the current implementations.

## Block vector clipping

There is block vector clipping process in the text (JCTVC-S1005), but not implemented in the software.

–	If cIdx is not equal to 0, trafoDepth is equal to 0, and nTbS is equal to 4, the following applies:

–	If ChromaArrayTyp is equal to 1, bv is set equal to bvIntra[ xTbY + 4 ][ yTbY + 4 ].

–	Otherwise, if ChromaArrayType is equal to 2, bv is set equal to bvIntra[ xTbY + 4 ][ yTbY ].

–	bv[ 0 ] = Max( −( xTbY % CtbSizeY + CtbSizeY ), bv[ 0 ] ) >> ( SubWidthC − 1 )

–	bv[ 1 ] = Max( −( yTbY % CtbSizeY ), bv[ 1 ] ) >> ( SubHeightC − 1 )

The above equations clip the block vector to the current CTU and the left CTU. However, the whole picture could be used for intra BC prediction. Thus, the above clipping operations do not make sense. This document proposes to remove the above clipping operation.

## Valid regions for small chroma blocks

After merging, the block vector of one PU (bottom right most PU) will be used in the whole merged region. Thus, it is necessary to check the pixels used to predict the whole region are all available. This document proposes to add the following bitstream conformance requirement:

–	When the derivation process for z-scan order block availability as specified in subclause Error: Reference source not found is invoked with ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbY, yNbY ) set equal to ( xPb + bvIntra[ xPb ][ yPb ][ 0 ], yPb + bvIntra[ xPb ][ yPb ][ 1 ] ) as inputs, it is a requirement of bitstream conformance that the output is set shall be equal to TRUE.

–	When the derivation process for z-scan order block availability as specified in subclause Error: Reference source not found is invoked with ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbY, yNbY ) set equal to ( xPb + bvIntra[ xPb ][ yPb ][ 0 ] + nPbSw − 1, yPb + bvIntra[ xPb ][ yPb ][ 1 ] + nPbSh – 1 ) as inputs, it is a requirement of bitstream conformance that the output is set shall be equal to TRUE.

When the ChromaArrayType is equal to 1, log2CbSize is equal to 3, PartMode is not equal to PART_2Nx2N, and the blkIdx is equal to (numPartitions − 1),

–	When the derivation process for z-scan order block availability as specified in subclause Error: Reference source not found is invoked with ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbY, yNbY ) set equal to ( xCb + bvIntra[ xPb ][ yPb ][ 0 ], yCb + bvIntra[ xPb ][ yPb ][ 1 ] ) as inputs, it is a requirement of bitstream conformance that the output shall be equal to TRUE.

–	When the derivation process for z-scan order block availability as specified in subclause Error: Reference source not found is invoked with ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbY, yNbY ) set equal to ( xCb + bvIntra[ xPb ][ yPb ][ 0 ] + nCbS − 1, yCb + bvIntra[ xPb ][ yPb ][ 1 ] + nCbS – 1 ) as inputs, it is a requirement of bitstream conformance that the output shall be equal to TRUE.

When the ChromaArrayType is equal to 2, log2CbSize is equal to 3, PartMode is equal to PART_Nx2N, and the blkIdx is equal to 1,

–	When the derivation process for z-scan order block availability as specified in subclause Error: Reference source not found is invoked with ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbY, yNbY ) set equal to ( xCb + bvIntra[ xPb ][ yPb ][ 0 ], yCb + bvIntra[ xPb ][ yPb ][ 1 ] ) as inputs, it is a requirement of bitstream conformance that the output shall be equal to TRUE.

–	When the derivation process for z-scan order block availability as specified in subclause Error: Reference source not found is invoked with ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbY, yNbY ) set equal to ( xCb + bvIntra[ xPb ][ yPb ][ 0 ] + nCbS − 1, yCb + bvIntra[ xPb ][ yPb ][ 1 ] + nCbS – 1 ) as inputs, it is a requirement of bitstream conformance that the output shall be equal to TRUE.

When the ChromaArrayType is equal to 2, log2CbSize is equal to 3, PartMode is equal to PART_NxN, and the blkIdx is equal to 1 or 3,

–	When the derivation process for z-scan order block availability as specified in subclause Error: Reference source not found is invoked with ( xCurr, yCurr ) set equal to ( xCb, yCb) and the neighbouring luma location ( xNbY, yNbY ) set equal to ( xCb + bvIntra[ xPb ][ yPb ][ 0 ], yCb +(blkIdx==1?0:nCbS/2) + bvIntra[ xPb ][ yPb ][ 1 ] ) as inputs, it is a requirement of bitstream conformance that the output shall be equal to TRUE.

–	When the derivation process for z-scan order block availability as specified in subclause Error: Reference source not found is invoked with ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbY, yNbY ) set equal to ( xCb + bvIntra[ xPb ][ yPb ][ 0 ] + nCbS − 1, yCb +(blkIdx==1?0:nCbS/2) + bvIntra[ xPb ][ yPb ][ 1 ] + nCbS – 1 ) as inputs, it is a requirement of bitstream conformance that the output shall be equal to TRUE.

Another possible solution to avoid the use of unavailable pixels when performing Intra BC prediction is padding the unavailable pixels with the mid-gray values (e.g., 128 for 8-bit content).

# Experimental results

The current SCM-3.0 does apply such constraints at the encoder side. So, there is no change on the coding efficiency.

# Patent rights declaration(s)

Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).