| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0044 |
| --- | --- |

| Title: | CE1 Test D1: Encoder modification of palette mode for escape pixels |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | 1Yao-Jen Chang, Chao-Hsiung Hung, 2Chun-Lung Lin, 3Ching-Chieh Lin, Jih-Sheng Tu 195, Sec. 4, Chung Hsing Rd., Chutung, Hsinchu 31040, Taiwan | Tel: Email: | +886-3-591-8774 1britpablo@itri.org.tw 2Chunlung@itri.org.tw 3JackLin@itri.org.tw |
| Source: | ITRI International |  |  |

_____________________________

# Abstract

This proposal reports the CE1 Test D1 results based on SCM 3.0 reference software. The proposed encoder adds one or two additional palette modes whose palette sizes are smaller than that of the original palette mode. A fast algorithm is proposed to select the modes for RD checks by simply comparing the palette size of the current CU with that of the last coded palette CU. There are two versions for the experiments according to the number of the RD checks compared to SCM 3.0: the first one has no additional RD check, and the second one has one additional RD check. Compared with SCM 3.0 reference software under the conditions of AI-lossy coding, the first version and the second version can, respectively, achieve up to 0.1% and 0.3% BD-rate saving for the class “text & graphics with motion, 1080p & 720p”. Note that the two versions of the fast algorithm has no loss on encoding time.

# Introduction

In palette mode, it may be a waste of bits to keep the palettes which simply represent few pixels. Therefore, JCTVC-R0075 and JCTVC-S0048 proposed methods to remove some palettes from the palette table. In JCTVC-S0048, three additional palette modes with smaller palette size were proposed, and the encoder would accordingly check three additional RD runs to decide the palette size. To further study the technical parts and evaluate its performance on top of SCM-3.0 and JCTVC-S0048, a fast algorithm by simply comparing the palette size of the current coding unit (CU) with that of the last coded palette CU is proposed and tested in CE1 Test D.1 [3]. Two versions are implemented for the experiments, where the first one has no additional RD check, and the second one has one additional RD check.

# Proposed algorithm

Suppose the palette sizes for the current CU and the last coded palette CU are M and S, respectively. The current encoder of the palette mode includes two modes for RD checks: the original palette mode and the sharing mode. Apart from the original palette mode and the sharing mode, the proposed encoder further considers N additional palette modes which have fewer palettes than the original palette mode. In other words, there are N+2 modes:

Mode 1: the original palette mode with the palette size M,

Mode 2: the sharing mode with the palette size S,

Mode 3 ~ Mode N+2: the additional palette modes with the palette sizes M-1, …, M-N, respectively.

Note that the palettes representing fewer pixels have higher priority for removal. A fast algorithm is proposed to decide which modes are selected for RD checks. If S << M, it is impossible to use S palettes to represent the whole pixels of the CU which are actually constituted by M palettes. Therefore, it is also impossible to use the sharing mode when S << M. For example, if S = 2 and M = 16, it is impossible to use the sharing mode with only 2 palettes to predict the whole pixels of the CU which should have 16 palettes. According to the above concept, the fast algorithm is designed as follows:

If S + TH < M: The proposed encoder will select Mode 1 and Mode 3 ~ Mode N+2 for RD checks.

Else: The proposed encoder will select Mode 1, Mode 2, and Mode 3 ~ Mode N+1 for RD checks.

There are two versions of the proposed algorithm for the experiments:

Version 1 (N = 1 and TH = 2): There are three modes, i.e., the original palette mode with the palette size M, the sharing mode with the palette size S and the additional palette mode with the palette size M-1. Using the proposed fast algorithm, there is no additional RD check for this version.

Version 2 (N = 2 and TH = 2): There are four modes, i.e., the original palette mode with the palette size M, the sharing mode with the palette size S, the additional palette mode with the palette size M-1 and the additional palette mode with the palette size M-2. Using the proposed fast algorithm, there is only one additional RD check for this version.

To further accelerate the algorithm, an early termination method is implemented on the two versions. Because the basic concept of the proposed algorithm is to remove the palettes which simply represent few pixels in the CU, it should avoid removing the palettes which can represent a lot of pixels in the CU. The additional palette mode used in the two versions is therefore enabled only when the removed palette simply represents few pixels. More specifically, the additional palette mode is enabled when the number of pixels represented by the removed palette is smaller than or equal to 5.

# Simulation results

Two versions of the proposed algorithm are simulated based on SCM 3.0 reference software under the common test condition [3]. Table 1 and Table 2 show the lossy results for Version 1 and Version 2, where the anchor is the results of SCM 3.0. Version 1 and Version 2 can achieve up to 0.1% and 0.3% BD-rate saving for the class “text & graphics with motion, 1080p & 720p”. Note that there is no coding time loss for both versions.

Table 1. Lossy results for Version 1.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | -0.2% |
| YUV, mixed content, 1440p & 1080p | 0.0% | -0.1% | 0.0% |
| YUV, Animation, 720p | 0.0% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.1% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | -0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.1% | 0.0% | -0.1% |
| YUV, mixed content, 1440p & 1080p | 0.2% | 0.3% | 0.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.1% | 0.1% | -0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | -0.2% | -0.2% |
| YUV, mixed content, 1440p & 1080p | 0.2% | 0.1% | 0.8% |
| YUV, Animation, 720p | 0.1% | -0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |

Table 2. Lossy results for Version 2.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.3% | -0.2% | -0.2% |
| RGB, mixed content, 1440p & 1080p | -0.1% | -0.2% | -0.2% |
| RGB, Animation, 720p | 0.0% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.1% | -0.2% | -0.4% |
| YUV, mixed content, 1440p & 1080p | -0.1% | -0.2% | -0.2% |
| YUV, Animation, 720p | 0.0% | -0.2% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1440p & 1080p | 0.1% | 0.0% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.1% | -0.1% | -0.2% |
| YUV, mixed content, 1440p & 1080p | 0.2% | 0.3% | 0.3% |
| YUV, Animation, 720p | 0.0% | -0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.2% | -0.2% | -0.2% |
| RGB, mixed content, 1440p & 1080p | -0.1% | 0.1% | -0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | -0.2% | -0.4% |
| YUV, mixed content, 1440p & 1080p | 0.2% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.1% | 0.4% | 0.1% |
| YUV, camera captured, 1080p | -0.1% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |

# Conclusion

This proposal proposes to add palette modes whose palette sizes are smaller than that of the original palette mode. To reduce the complexity, a fast algorithm is further proposed to select the modes for RD checks by simply comparing the palette size of the current CU with that of the last coded palette CU. Compared with SCM 3.0 reference software under the conditions of AI-lossy coding, the proposed algorithm with no additional RD check can be up to 0.1% BD-rate saving for the class “text & graphics with motion, 1080p & 720p”, and the proposed algorithm with one additional RD check can be up to 0.3% BD-rate saving for the class “text & graphics with motion, 1080p & 720p”. Note that the algorithm proposed is a non-normative change of SCM. Because the two versions of the fast algorithm have gains with no loss on encoding time, it is suggested to adopt one of the versions into the next version of SCM.

# Reference

- Y. J. Chang, C. L. Lin, C. C. Lin, J. S. Tu, C. H. Hung, “Non-SCCE3: Encoder modification of palette mode for escape pixels,” 18th JCT-VC meeting, JCTVC-R0075, Sapporo, Japan, June 2014.

- Y. J. Chang, C. H. Hung, C. L. Lin, C. C. Lin, J. S. Tu, “CE6 Test B.2: Encoder modification of palette coding for escape pixels,” 19th JCT-VC meeting, JCTVC-S0048, Strasbourg, France, Oct. 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common Test Conditions for Screen Content Coding,” 19th JCT-VC meeting, JCTVC-S1015, Strasbourg, France, Oct. 2014

# Patent rights declaration(s)

ITRI international may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)