| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0045 |
| --- | --- |

| Title: | AHG10: Memory bandwidth reduction for intra block copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jani Lainema Miska M. Hannuksela | Tel: Email: | +358 7180 08000 firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution aims at reducing the memory bandwidth associated with intra block copy operations. A typical intra block copy implementation may need to maintain two frame buffers and write a copy of a completed CTU to both of these (one with filtered output samples, one with non-filtered samples to be used as a source for the later intra block copy operations). The proposed approach signals in the bitstream if a CTU is to be used as reference area for intra block copy and if it is, switches off the in-loop post-processing (deblocking and SAO) for that and thus allows storing a single version of the CTU. The reported luma BD-bitrate impact of the proposed approach is on average 1.3%, 1.2% and 1.5% for 444 AI, RA and LB configurations, respectively.

# Introduction

Typical single-pass HEVC encoders and decoders perform deblocking and SAO operations right after encoding and decoding of a CTU. This allows them to operate at minimal memory bandwidth as the frame needs to be scanned through only once. Introduction of the intra block copy mode however complicates the processing somewhat. In order to be able to do single-pass encoding or decoding one needs to maintain now two buffers: one for the filtered output picture and one that contains the sample values prior to deblocking and SAO (which is used as the reference for the intra block copy prediction). Thus, two copies of each CTU needs to be written to the memory effectively doubling the memory bandwidth for the write operations.

# Proposal

It is proposed to add a new CTU level syntax element filter_flag to the HEVC Screen Content Coding extension in order to control the in-loop filtering. The flag simply switches of the deblocking and SAO operations for the CTU. In addition, it is proposed to allow intra block copy to use only input samples residing fully within the area of CTUs for which the filter_flag is indicating that in-loop filtering has been disabled.

When operating under these constraints, it is enough to work with only one memory buffer and write only one version of the CTU to the memory as the fully processed output picture is now also serving as the reference picture for the consequent intra block copy operations.

# Encoder algorithm

In the proposed approach it is encoder's responsibility to decide which CTUs are filtered and which of the CTUs become non-filtered source CTUs for the intra block copy operations. The proposed encoder algorithm counts the number of samples within one CTU that have been coded in either intra block copy mode or in palette mode and compares that to a threshold. In the case of the provided software, the threshold has been set to 500 samples. Thus, if intra block copy and palette coded samples within a CTU exceeds that number, the CTU remains unfiltered and is used as source area for future block copy operations.

As a result, the source area of the intra block copy operations is reduced. This can help improving the encoder runtimes of different implementations. In the case of SCM the proposed implementation excludes blocks overlapping with filtered CTUs from the hashing process and also reduces the horizontal search area if there are filtered CTUs on the left border of the original search area.

# Objective coding efficiency

As the proposal limits the search area of the intra block copy prediction and also switches off post-processing for some of the picture area, there is some negative impact of the coding efficiency. However, due to the same reasons there are also evident improvements in both encoder and decoder runtimes.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 1.0% | -0.7% | -0.5% |
| RGB, mixed content, 1440p & 1080p | 3.5% | 2.7% | 2.6% |
| RGB, Animation, 720p | 0.5% | 0.8% | 0.6% |
| RGB, camera captured, 1080p | 0.1% | 0.1% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | 1.4% | 0.3% | 0.2% |
| YUV, mixed content, 1440p & 1080p | 2.8% | 2.4% | 2.5% |
| YUV, Animation, 720p | 1.0% | 1.7% | 1.0% |
| YUV, camera captured, 1080p | 0.2% | 0.3% | 0.3% |
| Enc Time[%] | 96 % |  |  |
| Dec Time[%] | 83 % |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 2.7% | 0.4% | 0.6% |
| RGB, mixed content, 1440p & 1080p | 1.7% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.1% | 0.3% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 2.9% | 0.6% | 0.4% |
| YUV, mixed content, 1440p & 1080p | 1.7% | -0.2% | -0.1% |
| YUV, Animation, 720p | 0.4% | 0.2% | 0.5% |
| YUV, camera captured, 1080p | 0.1% | 0.2% | 0.0% |
| Enc Time[%] | 98 % |  |  |
| Dec Time[%] | 98 % |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 3.8% | 2.3% | 2.4% |
| RGB, mixed content, 1440p & 1080p | 2.0% | 0.5% | 0.2% |
| RGB, Animation, 720p | 0.1% | 0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.1% | 0.1% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | 3.9% | 1.9% | 2.0% |
| YUV, mixed content, 1440p & 1080p | 2.2% | -0.5% | -0.3% |
| YUV, Animation, 720p | 0.1% | 0.4% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 98 % |  |  |
| Dec Time[%] | 99 % |  |  |

# Impact of the non-normative changes

The table below reports the objective coding results when the non-normative aspects of the proposal are activated in SCM-3.0 environment. In this case the intra block copy search is limited to use classified input areas (with palette and intra block copy areas exceeding 500 pixels in CTU), but the post-processing operations are following strictly SCM-3.0. Results seem to indicate one can achieve around 5% encoder speed-up for AI case with quite negligible effects on coding efficiency.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.3% | 0.3% | 0.4% |
| RGB, mixed content, 1440p & 1080p | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.2% | 0.2% | 0.2% |
| RGB, camera captured, 1080p | 0.1% | 0.1% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | 0.4% | 0.4% | 0.4% |
| YUV, mixed content, 1440p & 1080p | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.4% | 0.4% | 0.3% |
| YUV, camera captured, 1080p | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 95 % |  |  |
| Dec Time[%] | 100 % |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.7% | 0.6% | 0.6% |
| RGB, mixed content, 1440p & 1080p | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.1% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | 0.7% | 0.5% | 0.5% |
| YUV, mixed content, 1440p & 1080p | 0.1% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.1% | 0.2% | 0.4% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.1% |
| Enc Time[%] | 96 % |  |  |
| Dec Time[%] | 100 % |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.8% | 0.8% | 0.8% |
| RGB, mixed content, 1440p & 1080p | 0.2% | 0.4% | 0.1% |
| RGB, Animation, 720p | 0.1% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.8% | 0.5% | 0.6% |
| YUV, mixed content, 1440p & 1080p | 0.2% | 0.0% | 0.5% |
| YUV, Animation, 720p | -0.1% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | -0.1% | 0.1% | 0.1% |
| Enc Time[%] | 99 % |  |  |
| Dec Time[%] | 100 % |  |  |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).